3GPP TSG-RAN WG4 Meeting #117	R4-2521217

Dallas, Tx, USA, 17th – 21st November 2025

Source: 	Ericsson

Title:  	Discussion on RRM requirements for NTN for NR Phase 3

Agenda Item:	4.25.3

Document for:	Discussion

# Introduction

In the light of the WI NR Non-Terrestrial Networks (NTN) for NR Phase 3, the outcomes of the last meeting's debate, encompassing consensuses and unresolved issues, were documented in meeting minutes [1].

This contribution outlines our point of view on on the underlying impacts to RRM requirements mandated by the issues listed in the meeting minutes.

# Discussion

## Topic #1: SSB periodicity larger than 20 ms

Issue 2-1-1: Decide on the value of Kmulti_SMTC,i

- Proposal 1: Confirm Kmulti_SMTC,i = maxj(Kmulti_SMTC,i,j), where Kmulti_SMTC,i,j is calculated in the legacy way for the j-the occasion of SMTC#i - Proposal 2: The value of K_multi_SMTC is to be calculated only considering SMTCs with same periodicity. - For SMTCs with shorter periodicity that overlayp with SMTCs with larger periodicity the formula for the measurement requirements shall consider a factor L_multi_SMTC, which corresponds to the number of measurement opportunities lost due to collision with SMTCs with larger periodicity within 5 SMTC occasions.

Kmulti_SMTC,i = maxj(Kmulti_SMTC,i,j) can be regarded as a simplification when handling the case of two SMTC periodicities and their potential collision between SMTC occasions. In our view, this approach may introduce some additional delay; however, the delay is expected. And the worst case happens when one SMTC periodicity is 20ms and the other one is 160ms. In this case, measurement delay for 160ms SMTC periodicity is extended significantly.

Proposal 2 can mitigate the degradation of long SMTC periodicity.

We understand that the core principle comprises:

- Only the collision of SMTCs with the same periodicity is counted in Kmulti_SMTC;

- When collisions between SMTCs of different periodicities occur on an occasion,

  - The SMTC with the longer periodicity occupies the occasion;

  - The SMTC with the shorter periodicity skips the occasion, with a scaling factor Koccupy,i

Wherein,

Koccupy,i = 1-SMTCi / MAX(SMTC), if SMTCi  < MAX(SMTC)

Koccupy,i = 1, if SMTCi  = MAX(SMTC)

Kmulti_SMTC,i  only calculates the collision between SMTCs with same periodicity

Table 9.2C.5.1-1: Time period for PSS/SSS detection, (Frequency range FR1-NTN)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max( 600 ms, ceil( 5 x Kp x Klayer1_measurement x Koccupy_i) x Kmulti_SMTC,i x SMTC period )Note 1 x CSSFintra |
| DRX cycle≤ 320 ms | max( 600 ms, ceil(1.5 x 5 x Kp x Klayer1_measurement x Koccupy_i) x Kmulti_SMTC,i x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | ceil(5 x Kp x Klayer1_measurement) x Kmulti_SMTC x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Proposal 1: We are open to the solutions of optimizing value of Kmulti_SMTC,i to mitigate the degradation of long SMTC periodicity.

Proposal 2: RAN4 to take below scaling factor into account of Kmulti_SMTC,i optimization.

- Koccupy,i :

= 1-SMTCi / MAX(SMTC), if SMTCi  < MAX(SMTC)

= 1, if SMTCi  = MAX(SMTC)

- Kmulti_SMTC,i  only calculates the collsion between SMTCs with same periodicity

Table 9.2C.5.1-1: Time period for PSS/SSS detection, (Frequency range FR1-NTN)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max( 600 ms, ceil( 5 x Kp x Klayer1_measurement x Koccupy_i) x Kmulti_SMTC,i x SMTC period )Note 1 x CSSFintra |
| DRX cycle≤ 320 ms | max( 600 ms, ceil(1.5 x 5 x Kp x Klayer1_measurement x Koccupy_i) x Kmulti_SMTC,i x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | ceil(5 x Kp x Klayer1_measurement) x Kmulti_SMTC x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Furthermore, K_satellite in inter-frequency measurement requirements functions same as Kmulti_SMTC, same approach shall be applied for it.

Proposal 3:  K_satellite in inter-frequency measurement requirements shall be updated in the same way as Kmulti_SMTC.

## Topic #2: Multiple SMTCs in idle mode

Issue 2-1-4: Clarify SMTC selection if more than 4 SMTCs are configured

- Proposal 1: For RRC Idle/Inactive mode RRM requirements, if the NW configures more than 4 SMTCs, the UE in idle/inactive selects up to 4 SMTCs (or less, depending on the UE capability) based on the closest reference locations for measurement. - Proposal 2: For supporting configuration of more than 4 SMTCs per frequency (i.e., 6), the SMTC selection is left to UE implementation and no update is needed for current RRC Idle/Inactive mode RRM requirements. - Proposal 3: Do not define larger M1/M2/M3/M4 values in cell reselection requirements.

We do not believe that having more than four SMTCs would lead to any change in the specification. Even if the network provides some location information, it does not obligate the UE to down-select which SMTCs to measure; this is entirely up to the UE implementation.

Another reason for not down-selecting SMTCs is that we do not require test cases such as “selects up to the supported SMTCs based on the closest reference locations for measurement.”

Proposal 4: For supporting configuration of more than 4 SMTCs per frequency (i.e., 6), the SMTC selection is left to UE implementation, and no update is needed for current RRC Idle/Inactive mode RRM requirements.

# Conclusion

Proposal 1: We are open to the solutions of optimizing value of Kmulti_SMTC,i to mitigate the degradation of long SMTC periodicity.

Proposal 2: RAN4 to take below scaling factor into account of Kmulti_SMTC,i optimization.

- Koccupy,i :

= 1-SMTCi / MAX(SMTC), if SMTCi  < MAX(SMTC)

= 1, if SMTCi  = MAX(SMTC)

- Kmulti_SMTC,i  only calculates the collsion between SMTCs with same periodicity

Table 9.2C.5.1-1: Time period for PSS/SSS detection, (Frequency range FR1-NTN)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max( 600 ms, ceil( 5 x Kp x Klayer1_measurement x Koccupy_i) x Kmulti_SMTC,i x SMTC period )Note 1 x CSSFintra |
| DRX cycle≤ 320 ms | max( 600 ms, ceil(1.5 x 5 x Kp x Klayer1_measurement x Koccupy_i) x Kmulti_SMTC,i x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | ceil(5 x Kp x Klayer1_measurement) x Kmulti_SMTC x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Proposal 3:  K_satellite in inter-frequency measurement requirements shall be updated in the same way as Kmulti_SMTC.

Proposal 4: For supporting configuration of more than 4 SMTCs per frequency (i.e., 6), the SMTC selection is left to UE implementation, and no update is needed for current RRC Idle/Inactive mode RRM requirements.

# References

- R4-2515066 Ad-hoc meeting minutes [116bis][325] NR_IoT_NTN_Maintenance_RRM