3GPP TSG-WG4 Meeting #117	R4-2521231

# Dallas, USA,  November 17, 2025 - November 21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101-4 | CR | draftCR | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR: Multiple PMI with 64TX Enhanced Type II Codebook TDD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_Ph5-Perf |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduce PMI reporting requirements with eTypeII-r19 with 64 CSI-RS ports in TDD (2Rx and 4Rx). |  |  |  |  |  |  |  |  |
| Summary of zchange: |  | Introduce PMI reporting requirements with eTypeII-r19 with 64 CSI-RS ports in TDD (2Rx and 4Rx). CSI-RS configuration: Scheduling: |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cannot verify the feature of eTypeII-r19 with 64 CSI-RS ports in TDD (2Rx and 4Rx). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.2.X2 (New), 6.3.3.2.Z2 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------- Beginning of Change 1 ------------------------------------------------------------

##### 6.3.2.2.X2	Multiple PMI with 64Tx Enhanced Type II Codebook

For the parameters specified in Table 6.3.2.2.X2-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.2.2.X2-2.

Table 6.3.2.2.X2-1: Test parameters (dual-layer)

| Parameter |  |  | Unit | Test 1 |
| --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 40 |
| Subcarrier spacing |  |  | kHz | 30 |
| Duplex Mode |  |  |  | TDD |
| TDD DL-UL configurations |  |  |  | FR1.30-1 as specified in Annex A |
| Propagation channel |  |  |  | TDLA30-5 |
| Antenna configuration |  |  |  | XP Medium 64 x 2 (N1,N2) = (8,4) |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 4 |
|  | CDM Type |  |  | FD-CDM2 |
|  | Density (ρ) |  |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,(5) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | Row 5,(7) |
|  | CSI-RS periodicity and offset |  | slot | 10/6 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource 1 | CSI-RS resource Type |  | Aperiodic |
|  |  | Number of CSI-RS ports (X) |  | 32 |
|  |  | CDM Type |  | CDM4 (FD2, TD2) |
|  |  | Density (ρ) |  | 1 |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 17, (2, 4, 6, 8) |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | Row 17, (2, 9) |
|  |  | CSI-RS interval and offset | slot | Not configured |
|  |  | aperiodicTriggeringOffset |  | 0 |
|  | CSI-RS resource 2 | CSI-RS resource Type |  | Aperiodic |
|  |  | Number of CSI-RS ports (X) |  | 32 |
|  |  | CDM Type |  | CDM4 (FD2, TD2) |
|  |  | Density (ρ) |  | 1 |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 17, (2, 4, 6, 8) |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | Row 17, (5, 12) |
|  |  | CSI-RS interval and offset | slot | Not configured |
|  |  | aperiodicTriggeringOffset |  | 0 |
| CSI-IM configuration | CSI-IM resource Type |  |  | Aperiodic |
|  | CSI-IM RE pattern |  |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (5, 7) |
|  | CSI-IM timeConfig interval and offset |  | slot | Not configured |
| ReportConfigType |  |  |  | Aperiodic |
| CQI-table |  |  |  | Table 1 |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |
| timeRestrictionForIChannelMeasurements |  |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |
| cqi-FormatIndicator |  |  |  | Wideband |
| pmi-FormatIndicator |  |  |  | Not configured |
| Sub-band Size |  |  | RB | 8 |
| csi-ReportingBand |  |  |  | 11111111111111 |
| CSI-Report interval and offset |  |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  |  | 12 |
| CSI request |  |  |  | 1 in slots i, where mod(i, 10) = 6, otherwise it is equal to 0 |
| reportTriggerSize |  |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  |  | etypeII-r19 |
|  | paramCombination-r19 |  |  | 6 (L =4, pν =1/2, β=1/2) |
|  | R(numberOfPMISubbandsPerCQISubband-r19) |  |  | 1 |
|  | (CodebookConfig-N1, CodebookConfig-N2) |  |  | (8,4) |
|  | (CodebookConfig-O1, CodebookConfig-O2) |  |  | (4,4) |
|  | (valueOfX1-typeII-CBSR-r19, valueOfX2-typeII-CBSR-r19) |  |  | [(1, 1)] |
|  | typeII-CBSR-r19 |  |  | 0x FFFF FFFF |
|  | RI Restriction (typeII-RI-Restriction-r19) |  |  | 0010 |
| Physical channel for CSI report |  |  |  | PUSCH |
| CQI/RI/PMI delay |  |  | ms | 9 |
| Maximum number of HARQ transmission |  |  |  | 4 |
| Measurement channel |  |  |  | TBD |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with i1 wideband granularity and i2 subband granularity |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (0.5 ms granularity) with equal probability of each applicable i1, i2 combination. The random precoder generation shall follow 'typeI-SinglePanel-r19' codebook configuration as specified in Table 6.3.2.2.X1-1. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-12), this reported PMI cannot be applied at the gNB downlink before slot#(n+6). Note 3: Randomization of the dual-cluster beam directions shall be used as specified in Annex B.2.3.2.3A. The value of relative power ratio (p) shall be fixed as 1 during the test. |  |  |  |  |

Table 6.3.2.2.X2-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
| TBD | TBD |

------------------------------------------------------------- End of change 1 ------------------------------------------------------------

----------------------------------------------------- Beginning of Change 2 ------------------------------------------------------------

##### 6.3.3.2.Z2	Multiple PMI with 64Tx Enhanced Type II Codebook

For the parameters specified in Table 6.3.3.2.Z2-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.3.2.Z2-2.

Table 6.3.3.2.Z2-1: Test parameters (dual-layer)

| Parameter |  |  | Unit | Test 1 |
| --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 40 |
| Subcarrier spacing |  |  | kHz | 30 |
| Duplex Mode |  |  |  | TDD |
| TDD DL-UL configurations |  |  |  | FR1.30-1 as specified in Annex A |
| Propagation channel |  |  |  | TDLA30-5 |
| Antenna configuration |  |  |  | XP Medium 64 x 4 (N1,N2) = (8,4) |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 4 |
|  | CDM Type |  |  | FD-CDM2 |
|  | Density (ρ) |  |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,(5) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | Row 5,(7) |
|  | CSI-RS periodicity and offset |  | slot | 10/6 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource 1 | CSI-RS resource Type |  | Aperiodic |
|  |  | Number of CSI-RS ports (X) |  | 32 |
|  |  | CDM Type |  | CDM4 (FD2, TD2) |
|  |  | Density (ρ) |  | 1 |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 17, (2, 4, 6, 8) |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | Row 17, (2, 9) |
|  |  | CSI-RS interval and offset | slot | Not configured |
|  |  | aperiodicTriggeringOffset |  | 0 |
|  | CSI-RS resource 2 | CSI-RS resource Type |  | Aperiodic |
|  |  | Number of CSI-RS ports (X) |  | 32 |
|  |  | CDM Type |  | CDM4 (FD2, TD2) |
|  |  | Density (ρ) |  | 1 |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 17, (2, 4, 6, 8) |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | Row 17, (5, 12) |
|  |  | CSI-RS interval and offset | slot | Not configured |
|  |  | aperiodicTriggeringOffset |  | 0 |
| CSI-IM configuration | CSI-IM resource Type |  |  | Aperiodic |
|  | CSI-IM RE pattern |  |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (5, 7) |
|  | CSI-IM timeConfig interval and offset |  | slot | Not configured |
| ReportConfigType |  |  |  | Aperiodic |
| CQI-table |  |  |  | Table 1 |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |
| timeRestrictionForIChannelMeasurements |  |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |
| cqi-FormatIndicator |  |  |  | Wideband |
| pmi-FormatIndicator |  |  |  | Not configured |
| Sub-band Size |  |  | RB | 8 |
| csi-ReportingBand |  |  |  | 11111111111111 |
| CSI-Report interval and offset |  |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  |  | 12 |
| CSI request |  |  |  | 1 in slots i, where mod(i, 10) = 6, otherwise it is equal to 0 |
| reportTriggerSize |  |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  |  | etypeII-r19 |
|  | paramCombination-r19 |  |  | 6 (L =4, pν =1/2, β=1/2 ) |
|  | R(numberOfPMISubbandsPerCQISubband-r19) |  |  | 1 |
|  | (CodebookConfig-N1, CodebookConfig-N2) |  |  | (8,4) |
|  | (CodebookConfig-O1, CodebookConfig-O2) |  |  | (4,4) |
|  | (valueOfX1-typeII-CBSR-r19, valueOfX2-typeII-CBSR-r19) |  |  | [(1, 1)] |
|  | typeII-CBSR-r19 |  |  | 0x FFFF FFFF |
|  | RI Restriction (typeII-RI-Restriction-r19) |  |  | 0010 |
| Physical channel for CSI report |  |  |  | PUSCH |
| CQI/RI/PMI delay |  |  | ms | 9 |
| Maximum number of HARQ transmission |  |  |  | 4 |
| Measurement channel |  |  |  | TBD |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with i1 wideband granularity and i2 subband granularity |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (0.5 ms granularity) with equal probability of each applicable i1, i2 combination. The random precoder generation shall follow 'typeI-SinglePanel-r19' codebook configuration as specified in Table 6.3.3.2.X1-1. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-12), this reported PMI cannot be applied at the gNB downlink before slot#(n+6). Note 3: Randomization of the dual-cluster beam directions shall be used as specified in Annex B.2.3.2.3A. The value of relative power ratio (p) shall be fixed as 1 during the test. |  |  |  |  |

Table 6.3.3.2.Z2-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
| TBD | TBD |

------------------------------------------------------------- End of change 2 ------------------------------------------------------------