3GPP TSG-RAN Working Group 4 Meeting #117	 R4-2521236

Dallas, USA, 17th – 21st November 2025

Source:	Ericsson

Title:	Discussion on requirements for ModifiedMPR-Behaviour [Modified_MPR]

Agenda item:	10.2.1

Document for:	Approval

# Introduction

In the August meeting, a WF [1] on modified MPR behaviour was approved to further discuss the following issues.

The followings are agreed to guide the handling on the modifiedMPR-Behaviour bits in TS 38.101-1 from release 20. - The following wording will be used in TS 38.101-1 and 38.101-2 spec to indicate whether the bit is set or not. - “The bit is set to 1” and “The bit is set to 0” - The following are clarifications for better understanding, but they will not be included in the spec. - If at least one modifiedMPR bit in the bit string for the band is set to 1 by UE, UE shall send the entire bit string including the bits with value 0. - If all bits are set to 0, UE may omit sending the bit string and then network interprets all the bits in the bit string are set to 0. - For the open release in which the MPR/AMPR requirements are modified, and also for the later releases, the new MPR/AMPR requirements are applied. For releases earlier than this modification, the new MPR/AMPR requirements may be applied depending on UE indication. - For example, if MPR requirement is changed/updated for R19 specification before Rel-19 ASN.1 freeze, the modifiedMPR-Behaviour bit “shall” be set to 1 for R19 UE. For early releases, the modifiedMPR-Behaviour bits “may” be set to 1. - Some exceptions can be discussed case by case and no impact to on-going product development is expected. - If different UE types have different requirements, separate ModifiedMPR-Behaviour bit is used. - For spec version number, the followings are some candidates for further discussion in next meeting. - Keep the version number and add the following note outside the table. - NOTE 2: Specification version in the “Definition” column in Table L.1-1 refer to the version N in which the requirements were modified. If UE supports release P which is later than release N then UE supports requirements defined in that release P. - Remove the version number following the approach similar with 38.101-2. - Remove the version number and using wording like “MPR/AMPR defined in clause xxx of present specification”. - Other solutions are not excluded. - If the requirements are changed (tighten or relaxation) more than once, further discuss the following Options: - Option 1: Use separate ModifiedMPR-Behaviour bits for the second changed MPR/AMPR - Note: Error correction can be made but the modifiedMPR-Behaviour bit is kept unchanged - Other options are not excluded

# Discussion

For spec version number, the followings are some candidates for further discussion in next meeting. - Keep the version number and add the following note outside the table. - NOTE 2: Specification version in the “Definition” column in Table L.1-1 refer to the version N in which the requirements were modified. If UE supports release P which is later than release N then UE supports requirements defined in that release P. - Remove the version number following the approach similar with 38.101-2. - Remove the version number and using wording like “MPR/AMPR defined in clause xxx of present specification”. - Other solutions are not excluded.

The version number is in the definition to indicate when the modification first occurred. This is especially important for UE from earlier releases to know when the modification occurred, so when UE is software upgraded, it sees from which version and onwards when it will find the modified requirements. Hence, we support option 1 to keep the version number and the following note outside the table.

Proposal 1: Keep the version number and add the following note outside the table.

NOTE 2: Specification version in the “Definition” column in Table L.1-1 refer to the version N in which the requirements were modified. If UE supports release P which is later than release N then UE supports requirements defined in that release P.

We do not see any gain in creating a rule that for every new release should have clean-up session before specifying it. That will cause bad practices of maintaining the specifications correctly.

- If the requirements are changed (tighten or relaxation) more than once, further discuss the following Options: - Option 1: Use separate ModifiedMPR-Behaviour bits for the second changed MPR/AMPR - Note: Error correction can be made but the modifiedMPR-Behaviour bit is kept unchanged - Other options are not excluded

If we ever change the requirements multiple times, then we propose we should

Proposal 2: Use separate ModifiedMPR-Behaviour bits for the second changed MPR/AMPR.

# Conclusion

In this paper, Ericsson has made the following observations and proposals:

Proposal 1: Keep the version number and add the following note outside the table.

NOTE 2: Specification version in the “Definition” column in Table L.1-1 refer to the version N in which the requirements were modified. If UE supports release P which is later than release N then UE supports requirements defined in that release P.

Proposal 2: Use separate ModifiedMPR-Behaviour bits for the second changed MPR/AMPR.

# References

[1]	R4-2511776, “WF on Modified MPR Behavior”, Xiaomi, MediaTek Inc., OPPO, RAN4#116