# 3GPP TSG-RAN WG4 Meeting #117	R4-2521241

# Dallas, USA, Nov. 17-21, 2025

Source: 	OPPO

Agenda Item:	7.9.3

Title: 	Inter-vendor training collaboration and interoperability for two-sided AIML models

Document for:	Approval

Introduction

In R20 WID, following objectives are approved for the inter-vendor training collaboration for two-sided AI/ML models

Inter-vendor training collaboration for two-sided AI/ML models - Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback

In this contribution, for CSI compression, performance evaluations under different model approaches are conducted based on the backbone provided by RAN1 in R19.

Discussion

Model structure

In R19, a CNN-based decoder was proposed and agreed in RAN4 to verify whether model performance alignment could be achieved, as shown in Appendix 1.

Additionally, a CNN-based CSI encoder, along with simplified transformer-based, simplified CNN-based, and MLP-based encoders, were utilized in conjunction with the proposed CNN-based decoder, as shown in Appendix 2.

In RAN1, for studying the standardized model structure, a CNN-based model structure is proposed as one example structure for study purpose.

Figure 1  CSI encoder and CSI decoder that proposed in RAN1 R19

Regarding hyperparameters for the CSI encoder and decoder, the following need to be specified:

| Transformer blocks | Number of transformer blocks NTF Dimension of transformer block dmodel Number of self-attention heads Nhead Dimension of attention head dhead Dimension of latent space inside feedforward module dFF Activation function choice |
| --- | --- |
| The decoder has a mirroring design as the encoder |  |

Regarding the model hyperparameters, the following need to be specified:

    - Number of transformer blocks NTF

    - Dimension of transformer block dmodel

    - Number of self-attention heads Nhead

    - Dimension of attention head dhead

    - Dimension of latent space inside feedforward module dFF

    - Activation function choice

Based on the above description, we propose two approaches as outlined below:

- Case A: Utilizing the hyperparameters defined for the lower-complexity Encoder2 in RAN4 R19 (with consistent hyperparameters maintained between the encoder and decoder, shown below)

| Model type | Transformer |
| --- | --- |
| d_model | 64 |
| d_feedforward | 256 |
| N_head | 8 |
| d_head | 64 |
| N_TF | 4 |

For case A, the FLOPs, number of trainable parameters and model size are summarized as below

| CSI feedback bits | FLOPs |  | Num. of Trainable Parameters |  | Model Size |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Encoder | Decoder | Encoder |  | Encoder | Decoder |
| 64 bits | 3.6M | 3.7M | 0.16M | 64 bits | 3.6M | 3.7M |

- Case B: The model complexity has been increased to achieve better performance (with consistent hyperparameters maintained between the encoder and decoder, shown below).

| Model type | Transformer |
| --- | --- |
| d_model | 256 |
| d_feedforward | 1024 |
| N_head | 4 |
| d_head | 256 |
| N_TF | 6 |

For case B, the FLOPs, number of trainable parameters and model size are summarized as below

| CSI feedback bits | FLOPs |  | Num. of Trainable Parameters |  | Model Size |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Encoder | Decoder | Encoder |  | Encoder | Decoder |
| 64 bits | 248.1M | 248.4M | 9.7M | 64 bits | 248.1M | 248.4M |

Simulation results

For both Case A and Case B, we train/test the encoder and decoder using the RAN4 R19 mixed dataset. Simulations are shown below

| Model | SGCS |
| --- | --- |
| Case A | 0.697 |
| Case B | 0.744 |

It can be observed that the introduction of the more complex transformer model yields higher SGCS gain. However, it is also important to note that in Case B, the model complexity reaches 248 MFLOPs on the UE side, with a model size of 42MB, which poses significant challenges for UE deployment. In contrast, the model corresponding to Case A achieves lower SGCS performance but has a complexity of only 3.6 MFLOPs and a model size of 1.6MB.

Therefore, in Release 20, RAN4 needs to clarify: the expected performance, along with the acceptable model complexity, parameter number, and model size. RAN4 needs to establish a set of constraints, otherwise it will be difficult to conduct effective standardization discussions or perform meaningful model performance comparisons and selections.

In R20, RAN4 needs to clarify the expected performance, along with the acceptable model complexity, parameter number, and model size.

Conclusion

In this contribution, issues regarding the development of RAN4 standardized models are analysed and got the following proposals:

- Proposal 1: 		Regarding the model hyperparameters, the following need to be specified:

    - Number of transformer blocks NTF

    - Dimension of transformer block dmodel

    - Number of self-attention heads Nhead

    - Dimension of attention head dhead

    - Dimension of latent space inside feedforward module dFF

    - Activation function choice

- Proposal 2: 		In R20, RAN4 needs to clarify the expected performance, along with the acceptable model complexity, parameter number, and model size.

Reference

RP-250792, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

RP-250308, Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2

RP-251870, New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements

Appendix

Appendix 1 – Simulation results from RAN4 R19

CSI encoder and CSI decoder that proposed after RAN4 #111 email discussion are shown as below:

CNN based CSI encoder 								CNN based CSI decoder

Figure 2  CSI encoder and CSI decoder that proposed after RAN4 #111 email discussion

FLOPs, Number of parameters, model size for CNN based CSI encoder and CNN based CSI decoder are evaluated as below:

| CSI feedback bits | FLOPs |  | Num. of Trainable Parameters |  | Model Size |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Encoder | Decoder | Encoder | Decoder | Encoder | Decoder |
| 64 bits | 94.1M | 1481.6M | 0.13M | 1.80M | 638KB | 7.03MB |
| 112 bits | 94.2M | 1481.7M | 0.14M | 1.82M | 677KB | 7.11MB |

Based on the CNN based CSI encoder and CNN based CSI decoder proposed by RAN4#111, performance evaluation results are shown as below(train by UMa channel data, test by different channel model)

| CSI feedback bits | Training dataset | Test dataset (SGCS) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | TDL-A30 | CDL-A30 | CDL-C30 | UMa |
| 64 bits | UMa | 0.759 | 0.705 | 0.603 | 0.725 |
| 112 bits | UMa | 0.784 | 0.768 | 0.645 | 0.782 |

Note：Baseline etype2 SGCS is 0.71

However, after adopting the same model structure and the same simulation assumptions (based on RAN4 conclusion), different companies still obtained different simulation results(e.g., in SGCS), as summarized in RAN4#112 meeting.

In RAN4#112 meeting, companies agreed to perform a second round of alignment and refined more simulation parameters as captured in R4-2414447. Simulation results from our side are as follows:

| CSI feedback bits | Quantization of encoder output | SGCS (UMa channel) |
| --- | --- | --- |
| 64 bits | 2 bits quantization for CSI report | 0.717 |
| 32 floats | without quantization of the FC layer outputs for validation purposes only, i.e., float32 is sent in CSI report | 0.771 |

In addition to the agreed CNN-based CSI encoder, three different low-complexity encoders will be evaluated. Their corresponding FLOPs, model size, and backbone descriptions are summarized as below, with detailed descriptions provided in Appendix-2.

|  | FLOPs | Model size | Description |
| --- | --- | --- | --- |
| Encoder-1 | 94.1M | 0.13Mbyte | CNN-based encoder |
| Encoder-2 | 3.63M | 1.6Mbyte | Transformer-based low-complexity encoder |
| Encoder-3 | 4.14M | 92Kbyte | CNN-based low-complexity encoder |
| Encoder-4 | 0.22M | 0.11Mbyte | MLP- based low-complexity encoder |

As discussed in 3GPP RAN4 Meeting #115, the verification procedures for these low-complexity encoders are outlined as below

- SGCS-0: Based on the agreed CSI Encoder-1 structure and the agreed decoder structure, trained on the mixed dataset to obtain SGCS-0.

- SGCS-1: Compared to SGCS-0, the CSI decoder is frozen. Encoder 1 is trained on the mixed dataset to obtain SGCS-1.

- SGCS-2: Compared to SGCS-1, multiple encoders are evaluated, including Encoder-2 (Transformer-based), Encoder-3 (CNN-based), and Encoder-4 (MLP-based). The decoder remains the Samsung-provided frozen decoder.

- SGCS-3: Compared to SGCS-2, the decoder is no longer frozen but is constrained to the agreed CSI decoder structure and participates in training. A dedicated decoder is joint-trained with each encoder using the mixed dataset, to obtain corresponding SGCS-3 per encoder.

- SGCS-4: Compared to SGCS-3, one decoder is trained to match different encoders, instead of training a dedicated decoder per encoder.

The updated simulation results for low-complexity encoders with different backbones are shown as below

|  | （1-to-1） Joint training | New encoder training with frozen decoder | （N-to-1）Joint training SGCS-4 Step-1 | New encoder training with frozen decoder from Step-1 SGCS-4 Step-2 |
| --- | --- | --- | --- | --- |
| Encoder-1 | SGCS-0 0.7054 | SGCS-1 0.7040 | SGCS-4 (step-1) 0.6871 | SGCS-4 (step-2) 0.6935 |
| Encoder-2 | SGCS-3 0.7175 | SGCS-2 0.6993 | SGCS-4 (step-1) 0.6926 | SGCS-4 (step-2) 0.7095 |
| Encoder-3 | SGCS-3 0.6988 | SGCS-2 0.6991 | SGCS-4 (step-1) 0.6851 | SGCS-4 (step-2) 0.6899 |
| Encoder-4 | SGCS-3 0.6775 | SGCS-2 0.6381 | SGCS-4 (step-1) 0.6698 | SGCS-4 (step-2) 0.6709 |

Appendix 2 – Model structure assumptions for encoder with lower complexity

(a)

(b)                                                                                                        (c)

Figure 3  Model assumptions for encoder with lower complexity

(a) Transformer based lower-complexity encoder (Encoder 2)

(b) CNN based lower-complexity encoder (Encoder 3)

(c) MLP based lower-complexity encoder (Encoder 4)