3GPP TSG-RAN WG4 Meeting #117	R4-25121245

Dallas, USA, November 17th – 21st, 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM performance requirements for On-demand SIB1

Agenda item:	6.15.2.3

Document for:	Discussion

- 1. 	Introduction

Performance aspects for Rel-19 network energy saving were discussed in last meeting. In this contribution we continue discussing the test case for On-demand SIB1 based on latest agreements and recommended WF [1]. Further discussion details can be found in the topic summary [2].

- Discussion

The following issue was left for further study at RAN4 #116bis on performance requirements for On-demand SIB1 [1].

Issue 2-2: Detail test case design Way Forward - Option 1: RAN4 to define a new test case to verify the cell reselection behaviour together with longer TSI-NR in case of OD-SIB1. - UE transmits UL-WUS transmission - UE receives and decodes RAR - UE receives OD-SIB1 within OD-SIB1 window - Further discuss the exact value on the longer TSI-NR and the ending point - Option 2: - Target Cell (Cell2) is a NES Cell. - The ending point of the TC is UL WUS transmission. - UE shall be able to transmit UL WUS no later than: Tdetect, NR_Intra + TSI-NR + TUL-WUS, where TSI-NR is 1280ms as defined in existing test cases, and TUL-WUS is the time to the first UL WUS occasion after TSI-NR.

In our view, option 1 is preferred. The cell reselection delay test in case of an OD-SIB1 based cell should encompass time for UL-WUS transmission, reception and decoding of RAR, reception of OD-SIB1 within the OD-SIB1 window until the UE performs PRACH transmission to perform the Tracking Area Update.

This is aligned to current cell reselection test case to target cell in FR1 or FR2 with OD-SIB1.

These test cases are specified in TS 38.133 Annex A.6.1.1 for FR1 and Annex A.7.1.1 for FR2 to verify the UE’s cell reselection behaviour. In the test cases, the cell reselection delay is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2 and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a registration procedure for mobility and periodic registration update on Cell 2, both in case of unknown cell (with cell detection delay) and known cell (without cell detection delay), as shown below for FR2

TS 38.133: ##### A.7.1.1.1.3 Test Requirements The cell reselection delay to a newly detectable cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration updateon Cell 2. The cell re-selection delay to a newly detectable cell shall be less than 130 s for PC2/3/4 devices and less than 258 s for PC1 devices. The cell reselection delay to an already detected cell is defined as the time from the beginning of time period T3, to the moment when the UE camps on cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration updateon cell 1. The cell re-selection delay to an already detected cell shall be less than 27 s for PC2/3/4 devices and less than 53 s for PC1 devices. …

If UE is performing cell reselection to an OD-SIB1 based cell, the cell reselection delay must additionally consider the time for requesting and acquiring OD-SIB1, i.e. the 3 components depicted above. Hence the required time for cell reselection in test requirement needs to be extended. A modified test case is needed to verify the cell reselection behaviour to an OD-SIB1 based cell.

Observation 1: If UE is performing cell reselection to an OD-SIB1 based cell, the cell reselection delay must additionally consider the time for requesting and acquiring OD-SIB1.

Proposal 1: 	Specify the new test case with longer TSI-NR to verify the cell reselection behaviour to an OD-SIB1 supporting NES cell.

As cell detection delay is already tested for legacy cells, the cell reselection to NES cell can be tested for known NES cell. No time interval for cell detection is needed for this test, same as for other cell reselection tests when low mobility relaxed measurement criterion or not-at-cell edge relaxed measurement criterion, respectively, is fulfilled.

Proposal 2: 	Specify the new test case for known target cell only.

Thus, the test case only includes 2 time intervals T1 and T2. Before start of T1, the UE has all information available for sending UL-WUS, as soon as the SNR of cell 2 exceeds the cell reselection threshold. Thereafter in T2, the UE performs cell reselection back to the non-NES cell.

Proposal 3:  Specify the new test case to verify the cell reselection behaviour to an OD-SIB1 supporting NES cell and thereafter cell reselection back to non-NES cell with two time periods T1 and T2.

The additional cell reselection delay for the 3 components depicted above is named TOD-SIB1. It is proposed to set this to [5]s. Further study is needed to verify this figure depending e.g. on selected resource configurations for RACH transmissions, RAR configuration and on the OD-SIB1 repetition period.

Proposal 4: The additional cell reselection delay due to OD-SIB1 is named TOD-SIB1 and set to [5]s with further study being needed to verify this figure.

The accompanying CR for cell reselection TC for FR2 target cell is found in [3].

Conclusion

In this contribution, we discussed the RRM performance requirements for Rel-19 network energy saving for the feature On-demand SIB1.

The following observation and proposals are made.

Observation 1: If UE is performing cell reselection to an OD-SIB1 based cell, the cell reselection delay must additionally consider the time for requesting and acquiring OD-SIB1.

Proposal 1: 	Specify the new test case with longer TSI-NR to verify the cell reselection behaviour to an OD-SIB1 supporting NES cell.

Proposal 2: 	Specify the new test case for known target cell only.

Proposal 3:  Specify the new test case to verify the cell reselection behaviour to an OD-SIB1 supporting NES cell and thereafter cell reselection back to non-NES cell with two time periods T1 and T2.

Proposal 4: The additional cell reselection delay due to OD-SIB1 is named TOD-SIB1 and set to [5]s with further study being needed to verify this figure.

The accompanying CR for cell reselection TC for FR2 target cell is found in [3].

- References

[1]	R4-2514809, “WF on Netw_Energy_NR_enh_Part2”, Huawei, RAN4 #116bis

[2]	R4-2513515, “Topic summary for [116bis][209] Netw_Energy_NR_enh_RRM_Part2”, Moderator (Huawei), RAN4 #116bis

[3]	R4-2521246, “Draft CR 38.133 TC2: OD-SIB1 based Cell reselection FR2 to FR2 target Cell”, Nokia, Nokia Shanghai Bell