3GPP TSG-RAN WG4 Meeting #117	R4-2521252

Dallas, USA, November 17th – 21st, 2025

Source:	Nokia

Title:	Sensing in 6G networks

Agenda item:	8.10

Document for:	Discussion

Introduction

During RAN4 #116bis, the discussion on 6G sensing was started as the sensing objective is part of the 6G radio study item. The SID [1] specifies following objective.

- Sensing – Studies to be based on use cases and associated requirements, as defined in [TR38.914] - PHY functions and procedures for sensing technology (e.g., waveform. reference signals, measurement feedback, etc…) [RAN1, RAN4] - Evaluate sensing performance and if necessary, extend channel modelling, for the selected use cases [RAN1] - Aspects of integration with communication services [RAN1] - higher layer procedures and protocol aspects [RAN2] - RAN4 aspects of sensing including RF, coexistence, and testability in coordination with other WGs [RAN4] Note: RAN1 identify detailed requirements, if triggered by TSG RAN

At RAN4 #116bis, following agreement was captured in the WF [2]. # Agreement for RAN4#116bis Before April 2026, the agenda for sensing will be kept. RAN4 discussion will primarily focus on the following aspect - Sensing related regulatory status and requirement survey - Potential architecture consideration based on the RAN plenary use case study - views sharing on the potential RAN4 scope based on the identified use cases out of RAN plenary study - Identify less RAN1 design dependent RAN4 aspects if any.

This contribution investigates RAN4 aspects for sensing in 6G networks based on the above objective in the SID and the RAN4 #116bis agreement.

Discussion

In this section, we discuss RAN4 relevant aspects for sensing in 6G networks.

Use cases for 6G sensing

In RAN#109, the use cases listed below were captured in the moderator’s summary for RAN led 6G SI: Requirements of new and existing services [3]. The first proposal below considers the use cases of detection and/or tracking of passive objects, including UAVs, humans, vehicles, and AGVs. The other use cases listed in the second proposal below are from TR 22.837 [11] and TR 22.870 [12].

Proposal 2-1: The following 5G-A sensing use cases for detection and tracking shall be supported in 6G, including: - UAV - Human - Automotive - AGV Proposal 2-1: Further study the following use cases: - Moving target detection and tracking - Including road digitization, intrusion detection for highway/railway, ship detection etc. - Environment reconstruction - Including RAN digital twin, environment awareness to assist communication etc. - Human health and motion monitoring - Others Including privacy focused, public safety related cases etc.

The text below is quoted from the latest version of TR 38.914 [4], where the use cases of detection and/or tracking of passive objects, at least including UAVs, humans, vehicles and AGVs are highlighted.

5.4.4 Sensing Editor note: More sensing use cases can be included depending on further discussion. The 6GR and 6G RAN architecture shall at least support use cases of detection and/or tracking of passive objects, at least including UAVs, human, vehicles and AGVs.

We believe that the use cases of detection and/or tracking of passive objects can also be considered for the RAN4 study on 6G sensing. As the NR ISAC study focuses on UAVs as passive objects [5], RAN4 can adopt this use case to be able to leverage the NR ISAC study for the 6G sensing study.

RAN4 to start with the use cases of detection and/or tracking of passive objects, at least including UAVs, humans, vehicles and AGVs. UAVs as passive objects can be considered to leverage the NR ISAC study for the 6G sensing study.

Overview on sensing modes

In the Rel-19 study on channel modelling for Integrated Sensing And Communication (ISAC) for NR, six sensing modes were considered: TRP monostatic, UE monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-TRP bistatic, and UE-UE bistatic [6]. Bistatic modes include multi-static modes according to the RAN1 study.

As shown in Figure 1, in the TRP monostatic scenario, the same BS acts as transmitter of the sensing signals and as receiver of the signals reflected from the target. Similarly in the UE monostatic sensing mode, the same UE acts as transmitter of the sensing signals and as receiver of the reflected sensing signals. In the TRP-TRP bistatic or multi-static sensing modes, a BS acts as transmitter of the sensing signals, while one or more other BSs act as receiver(s) of the signals reflected from the target. The same latter principle is used in the UE-UE bistatic or multi-static sensing modes, where a UE acts as transmitter of the sensing signals, while one or more other UEs act as receiver(s) of the signals reflected from the target. The last two sensing modes involving both the UE and the BS are TRP-UE and UE-TRP bistatic scenarios. A BS acts as transmitter of the sensing signals and one or more UEs receive the reflected sensing signals in the TRP-UE bistatic or multi-static sensing modes. A UE acts as transmitter of the sensing signals and one or more BSs receive the sensing signals reflected from the target in the UE-TRP bistatic or multi-static sensing modes.

Figure 1: Overview of the six sensing modes.

These sensing modes were also discussed in RAN#109, where the proposals stated below were captured in the moderator’s summary for RAN led 6G SI: Requirements of new and existing services [3]. However, these proposals were not agreed due to the question of whether to discuss this in RAN plenary or RAN1.

Proposal 2-2-1: 6GR should study the following sensing modes: - TRP monostatic - TRP-TRP bistatic, - TRP-UE DL - UE-TRP UL - UE-UE bistatic - UE monostatic Proposal 2-2-2: 6GR should study cooperative sensing/multi-static sensing mechanism involving multiple UEs and base stations.

For RAN4 study on 6G sensing, these sensing modes can be considered when discussing measurements, requirements, and testability aspects. In case there will be updates from RAN1 during the study regarding the sensing modes (e.g., deprioritized scenarios and/or new modes introduced), RAN4 can take such updates into account.

RAN4 to consider the six sensing modes (i.e., TRP monostatic, UE monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-TRP bistatic, and UE-UE bistatic) at the start of the 6G study. Further updates on the considered sensing modes can be made based on input from RAN1.

Regulatory status and aspects

As agreed at RAN4#116bis, the sensing related regulatory status should be clarified. Below, we depict the regulatory status for the following 3 regions: US, Europe, and China.

US

In the US, the Federal Communications Commission (FCC) has established specific spectrum regulations that support radar-based technologies. According to FCC Radio Spectrum Allocation, radar systems operating in the 76 - 81 GHz band must be certified and are permitted for vehicular use or fixed/mobile applications within airport air operations areas, such as foreign object debris (FOD) detection.

Additionally, the FCC has enabled short-range radar operations in the 60 GHz band, particularly through its 2016 decision to allow limited mobile radar use for interactive motion sensing. This includes technologies like Google’s Soli sensor. Soli uses millimeter-wave radar to detect fine hand gestures at close range, enabling touchless control of devices. To support its deployment, the FCC’s Office of Engineering and Technology granted Google a waiver in 2018 allowing higher power levels than previously permitted: 10 dBm peak transmitter conducted output power, 13 dBm peak EIRP, and 13 dBm/MHz power spectral density, with a duty cycle capped at 10% within any 33 ms interval.

FCC considers key technical aspects such as frequency band allocation, power limits, spectral density, and duty cycle to ensure safe and effective operation of short-range radar systems.

Europe

In Europe, the Hexa-X project [7] ending in 2023 was dealing with localization and sensing and the Hexa-X-II project [8] ending mid 2025 has provided a view on 6G vision and on potential 6G system architecture. The latter project also considered regulatory aspects and liaised with BEREC (Body of European Regulators for Electronic Communication). It is unclear to which extent sensing aspects were part of this exchange and further study is needed.

ETSI has also established an Industry Specification Group (ISG) [9] which is doing pre-standard research on integrated sensing and communication technology. They investigated regulatory aspects for different use cases [10]. The below list of use cases is taken from this deliverable, published in March 2025.

Use case: Body proximity use case

For instance, for the use case on body proximity sensor, it is expected that the regulator sets guidelines to limit exposure to protect against known thermal effects from electromagnetic fields. The two main metrics quantify RF exposure: Specific Absorption Rate (SAR) and Power Density (PD). A potential requirement for the 6GS is formulated as follows: The 6GS should be able to adjust the transmission parameter (e.g. power) of the 6G wireless

transceiver, based on the sensing results (i.e. the distance and the direction), within the regulatory limits.

User case: Personal and health-related sensing data

Regulatory compliance and data privacy issues are judged to have key importance. The 6G system operating within stringent data protection frameworks has to ensure that all personal and health-related sensing data captured through 6G sensing signals are processed and stored in compliance with global privacy standards.

Use case: Sensing results used for traffic management

Sensing results have been exposed to a traffic management system, which optimizes road traffic for all traffic participants according to the regulatory policy.

Use case: Safe & economic UAV transport

When Unmanned Aerial Vehicles (UAVs) are used in rural regions for transporting persons or goods, the

current regulatory framework presents a challenge, as obtaining operational permits is both time-consuming and

Expensive. While UAVs could potentially fly anywhere, widespread approval by the regulatory authority is unlikely in the near future. Hence, the envisioned UAV infrastructure for 2030 (as illustrated in Figure 14) includes parcel drop-off points/ vertiports and 100 m airstrips for fixed-wing UAVs.

Use case: Automated guided vehicles travelling in airports

Regulatory compliance and data privacy issues are judged to have key importance. Collected data through 5G-Advanced and 6G signals need to be processed, saved and stored in compliance with global privacy standards that do not violate privacy and security.

Regulatory aspects in Europe for the various envisaged use cases of integrated sensing and communication are being considered in EU funded projects such as Hexa-X-II and in pre-standard research such as ETSI ISG ISAC.

China

In 2025, China’s Ministry of Industry and Information Technology (MIIT) issued the Interim Provisions on Radar Radio Management, introducing a structured regulatory framework for radar spectrum usage. For instance, the provisions clearly designate frequency bands for specific land mobile radar applications, including 76 - 79 GHz for automotive radar and 24.05 - 24.25 GHz for short-range radar. These allocations are intended to reduce cross-system interference and improve spectrum efficiency. The regulation also outlines compliance requirements for the development, importation, and operation of radar equipment, reinforcing technical standards and approval protocols.

It is expected RF requirements will comply with regional spectrum regulations, including designated frequency usage and technical compliance.

Architecture considerations

UE RF impact

Based on the progress in RAN1, there are some concerns that may impact the RF specification in RAN4.

- The DL waveform, i.e., DFT-S-OFDM, which may be used by UE to transmit the sensing signals. Since only CP-ODFM signal is defined in the current specification for UL, then, if UE need to use DFT-S-OFDM for sensing, the ACLR, SEM, etc. RF specification needs to be re-visited.

- UE needs to do Tx and Rx at the same time for sensing since the reflected signals can come back to UE at any time depending on the distance of the objects from UE. Therefore, the Tx and Rx power separation in full-duplex mode needs to be analysed. RAN1 is expecting 65-80 dB separation/isolation and we need to verify if that is reasonable.

- RAN1 may propose to use higher than 31 dBm signals in the sensing case. Since the highest maximum power of UE types in RAN4 is PC1 at 31 dBm, therefore, higher than 31 dBm output power will need to be analysed in terms of co-existence, ACLR, SEM, etc.

The concerns from RAN1 related to UE using DL waveform, Tx and Rx separation for full-duplex mode, and higher than 31 dBm maximum output power will impact RF.

A challenge in detecting/localizing/tracking ground targets is the environment [RP-251989]. The line-of-sight link between base stations serving as transmitter and receiver and the target may often be blocked by objects in the environment, such as buildings. To address this challenge, we can utilize a radar technique, such as sync radars, to transmit sensing signals simultaneously. Also, to improve sensing accuracy, increase the accuracy of positioning estimate, and increase the accuracy of velocity estimate, we can consider modifying radar technology “chirping” in the frequency domain based on LTE and 5G technologies to be used in 3GPP.

Reusing radar technologies can help to solve the issues related to detecting/localizing/tracking ground targets, improve sensing accuracy, increase the accuracy of positioning estimate, and increase the accuracy of velocity estimate.

In general, we propose to keep using the 5MHz, 10MHz, 15MHz, or 20MHz bandwidth, which is compatible to current LTE and NR OFDM signals. When reusing radar technologies, we try to reuse the same technologies used in LTE and NR to minimize the impact to the waveform, ACLR, and other RF specifications.

When adopting the benefits of other technologies, such as radar technologies, into 3GPP domain to solve sensing issues mentioned in RAN and RAN1, consider adjusting them to be LTE/NR compatible to reduce the workload and reduce the impact to RF.

RAN4 to study the RAN4 impact on UE RF requirements once RAN1 makes sufficient agreements.

- To enable bi-static sensing, the synchronization between the UE and gNB must be achieved with much higher accuracy than current systems provide. For example, the delay of the scattered sensing waveform can be used to estimate the distance between objects and the receiver, and these delays are typically on the nanosecond scale. In contrast, current OFDM frames, subframes, and slots are measured at the millisecond scale, which is insufficient for accurate distance estimation. Therefore, a higher-precision synchronization mechanism is required to support bi-static sensing.

To enable bi-static sensing, much more accurate synchronization, on the order of nanoseconds, between the UE and gNB is required, since the delay (measured in nanoseconds) is used to estimate the distance of objects from the receiver.

Conduct further studies on mechanisms to improve synchronization accuracy between UEs and TRPs to enable bi-static sensing.

BS RF impact

The scope of the RAN4 work area for BS is similar as for UE. Relevant aspects are architecture aspects, such as inter-TRP synchronization for bi-static TRP sensing, BS type, output power and spectrum emission characteristics, which are all depending on wave form characteristics (radar signal, OFDM signal, etc.), Rx sensitivity degradation, suitable channel bandwidth and other PHY channel parameters as well as required RF separation for TRP mono-static sensing. We propose to study them in greater detail, once RAN1 has made fundamental decisions on the wave form and the PHY channel characteristics for the identified prioritized ISAC use cases.

RAN4 to study the RAN4 impact on BS RF requirements once RAN1 makes sufficient agreements.

Testability

The integration of sensing into 6G systems supports a variety of use cases including UAV detection, human respiration monitoring, AGV navigation, and smart transportation. These applications require high accuracy, low latency, and robust performance. To support these requirements, it is important for RAN4 to define a dedicated testing framework that can evaluate sensing capabilities across different scenarios. While RAN4 can initiate the study of testability aspects that are independent of RAN1, the overall testing environment and procedures will depend on signal structures, waveform definitions, and measurement types that must first be specified by RAN1.

A testing framework is essential for validating 6G sensing performance, and its development in RAN4 depends on foundational definitions from RAN1.

Sensing test metrics must be tailored to the specific sensing mode and scenario. The six representative use cases include TRP monostatic, UE monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-TRP bistatic, and UE-UE bistatic, and each case involves different measurement requirements. For example, TRP monostatic may mainly focus on sensing accuracy such as success/missed detection probability, while UE monostatic and bi-static modes may additionally require evaluation of RF synchronization, micro-Doppler accuracy, and velocity estimation. These differences necessitate scenario-specific metrics to ensure meaningful and consistent performance evaluation.

The sensing test metrics may vary depending on the considered use cases and sensing modes.

Each sensing scenario requires a distinct test framework configuration. Scenarios involving the UE, such as UE monostatic, UE-TRP bistatic, and TRP-UE bistatic, are more practical for testing because the UE can act as the DUT and report relevant measurements. In contrast, scenarios without UE involvement, such as TRP-only sensing, present challenges in defining test procedures and validating performance solely from the gNB side. Therefore, it is recommended to prioritize the development of test cases where the UE is actively involved in the sensing process.

RAN4 to prioritize test cases involving UE, as they provide a clearer basis for defining DUT behavior and evaluating sensing performance.

RAN4 to start the study by focusing on testability aspects with priority given to test cases that involve UE participation.

Conclusion

This contribution investigated RAN4 aspects for sensing in 6G networks based on the objective in the SID [1] and the agreement found in RAN4 #116bis [2].

The following Observations and Proposals were made:

- RAN4 to start with the use cases of detection and/or tracking of passive objects, at least including UAVs, humans, vehicles and AGVs. UAVs as passive objects can be considered to leverage the NR ISAC study for the 6G sensing study.

RAN4 to consider the six sensing modes (i.e., TRP monostatic, UE monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-TRP bistatic, and UE-UE bistatic) at the start of the 6G study. Further updates on the considered sensing modes can be made based on input from RAN1.

- FCC considers key technical aspects such as frequency band allocation, power limits, spectral density, and duty cycle to ensure safe and effective operation of short-range radar systems.

Regulatory aspects in Europe for the various envisaged use cases of integrated sensing and communication are being considered in EU funded projects such as Hexa-X-II and in pre-standard research such as ETSI ISG ISAC.

It is expected RF requirements will comply with regional spectrum regulations, including designated frequency usage and technical compliance.

The concerns from RAN1 related to UE using DL waveform, Tx and Rx separation for full-duplex mode, and higher than 31 dBm maximum output power will impact RF.

Reusing radar technologies can help to solve the issues related to detecting/localizing/tracking ground targets, improve sensing accuracy, increase the accuracy of positioning estimate, and increase the accuracy of velocity estimate.

When adopting the benefits of other technologies, such as radar technologies, into 3GPP domain to solve sensing issues mentioned in RAN and RAN1, consider adjusting them to be LTE/NR compatible to reduce the workload and reduce the impact to RF.

RAN4 to study the RAN4 impact on UE RF requirements once RAN1 makes sufficient agreements.

To enable bi-static sensing, a more accurate synchronization between UE and gNB, in nanoseconds, is needed since the delay (in nanoseconds) is used to estimate the distance of the objects from the receiver.

Further study the mechanisms to increase the synchronization accuracy level between UEs and TRPs to enable bi-static sensing.

RAN4 to study the RAN4 impact on BS RF requirements once RAN1 makes sufficient agreements.

A testing framework is essential for validating 6G sensing performance, and its development in RAN4 depends on foundational definitions from RAN1.

The sensing test metrics may vary depending on the considered use cases and sensing modes.

RAN4 to prioritize test cases involving UE, as they provide a clearer basis for defining DUT behavior and evaluating sensing performance.

RAN4 to start the study by focusing on testability aspects with priority given to test cases that involve UE participation.

- References

- RP-2501881, “New SID: Study on 6G Radio”, NTT DOCOMO (Moderator).

- R4-2514592, “WF on 6G sensing”, ZTE Corporation.

- RP-252943, “Moderator's summary for RAN led 6G SI: Requirements of new and existing services”, CMCC.

- 3GPP TR 38.914 v0.2.0 “Study on 6G Scenarios and Requirements (Release 20)”.

- RP-251861, “New SID: Study on Integrated Sensing And Communication (ISAC) for NR”, AT&T.

- RP-240799, “Revised SID: Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR”, Xiaomi, AT&T.

- Main - Hexa-X, https://hexa-x.eu/

- Hexa-X-II - European level 6G Flagship project, https://hexa-x-ii.eu/

- ETSI Industry Specification Group (ISG) Integrated Sensing And Communications (ISAC), https://www.etsi.org/committee/2295-isac

- ETSI GR ISC 001 V1.1.1, “Integrated Sensing And Communications (ISAC); Use Cases and Deployment Scenarios“.

- 3GPP TR 22.837 v19.4.0 “Study on Integrated Sensing and Communication”.

- 3GPP TR 22.870 v0.3.1 “Study on 6G Use Cases and Service Requirements”.