3GPP TSG RAN4 Meeting #117						    	         R4-2521274

Dallas, USA, Nov 17-21, 2025

Agenda item:	7.7.4

Source:	Huawei, HiSilicon

Title:	Discussion on UAV RF requirements

Document for:	Discussion

Background

The new WID for enhancement of NR RF and RRM requirements for uncrewed aerial vehicle (UAV) was approved at RAN#109 [1]. In RAN4#116bis, the discussion on co-existence evaluation was initiated, and the agreements were captured in [2]. This document provides our further thinking on the RF requirements.

Discussion

In RAN4#116bis, two options are listed for maximum output power: rated maximum output power (similar as ATG), and normal power class (new or existing). We prefer to use the rated maximum output power approach due to the limited TU for this WI. To specify a normal power class, companies needs to spend much effort on simulating and deciding a set of MPR/AMPR requirements for all the UAV UEs. It could be a challenge considering we only have 0.25 TU for RF session in each meeting. The ATG approach has been approved feasible in the past. We think it could be reused by UAV UEs.

In addition, knowing a fixed power class seems no apparent benefit for the network. The power class information might be used for power control by the network, but UE could report PHR and provide more accurate information according to UE’s working status. The power control itself might also be easier for UAV UEs since there is little fast fading using LOS channel.

Proposal 1: The maximum output power is specified as rated output power, i.e. up to 33dBm based on UE declaration.

For PC3 UAV UE, the additional spurious emission requirements are specified according to the ECC Decision (22)07 for CEPT countries. In Rel-18 discussion companies clarified the regulatory requirements apply to PC3 only. For the high power UAV UE, it’s not clear how to handle the regulatory aspect. In addition, the ECC Decision (22)07 applies to non-AAS aerial UE only. For high power UE equipped with antenna array, the ECC Decision doesn’t apply.

Proposal 2: RAN4 to study how to handle the regulatory requirements for high power UAV UE.

The FDD band n3 is considered as the example band for studying the high power UAV UE. There is potentially impact on the REFSENS requirements when the transmit power is increased. In the case of ATG UE, the REFSENS requirements refer to TN UE requirements although the transmit power could be up to 40dBm. RAN4 needs to decide whether the same principle could be used for high power UAV UEs.

Proposal 3: RAN4 to study the impact on REFSENS from increased transmit power.

Conclusion

This document provides our view on the UAV RF requirements. Following proposals and observations are for RAN4 discussion.

Proposal 1: The maximum output power is specified as rated output power, i.e. up to 33dBm based on UE declaration.

Proposal 2: RAN4 to study how to handle the regulatory requirements for high power UAV UE.

Proposal 3: RAN4 to study the impact on REFSENS from increased transmit power.

References

[1] RP-252954, New WID on enhancement of NR RF and RRM requirements for uncrewed aerial vehicle (UAV), CMCC

[2] R4-2515159, Way Forward for [116bis][319] NR_UAV, Qualcomm, CMCC