3GPP TSG-RAN WG4 Meeting #117	                                                                                   R4-2521348

Dallas, USA, November 17th – 21st, 2025

Agenda Item:	7.12.4

Source: 	OPPO

Title: 	Discussion on RAN4 requirements for measurement event prediction

Document for:	Approval

- Introduction

The WID for Rel-20 AI mobility was agreed in [1] and revised in [2]. The objectives of core part are as following:

The objectives of this work item focused on AI/ML-assisted mobility in RRC_CONNECTED mode, wherein RRM measurement prediction and measurement event prediction could be applied. Only functionality-based LCM procedures are supported. In general, the RRM measurement configuration and reporting frameworks in RRC layer are taken as baseline. The objectives are as the following: - Specify support for RRM measurement prediction and measurement event prediction for UE sided models [RAN2]: - Temporal domain prediction case A and case B, and Frequency domain prediction for co-located cases are supported. - L3 Cell level predication is supported Note 1: For UE sided model, spatial domain prediction is not supported. Note 2: For UE sided model, L3 beam level prediction is not supported. - For both RRM measurement prediction and measurement event prediction, specify signalling and protocol aspects to enable LCM functionality management for UE sided model including [RAN2]: - UE capability request and response procedure - Applicability report for both full and partial configuration approach - Inference configuration and inference report - Performance monitoring performed in UE or network side, based on which network can manage UE sided functionality - UE sided data collection - For measurement event prediction: - Measurement event A1~A6 are in the scope in this release Note3: Data transfer and model delivery for UE sided model are out of scope of this WI. And model related choices can be up to UE’s implementation. - For RRM measurement prediction, specify signalling and protocol aspects to enable LCM functionality management for network sided model including [RAN2]: - Inference input report for at least RRM sub-case 2 of intra-frequency temporal domain case A for L3 cell prediction - Inference input report for RRM sub-case 5 of all scenarios for L3 beam level prediction - Network sided data collection for all scenarios and all RRM sub-cases of both L3 cell level and L3 beam level prediction, at least - UE is configured to log RRM measurement results. - UE supports periodical logging and L3 event triggered logging. - UE can indicate logged data availability. Note 4: All scenarios in this objective refer to intra-frequency domain case A and case B, inter-frequency prediction and spatial domain prediction. Note5: The above objectives are to be based on the LCM framework for Rel-19 AI/ML for NR air interface WI as much as possible. - For both RRM measurement prediction and measurement event prediction for UE sided model [RAN4]: - Specify core requirements for the use cases and scenarios supported by the previous objectives for RRM measurement prediction and indirect measurement event prediction - Note 6: RAN4 is expected to investigate whether the core requirement for RRM measurement prediction can be reused for indirect measurement event prediction - For direct event prediction, study feasibility of core requirement in Q2 2026. - Specify LCM-related core requirements to support the LCM functionality management , if any Note 7: Can check in RAN#112 whether direct event prediction can be supported depending on RAN2 and RAN4 conclusions

According to the objectives, RAN4 need to define core requirements for the supported use cases and LCM related requirements for LCM functionality management for UE sided model. In last meeting, the requirements for AI mobility including measurement prediction and indirect event prediction were discussed. The agreements and open issues are captured in the approved WF [3]. we will further discuss the identified requirements and open issues for indirect measurement event prediction as captured in the topic summary [4] and give our proposals.

- Discussion

According to the objectives in WID, the indirect measurement event prediction will be supported with the same scenarios as measurement prediction while the feasibility of core requirements for direct event prediction needs to be discussed in Q2 2026. So in this section, the discussion will focus on indirect measurement event prediction.

Observation 1: The feasibility of direct measurement event prediction needs to be studied in Q2 2026.

For indirect measurement event prediction, the inference procedure is illustrated as below (Figure in TR 38.744):

Figure 1: Indirect measurement event prediction

Based on the studies in the SI, for indirect event prediction method, measurement result(s) is predicted by a RRM measurement prediction model for intra-frequency temporal domain case A, temporal domain case B, frequency domain or spatial domain at first. Afterwards, predicted and optionally actual historical measurement result(s) of the same cell(s) are used to derive whether a measurement event at one future time instance occurs, without further involvement of an AI/ML model.

With this definition, we understand the AI/ML model implementation to get predicted L3-RSRP for measurement prediction and indirect measurement event prediction are the same. The model related requirements including measurement delay, prediction delay and LCM related requirements for measurement prediction can be reused to indirect measurement event prediction. And since the occurrence of the measurement event is based on the calculation of measured and predicted results according to the legacy procedure, we think no additional delay is needed between the report of predicted L3-RSRP and report of event occurrence, i.e., the two results are reported together. So the reporting delay requirements can also be reused to indirect measurement event prediction.

Proposal 1: The core requirements defined for measurement prediction e.g., measurement/reporting delay and LCM related requirements can be reused to indirect measurement event prediction.

In study item, it was agreed that RAN4 will define requirement for the absolute and/or relative accuracy of predicted RSRP if the report includes the predicted RSRPs corresponding to the predicted event occurrence time. According to RAN2 discussion, the spec impact of measurement prediction can be applicable for measurement event prediction unless otherwise described explicitly. Based on this design, we think the absolute and/or relative accuracy requirements defined for measurement prediction can be also reused for the predicted RSRP in measurement event prediction.

Proposal 2: The absolute and/or relative accuracy requirements defined for measurement prediction can be also reused for the predicted L3-RSRP in indirect measurement event prediction.

The difference between measurement prediction and indirect measurement event prediction is the final output of indirect event prediction, i.e., the occurrence of measurement event. The main issue is the metrics/KPI for this final output which has been discussed in study item. In last meeting, there are many proposals on the potential metrics/KPI as captured in [4]. And they can be categorized as the following directions:

Issue 3-2-1: Performance metrics for indirect measurement event prediction Further discuss based on the open issues in [1] and the following way forward: - Define the accuracy requirements for predicted L3-RSRP for indirect measurement event prediction. - Discuss whether the following performance metrics need to be defined and align the understanding on the definitions: - Option 1: Required window length under certain precision/accuracy - Option 1-1: define ETD of event prediction as a requirement, under the conditions of certain fixed precision and recall or F1 score. - Option 1-2: define ETD of event prediction as a requirement, under the condition of certain event prediction accuracy (e.g., true event prediction or false event prediction). - Option 2: Required precision/accuracy under certain window length - Option 2-1: define precision and recall or F1 score of event prediction as a requirement, under the condition of certain fixed ETD. - Option 2-2: define event prediction accuracy (e.g., true event prediction or false event prediction) of event prediction as a requirement, under the condition of certain fixed ETD. - Option 2-3: Windowed precision ≥ P_min - Precision is the ratio of correctly predicted A3 events to all predicted events, counting only matches within the allowed lead-time window. - Option 3: Lead-time of event prediction - Option 3-1: Lead-time window [L_min, L_max] - Each accepted prediction must arrive at least L_min but no more than L_max before the actual A3 trigger. - Option 3-2: Event prediction latency (i.e., how far ahead of the actual ground truth event occurrence time the event is predicted): - It is defined as the time gap between the report time of the predicted event and the actual occurrence time of the ground truth event.

According to the above directions, we give the following illustration for the performance metrics of indirect event prediction:

Figure 2: Performance metrics for indirect measurement event prediction

In our understanding, option 1 and option 2 are similar which is based on the ETD as illustrated above. Assuming the prediction is correct when the timing difference between ground-truth event and predicted event is less than or equal to ETD, the correct prediction ratio can be counted by multiple times evaluation. Here, the maximum ETD reflects the accuracy of single event prediction while the correct prediction ratio reflects the system performance. Theoretically, they are negatively related to each other, e.g., if we want to get high system performance, i.e., high correct prediction ratio, the larger ETD will be required, but in this case, it means for each single event prediction, the ground truth can be far away from the predicted event which may be not tolerated. Conversely, if the maximum ETD is defined smaller, the accuracy for single prediction is reasonable, but total correct prediction ratio may be decreased. To define this requirement, we need to evaluate the performance for each combination of ETD and correct prediction ratio, and choose a reasonable requirement considering the balance between both sides.

For option 3 (Lead-time of event prediction) which is the time difference between the report of event prediction and ground truth event, we understand it reflects the UE capability on how far ahead of the time UE can give the event prediction which may be related to PW length configuration, but we don’t think it represents the performance of prediction. We think as long as the correct prediction ratio or maximum ETD in option 1/2 are fulfilled, how long ahead of time to give the prediction can be left to UE implementation. Even if we want to clarify this time in the specification, we think it should be the UE capability on the support of PW length which may be discussed in RAN2 rather than the performance metrics.

Based on the above discussion, we think we can start from option 1 or option 2 to define the performance metrics for indirect event prediction. But considering RAN2 has not started the normative work for indirect event prediction, we are also OK to wait for more progress in case of any impact on this definition from RAN2 conclusions.

Proposal 3: RAN4 to start from the following options to discuss the performance metrics for indirect measurement event prediction.

  - Option 1: Required window length tolerance under certain precision/accuracy.

  - Option 2: Required precision/accuracy under certain window length tolerance.

- Conclusions

In this contribution, we discussed the RAN4 requirements for RRM measurement event prediction and got the following observations and proposals:

Observation 1: The feasibility of direct measurement event prediction needs to be studied in Q2 2026.

Proposal 1: The core requirements defined for measurement prediction e.g., measurement/reporting delay and LCM related requirements can be reused to indirect measurement event prediction.

Proposal 2: The absolute and/or relative accuracy requirements defined for measurement prediction can be also reused for the predicted L3-RSRP in indirect measurement event prediction.

Proposal 3: RAN4 to start from the following options to discuss the performance metrics for indirect measurement event prediction.

  - Option 1: Required window length tolerance under certain precision/accuracy.

  - Option 2: Required precision/accuracy under certain window length tolerance.

- References

RP-251864 New WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR, RAN#108

RP-252899 Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR, RAN#109

R4-2514827 WF on NR_AIML_Mob_RRM, RAN4#116bis

R4-2513532 Topic summary for [116bis][226] NR_AIML_Mob_RRM, RAN4#116bis

3GPP TR 38.744: “Study on Artificial Intelligence (AI)/Machine Learning (ML) for Mobility in NR”