3GPP TSG-RAN WG4 Meeting #117	R4-2521356

# Dallas, USA, 17 - 21 Nov, 2025

Title: 	Discussion on NTN testing for NGSO demodulation performance requirements

Source: 	Huawei,HiSilicon

Agenda item:	6.6.3.3

Document for:	Discussion

Background

During RAN4#116bis meeting, Way Forward [1] for NTN_testing_NGSO_channel_model_and_demod was approved. In this contribution, we share our view on NTN testing for NGSO demodulation performance requirements.

Discussion

How to ensure the satellite elevation angles more than 30 degrees during the test

Background: RAN5 conformation test procedure specifies to run multiple subtest to measure the average throughput for a duration sufficient to achieve statistical significance (See TS 38.521-5 8.2.1.2.2.1.1_1.3.2).. - Interested companies are encouraged to study how TE ensures to apply the same Doppler shift and propagation delay model for each subtest. FFS how to capture in the specification. - Option 1: For each subtest, TE should start from the same initial ephemeris information (i.e., positionX, positionY, positionZ, velocityVX, velocityVY, velocityVZ) = (-2654249, 4386991, 1594205, 14581, -34487, 120182).

As per the test procedure defined in RAN5 test specification TS 38.521-5 section 8.2.1.2.2.1.1_1.3:

5. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for NR/5GC with Connected without Release On, Test Mode On according to TS 38.508-1 [12] clause 4.5. Message contents are defined in clause 8.2.1.2.2.1.1_1.3.3. 8.2.1.2.2.1.1_1.3.2 Test procedure 1. UE location according to TS 38.508-1 [12] clause 5.6.1 is provided to the UE through any preconfigured means. 2 Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 38.508[12] table 5.6.2.1-2 for NGSO (LEO-600) if UE supports only NGSO satellites or both GSO and NGSO satellites. Test system shall send same SIB19 information during the duration of the test as defined in clause 8.2.1.2.2.1.1_1.3.3. 3. Deactivate UE prediction of satellite trajectory by any preconfigured means. 4. SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Tables Table 8.2.1.2.2.1.1-3. The SS sends downlink MAC padding bits on the DL RMC. 5. Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR according to Table 8.2.1.2.2.1.1-2 and Table 8.2.1.2.2.1.1-3 as appropriate. 6. Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest and decide pass or fail according to [Annex G]. 4. Repeat steps from 1 to 3 for each subtest in Table 8.2.1.2.2.1.1-3 as appropriate.

The demodulation requirements are defined with the starting elevation angle 30°assumed to verify the case of the largest delay shift.

From the test step yellow highlighted, we can know that UE firstly needs to enter into the RRC_CONNECTED state no later than elevation angle 30°, and the test needs to be repeated for many times. If the timing varying channel model is applied, it means that UE needs to perform the initial access procedure and ensures to enter into the RRC_CONNECTED state no later than elevation angle 30°for each subtest, the satellite needs to disappear suddenly and appears again with the Doppler shift and delay shift jumping during the testing. To avoid such jumping, a feasible way is to use different cell ID for each sub-test so that UE identify the disappeared satellite is different from the new satellite. RRC Release after each sub-test and re-enter RRC Connect mode before each sub-test is needed. With such method, the UE will not suffer unexpected satellite teleportation and will spend only a few seconds more for each sub-test comparing to the legacy test procedure.

Use different cell ID for each sub-test so that UE identify the disappeared satellite is different from the new one. RRC is Release after each sub-test running, and UE re-enters RRC Connect mode for a different cell id before each sub-test. The proposed test procedure is shown as following:

8.2.1.2.2.1.1_1.3.2 Test procedure 1. UE location according to TS 38.508-1 [12] clause 5.6.1 is provided to the UE through any preconfigured means. 2 Test equipment shall emulate the signal with doppler and delay according to initial ephemeris defined in TS 38.508[12] table 5.6.2.1-x for NGSO (LEO-600) if UE supports only NGSO satellites or both GSO and NGSO satellites. Test system shall send SIB19 information during the duration of the test as defined in clause 8.2.1.2.2.1.1_1.3.3. 3. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for NR/5GC with Connected without Release On, Test Mode On according to TS 38.508-1 [12] clause 4.5. Message contents are defined in clause 8.2.1.2.2.1.1_1.3.3. 4. SS continue transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Tables Table 8.2.1.2.2.1.1-3. The SS sends downlink MAC padding bits on the DL RMC. 5. Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR according to Table 8.2.1.2.2.1.1-2 and Table 8.2.1.2.2.1.1-3 as appropriate. 6. Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest and decide pass or fail according to [Annex G]. 7. The SS shall send an RRCRelease message to ensure that the UE is in state RRC_IDLE on Cell 1 8. Set Cell 1 physical cell identity = ((current Cell 1 physical cell identity + 3) mod1008) for next iteration of the test procedure loop. 9. Repeat steps from 2 to 8 for each subtest in Table 8.2.1.2.2.1.1-3 as appropriate.

Applicability of Rel-19 tests with time-varying Doppler shift and propagation delay model

Background In Rel-17 RAN4 defined four PDSCH demodulation requirements for NR NTN (1-1, 1-2, 1-3, and 1-4). In this WI, RAN4 has agreed to add 2 new test cases (1-5 and 1-6) by applying the time-varying Doppler shift and propagation delay model for NGSO to tests 1-1 and 1-2, as follows. Test num.; Reference channel; Bandwidth (MHz) / Subcarrier spacing (kHz); Modulation format and code rate; Propagation condition; Correlation matrix and antenna configuration; Reference value. Fraction of maximum throughput (%); SNR (dB). 1-1; R.PDSCH.1-1.1 FDD; 10 / 15; QPSK, 0.30; NTN-TDLA100-200; 1x2, ULA Low; 70; 0.3. 1-2; R.PDSCH.1-2.1 FDD; 10 / 15; 16QAM, 0.48; NTN-TDLC5-200; 1x2, ULA Low; 70; 7.6. 1-3; R.PDSCH.1-1.1 FDD; 10 / 15; QPSK, 0.30; NTN-TDLC5-200; 1x2, ULA Low; 70; -0.4. 1-4; R.PDSCH.1-1.1 FDD(1); 10 / 15; QPSK, 0.30; NTN-TDLA100-200; 1x2, ULA Low; 70; 1.1. 1-5; R.PDSCH.1-1.1 FDD; 10 / 15; QPSK, 0.30; NTN-TDLA100-200; 1x2, ULA Low; 70; [0.3+0.5]. 1-6; R.PDSCH.1-2.1 FDD; 10 / 15; 16QAM, 0.48; NTN-TDLC5-200; 1x2, ULA Low; 70; [7.6+0.5]. Note 1: The Maximum throughput is based on the HARQ processes with HARQ feedback enabled. Note 2: For Tests 1-5 and 1-6, the time-varying Doppler shift and propagation delay model, specified in Annex E, is applied... For UE declaring Rel-19 UE, - Option 1 - Applicable tests are 1-5, 1-6, 1-3 and 1-4. - Option 2 - Applicable tests are 1-5, 1-6. For UE declaring Rel-17/18 UE, - Option 1a: - If UE declares to meet the requirements with time-varying Doppler shift and propagation delay, applicable tests are 1-5, 1-6, 1-3 and 1-4. - Otherwise, applicable tests are 1-1, 1-2, 1-3 and 1-4. - Option 1b: - If UE declares to meet the requirements with time-varying Doppler shift and propagation delay, applicable tests are 1-5, 1-6. - Otherwise, applicable tests are 1-1, 1-2, 1-3 and 1-4. - Option 2: - Applicable tests are 1-1, 1-2, 1-3 and 1-4. Note the same test applicability rule is used for IoT-NTN UE demodulation requirements.

From our understanding, the new NGSO requirements should applied only for Rel-19 UE. If UE can pass new NGSO requirements applying the new channel model, then it can skip the corresponding NGSO requirements instead of all NGSO requirements defined in Rel-17/Rel-18. Because only two test cases are selected from the NGSO requirements defined in Rel-17/Rel-18 for new NGSO requirements definition with timing varying channel model and additional 0.5dB margin added, the others are still applicable for testing with NTN-TDL channel model without additional 0.5dB margin added.

For Rel-19 UE

- The NGSO requirements with timing varying channel model should be applied

  - If UE can pass the NGSO requirements with the timing varying new channel model, then it can skip the corresponding NGSO requirements with NTN-TDL defined in Rel-17/Rel-18.

- All other test cases defined in Rel-17/18 that are only feasible for testing under NTN-TDL channel model are still applicable for Rel-19 UE.

- Applicable tests are 1-5, 1-6, 1-3 and 1-4

Some company proposed to test all NTN requirements including Rel-17/18/19 under timing varying channel model, but the Rel-17/18 NGSO requirements are defined under NTN-TDL channel modeling, it means that will requires RAN4 to re-simulate all Rel-17/18 test cases under timing varying channel model, this will be very heavy workload for RAN4 considering the parallel discussion for NR Rel-19/Rel-20 and 6GR study for demodulation requirements. It is due to the workload concerns and consideration the key motivation for this Rel-19 NGSO testing WI is to test that UE can properly pre-compensate the frequency offset shift and timing offset shift, the selected RF frequency error test, RRM UL timing test and limited number of demodulation test cases are enough to verify the UE’s pre-compensation on the frequency offset shift and timing offset shift under timing varying channel model.

RAN4 needs to re-simulate all Rel-17/18 demodulation requirements with the timing varying channel model that will be very heavy workload for RAN4, with also consideration of the parallel discussion for NR Rel-19/Rel-20 and 6GR study for demodulation requirements. i.e. Rel-17/18 test 1-3 and 1-4 are not applicable for testing under NTN timing varying channel model

The selected RF frequency error tests, RRM UL timing tests and limited number of demodulation test cases are enough to verify the UE’s pre-compensation on the frequency offset shift and timing offset shift under timing varying channel model as per the discussion on this WI.

Now it is in the late stage of Rel-19, some NTN UEs already finished some Rel-17/18 NTN testing, it means that both UE and TE vendors already support the NTN testing with NTN-TDL channel emulated, there are no technical feasibility issues to conduct the NGSO testing under NTN-TDL channel model.

Observation 3: There are commercial NTN UE finished Rel-17/18 NTN testing, it means that no technical feasibility issues to conduct the NGSO testing under NTN-TDL.

For Rel-17/18 UE

- The NGSO requirements with timing varying channel model is not applicable

- Test 1-3 and 1-4 are not applicable for testing under NTN timing varying channel model without re-simulation to decide the additional margin

- Applicable tests are 1-1, 1-2, 1-3 and 1-4.

Proposals

In this contribution, we share our view on NTN testing for NGSO for demodulation performance requirements. Our observations and proposals are:

- Use different cell ID for each sub-test so that UE identify the disappeared satellite is different from the new one. RRC is Release after each sub-test running, and UE re-enters RRC Connect mode for a different cell id before each sub-test. The proposed test procedure is shown as following:

8.2.1.2.2.1.1_1.3.2 Test procedure 1. UE location according to TS 38.508-1 [12] clause 5.6.1 is provided to the UE through any preconfigured means. 2 Test equipment shall emulate the signal with doppler and delay according to initial ephemeris defined in TS 38.508[12] table 5.6.2.1-x for NGSO (LEO-600) if UE supports only NGSO satellites or both GSO and NGSO satellites. Test system shall send SIB19 information during the duration of the test as defined in clause 8.2.1.2.2.1.1_1.3.3. 3. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for NR/5GC with Connected without Release On, Test Mode On according to TS 38.508-1 [12] clause 4.5. Message contents are defined in clause 8.2.1.2.2.1.1_1.3.3. 4. SS continue transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Tables Table 8.2.1.2.2.1.1-3. The SS sends downlink MAC padding bits on the DL RMC. 5. Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR according to Table 8.2.1.2.2.1.1-2 and Table 8.2.1.2.2.1.1-3 as appropriate. 6. Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest and decide pass or fail according to [Annex G]. 7.The SS shall send an RRCRelease message to ensure that the UE is in state RRC_IDLE on Cell 1 8. Set Cell 1 physical cell identity = ((current Cell 1 physical cell identity + 3) mod1008) for next iteration of the test procedure loop. 9. Repeat steps from 2 to 8 for each subtest in Table 8.2.1.2.2.1.1-3 as appropriate.

For Rel-19 UE

- The NGSO requirements with timing varying channel model should be applied

  - If UE can pass the NGSO requirements with the timing varying new channel model, then it can skip the corresponding NGSO requirements with NTN-TDL defined in Rel-17/Rel-18.

- All other test cases defined in Rel-17/18 that are only feasible for testing under NTN-TDL channel model are still applicable for Rel-19 UE.

- Applicable tests are 1-5, 1-6, 1-3 and 1-4

- RAN4 needs to re-simulate all Rel-17/18 demodulation requirements with the timing varying channel model that will be very heavy workload for RAN4, with also consideration of the parallel discussion for NR Rel-19/Rel-20 and 6GR study for demodulation requirements. i.e. Rel-17/18 test 1-3 and 1-4 are not applicable for testing under NTN timing varying channel model

The selected RF frequency error tests, RRM UL timing tests and limited number of demodulation test cases are enough to verify the UE’s pre-compensation on the frequency offset shift and timing offset shift under timing varying channel model as per the discussion on this WI.

Observation 3: There are commercial NTN UE finished Rel-17/18 NTN testing, it means that no technical feasibility issues to conduct the NGSO testing under NTN-TDL.

For Rel-17/18 UE

- The NGSO requirements with timing varying channel model is not applicable

- Test 1-3 and 1-4 are not applicable for testing under NTN timing varying channel model without re-simulation to decide the additional margin

- Applicable tests are 1-1, 1-2, 1-3 and 1-4.

Reference

R4-2515053, Way Forward for [116bis][328] Rel-19 Demodulation_Part3, RAN4#116bis, Ericsson