3GPP TSG-RAN WG4 Meeting #117	 R4-2521403

Dallas, USA, 17th – 21st November, 2025

Source:	Nokia

Title:	On High power UE (HPUE) for NR single carrier operation

Agenda item:	7.1.4

Document for:	Discussion

# 1				Introduction

New Rel-19 WI for 5G-A UE RF was approved [1]. In this contribution we focus on High power UE (HPUE) for NR single carrier operation objectives

As a re-cap of the objctive:

High power UE (HPUE) for NR single carrier operation

- Specify the UE RF requirements for single carrier TDD to support 4Tx for CPE/FWA UE with 4x26dBm PA configurations

  - Co-existence study for ACLR, specify ACLR requirements if needed

  - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc.

    - Supporting 4x4 UL MIMO and TxD

  - Specify the band specific requirements for exemplary bands

    - Example bands: n41, n77/n78, n79, n104

  - Maximum output power is up to 32 dBm

    - RAN4 to determine whether to reuse PC1 or introduce new power class

- PC1.5 with 2Tx for FDD bands for handheld and FWA UE

  - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc.

    - Supporting 2x2 UL MIMO and TxD

    - Baseline PA configuration: 2 x 26 dBm

  - Specify the band specific requirements for exemplary bands

  - Example bands: n1, n25

- PC1.5 1Tx for TDD bands for FWA UE

  - Study the possibility of applying PC1.5 with 2Tx MPR/A-MPR requirements to PC1.5 with 1Tx, if feasible, specify the corresponding UE RF requirements, including maximum output power (MOP), MPR/A-MPR requirements, etc.

    - Baseline PA configuration: 1 x 29 dBm

    - Example bands: n41, n77/n78, n79

  - This work is to start in 2026 Q4

- 2Tx for TDD bands for FWA UE

  - Study the possibility of applying PC1 with 4Tx A-MPR/MPR requirements to PC1 with 2Tx, if feasible, specify the corresponding UE RF requirements, including maximum output power (MOP), MPR/A-MPR requirements, etc.

    - Baseline PA configuration: 2 x 29 dBm

    - Example bands: n41, n77/n78, n79, n104

  - Maximum output power is up to 32 dBm

    - RAN4 to determine whether to reuse PC1 or introduce new power class

  - This work is to start in 2026 Q4

# 2	Discussion

## 2.1	4Tx PC1

Purpose of the first objective “single carrier TDD to support 4Tx for CPE/FWA UE with 4x26dBm PA configurations” is to enable to build FWA device that can produce power class 1 level of output power using current off-the-self components. Currently power class 1 specifications assume that single PA Tx architecture is used and availability of these PC1 PAs and filters that can tolerate such a high-power level is limited.

26 dBm PAs are nowadays common for TDD bands and all example bands of the WI are TDD bands. Using four 26 dBm PA the output power would be 32 dBm if one does the math. Now power class 1 is 31 dBm. In RAN#109 there was discussion how to solve the discrepancy, but no conclusion was reached. One way is to reuse PC1 and rely to the fact that 32 dBm is still within tolerances. For this option a concearn was raised that in this case then a UE capability is needed to separate 1Tx and 4Tx UEs. Another proposed way was to define new power class 1bis which would be 32 dBm. With new power class there is no need for new UE capability to distinguish 1Tx and 4Tx.

In previous RAN4 meeting WF for co-ex analysis was agreed [2] which listed two options to do the analysis.

- Option 1 Average throughput loss and 5%-ile throughput loss should be less than 5%. - Option 2: The following procedure shall be performed to obtain the performance metric: 1) Run the UL to UL coexistence study, assuming UE max TX power of both systems are 23 dBm. This corresponds to the coexistence of two commercial networks operating in adjacent channel and with similar deployment parameters. This is used as the reference. The victim system performance degradation results in this scenario are used as the baseline. 2) Run the UL to UL coexistence study, assuming 32 dBm power class UE is deployed in interfering system only and obtain the victim system performance degradation results. 3) Compare the victim system performance degradation obtaining in steps 1) and 2), choose the 32 dBm UE ACLR value so that the victim system performance degradation due to 32 dBm UE in 2) is similar to 1).

Based on our analysis [3] if RAN4 follows option 1 above then 32 dBm is feasible with 31 dB ACLR

Proposal 1: If average throughput loss and 5%-ile throughput loss should be less than 5% method for co-ex is chosen, then define new power class PC1Bis with 32 dBm of maximum output power.

Regarding the release independence, it can only be determined once the decision regarding power class is made and we know if any additional signalling is required or not. Thus, we propose to wait before making any decision on release independence regarding 4 Tx.

Proposal 2: Wait for further agreements on power class and need for additional signalling before deciding release independence.

## 2.2	2Tx PC1.5 FDD

The second objective “PC1.5 with 2Tx for FDD bands for handheld and FWA UE” is building on existing specifications as PC1.5 with 2Tx for TDD feature is already specified.

Observation 1: PC1.5 with 2Tx for TDD feature is already specified and those emission requirements can be extended into FDD operation.

It is quite likely that MPR requirements for PC1.5 with 2Tx for TDD are also applicable for FDD. In fact the PC1.5 TDD feature is reusing PC2 MPR as it is realized by using two 26 dBm PAs.

Observation 2: PC1.5 with 2Tx for TDD MPR maybe be extendedable for FDD operation.

FWA devices generally have a lot more room avaialble in them and thus can have have higher antenna isolation between them. Thus, a lager antenna isolaiton is possible for them.

Proposal 3: 20 dB or larger antenna isolation should be used for FWA devices.

The example bands n1, n25 do need A-MPR study which is the major part of the work for PC1.5 with 2Tx for FDD. Regarding NS_03 emission requirements, they include CBWs up to 45 MHz. However, if we can restrict to the UL CBWs of n25, then the 45 MHz CBW doesn’t need to be studied, since in n25 it is for DL only. Our initial simulation A-MPR simulaiton results as presented for information in R4-2521305.

Proposal 4: RAN4 shall discuss if 45 MHz needs to be studied for NS_03 A-MPR.

Observation 3: Initial A-MPR results for NS_03 and NS_05 are presented in R4-2521305.

# 3	Conclusion

In this contribution we have made following proposals and observations.

Proposal 1: If Average throughput loss and 5%-ile throughput loss should be less than 5% method for co-ex is chosen then define new power class 1Bis for 32 dBm.

Proposal 2: Wait for further agreements on power class and need for additional signalling before deciding release independence.

Observation 1: PC1.5 with 2Tx for TDD feature is already specified and those emission requirements can be extended into FDD operation.

Observation 2: PC1.5 with 2Tx for TDD MPR maybe be extendedable for FDD operation.

Proposal 3: 20 dB or larger antenna isolation should be used for FWA devices.

Proposal 4: RAN4 shall discuss if 45 MHz needs to be studied for NS_03 A-MPR.

Observation 3: Initial A-MPR results for NS_03 and NS_05 are presented in R4-2521305.

- References

- RP-252942, UE RF enhancements for NR FR1, Phase 5, RAN#109,

- R4-2515085, Way Forward for [116bis][329] NR_UE_RF_Ph5_Part2 - HPUE coexistence,

- R4-2515087, Way Forward for [116bis][316] NR_UE_RF_Ph5, Nokia, Murata, RAN4#116bis,

- R4-2520279, Simulation results for HPUE coexistence study.