# 3GPP TSG-RAN WG4 Meeting #117	R4-2521422

# Dallas, USA, Nov. 17-21, 2025

Title:	Discussion on IoT-NTN PC1.5 single Tx

Source:	Samsung

Agenda item:	7.6.3

Document for:	Approval

Introduction

In last RAN #109 meeting, the meeting approved the WID [1] for further enhancement of both NR-NTN and IoT-NTN. In this agenda, it includes three major aspects: HPUE for NTN, HD-FDD for NTN and DL intra-band intra-SAN CA. And in this paper, we will focus on the NR-NTN objectives as follows:

HPUE for NTN Core Part

- Specify high power UE (HPUE) NB-IoT based IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario

  - IoT-NTN PC1.5 with 1Tx for handheld and non-handheld device types

  - Example bands are Band 256 and 255

  - Necessary Tx requirements, at least including

    - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary

  - Necessary Rx requirements, at least including

    - Specify the RSD requirements on the example bands, if necessary

  - Specify the necessary signaling to support the above objectives

- The band agnostic requirements for UE power class specified for TN are expected to apply also to corresponding NR NTN power classes.

Discussion

In last meeting, #116bis had agreed on almost every requirement in WF [2] for PC1.5 NB-IoT based IoT-NTN in RF, and we will continue discussing the remaining parts in this paper.

In last meeting, one issue that was not resolved is the overheating solution. The WF[2] captured the following:

R4-2515081 [2] - Issue 2-2-3: UE RF front end assumptions and related accommodations WayFoward: - FFS on potential accommodations to UE maximum output power requirements when UE is scheduled for long duration in UL.

In previous discussions, the IoT NTN HPUE thermal solution was following the existing TN spec in general. In the current IoT-NTN PC2 and PC1 output power requirements, the scheme for thermal issue uses P-MPR, same as 36.101.

- TS 36.102 - 6.2B.4 Configured transmitted Power for category NB1 and NB2 For category NB1 and NB2 UE, the configured transmitted power requirements in clause 6.2.5F of TS 36.101 [7] shall apply except for: The definition of PCMAX_L,c and PCMAX_H,c which would be substituted by:  PCMAX_L,c = MIN { PEMAX,c , PPowerClass – MAX(MPRc + A-MPRc, P-MPRc) + ΔPNBBoost}  PCMAX_H,c = MIN { PEMAX,c, PPowerClass + ΔPNBBoost } , wherein - PPowerClass is the maximum UE power specified in Table 6.2B.1-1 without taking into account the tolerance specified in the Table 6.2B.1-1 - ΔPNBBoost is defined as 2dB for single tone positions 3 to 8 for 15kHz SCS and 8 to 39 for 3.75kHz 1dB for other single tone positions and all multi-tone cases, when all of the following conditions are met: if the UE indicates support for UE capability [powerBoosting-NBIotNTN-r19] if IE [powerBoostNBIotNTN-r19] is set to 1 if UE indicates support for power class 2. - The Maximum output power requirements are specified in subclause 6.2B.1 - The MPR requirements are specified in subclause 6.2B.2 - The A-MPR requirements are specified in subclause 6.2B.3. - PEMAX,c and P-MPRc are specified in subclause 6.2.5 of TS 36.101 [7]

Observation 1: The current PC2 and PC1 IoT-NTN re-uses P-MPR as thermal issue solution, and PC 1.5 should have similar issue and scheme as PC1 and PC2.

And even if have identified the issues for IoT-NTN HPUE in general for thermal issues, which means there is new identified issue for PC2, PC1.5 and PC1 in general, we believe it would also apply to TN HPUE. By saying this, we believe the logic is to introduce the scheme, if agreed, for TN spec first and then NTN UE can adopt it smoothly.

Proposal 1: To move the discussion to TN HPUE topic, given it should be a common scenario for both TN and NTN and for all HPUE cases. NTN can further discuss and even adopt the agreements from TN.

Conclusion

In this contribution, we proposed following for the IoT-NTN PC1.5 single Tx:

Observation 1: The current PC2 and PC1 IoT-NTN re-uses P-MPR as thermal issue solution, and PC 1.5 should have similar issue and scheme as PC1 and PC2.

Proposal 1: To move the discussion to TN HPUE topic, given it should be a common scenario for both TN and NTN and for all HPUE cases. NTN can further discuss and even adopt the agreements from TN.

Reference

[1] RP-252962, New WID on Enhanced requirements for NR NTN and IoT NTN Phase 2, Moderator (RAN4 VC, Qualcomm Incorporated)

[2] R4-2515081, WF for [116bis][318] NR_IoT_NTN_Ph2, Moderator (CHTTL)