3GPP TSG-RAN WG4 Meeting #117			R4-2521424

Dallas, TX, USA, Nov 17th – 21th, 2025

Agenda item:	7.2.4

Title:	On SBFD BS to SBFD BS Adjacent Channel Coexistence

Source:	Samsung

Document for:	Discussion

Introduction

In Rel-18, Sub-Band non-overlapping Full Duplex (SBFD) was adopted as a new study item (SI) to enable the simultaneous transmission and reception capability of NR base stations (BSs) and to enhance the capacity of NR system. Since then, comprehensive studies have been conducted on the SBFD topics and conclusions have been made and documented in technical report [1].

To explore the potential deployment of SBFD BSs, the adjacent channel co-existence evaluation has been proposed in Rel-19, and the TDD-SBFD co-existence problem have been investigated. However, due to the work load and time limitation, RAN4 agreed not to introduce specific requirements for SBFD-SBFD co-ex case for FR1 WA or MR BS in Rel-19 [2].

- Issue 1-1: Whether SBFD-SBFD co-ex case should be in Rel-19 scope - Way forward - RAN4 do not introduce the specific requirements for SBFD-SBFD co-ex case for FR1 WA or MR BS in Rel-19. SBFD-SBFD co-ex case can be considered for Rel-20.

In RAN#108, it is agreed to continue investigating the SBFD BS to SBFD BS adjacent channel coexistence, but focusing on the more realistic deployment scenarios.

6 BS RF centric evolution 6.1 Agreements: The high-level scope for WI on BS RF centric evolution (the exact scope and WID draft will be discussed and decided in RAN#109): - Enhancement of co-location requirements - It is FFS on the applicable frequency ranges, e.g. >3GHz requirements, for OTA - R19 outcome should be considered - Coexistence requirement enhancement can be further discussed in RAN#109 - Down select from the following four options in RAN#109 - Additional Hybrid requirement sets for BS type 1-H/large antenna array - SBFD BS to SBFD BS adjacent channel coexistence – focusing on realistic deployment scenarios - It is limited to RAN4 only solution - Neither - Both - No RAN1/2 impact is expected - Target 0.5RF TU in Q4’25 and 1 TU allocation from Q1’26.

In RAN#109, the new work item (WI) entitled “NR base station (BS) RF requirement evolution for FR1 and testing phase 2” has been approved to further investigate the co-location requirements enhancement and SBFD BS to SBFD BS adjacent co-existence problems in Rel-20 [3].

SBFD BS to SBFD BS adjacent channel co-existence - SBFD BS to SBFD BS adjacent channel coexistence - Focus on FR1 WA. - Consider both scenarios including SBFD capable BS being co-located and non-collocated. - Focus on bands: n104, n79, n41. - Specify necessary (and if feasible) BS TX and RX RF requirement for SBFD-capable BS for the scenarios of SBFD BS to SBFD BS adjacent channel coexistence. The BS RF requirements include - TX unwanted emission - RX in-band blocking - Note 1: whether to comply with requirements is declared by BS. - Note 2: The existing TDD requirements apply for non-SBFD BSs

In this contribution, we discuss the SBFD BS to SBFD BS (SBFD-SBFD) adjacent channel co-existence problem and corresponding evaluation requirements.

SBFD BS to SBFD BS Adjacent Channel Coexistence Discussion

2.1 Frequency Band Discussion

In RAN#109, it is approved that the new SBFD––SBFD adjacent coexistence study focuses on bands n104, n79, n41. Based on TS 38.104, the definitions of these operating bands are summarized below:

Table 1: NR operating bands in FR1 for SBFD [4]

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL, low – FUL, high (MHz) | Downlink (DL) operating band BS transmit / UE receive FDL, low – FDL, high (MHz) | Duplex mode | Bandwidth (MHz) |
| --- | --- | --- | --- | --- |
| n41 | 2496 – 2690 | 2496 – 2690 | TDD | 194 |
| n79 | 4400 – 5000 | 4400 – 5000 | TDD | 600 |
| n104 * | 6425 – 7125 | 6425 – 7125 | TDD | 700 |
| * Note: This band is applicable only in countries/regions designating this band for IMT licensed operation subject to country-specific conditions. |  |  |  |  |

For the SBFD co-existence study in NR, the carrier frequency of the SBFD BS is assumed to be 4GHz under FR1 deployment scenarios [7], which is documented in Table E.2.1-1 TR 38.858 Annex E, and the SBFD-TDD co-existence studies have been conducted under this carrier frequency requirement.

Noticed that in Table 1, the bandwidths for band n79 and band n104 are 600MHz and 700MHz, respectively. But there is only 196MHz for band n41. For band n104, since the frequency range has not been used extensive, it is convenient for vendors to deploy SBFD BS in this band with less coexistence issues. Concerning the future evolutions of full-duplex techniques as well as the freedom of deployment, it is recommended that RAN4 prioritize the SBFD–SBFD adjacent coexistence study on bands n104. However, band n79 and n49 can also be studied in future if it is allowed.

RAN4 prioritize the SBFD–SBFD adjacent coexistence study on bands n104, and band n41 and n79 can be further considered if allowed.

2.2 Adjacent Channel Co-existence Scenarios

Based on the scenarios defined in Table 11.1-1 in TR38.858 [1], Scenario 1 and 6 are related to the wide area coverage co-existence and Scenarios 1-5 are related to the co-existence simulation with FR1. According to the agreement in RAN#109 [3], the SBFD-SBFD co-existence study will focus on FR1 WA. Thus, only Scenario 1 should be adopted for the SBFD-SBFD co-existence simulation.

Table 2: Adjacent channel co-existence scenarios [1]

| Scenario | FR | Aggressor | Victim |
| --- | --- | --- | --- |
| 1 | FR1 | Urban Macro | Urban Macro |
| 2 | FR1 | Urban Hotspot | Urban Hotspot |
| 3 | FR1 | Indoor | Indoor |
| 4 | FR1 | Urban Macro | Micro |
| 5 | FR1 | Micro | Micro |
| 6 | FR2-1 | Urban Macro | Urban Macro |
| 71 | FR2-1 | Urban Hotspot | Urban Hotspot |
| 8 | FR2-1 | Urban Dense | Urban Dense |
| 9 | FR2-1 | Indoor | Indoor |
| Note 1: This scenario has been down-selected. |  |  |  |

Only Scenario 1 should be adopted for the current SBFD-SBFD co-existence simulation.

2.3 Adjacent Channel Co-existence Slot Allocation

In RAN4#116Bis meeting, several SBFD-SBFD adjacent channel coexistence cases have been discussed, and it is agreed that the coexistence case optional need to be down-select.

Agreements: - For next meeting further consider following options, with the intention to down-select: Option 1: UD (Operator 1) and DU (Operator 2) Option 2: DUD (Operator 1 and 2) Option 3: UD and DUD Provide more description on each specific case.

Generally, for the SBFD–SBFD adjacent channel coexistence problem, the basic principle is to maximize guard band size in exchange for the better suppression of bandpass filter (BPF) so that the effect of adjacent channel interference is minimized with best effort.

For the SBFD–SBFD coexistence problem, the size of guard band should be maximized in exchange for the better suppression of bandpass filter (BPF).

Based on this principle, for the two operator coexistence cases, the best deployment strategy is to adopt UD–DU option, in which the downlink bands are adjacent for the two operator and the guard band is maximized. DUD–DUD should be deprioritized for the two-operator coexistence case, since the guard band size is not optimal.

For the three operation coexistence cases, the operator who possess the middle frequency band should adopt DUD slot allocation and put the downlink band at both sides to maximize guard band size of BPF. For the three or more operator coexistence cases, the deployment strategies for the operator are similar to the two-operator case and the operators in the middle should adopt DUD slot allocation to maximize guard band size as shown in Fig. 1. However, in this circumstance, there would be both UD–DUD slot allocation and DUD–DUD slot allocation, and the DUD–DUD slot allocation option has smaller guard band size, which would lead to more stringent coexistence requirement.

Fig.1 Illustration of coexistence case options.

DUD–DUD slot allocation has smallest guard band size, which would lead to the most stringent coexistence requirement.

Based on the agreement in RAN#109, the SBFD-SBFD coexistence study is focused on FR1, and n104, n79, n41 band might be selected for the coexistence simulation. For the frequency band 2496 – 2690MHz (n41), there would generally be no more than two operators. Thus, the coexistence case option 1 would be sufficient. However, for the frequency band 4400 – 5000MHz (n79) and frequency band 6425 – 7125MHz (n104), there could be two or more than two operators coexisting. In this case, either option 1, option 2 or option 3 coexistence case can be adopted.

On the one hand, two-operator coexistence is the most common coexistence case, and the corresponding UD–DU slot allocation (option 1) should be prioritized.

RAN4 SBFD–SBFD coexistence study could start with UD–UD slot allocation for the SBFD–SBFD adjacent coexistence study.

On the other hand, more than two-operator coexistence cases are also critical for bands n104 & n79, and UD–DUD slot allocation (option 3) should also be studied. Based on the above analysis, it can be derived that the DUD–DUD slot allocation is the worst case among the SBFD–SBFD adjacent coexistence cases. For the simple two-operator and three-operator coexistence cases, there is no need to adopt this slot allocation scheme.

RAN4 can discuss how to apply the co-existence study results to other cases.

According to the above analysis, the adjacent channel co-existence case table in TR38.858 Annex E can be updated as.

Table 11.1-2: (Updated) Adjacent channel co-existence cases

| Aggressor | Victim | Slot allocation Aggressor Victim |
| --- | --- | --- |
| SBFD UD | SBFD DU |  |
| SBFD UD | SBFD DUD |  |
| SBFD DUD | SBFD DUD |  |

2.4 Grid Shift Parameters for the Coexistence Simulation

Generally, the site distance and grid shift are critical parameters for the adjacent channel coexistence study. They could influence the performance of the coexistence simulations to a large extent and thus decide the final RF requirements for the coexistence scenarios, including TX unwanted emission, RX in-band blocking. For example, in TR38.104 [4], it is noted that the in-band blocking requirements are derived based on the simulation assumption of 4GHz with 288 meters minimum distance coming from 100% grid shift between SBFD-capable BSs and neighbouring adjacent-channel TDD BSs.

In TDD-SBFD coexistence study, it is reported that coexistence is only feasible for 100% grid-shift. For the SBFD-SBFD co-existence study, the inherent full-duplex nature of SBFD BS would relax ACIR requirement to be less stringent. Considering the flexibility of the deployment and the feasibility of the coexistence, RAN4 could consider the 10% grid shift as a baseline for the SBFD–SBFD coexistence study. Other options, such as 0% grid shift, could be further considered based on the outcome of the 10% grid shift coexistence study if needed. Thus, it is recommended RAN4 can consider 10% grid shift as starting point for SBFD–SBFD coexistence simulation.

RAN4 to consider 10% grid shift as a starting point for SBFD–SBFD coexistence simulation.

Conclusion

- RAN4 prioritize the SBFD–SBFD adjacent coexistence study on bands n104, and band n41 and n79 can be further considered if allowed.

- Only Scenario 1 should be adopted for the current SBFD-SBFD co-existence simulation.

For the SBFD–SBFD coexistence problem, the size of guard band should be maximized in exchange for the better suppression of bandpass filter (BPF).

DUD–DUD slot allocation has smallest guard band size, which would lead to the most stringent coexistence requirement.

RAN4 SBFD–SBFD coexistence study could start with UD–UD slot allocation for the SBFD–SBFD adjacent coexistence study.

RAN4 can discuss how to apply the co-existence study results to other cases.

RAN4 to consider 10% grid shift as a starting point for SBFD–SBFD coexistence simulation.

References

TR 38.858, “Study on Evolution of NR Duplex Operation”

R4-2508713, “Way Forward for [115][307] NR_duplex_evo_BSRF”, Huawei, HiSilicon, 3GPP TSG RAN4#115, Malta, MT, May 19-23, 2025

RP-251879, “WF for RAN4 led REL-20 topics”, RAN4 Chair (Apple), 3GPP TSG RAN Meeting #108, Prague, Czech Republic, Jun. 9-13, 2025

3GPP TS 38.104, “NR; Base Station (BS) Radio Transmission and Reception”

RP-252067, “Views on 5G A Rel 20 BS RF topics”, Samsung, 3GPP TSG RAN Meeting #109, Beijing, China, Sep. 15-18, 2025

RP-252739, “Proposals for RAN4-led Rel-20 BS RF topics”, Huawei, HiSilicon, 3GPP TSG RAN Meeting #109, Beijing, China, Sep. 15-18, 2025

TR 38.858, “Study on Evolution of NR Duplex Operation: Annex E”

R4-2511255, “CR to TS38.104 to Introduce BS RF Requirement for SBFD-Capable BS”, Samsung, 3GPP TSG RAN4#116, Bengaluru, India, Aug. 25-29, 2025

TR 38.922 “Study on International Mobile Telecommunications (IMT) parameters for 4400 - 4800 MHz, 7125 - 8400 MHz and 14800 - 15350 MHz;”