3GPP TSG-RAN WG4 Meeting #117	R4-2521573

Dallas, USA, Nov. 17th – 21st 2025

Agenda item:			7.7.4

Source:	Ericsson

Title:	Discussion on UAV UE RF requirements

Document for:	Discussion

# Introduction

A new WI on enhancement of NR RF and RRM requirements for UAV was approved in RAN#109 meeting. The objectives of the core part of the WI are shown below [1]:

- Scenario: - BSs for UAV service are deployed on the ground by adjusting the tilt angle upwards. The coverage height is up to 600~1000m. The ISD for BSs for UAV service is about 2~4km. - A direct radio link between BS on the ground and UAV UE. - Specify RF requirements for UAV UE to support higher UL output power. - Example bands: - TDD with same TDD pattern, including n41, n77, n78, n79, - FDD: n3 - Consider UAV UE with omni-direction antenna or with antenna array - Consider UAV UE with up to 33dBm max total conductive output power - Analyse co-existence for UAV and considering the details relevant to UAV including antenna array model and specify requirements if needed. - Specify UE RF requirements, e.g., maximum output power, configured transmission power, SEM. - Specify RRM requirements, if needed Note 1: No RAN1 impact are expected

The part of the WF containing the RF requirement was shown below [2]:

### Sub-topic 3-1: Tx RF requirements

Issue 3-1-1: General aspects

- Agreements:

- The high-power UAV UE applies for the listed example bands in Rel-20 WID.

- RF requirements need to be defined on both omni-directional antenna connectors and transceiver array boundary (TAB) connectors.

- The same UE can support either omni-directional antenna and phase array antenna.

### Sub-topic 3-2: Tx RF requirements

Issue 3-2-1: Maximum output power

- Agreements:

- RAN4 to specify maximum output power for UAV higher power UE considering the following two options:

- Option 1: Introduce the new capability for high power UAV to indicate the rated maximum output power (Similar as ATG)

- Option 2: Introduce a new power class for UAV UE for up to 33dBm max total conductive output power for the example bands (Similar as legacy TN UE power class definition)

- Option 3: Other options are not precluded.

Issue 3-2-3: Output dynamic range - Agreements: - FFS on RAN4 needs to evaluate the minimum output power for high power UAV equipped with omni-directional antenna and directional antenna array considering the different center frequency, pass loss and antenna gain, etc. Issue 3-2-4: Transmit signal quality - Agreements: - As a starting point, the legacy frequency error and transmit modulation quality requirements can be reused for high power UAV. Issue 3-2-5: Output RF spectrum emissions - Agreements: - Take the following conclusions for output RF spectrum emissions as the starting point. Requirements; Conclusions. Occupied bandwidth; Legacy requirements can apply to high power UAV UE.. SEM requirements; The discussion on SEM can be held until ACLR is concluded.. ACLR requirements; RAN4 needs to evaluate ACLR for high power UAV based on co-existence analysis.. Spurious emissions; Legacy requirements can apply to high power UAV UE.. Spurious emissions for UE co-existence; Legacy requirements can apply to high power UAV UE.. Transmit intermodulation; FFS on whether this requirement is needed or not for high power UAV UE.. ### Sub-topic 3-3: Rx RF requirements Issue 3-3-2: ACS - Agreements: - FFS on how to specify ACS requirements

This paper presents some preliminary views and observations on UAV UE RF requirements.

# Discussion

## Tx Requirements

One of the key issues within the Tx requirements discussion is how to specify the maximum output power. According to the stated objectives for this WI, the maximum conductive output power to be considered in this WI is up to 33 dBm, which exceeds any existing any power class defined in the current specification. Therefore, the necessity of whether adopting 33 dBm as the maximum output power should be carefully assessed based on the outcome of co-existence simulation. For instance, if the current UE PC1(31 dBm with +2/-3 dB tolerance) could achieve the coverage goal, given that the majority of UAV deployment scenario are expected to operate under LoS conditions, then 33 dBm as maximum output power maybe not be justified.

- The current UE PC1 may achieve the coverage goal given that majority of UAV deployment scenario are under LoS conditions.

- The necessity of whether to adopt 33 dBm as maximum output power should be further discussed.

For ATG UEs, the maximum output power of ATG UE is vendor-declared via UE capability signaling, ranging from 23 dBm to 40 dBm. This flexibility suits ATG’s sparse deployment, high altitude (Vertical distribution between 3 km and 10 km) where interference is less of a concern due to low density.

Compared to the ATG network, UAV deployment scenario involves lower coverage height (up to 600 – 1000 m), higher deployment density, potentially many drones in urban and suburban areas. Though, the assumption of the number of UAV UEs served from the UAV BS is yet to be defined (e.g. maybe 5 or more per cell), such elevated densities amplify aggregate interference risks, including uplink noise rise from multiple UAVs transmitting at higher powers, and inter-drone sidelobe effects in antenna array configurations. These factors highlight key questions in defining the UAV’s maximum output power, out of band emission and other Tx parameters.

- Compared to the ATG network, UAV scenario involves lower coverage height, higher deployment density.

- Consider further how to specify the maximum output power for UAV UE based on the outcome of co-existence simulation.

An increase in output power level generally leads to higher adjacent channel leakage. Therefore, additional measures and evaluation are needed to ensure the coexistence of the UAV operated through ground BS with other services in-band and in adjacent bands. Such services are FSS, RAS, etc.

The ACLR requirements should be determined according to the outcome of co-existence analysis.

- Further evaluate the ACLR, SEM and spurious emissions, considering the co-existence and regulations.

Normally, handheld UE on the ground is likely to encounter strong nearby interfering signals. These can cause non-linearities in the UE's transmitter, generating intermodulation products that could degrade performance or cause interference.

Transmit intermodulation occurs due to non-linearities in the transmitter's power amplifier and other components. When a wanted signal and an interfering signal (e.g., from nearby transmitters) mix, they can produce unwanted intermodulation products.

For altitudes below 1000m, the interference environment is considered closer to terrestrial scenarios, where potential interferers can still reach the UAV UE with sufficient power, all depends on how close between two devices.

- Depends on how close between UAV and other devices, the transmit intermodulation requirement may still be relevant.

## Rx Requirements

The situation of the out-of-band blocking differs from a ground-based UE, and it may not be similar to ATG UE either, as overall shape, weight for the UAV under study is unknow. Currently, there is no one standard of classification used in the industry. According to the US DoD, the UAV with service height below 3500 ft (1067 m) belongs to Group 2, medium size UAV weight between 9.5 -25 kg [4]. EASA divides UAS (unmanned aerial system) operation into three main categories: open (low risk), specific (medium risk) and certified (high risk). The category is determined by factors such as operation environment, drone weight, proximity to people, flight altitude, etc. Furthermore, UAV may contain other type of radio transmitters that are yet to know. Therefore, OOBB requirement needs further investigation.

As stated in the previous section, the UAV deployment scenario differs from that of the ATG network. Therefore, several receiver requirements require further study, a few examples listed below:

- OOBB requirements

- The maximum input level considering the worst-case scenario yet to be defined.

- Adjacent channel selectivity

- The OOBB requirements, maximum input level needs further investigation and the adjacent channel selectivity should be specified based on the co-existence analysis.

# Conclusion

Based on the discussion in the previous sections, we made the following observations:

Observation 1	The current UE PC1 may achieve the coverage goal given that majority of UAV deployment scenario are under LoS conditions.

Observation 2	Compared to the ATG network, UAV scenario involves lower coverage height, higher deployment density.

Observation 3	Depends on how close between UAV and other devices, the transmit intermodulation requirement may still be relevant.

Based on the discussion in the previous sections, we propose the following:

Proposal 1	The necessity of whether to adopt 33 dBm as maximum output power should be further discussed.

Proposal 2	Consider further how to specify the maximum output power for UAV UE based on the outcome of co-existence simulation.

Proposal 3	Further evaluate the ACLR, SEM and spurious emissions, considering the co-existence and regulations.

Proposal 4	The OOBB requirements, maximum input level needs further investigation and the adjacent channel selectivity should be specified based on the co-existence analysis.

# References

- RP-252908, New WID on enhancement of NR RF and RRM requirements for uncrewed aerial vehicle (UAV), Moderator (Huawei).

- R4-2515159, Way Forward for [116bis][319] NR_UAV, Qualcomm Incorporated, CMCC.

- TR 38.876, Air-to-ground network for NR.

- U.S Army Unmanned aircraft system roadmap 2010-2035.