3GPP TSG-RAN WG4 Meeting#117                                R4-2521761

# Dallas, USA, 17 – 21 Nov, 2025

Agenda item:	6.8.3.1

Source: 	ZTE Corporation, Sanechips

Title: 	Further discussion on Coexistence spurious emission requirements

Document for:	Approval

Introduction

In the RAN#103 meeting, new WID for Rel-19 BS RF evolution has been approved and it’s expected to start the normative work from this April meeting. In the RAN#106 meeting, the following objective is added for the reply LS to ETSI TFES. In this contribution, we would like to share our understanding on the simulation assumptions for the further evaluation in RAN4 and initial calibration results captured in the attached excel sheet.

Transmitter co-existence spurious emission requirements - Evaluate co-existence transmitter spurious emission limits according to the request from ETSI/TFES - Improve transmitter co-existence spurious emission requirements if needed. - Note: start the work after March, 2025 and focus on bands in the upper region of FR1 with priority above 5.9 GHz.

Discussion

## 2.1. Simulation assumption for BS2BS coupling loss analysis

Details on urban macro network layout model for BS2BScoupling loss analysis are listed in Table 2.1-1 and 2.1-2.

Table 2.1-1: Single operator layout for urban macro

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |  |
| Inter-site distance |  | 500 m |  |
| BS antenna height |  | 25 m |  |
| UE location | Outdoor/indoor | Outdoor and indoor |  |
|  | Indoor UE ratio | 20% | ` |
|  | Low/high Penetration loss ratio | 50% low loss, 50% high loss |  |
|  | LOS/NLOS | LOS and NLOS |  |
|  | UE antenna height | Same as 3D-UMa in TR 36.873 |  |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 35 m |  |
| Channel model |  | UMa |  |
| BS2 UE Shadowing correlation |  | Between cells: 1.0 Between sites: 0.5 |  |
| Pathloss model for BS2BS |  |  | Free space pathloss model |
| Pathloss model for BS2UE |  | Please use the UMa model in the TR 38.901 7.4.1 Uma channel model |  |

Table 2.1-2: Multi operators layout for urban macro

| Parameters | Values | Remark |
| --- | --- | --- |
| Multi operators layout | Un-coordinated operation (100% Grid Shift) |  |

| Coordinated Operation: each network with co-location of sites |  |
| --- | --- |

The antenna configurations for 2GHz and U6GHz WA BS are summarized in the following Table 2.1-3.

Table 2.1-3: Antenna configurations for 2GHz and U6GHz BS in Macro Urban scenario

| U6GHz BS antenna configuration |  | 2GHz BS antenna configuration |  |
| --- | --- | --- | --- |
| Parameter | Macro urban | Parameter | Macro urban |
| Am | 30 dB | Am | 30 dB |
| SLAv | 30 dB | SLAv | 30 dB |
| j3dB | 90 deg. | j3dB | 90 deg. |
| q3dB | 65 deg. | q3dB | 65 deg. |
| GE,max | 6.4 dBi | GE,max | 6.4 dBi |
| Msub | 3 | Msub | 3 |
| dv,sub | 0.7l m | dv,sub | 0.7l m |
| qsubtilt | 3 deg. | qsubtilt | 3 deg. |
| M | 8 | M | 4 |
| N | 16 | N | 8 |
| dh | 0.5l m | dh | 0.5l m |
| dv | 2.1l m | dv | 2.1l m |
| jescan | deg. | jescan | deg. |
| r | 1 | r | 1 |
| Ptx | 46 dBm | Ptx | 46 dBm |
| qmech | 6 deg. | qmech | 6 deg. |

The extended AAS antenna modelling is provided in the following Table 2.1-4.

Table 2.1-4: Extended AAS antenna modelling

| Description | Equation |
| --- | --- |
| Peak normalized element radiation pattern | Aθ,φ=−min−−min12φφ3dB2,Am−min12θ−90θ3dB2,SLAv ,Am |
| Peak gain normalized element radiation pattern | AEθ,φ=GE,max+Aθ,φ |
| Sub-array excitation | wm=1Msubexpj2πm−1dv,subλsinθsubtilt |
| Sub-array radiation pattern | Asubθ,φ=AEθ,φ+10log10m=1Msubwmvm2 , where vm=expj2πm−1dv,subλcosθ |
| Array excitation | wm,n=1MNexpj2πm−1dvλsinθetilt−n−1dhλcosθetiltsinφescan |
| Composite array radiation pattern | AAθ,φ=Asubθ,φ+10log101+ρm=1Mn=1Nwm,nvm,n2−1 , where vm,n=expj2πm−1dvλcosθ+n−1dhλsinθsinφ |

For the calibration procedures for BS2BS coupling loss:

Step 1: Calibrate the pathloss between aggressive BSs and victim BSs without antenna gain;

Step 2: Calibrate the pathloss between aggressive BSs and victim BSs with aggressor’s element gain (including 3dB diversity gain)

Step 3: Calibrate the pathloss between aggressive BSs and victim BSs with Aggressor_Tx_element_gain+ Aggressor_Tx_sub_array gain+Aggressor_Tx_array gain;

Step 4: Calibrate the pathloss between aggressive BSs and victim BSs with Aggressor_Tx_element_gain+ Aggressor_Tx_sub_array gain+Aggressor_Tx_array gain+Victim_Rx_element_gain+Victim_Rx_sub_array gain+Victim_Rx_array_gain;

## 2.2. Simulation results for BS2BS case and conclusion

### 2.2.1. Coupling loss metric

In the following figure 2.2.1-1/6, we provide the CDF of coupling loss between BS2BS, then detailed values at different percentile and also correlation level and harmonic signal types are summarized in the Table 2.2.1-1.

Figure 2.2.1-1. Coupling loss for BS2BS with 50% correlation level for 3rd harmonic signal

Figure 2.2.1-2. Coupling loss for BS2BS with 0% correlation level for 3rd harmonic signal

Figure 2.2.1-3. Coupling loss for BS2BS with 100% correlation level for 3rd harmonic signal

Figure 2.2.1-4. Coupling loss for BS2BS with 50% correlation level for 2nd harmonic signal

Figure 2.2.1-5. Coupling loss for BS2BS with 0% correlation level for 2nd harmonic signal

Figure 2.2.1-6. Coupling loss for BS2BS with 100% correlation level for 2nd harmonic signal

Table 2.2.1-1. The summary of BS2BS coupling loss analysis at different percentile

| Signal type | Correlation level between sub-array | Percentile of BS2BS coupling loss CDF |  |  |
| --- | --- | --- | --- | --- |
|  |  | 99% | 99.9% | 99.99% |
|  |  | CL [dB] | CL [dB] | CL [dB] |
| 3rd harmonic signal | 50% | -94.8596 | -81.5196 | -73.5077 |
|  | 100% | -99.5072 | -83.5311 | -71.7873 |
|  | 0% | -93.7391 | -80.4932 | -72.8547 |
| 2nd harmonic signal [NOTE 1] | 50% | -88.3497 | -75.4640 | -66.5155 |
|  | 100% | -92.5781 | -76.9807 | -64.6391 |
|  | 0% | -87.3982 | -75.1114 | -66.0110 |
| NOTE 1: for antenna with slot cut, the 2nd harmonic signal as elaborated in the following can be counteracted due to the antenna physical structure; In other words, the antenna element gain/ fully/partial beamforming gain can be removed from the simulation. NOTE 2: 10000 snapshot is considered. |  |  |  |  |

In the following section, radiation pattern at 2GHz and U6GHz of 2GHz antenna element and 3.5GHz antenna elements are provided to check the performance of wanted signal and spurious emission signal.

The simulation results for 2GHz antenna element are summarized in the following table 2.2.1-2:

Table 2.2.1-2. summarized simulation results for 2G antenna element

|  | 2G | U6G |
| --- | --- | --- |
| Operating frequency | 1710-2170MHz | 6425-7125MHz |
| Maximum Antenna gain | 8.1dB | 6.22dB |
| VSWR | <2 | 2.2-4.4 |

Figure 2.2.1-7. antenna gain diagram at 2GHz antenna element [2GHz left, U6GHz right]

The simulation results for 2GHz antenna sub-array with 1x4 configuration are summarized in the following table 2.2.1-3:

Table 2.2.1-3. summarized simulation results for 2GHz antenna sub-array with 1x4 configuration

|  | 2G | U6G |
| --- | --- | --- |
| Operating frequency | 1710-2170MHz | 6425-7125MHz |
| Maximum Antenna gain | 13.4dBi | 10.4dBi |
| VSWR | <1.7 | <4.56 |

Figure 2.2.1-8. antenna gain diagram at 2GHz antenna element [2GHz left, U6GHz right]

For the 3rd harmonic of 2GHz falling into U6GHz bands, the antenna aperture size of 2GHz antenna element/sub-array is indeed about 3 times of U6GHz which make the suitable for 3rd harmonic transmission. Based on the initial simulation results, comparable antenna gain could be still achieved at U6GHz to 2GHz band.

Observation 1: antenna elements/antenna sub-array of 2GHz can still achieve comparable gain at the 3rd harmonic freq within U6GHz.

The simulation results for 3.5GHz antenna element are summarized in the following table 2.2.1-4:

Table 2.2.1-4. summarized simulation results for 3.5G antenna element

|  | 3.5G | U6G |
| --- | --- | --- |
| Operating frequency | 3300-3700MHz | 6425-7125MHz |
| Maximum Antenna gain | 8.7dB | -4.1dB |
| VSWR | <1.7 | 2.1-8.3 |

Figure 2.2.1-9. antenna gain diagram at 3.5GHz antenna element [3.5GHz left, U6GHz right]

For the 2rd harmonic of 3.5GHz falling into U6GHz bands, however due to the internal antenna cancellation in the antenna, this kind of even harmonic signal could be well alleviated as shown in the figure 2.2.1-9.

Observation 2: the antenna gain of 2nd harmonic of 3.5GHz signal is much reduced compared with 3.5GHz due to the internal counteract of the antenna’s physical structure design.

Observation 3: for different percentile of BS2BS CDF to derive the coexistence spurious emission requirement, there are around 20dB coupling loss difference between 99% to 99.99%.

In the previous RAN4 meetings, there were some discussions on the percentile to derive the coexistence spurious emission requirements, however no consensus has been reached. As shown in Figure 2.2.1-10, for two UTRA BSs with passive antenna installation, coupling loss between two BSs are 57dBc with 100% probability due to the fixed beam pattern and fixed pathloss. If UTRA/LTE BS with passive antenna is failing to meet the coexistence spurious emission requirements, then it would cause the 0.8dB performance degradation with 100% probability. That is part of reasons that MCL was used in the past to derive the coexistence spurious emission requirement.

Figure 2.2.1-10. The illustration of UTA BS coexisting with UTRA BS [passive antenna assumption]

For NR BS coexisting with NR BS with AAS BS deployed as shown in Figure 2.2.1-11, the probability of the worst case (e.g. two beams steering towards each other or with worst coupling loss) is quite low which is different from passive antenna assumption. If we choose the 99.9% percentile of coupling loss as the worst case to derive the coexistence spurious emission requirement, then in theory, NR BS will have 0.1% probability to cause more than 0.8dB performance degradation which is still acceptable compared with other in-band blocking requirement defined in the past (check the 99%-99.9% of interference signal power level and 6dB performance degradation). If we choose the 99.99% percentile of coupling loss as the worst case, then current coexistence requirement could be kept unchanged.

Figure 2.2.1-11. The illustration of NR BS coexisting with NR BS [AAS antenna]

Proposal 1: For coexistence spurious emission requirement between aggressive BS and victim BS, propose to use the more stringent requirement between the 99.9% of coupling loss and 5% throughput loss criteria to derive the requirement.

### 2.2.2. Throughput loss metric

In the following figure 2.2.2-1/6, we provide the throughput loss due to BS2BS 2nd and 3rd harmonic interference signal, then detailed ACIR offset on top of baseline ACIR and required coexistence spurious emission limit according to the legacy 5% throughput loss are summarized in the Table 2.2.2-1.

Figure 2.2.2-1. Throughput loss for BS2BS with 50% correlation level for 3rd harmonic signal

Figure 2.2.2-2. Throughput loss for BS2BS with 0% correlation level for 3rd harmonic signal

Figure 2.2.2-3. Throughput loss for BS2BS with 100% correlation level for 3rd harmonic signal

Figure 2.2.2-4. Throughput loss for BS2BS with 50% correlation level for 2nd harmonic signal

Figure 2.2.2-5. Throughput loss for BS2BS with 0% correlation level for 2nd harmonic signal

Figure 2.2.2-6. Throughput loss for BS2BS with 100% correlation level for 2nd harmonic signal

Table 2.2.2-1. The summary of required ACIR offset to meet 5% throughput loss criteria

| Signal type | Correlation level between sub-array | ACIR offset in dB | Coexistence spurious emission dBm/MHz | NOTE |
| --- | --- | --- | --- | --- |
| 3rd harmonic signal | 50% | 21 | -34.4600 | 49-10*log10(100)-42.46-21 |
|  | 100% | 22 | -35.4600 | 49-10*log10(100)-42.46-22 |
|  | 0% | 19 | -32.4600 | 49-10*log10(100)-42.46-19 |
| 2nd harmonic signal [NOTE 1] | 50% | 26 | -39.4600 | 49-10*log10(100)-42.46-26 |
|  | 100% | 26 | -39.4600 | 49-10*log10(100)-42.46-26 |
|  | 0% | 24 | -37.4600 | 49-10*log10(100)-42.46-24 |
| NOTE 1: for antenna with slot cut, the 2nd harmonic signal as elaborated in the following can be counteracted due to the antenna physical structure; In other words, the antenna element gain/ fully/partial beamforming gain can be removed from the simulation. |  |  |  |  |

Observation 4: there are roughly around addition 19-26dB ACIR offset needed on top of baseline ACIR 42.46dB to meet the 5% throughput loss.

According to the coexistence simulation result as summarized in Table 2.2.2-1, it also indicate that 99.99% of coupling loss to keep the current requirement unchanged is not necessary, there are still around 13dB relaxation allowed according to the legacy coexistence criteria.

Observation 5: using 99.99% of coupling loss to keep the current requirement unchanged is not necessary, there are still around 13dB relaxation allowed according to the legacy coexistence criteria.

### 2.2.3. Conclusion

Based on the coupling loss analysis in section 2.2.1 and throughput loss analysis in section 2.2.2, we propose to define the coexistence spurious emission requirement as -39dBm/MHz for 2GHz/3.5GHz BS coexisting with band n104

Proposal 2: Propose to define -39dBm/MHz as coexistence spurious emission requirement for 2GHz and 3.5GHz BS coexisting with band n104.

## 2.3. Simulation assumption for BS2UE coupling loss analysis

Details on Dense Urban network layout model for BS2UE coupling loss analysis are listed in Table 2.2-1 and 2.2-2.

Table 2.2-1: Single operator layout for dense urban

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | Fixed cluster circle within a macro cell. | note1 |
| Number of micro BSs per macro cell As |  | 3 | 3 cluster circles are in a macro cell. 1 cluster circle has 1 micro BS. As shown in Figure 2.2-1 |
| Radius of UE dropping within a micro cell |  | < 72.17 m | Calculated based ISD. Nokia CR number: Sqrt(3)/4*ISD/3 |
| BS antenna height |  | 10 m |  |
| UE location | Outdoor/indoor | Outdoor and indoor |  |
|  | Indoor UE ratio | 80 % |  |
|  | 50% low loss, 50% high loss | Low/high Penetration loss ratio |  |
|  | LOS/NLOS | LOS and NLOS |  |
|  | UE antenna height | Same as 3D-UMi in TR 36.873 |  |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 3m |  |
| Channel model |  | UMi | TR 38.901 Umi channel model should be used |
| Shadowing correlation |  | Between Micro site: 0.5 |  |
| Note 1: Micro BS is randomly dropped on an edge of the cluster circle. All UEs communicate with micro BS, i.e. macro cell is only used for determining position of micro BS. As a layout of macro cell, hexagonal grid, 19 macro sites, 3 sectors per site model with wrap around with ISD =500 m is assumed. |  |  |  |

Figure 2.2-1: Network layout for dense urban

Table 2.2-2: Multi operators layout for dense urban

| Parameters | Values | Remark |
| --- | --- | --- |
| Multi operator layout | Cluster circle is coordinated | Note 1 |
| Minimum distance between micro BSs in different operator | 10 m |  |
| Note 1: Macro cell is collocated. Micro BS itself is randomly dropped. |  |  |

The antenna configurations for 2GHz and U6GHz WA BS are summarized in the following Table 2.2-3

Table 2.2-3: Antenna configurations for 2GHz and U6GHz BS in Dense Urban scenario

| U6GHz BS antenna configuration |  | 2GHz BS antenna configuration |  |
| --- | --- | --- | --- |
| Parameter | Micro BS | Parameter | Micro BS |
| Am | 30 dB | Am | 30 dB |
| SLAv | 30 dB | SLAv | 30 dB |
| j3dB | 90 deg. | j3dB | 90 deg. |
| q3dB | 65 deg. | q3dB | 65 deg. |
| GE,max | 6.4 dBi | GE,max | 6.4 dBi |
| Msub | N/A | Msub | N/A |
| dv,sub | N/A | dv,sub | N/A |
| qsubtilt | N/A | qsubtilt | N/A |
| M | 8 | M | 8 |
| N | 8 | N | 8 |
| dh | 0.5l m | dh | 0.5l m |
| dv | 0.7l m | dv | 0.7l m |
| r | 1 | r | 1 |
| Ptx | 37 dBm | Ptx | 37 dBm |
| qmech | N/A | qmech | N/A |

The AAS antenna modelling without sub-array configuration is provided in the following Table 2.2-4.

Table 2.2-4: Composite antenna pattern for AAS antenna modelling without sub-array configuration

| Parameter | Values |
| --- | --- |
| Composite Array radiation pattern in dB | For beam i: the super position vector is given by: the weighting is given by: |

For the calibration procedures for BSUE coupling loss:

Step 1: Calibrate the pathloss between aggressive BSs and victim UEs without antenna gain;

Step 2: Calibrate the pathloss between aggressive BSs and victim UEs with aggressor’s element gain (including 3dB diversity gain)

Step 3: Calibrate the pathloss between aggressive BSs and victim UEs with Aggressor_Tx_element_gain

+Aggressor_Tx_array gain;

## 2.4. Simulation results for BS2UE case

### 2.4.1. Coupling loss metric

In the following figure 2.2.4-1/6, we provide the CDF of coupling loss between BS2UE, then detailed values at different percentile and also correlation level and harmonic signal types are summarized in the Table 2.4.1-1.

Figure 2.4.1-1. Coupling loss for BS2UE with 100% correlation level for 3rd harmonic signal

Figure 2.4.1-2. Coupling loss for BS2UE with 50% correlation level for 3rd harmonic signal

Figure 2.4.1-3. Coupling loss for BS2UE with 0% correlation level for 3rd harmonic signal

Figure 2.4.1-4. Coupling loss for BS2UE with 50% correlation level for 2nd harmonic signal

Figure 2.4.1-5. Coupling loss for BS2UE with 100% correlation level for 2nd harmonic signal

Figure 2.4.1-6. Coupling loss for BS2UE with 0% correlation level for 2nd harmonic signal

Table 2.2.1-1. The summary of BS2UE coupling loss analysis at different percentile

| Signal type | Correlation level between sub-array | Percentile of BS2UE coupling loss CDF |  |  |
| --- | --- | --- | --- | --- |
|  |  | 99% | 99.9% | 99.99% |
|  |  | CL [dB] | CL [dB] | CL [dB] |
| 3rd harmonic signal | 50% | -117.0993 | -93.9449 | -77.9506 |
|  | 100% | -116.8091 | -94.3013 | -77.7408 |
|  | 0% | -116.4873 | -95.5220 | -78.6758 |
| 2nd harmonic signal [NOTE 1] | 50% | -117.1788 | -95.8344 | -81.7833 |
|  | 100% | -116.7631 | -94.4136 | -76.6655 |
|  | 0% | -117.2344 | -93.3777 | -78.7230 |
| NOTE 1: for antenna with slot cut, the 2nd harmonic signal can be counteracted due to the antenna physical structure; In other words, the antenna element gain/ fully/partial beamforming gain can be removed from the simulation. |  |  |  |  |

Observation 6: For different percentile of BS2UE CDF to derive the coexistence spurious emission requirement, there are around 40dB coupling loss difference between 99% to 99.99%.

Observation 7: Compared with the legacy MCL assumption 53dB between aggressive BS and victim UE, there is a big gap between 99.9 or 99.99% of coupling loss and 53dB.

### 2.4.2. Throughput loss metric

For this coexistence spurious emission case which is also quite similar as adjacent channel coexistence cases, in other words, once ACIR 32.73dBc can be met by the aggressor BS and victim UE, then it should be acceptable to maintain the 5% throughput loss. In other words, the coexistence spurious emission requirement for aggressor BS and victim UE is set as -15.73dBm/MHz (namely 37dBm-10*log10(100)-32.73=-15.73dBm/MHz), then the 5% throughput loss could be maintained.

Observation 8: For dense urban coexistence case, once the coexistence spurious emission -15.73dBm/MHz is met, the it should be acceptable to maintain the 5% throughput loss.

In the following section, we also provide the coexistence evaluation results for 2GHz/3.5GHz BS spurious emission at U6GHz towards the victim UE. According to the simulation result as shown in Figure 2.4.2-1/2/3/4/5/6 and summarized in table 2.4.2-1, it clearly indicates that 5% throughput loss including cell edge and cell average case can be met by assuming ACIR as 32.73dB.

Figure 2.4.2-1. Throughput loss for BS2UE with 100% correlation level for 3rd harmonic signal

Figure 2.4.2-2. Throughput loss for BS2UE with 50% correlation level for 3rd harmonic signal

Figure 2.4.2-3. Throughput loss for BS2UE with 0% correlation level for 3rd harmonic signal

Figure 2.4.2-4. Throughput loss for BS2UE with 100% correlation level for 2nd harmonic signal

Figure 2.4.2-5. Throughput loss for BS2UE with 50% correlation level for 2nd harmonic signal

Figure 2.4.2-6. Throughput loss for BS2UE with 0% correlation level for 2nd harmonic signal

Table 2.4.2-1. The summary of required ACIR offset to meet 5% throughput loss criteria

| Signal type | Correlation level between sub-array | ACIR offset in dB | Coexistence spurious emission dBm/MHz | NOTE |
| --- | --- | --- | --- | --- |
| 3rd harmonic signal | 50% | 0 | -15.73 | 37dBm-10*log10(100)-32.73=-15.73dBm/MHz |
|  | 100% | 0 | -15.73 | 37dBm-10*log10(100)-32.73=-15.73dBm/MHz |
|  | 0% | 0 | -15.73 | 37dBm-10*log10(100)-32.73=-15.73dBm/MHz |
| 2nd harmonic signal [NOTE 1] | 50% | 0 | -15.73 | 37dBm-10*log10(100)-32.73=-15.73dBm/MHz |
|  | 100% | 0 | -15.73 | 37dBm-10*log10(100)-32.73=-15.73dBm/MHz |
|  | 0% | 0 | -15.73 | 37dBm-10*log10(100)-32.73=-15.73dBm/MHz |
| NOTE 1: for antenna with slot cut, the 2nd harmonic signal as elaborated in the following can be counteracted due to the antenna physical structure; In other words, the antenna element gain/ fully/partial beamforming gain can be removed from the simulation. |  |  |  |  |

### 2.4.3. Conclusion

Compared with simulation results as captured in 2.2. the required coexistence spurious emission between aggressive BS and victim UE is much more relaxed compared with the required coexistence spurious emission between aggressive BS and victim BS. In other words, the coexistence spurious emission requirement between 2GHz/3.5GHz BS and U6GHz system, it should be decided by the BS2BS spurious emission limit instead of BS2UE spurious emission limit.

Proposal 3: for coexistence spurious emission requirement between 2GHz/3.5GHz BS and band n104, propose to define the requirement based on the BS2BS spurious emission requirement;

Conclusions

In this contribution, we shared some further analysis on coexistence spurious requirements between 2GHz and 3.5GHz BS and n104, observations and proposals are made as following:

Observation 1: Antenna elements/antenna sub-array of 2GHz can still achieve comparable gain at the 3rd harmonic freq within U6GHz.

Observation 2: The antenna gain of 2nd harmonic of 3.5GHz signal is much reduced compared with 3.5GHz due to the internal counteract of the antenna’s physical structure design.

Observation 3: For different percentile of BS2BS CDF to derive the coexistence spurious emission requirement, there are around 20dB coupling loss difference between 99% to 99.99%.

Proposal 1: For coexistence spurious emission requirement between aggressive BS and victim BS, propose to use the more stringent requirement between the 99.9% of coupling loss and 5% throughput loss criteria to derive the requirement.

Observation 4: There are roughly around addition 19-26dB ACIR offset needed on top of baseline ACIR 42.46dB to meet the 5% throughput loss.

Observation 5: Using 99.99% of coupling loss to keep the current requirement unchanged is not necessary, there are still around 13dB relaxation allowed according to the legacy coexistence criteria.

Proposal 2: Propose to define -39dBm/MHz as coexistence spurious emission requirement for 2GHz and 3.5GHz BS coexisting with band n104.

Observation 6: For different percentile of BS2UE CDF to derive the coexistence spurious emission requirement, there are around 40dB coupling loss difference between 99% to 99.99%.

Observation 7: Compared with the legacy MCL assumption 53dB between aggressive BS and victim UE, there is a big gap between 99.9 or 99.99% of coupling loss and 53dB.

Observation 8: For dense urban coexistence case, once the coexistence spurious emission -15.73dBm/MHz is met, the it should be acceptable to maintain the 5% throughput loss.

Proposal 3: for coexistence spurious emission requirement between 2GHz/3.5GHz BS and band n104, propose to define the requirement based on the BS2BS spurious emission requirement;

References

- RP-240829,New WID: WI resulting from discussion on BS RF requirement evolution, Approved.

- R4-2419797	Way Forward for [113][303] NR_BS_RF_Part2_CLTA

- R4-2419827	Way Forward for [113][302] NR_BS_RF_Part1_E_EIRP
[4] R4-2419828	Way Forward for [113][304] NR_BS_RF_Part3_OTA_TRP

- R4-2504769	Way Forward for Tx spurious emissions co-existence limits re-evaluation, Huawei, approved.

- R4-2504298   Huawei, HiSilicon

- R4-2504267,   Ericsson