3GPP TSG-RAN WG4 Meeting #117	R4-2521932

# Dallas, USA, Nov. 17-21, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on maintenance of RRM aspects of R19 Low band

carrier aggregation via switching

Agenda Item:	4.8.2

Document for:	Discussion

- Introduction

During previous several RAN4 meetings, good progress were achieved and the core part has been completed. All these progress were captured in [1][2][3][4][5]. In this paper, we provide our further analysis on the remaining issues for the maintenance phase.

- Discussion

- Application time of disabling switching pattern in timer triggered SCell deactivation

- During last meeting, after warm discussion, the application time of disabling switching pattern in MAC CE triggered SCell deactivation has already reached consensus as below, i.e. reuse similar timeline as the enabling switching pattern by MAC CE triggered SCell activation. For the timer triggered SCell deactivation, still suspending.

Agreement: - The switching pattern is NOT applied after n+ ((THARQ+3ms+ TLBCA)/slot-length) for MAC-CE based SCell deactivation - TLBCA =1 ms for MAC-CE based - Further discuss for timer triggered SCell deactivation

- For the timer triggered SCell deactivation, the UE processing timeline of SCell deactivation is specified in clause 8.3.3 of TS 38.133 as below:

- Upon expiry of the sCellDeactivationTimer in slot n, the UE shall accomplish the deactivation actions for the SCell being deactivated no later than in slot n + 3msNR slot lengtℎ.

- Compared with the MAC CE triggered SCell deactivation, the component of THARQ is not necessary since UE does not need to decode the TB in which MAC CE is loaded and not need to feedback ACK/NACK. As long as the timer expires, UE needs to process the switching pattern buffered in RRC layer, considering the timer monitoring is also MAC layer processing, similar as the MAC CE processing, so we prefer to reuse the MAC CE triggered timeline, i.e. TLBCA =1 ms.

Proposal 1: For timer triggered SCell deactivation, the switching pattern is not applied after slot n + (3ms + TLBCA)/slot-length, TLBCA =1 ms.

- Another relevant issue is when UE could switches to PCell after SCell deactivation triggered by timer. Since both timer monitoring and MAC CE processing are MAC layer processing, similar as above, only the component of THARQ is not necessary, so if UE is on SCell after the completion of SCell deactivation, UE switches to PCell immediately after slot n+(3ms+TLBCA)/slot-length.

- Proposal 2: For timer triggered SCell deactivation, if UE is on SCell after the completion of SCell deactivation, UE switches to PCell immediately after slot n+(3ms+TLBCA)/slot-length.

- Delay requirement in SDL SCell activation

- Since the switching pattern is also applicable to the SCell activation procedure, it seems the partial available case of SSB occasion/L1 RS occasion is possible, so the measurement extension is needed to address the partial availability issue. While how to capture this in the spec, which needs to be considered. It seems hard to simply introduce Kp factor since the delay requirements of SCell activation is split into multiple component, rather than an integrated formula. So one solution is to explain all the component is the available RS occasion based on switching pattern.

- Proposal 3: The partial availability of L3/L1 RS occasions during the SCell activation procedure should be considered. It could be captured into the spec with the clarification on the available RS occasions based on the switching pattern.

- RLM requirement for PCell

- Around the RLM/BFD on PCell, the following agreement was achieved with one FFS point.

- For RLM in LB-CA, follow principle in issue 1-1-1 to define RLM requirements for PCell in LB CA. - P factor in the existing RLM measurement requirements can be updated, where, - W window is max(TL1, xRP_max, switching pattern periodicity) - P = Ntotal/ Navailable - Ntotal is total number of RLM-RS in W window. - Navailable is number of available RLM-RS in W window. - No requirement when Navailable = 0. - FFS in maintenance stage: RAN4 to agree that the switch pattern is not applicable when the RLM or BFD is triggered on the PCell and UE should prioritize to receive RS for faster link recovery. Principle of scheduling restriction or interruption requirement agreed for the SCell activation to be reused for the link recovery procedure.

- We can understand the intention of the optimization to prioritize the RLM/BFD measurement over the switching pattern for the sake of faster link recovery. However as long as the RLM/BFD relevant parameters are configured to UE, actually UE performs the RLM/BFD measurement persistently in PHY layer, the evaluates whether the Ratio link failure or beam failure happens in higher layer. So from the perspective of RLM/BFD measurements, they are done by UE persistently as long as configured by NW. So if the RLM/BFD measurements prioritize the switching pattern, it is hard to decide the starting point and ending point of such prioritization, in other ward, such optimization is less operability.

- Proposal 4: Not to consider additional optimization for RLM/BFD for PCell.

- Conclusion

In this contribution, we have the following proposals on the RRM enhancements:

Proposal 1: For timer triggered SCell deactivation, the switching pattern is not applied after slot n + (3ms + TLBCA)/slot-length, TLBCA =1 ms.

- Proposal 2: For timer triggered SCell deactivation, if UE is on SCell after the completion of SCell deactivation, UE switches to PCell immediately after slot n+(3ms+TLBCA)/slot-length.

- Proposal 3: The partial availability of L3/L1 RS occasions during the SCell activation procedure should be considered. It could be captured into the spec with the clarification on the available RS occasions based on the switching pattern.

- Proposal 4: Not to consider additional optimization for RLM/BFD for PCell.

- Reference

- R4-2500527, “WF on RRM requirements for NR_LBCA_Sw”, Apple,3GPP RAN4#114, 2025.

- R4-2504895, “WF on RRM requirements for NR_LBCA_Sw”, Apple,3GPP RAN4#114bis, 2025.

- R4-2508259, “WF on RRM requirements for NR_LBCA_Sw”, Apple,3GPP RAN4#115, 2025.

- R4-2512119, “WF on RRM requirements for NR_LBCA_Sw”, Apple,3GPP RAN4#116, 2025.

- R4-2514870, “WF on RRM requirements for NR_LBCA_Sw”, Apple,3GPP RAN4#116bis, 2025.