3GPP TSG-RAN WG4 Meeting #117	R4-2522059

Dallas (TX), USA, November 17-21, 2025

Source: 	Qualcomm Incorporated

Title: 	Views on UE Performance requirements for FR1 SU-MIMO with SCM

Agenda item:	 7.5.3

Document for:	Agreement

- Introduction

According to the agreements in the Way Forward outlined in R4-2514823, we share our company’s view on the ongoing discussions and proposed agreements regarding UE performance requirements for FR1 SU-MIMO with SCM.

# Updates to the rCDL channel model

## rCDL-C2 Proposal

Based on discussions during and following the previous RAN4#116bis meeting, we propose to adopt the new rCDL-C2 table as the baseline for SU-MIMO evaluations. This table has been generated according to the procedure described in 38.753, apart from the fixes agreed in the last meeting.

In particular, this table follows the updated angular scaling procedure defined in TR 38.901 v19.2.0, and we have used the agreed desired AS [deg]: ASD=25, ASA=60, ZSD=5, ZSA=15.

Furthermore, we have not performed the second step of RMS delay rescaling on the surviving 12 clusters, and the delays have remained consistent with respect to the pre-reduction rescaling.

Observation 1: The proposed rCDL-C2 table has been generated according to TR 38.753, plus the following fixes: updated angular scaling procedure and no RMS delay rescaling post cluster reduction.

Proposal 1: Regarding the open issue of delay scaling after cluster reduction, we support Option 1 for rCDL (omit the post-reduction delay rescaling).

Proposal 2: RAN4 to discuss how to capture the AS/“s” value chosen for each channel model in the channel profile name (e.g. rCDL-C2-Asx or similar).

With respect to the updated angular scaling procedure, we have followed the procedure for “s” value computation in TR 38.901 and noticed minor differences with respect to the tabulated values included in the 38.901 spec. Previous agreements recommended using the tabulated values for s, however it is our view this would restrict the number of possible AS values for RAN4. To that end, we recommend RAN4 to capture the s values used for the generation of any table for future reference, and use the tabulated values in 38.901 for reference only

Observation 2: “s” scaling values computed based on the formula in TR 38.901 presented minor differences with respect to the tabulated values in the same spec. Using only the tabulated values restricts the available choices of Angular Scaling values.

Proposal 3: For future channel models selected, RAN4 companies should compute “s” scaling values according to the AS parameters agreed, and include them alongside the channel tables for reference.

Proposed rCDL-C2 Table

| CLID | Delay [nsec] (pre-reduction scaling) | Power [dB] | AoD [deg] | AoA [deg] | ZoD [deg] | ZoA [deg] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4 | -38.3 | -117.2 | 95.5 | 101.1 |
| 2 | 76 | -1.2 | -22.9 | 123.5 | 98 | 70.9 |
| 3 | 80 | -5.5 | -34.5 | -140 | 101.6 | 67 |
| 4 | 232 | 0 | -7.8 | 166.7 | 99.1 | 77.1 |
| 5 | 240 | -10.4 | 39.6 | 68 | 110 | 61.7 |
| 6 | 290 | -10.1 | -50 | 77.5 | 92 | 54.7 |
| 7 | 300 | -13.7 | 44.2 | -71.8 | 111.7 | 69.5 |
| 8 | 448 | -8.1 | -44 | 78.9 | 107.3 | 106.9 |
| 9 | 476 | -9.8 | -49.9 | -89.5 | 108.2 | 47.4 |
| 10 | 792 | -11.7 | -59.1 | 90.4 | 87.8 | 49.2 |
| 11 | 988 | -16.2 | 58.9 | 46.8 | 108.2 | 126.6 |
| 12 | 1676 | -16.9 | 49.8 | 23.7 | 88 | 60.3 |

And the related “s” scaling values used

| ‘s’ according to 38.901 V19 |  |  |  |
| --- | --- | --- | --- |
| AoD | AoA | ZoD | ZoA |
| 0.65121 | 0.85833 | 1.82164 | 1.9501 |

Proposal 4: RAN4 to agree on rCDL-C2 as baseline channel model for the evaluation of PDSCH and PMI demod requirements in this WI. Other options for channel models (e.g. different profiles, different AS choice) can be evaluated.

For reference, we have created a step-by-step table for other companies’ to validate the results modeled after similar tables in TR 38.753.

Observation 3: Annex A in this contribution contains the step-by-step table including spatial filtering for the derivation of the rCDL-C2.

## Capturing rCDL channel models

It is our view that future rCDL channel models used for the definition of UE Demod requirements in 3GPP will be captured in a dedicated section in TR 38.101-4, in line with the existing TDL models. To that end, channel models defined in this WI do not need to be captured in TR 38.753.

Proposal 5: RAN4 to capture rCDL channel models defined in this WI and used for the definition of Demodulation requirements in TR 38.101-4. No modifications are expected to rCDL channel models already captured in TR 38.753.

# SCM SU-MIMO requirements

## Test applicability for SCM SU-MIMO requirements

According to the proposals included in the WF, we support the introduction of a declaration for SCM testing, with the favourite options being a declaration per each type of test (e.g. PDSCH 4L, 8L, PMI Type I, etc). We do not support the introduction of a dedicated UE capability.

Proposal 6: Support the introduction of a declaration expressing support of the SCM requirements, with multiple declarations for each type of requirement defined with SCM (e.g. PDSCH 4L, 8L, PMI Type I, etc).

With respect to the release independence, we can discuss

Proposal 7: If RAN4 defines release independent requirements for SCM, SCM requirements should be applicable starting from R.19.

## Test scope and test parameters for PDSCH

In the last meeting RAN4 discussed the choice of MCS and test metric, with multiple candidate options. Our view is that for test scenarios with >4 layers, the UE expects to be scheduled with different MCS per codeword in a realistic scenario. Indeed preliminary studies in the SI phase has shown that SNR can be different for the two CWs using rCDL, so the test should follow that configuration.

Proposal 8: For simulations with >4L, RAN4 should configure different MCS per CW.

According to this choice, and to keep consistency with existing requirements, RAN4 should use as test metric the per-CW throughput in Rel.20 requirements.

Proposal 9: For requirements in this WI, RAN4 should consider 70% of the maximum throughput for different MCS per CW.

Proposal 10: MCS choice should be based on simulation results according to the following procedure. The requirement SNR is chosen based on the CW0 performance using an agreed MCS. The MCS for CW1 is then chosen as the highest MCS that can achieve at least 70% of the maximum throughput at the chosen SNR based on the collected simulation results.

## Unbiasing PMI statistics

It has been highlighted multiple times that rCDL is characterized by stationary spatial properties, that is fixed angular characteristics that do not change significantly during the test duration time. While this is part of what makes rCDL better suited for requirements and testing compared with the baseline CDL models, This stationarity leads to non-uniform PMI (Precoder Matrix Indicator) statistics: certain precoder indices are favored regardless of the actual channel conditions. As a result, a fixed precoder may outperform adaptive PMI selection algorithms in test scenarios, since these do not reflect real-world channel variability.

This issue is explicitly recognized in TR 38.753: “All spatial channel models in this study have fixed angular properties and have non-uniform PMI statistics, hence a fixed precoder may perform better than a channel adaptive precoder found by PMI selection algorithm.”

Observation 4: SCM (including rCDL) presents fixed angular properties and non-uniform PMI statistics.

To address this bias, we our proposal for RAN4 would be to follow an approach which combines the use of a directional antenna element model at the UE, as already proposed by some delegates in the last RAN4 meeting #116b, and simulated periodic rotation of the UE orientation in the azimuth direction during the execution of the test. By introducing a realistic antenna pattern and simulating UE rotation, the angular properties of the channel become dynamic, resulting in a richer and more uniform distribution of PMI statistics.

This method ensures that the UE cannot simply report the same PMI index repeatedly, and that the performance evaluation of PMI reporting algorithms is less biased, through the following aspects:

- Instead of assuming isotropic reception, the UE is modeled with a directional antenna element, which introduces angular selectivity and spatial filtering. This change increases the diversity of reported PMI indices, as the antenna pattern interacts with the channel’s angular spread.

- By periodically rotating the UE in the simulation, the angular relationship between the UE and the channel clusters changes over time. This dynamic process prevents the channel from favoring specific precoder indices and ensures that the PMI statistics reflect a realistic operational environment.

Preliminary results show that, with this approach, the distribution of PMI indices becomes significantly more varied, and the channel can be considered “unbiased”, and the UE is no longer always reporting the same index.

Proposal 11: RAN4 to investigate the use of directional Antenna Element response and periodic UE orientation rotation in the azimuth direction for the purposes of PMI unbiasing.

In any case, to keep the cluster reduction algorithm simple and repeatable, even if RAN4 decides to use directional antenna elements at the UE, we recommend to stick to omnidirectional UE antenna for the purposes of cluster reduction, in order to keep the reduced channel profile generic and applicable to all possible rotations.

Proposal 12: RAN4 to use Omnidirectional AE assumption at the UE for the purposes of CDL cluster reduction in all cases, including for channel use with directional AE, keeping the rCDL profile generic and applicable to all rotation values.

## Test scope for PMI requirements

Some companies have proposed multiple agreements related to eType II PMI reporting requirements. It is our view that in this work item we should first address, investigate and familiarize with the PMI unbiasing setup and results for TypeI PMI reporting, and only then focus on eTypeII PMI reporting.

Proposal 13: RAN4 to address PMI biasing and evaluate preliminary alignment on TypeI PMI reporting before concluding on parameters and test scope for eTypeII PMI requirements.

- Conclusions

Observation 1: The proposed rCDL-C2 table has been generated according to TR 38.753, plus the following fixes: updated angular scaling procedure and no RMS delay rescaling post cluster reduction.

Proposal 1: Regarding the open issue of delay scaling after cluster reduction, we support Option 1 for rCDL (omit the post-reduction delay rescaling).

Proposal 2: RAN4 to discuss how to capture the AS/“s” value chosen for each channel model in the channel profile name (e.g. rCDL-C2-Asx or similar).

Observation 2: “s” scaling values computed based on the formula in TR 38.901 presented minor differences with respect to the tabulated values in the same spec. Using only the tabulated values restricts the available choices of Angular Scaling values.

Proposal 3: For future channel models selected, RAN4 companies should compute “s” scaling values according to the AS parameters agreed, and include them alongside the channel tables for reference.

Proposal 4: RAN4 to agree on rCDL-C2 as baseline channel model for the evaluation of PDSCH and PMI

demod requirements in this WI. Other options for channel models (e.g. different profiles, different AS choice) can be evaluated.

Observation 3: Annex A in this contribution contains the step-by-step table including spatial filtering for the derivation of the rCDL-C2.

Proposal 5: RAN4 to capture rCDL channel models defined in this WI and used for the definition of Demodulation requirements in TR 38.101-4. No modifications are expected to rCDL channel models already captured in TR 38.753.

Proposal 6: Support the introduction of a declaration expressing support of the SCM requirements, with multiple declarations for each type of requirement defined with SCM (e.g. PDSCH 4L, 8L, PMI Type I, etc).

Proposal 7: If RAN4 defines release independent requirements for SCM, SCM requirements should be applicable starting from R.19.

Proposal 8: For simulations with >4L, RAN4 should configure different MCS per CW.

Proposal 9: For requirements in this WI, RAN4 should consider 70% of the maximum throughput for different MCS per CW.

Proposal 10: MCS choice should be based on simulation results according to the following procedure. The requirement SNR is chosen based on the CW0 performance using an agreed MCS. The MCS for CW1 is then chosen as the highest MCS that can achieve at least 70% of the maximum throughput at the chosen SNR based on the collected simulation results.

Observation 4: SCM (including rCDL) presents fixed angular properties and non-uniform PMI statistics.

Proposal 11: RAN4 to investigate the use of directional Antenna Element response and periodic UE orientation rotation in the azimuth direction for the purposes of PMI unbiasing.

Proposal 12: RAN4 to use Omnidirectional AE assumption at the UE for the purposes of CDL cluster reduction in all cases, including for channel use with directional AE, keeping the rCDL profile generic and applicable to all rotation values.

Proposal 13: RAN4 to address PMI biasing and evaluate preliminary alignment on TypeI PMI reporting before concluding on parameters and test scope for eTypeII PMI requirements.

# Annex A

## rCDL-C2 cluster reduction procedure using AAV(1,1)

Note that removed clusters after spatial filtering are highlighted in orange.

| Scale ‘s’ in 38.901 V19.1.0 |  |  |  |
| --- | --- | --- | --- |
| AoD | AoA | ZoD | ZoA |
| 0.65121 | 0.85833 | 1.82164 | 1.95018 |

| CLID | FILTERED POWER LIN | FILTERED POWER LIN NORMALIZED | FILTER POWER DB NORMALIZED | AoD, deg | AoA, deg | ZoD, deg | ZoA, deg |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0.037250568 | 0.071361266 | -11.47 | -38.3 | -117.2 | 95.5 | 101.1 |
| 2 | 0.289891686 | 0.555348255 | -2.55 | -22.9 | 123.5 | 98 | 70.9 |
| 3 | 0.070909651 | 0.135842292 | -8.67 | -34.5 | -140 | 101.6 | 67 |
| 4 | 0.521999814 | 1 | 0 | -7.8 | 166.7 | 99.1 | 77.1 |
| 5 | 0.017315621 | 0.0331717 | -14.79 | 39.6 | 68 | 110 | 61.7 |
| 6 | 0.009855692 | 0.018880643 | -17.24 | -50 | 77.5 | 92 | 54.7 |
| 7 | 0.006247331 | 0.011968072 | -19.22 | 44.2 | -71.8 | 111.7 | 69.5 |
| 8 | 0.000697614 | 0.001336426 | -28.74 | -71.2 | 60.7 | 88.7 | 48.2 |
| 9 | 0.023860033 | 0.045708892 | -13.4 | -44 | 78.9 | 107.3 | 106.9 |
| 10 | 0.011347478 | 0.021738472 | -16.63 | -49.9 | -89.5 | 108.2 | 47.4 |
| 11 | 0.003310904 | 0.006342731 | -21.98 | -59.1 | 90.4 | 87.8 | 49.2 |
| 12 | 1.40E-03 | 0.00269049 | -25.7 | 58.9 | 46.8 | 108.2 | 126.6 |
| 13 | 0.001290603 | 0.00247242 | -26.07 | 56.6 | 6.4 | 91.3 | 51.7 |
| 14 | 0.001944221 | 0.003724563 | -24.29 | 49.8 | 23.7 | 88 | 60.3 |
| 15 | 0.000166718 | 0.000319383 | -34.96 | -74.4 | 8.7 | 86.3 | 33.4 |
| 16 | 8.85E-04 | 0.001694998 | -27.71 | 52 | 28.8 | 112.8 | 50.8 |
| 17 | 9.81E-04 | 0.00187971 | -27.26 | 52.8 | 25.3 | 87.8 | 31.5 |
| 18 | 5.50E-04 | 0.001053036 | -29.78 | 61.4 | 21.1 | 87.6 | 50.6 |
| 19 | 8.28E-05 | 0.000158548 | -38 | 69.8 | 1.7 | 110 | 43.4 |
| 20 | 9.19E-06 | 1.75968E-05 | -47.55 | -88.6 | 49.3 | 114.8 | 41.4 |