3GPP TSG-RAN4 Meeting #117						           R4-2522244

Dallas, Texas US, 17 – 21 November 2025

Agenda Item:	   8.4.5

Source:	Thales, ESA, Viasat, Eutelsat Group, Airbus, SES, Hispasat

Title:	SAN Characteristics for 6GR

WI code: 	FS_6G_Radio

Release:	Rel-20

Document for:	Discussion

# Introduction

The 6G Radio Study Item approved at RAN#108 (RP-251881) [1] and revised at RAN#109 in RP-252912 [2] highlights the importance of Non-Terrestrial Network (NTN) support in 6G from the outset.

Preliminary discussions on evaluation assumptions for the 6GR air interface were held at RAN1#122 and RAN1#122bis [3]. These discussions covered models, scenarios, parameters and methodologies; evaluation metrics and criteria for technology proposals; energy efficiency; and sensing performance, including potential extensions to the channel model.

In this contribution, we provide inputs for the satellite access node characteristics (e.g., EIRP, antenna size, beam diameter, etc.) that can be used for coexistence. Some of the inputs for existent bands were also updated.

# Recap of agreements from RAN1 #122bis meeting

RAN1 #122bis made the following agreements related to NTN evaluation:

Agreement

The attached templates for NTN in R1-2507956 are endorsed in principle.

Agreement

At least the following carrier frequencies could be considered (from RAN1 perspective) for 6GR NTN evaluations:

- L-band (i.e., 1.5GHz)

- S-band (i.e. 2 GHz)

- Ku-band (FFS detailed frequency range)

- Ka-band (i.e. 30 GHz for UL, 20GHz for DL)

# Frequency bands to be considered

Proposal 1: RAN4 to consider at least the following lists of bands for 6G NTN:

- L-band (i.e., 1.5GHz)

- S-band (i.e., 2 GHz)

- C-band (i.e., 4 GHz)

- Ku-band (FFS detailed frequency range)

- Ka-band (i.e., 30 GHz for UL, 20 GHz for DL)

- Q/V-band (i.e., 37 GHz for UL, 47 GHz for DL)

# Satellite Payload Parameters

In the following, we provide new satellite access node characteristics for the evaluation of NTN based 6GR access. The aim is to update the satellite parameters listed in Table 6.1.1.1-1 from TR 38.821 (Release 16) with the current state-of-the-art space technologies for 6G network deployments.

The next tables are reporting SAN parameters (like EIRP, G/T, antenna size, and beam diameter) for L-, S-, C-, Ku-, Ka-, and Q/V bands.

NOTE-1: The reported beam diameters refer to the Nadir pointing of the satellite.

NOTE-2: For all frequency ranges, the number of simultaneous active beams may be as low as 1.5% with respect to the total visible beams in the field of view.

Table 1: Satellite parameters in L- and S- bands.

|  |  | Around 1.5 GHz |  | Around 2 GHz |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | L-band LEO 1200km | L-band LEO 600km | S-band LEO 1200km | S-band LEO 600km | S-band VLEO 300km |
| DL | Equivalent satellite antenna aperture | 2.5 m | 2.5 m | 2.5 m | 2.5 m | 1.5 m |
|  | Satellite EIRP density | 38 dBW/MHz | 38 dBW/MHz | 40 dBW/MHz | 40 dBW/MHz | 24 dBW/MHz |
|  | Max Tx Antenna Gain | 30 dBi | 30 dBi | 32 dBi | 32 dBi | 27 dBi |
|  | 3dB beamwidth | 4.50 deg | 4.50 deg | 3.6 deg | 3.6 deg | 6.0 deg |
|  | Beam diameter | 95 km | 50 km | 75 km | 40 km | 30 km |
| UL | Equivalent satellite antenna aperture | 2.5 m | 2.5 m | 2.5 m | 2.5 m | 1.5 m |
|  | Max Rx Antenna Gain | 30 dBi | 30 dBi | 32 dBi | 32 dBi | 27 dBi |
|  | G/T | 3 dB/K | 3 dB/K | 5 dB/K | 5 dB/K | 0.5 dB/K |

Table 2: Satellite parameters in C-band [5]

|  |  | Around 4 GHz |  |
| --- | --- | --- | --- |
|  |  | C-band GEO | C-band LEO 600km |
| DL | Equivalent satellite antenna aperture | 12 m | 2 m |
|  | Satellite EIRP density | 59 dBW/MHz | 34 dBW/MHz |
|  | Max Tx Antenna Gain | 50.5 dBi | 35 dBi |
|  | 3dB beamwidth | 0.42 deg | 2.50 deg |
|  | Beam diameter | 260 km | 30 km |
| UL | Equivalent satellite antenna aperture | 12 m | 2 m |
|  | Max Rx Antenna Gain | 50.5 dBi | 35 dBi |
|  | G/T | 23 dB/K | 7.5 dB/K |

Table 3: Satellite parameters in Ku-band

|  |  | Around 11/14GHz |  |  |
| --- | --- | --- | --- | --- |
|  |  | Ku-band MEO 8000km | Ku-band LEO 1200km | Ku-band LEO 600km |
| DL | Equivalent satellite antenna aperture | 1.50 m | 0.45 m | 0.40 m |
|  | Satellite EIRP density | 28 dBW/MHz | 13 dBW/MHz | 7.0 dBW/MHz |
|  | Max Tx Antenna Gain | 42.5 dBi | 32 dBi | 31 dBi |
|  | 3dB beamwidth | 1.00 deg | 3.33 deg | 3.75 deg |
|  | Beam diameter | 140 km | 75 km | 40 km |
| UL | Equivalent satellite antenna aperture | 1.10 m | 0.33 m | 0.30 m |
|  | Max Rx Antenna Gain | 42.5 dBi | 32 dBi | 31 dBi |
|  | G/T | 15.5 dB/K | 5.0 dB/K | 4.0 dB/K |

Table 4: Satellite parameters in Ka-band

|  |  | Around 20/30GHz |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Ka-band GEO | Ka-band MEO 8000km | Ka-band LEO1200km | Ka-band LEO 600km | Ka-band LEO 300km |
| DL | Equivalent satellite antenna aperture | 6.0 m | 1.0 m | 0.35 m | 0.30 m | 0.20 m |
|  | Satellite EIRP density | 42 dBW/MHz | 32 dBW/MHz | 17 dBW/MHz | 11 dBW/MHz | 5.0 dBW/MHz |
|  | Max Tx Antenna Gain | 60 dBi | 43 dBi | 34 dBi | 32.5 dBi | 29 dBi |
|  | 3dB beamwidth | 0.16 deg | 0.95 deg | 2.75 deg | 3.20 deg | 4.75 deg |
|  | Beam diameter | 100 km | 130 km | 60 km | 40 km | 25 km |
| UL | Equivalent satellite antenna aperture | 4.00 m | 0.67 m | 0.23 m | 0.20 m | 0.13 m |
|  | Max Rx Antenna Gain | 60 dBi | 43 dBi | 34 dBi | 32.5 dBi | 29 dBi |
|  | G/T | 31 dB/K | 16 dB/K | 6.7 dB/K | 5.5 dB/K | 2 dB/K |

Table 5: Satellite parameters in Q/V-band [5]

|  |  | Around 37/47GHz |  |
| --- | --- | --- | --- |
|  |  | Q/V-band GEO | Q/V-band LEO 600km |
| DL | Equivalent satellite antenna aperture | 2.7 m | 0.27 m |
|  | Satellite EIRP density | 45 dBW/MHz | 15 dBW/MHz |
|  | Max Tx Antenna Gain | 60.4 dBi | 40.4 dBi |
|  | 3dB beamwidth | 0.18 deg | 1.88 deg |
|  | Beam diameter | 120 km | 20 km |
| UL | Equivalent satellite antenna aperture | 2.1 m | 0.21 m |
|  | Max Rx Antenna Gain | 60.4 dBi | 40.4 dBi |
|  | G/T | 31.8 dB/K | 11.8 dB/K |

Proposal 2: RAN4 to consider for NTN-based 6GR access the SAN characteristics reported in the R1-2509055 from Table 1 to Table 5.

# Satellite Antenna Model

In 5G analysis and studies, the reference satellite antenna model was reflector-based. In 6G, we would like to add the satellite antenna model based on phased-array antennas for NGSO. Mostly of the basic considerations and formulas were already introduced in [4].

Indeed, given the maximum Tx/Rx antenna gain and the equivalent aperture from the previous tables, it is straightforward to compute the necessary single element gain and the total number of elements. The proposed methodology for deriving these parameters of the phased-array antennas is shortly reported below:

- in FR1, the array spacing is exactly dx=dy=0.5λ, which ensures the absence of grating lobes and still a reasonable number of elements in the whole array;

- in FR2, the primary goal is to keep the number of elements not greater than 2000 and at the same time to guarantee all grating lobes off the visible Earth.

Table 6: Satellite phased-array parameters in FR1

|  | L-band | S-band | S-band | C-band |
| --- | --- | --- | --- | --- |
| Equivalent satellite antenna aperture | 2.5 m | 2.5 m | 1.5 m | 2.0 m |
| Array spacing | 0.5 | 0.5 | 0.5 | 0.5 |
| Total array elements | 676 | 1024 | 400 | 2116 |
| Single element gain | 1.14 dBi | 1.28 dBi | 0.92 dBi | 1.74 dBi |

Table 7: Satellite phased-array parameters in Ku-band

|  | Ku-band | Ku-band | Ku-band |
| --- | --- | --- | --- |
| Equivalent satellite antenna aperture | 1.5 m | 0.45 m | 0.40 m |
| Array spacing | 1.35 | 0.66 | 0.6 |
| Total array elements | 1936 | 676 | 676 |
| Single element gain | 9.64 dBi | 3.75 dBi | 2.73 dBi |

Table 8: Satellite phased-array parameters in Ka-band

|  | Ka-band | Ka-band | Ka-band |
| --- | --- | --- | --- |
| Equivalent satellite antenna aperture | 1.0 m | 0.35 m | 0.30 m |
| Array spacing | 1.35 | 0.66 | 0.6 |
| Total array elements | 2116 | 1024 | 900 |
| Single element gain | 9.72 dBi | 3.75 dBi | 2.98 dBi |

Table 9: Satellite phased-array parameters in Q/V-band [5]

|  | Q/V-band |
| --- | --- |
| Equivalent satellite antenna aperture | 0.27 m |
| Array spacing | 0.5 |
| Total array elements | 3481 |
| Single element gain | 5 dBi |

Proposal 3: RAN4 to consider new phased-array parameters for NTN antenna model reported in R1-2509055 from Table 6 to Table 9.

# Conclusion

In this contribution, we made the following observations and proposals:

Proposal 1: RAN4 to consider at least the following lists of bands for 6G NTN:

- L-band (i.e., 1.5GHz)

- S-band (i.e., 2 GHz)

- C-band (i.e., 4 GHz)

- Ku-band (FFS detailed frequency range)

- Ka-band (i.e., 30 GHz for UL, 20 GHz for DL)

- Q/V-band (i.e., 37 GHz for UL, 47 GHz for DL)

Proposal 2: RAN4 to consider for NTN-based 6GR access the SAN characteristics reported in the R1-2509055 from Table 1 to Table 5.

Proposal 3: RAN4 to consider new phased-array parameters for NTN antenna model reported in R1-2509055 from Table 6 to Table 9.

# References

- RP-251881, New SID: Study on 6G Radio, RAN#108

- RP-252912, Revised SID: Study on 6G Radio, RAN#109

- Chairman’s Notes, 3GPP RAN1 #122bis, Prague, CZ, August 2025

- R1-2403403, “On the SAN phased-array antenna characteristics”, ESA, April 2024

- Carla Amatetti, Juan Bucheli, Dorin Panaitopol, Sorya Tong, Xavier Leturc et al., “6G NTN in C and Q/V-Bands: Link Budget Analysis and Waveforms Performance”, IEEE Globecom 2024.