3GPP TSG RAN WG4 #118	R4-2602004

Gothenburg, Sweden, February 9-13, 2026

Agenda item:		7.8.4

Source:				Ofinno

Title:					RRM Requirements for Low Band CA via Switching Enhancement

Document for:		Discussion

# Introduction

In the last RAN4 meeting (RAN4#118) a way forward on RRM core requirements for Rel-20 WI on “Enhancement of low NR band carrier aggregation via switching” (NR_LBCA_Sw_enh-Core) was approved [1].

The approved WF contains a list of Rel-19 LBCA RRM core requirements which can be made applicable to corresponding Rel-20 LBCA RRM features. In addition, the approved WF also identified Rel-20 specific RRM core requirements for LBCA.

In this contribution we provide our views on the need for some additional RRM core requirements for LBCA enhancements in Rel-20.

# Analysis of RRM core requirements

During the early phase of Rel-19 WI on Low Band CA via Switching (NR_LBCA_Sw-Core), the need for defining RRM core requirements related to LTM cell switch was discussed [2][3]. However, there was no further analysis or discussion related to LTM cell switch RRM requirements under low and CA via switching. As of now, LTM cell switch RRM requirements under low and CA via switching are not supported in Rel-19 [4].

Therefore, Rel-19 UE configured with low band CA via switching will not be capable of meeting the RRM requirements defined for LTM cell switch delay in clause 6.3.1.2 of TS 38.133. This means the network can mainly rely on layer-3 handover for mobility purpose. The LTM is highly beneficial feature in terms of reducing handover delay and thereby enhances mobility performance. We therefore propose that LTM cell switch related requirements are supported for the UE configured with low and CA via switching in Rel-20. The impact on the LTM cell switch related requirements is analysed below:

- Observation 1: LTM cell switch related core RRM requirements under low and CA via switching are not supported in Rel-19.

- Observation 2: For a UE conigured with low band CA via switching, network has to mainly rely on layer-3 handover for mobility resulting in much longer handover delay compared to LTM cell switch delay.

## LTM cell switch delay requirements

Section 6.3.1.2 of TS 38.133 specifies LTM cell switch delay (DLTM) within which the UE is required to complete the LTM cell switch to the target cell, i.e.

DLTM = Tcmd + TLTM-interrupt

The UE may be configured with low band CA switching pattern while the UE is requested by the network (via MAC-CE command) to perform the LTM cell switch to the target cell.

After processing the MAC-CE command for LTM cell switch, the UE will have to deactivate the switching pattern (if configured) before initiating/performing necessary tasks for completing the LTM cell switch, i.e., during Tcmd. These necessary tasks (being part of the interruption time (TLTM-interrupt)) as described in section 6.3.1.3 comprise: LTM RRC processing time (TLTM-RRC-processing), LTM processing time for target cell parameters (TLTM-processing), fine time tracking of the target cell (Tfirst-RS), SSB processing time (TRS-proc), and LTM interruption uncertainty (TLTM-IU).

According to the RAN4 agreement, the UE processing delay (TLBCA) for deactivating the switching pattern in Rel-19 is 1 ms as captured in SCell deactivation delay requirements (e.g., clause 8.8.3 of TS 38.133).

Furthermore, in Rel-20 it has been agreed to use the same value of TLBCA as defined in Rel-19 as captured in RAN4 WF [1]:

Issue 1-2-2: Application time of switching pattern in SCell activation/deactivation

Tuesday online Agreement: - The application time of enabling/disabling switching pattern including TLBCA for the SCell which is in activation/deactivation procedure defined in Rel-19 can be reused, containing - MAC CE based SCell activation/deactivation - RRC based direct SCell activation - timer triggered SCell deactivation - For CQI/L1-RSRP reporting during SDL SCell activation in LB-CA requirement. - The issue can be closed (without any additional clarification or spec change in RAN4)

Therefore, the UE processing delay (TLBCA=1 ms) for deactivating the switching pattern should be included in the LTM cell switch delay (DLTM) and preferably within Tcmd.

- Observation 3: A UE configured with a switching pattern will have to deactivate the switching pattern (if configured) after processing the MAC-CE cell switch command and before initiaing the necessary tasks for completing the LTM cell switch.

- Proposal 1: LTM cell switch related RRM core requirements are supported for Rel-20 low band CA via switching.

- Proposal 2: Include the UE processing delay (TLBCA=1 ms) for deactivating the switchingPattern within Tcmd of the LTM cell switch delay (DLTM).

# Conclusion

In this contribution we have analyzed RRM requirements related to LTM cell switch in Rel-20 for the UE configured with low band CA switching pattern. Based on this analysis our observations and proposals are as follows:

- Observation 1: LTM cell switch related core RRM requirements under low band CA via switching are not supported in Rel-19.

- Observation 2: For a UE conigured with low band CA via switching, network has to mainly rely on layer-3 handover for mobility resulting in much longer handover delay compared to LTM cell switch delay.

- Observation 3: A UE configured with a switching pattern will have to deactivate the switching pattern (if configured) after processing the MAC-CE cell switch command and before initiaing the necessary tasks for completing the LTM cell switch.

- Proposal 1: LTM cell switch related RRM core requirements are supported for Rel-20 low band CA via switching.

- Proposal 2: Include the UE processing delay (TLBCA=1 ms) for deactivating the switchingPattern within Tcmd of the LTM cell switch delay (DLTM).

# References

- R4-2522653, “WF on NR_LBCA_Sw_enh_RRM”, Samsung

- R4-2500527, “Topic summary for [114][213] NR_LBCA_Sw”, Moderator (Apple)

- R4-2502663, “Ad-hoc minutes for [114][213] NR_LBCA_Sw”, Apple

- R4-2522630, “WF on NR_LBCA_Sw”, Moderator (Apple).