3GPP TSG-RAN4 Meeting #116bis	        R4-251xxxx  Prague, Czech Republic, Oct. 13-17, 2025

Title:	LS to RAN5 on LP-WUS/WUR Testability

Response to:

Release:	Release 19

Work Item:	NR_LPWUS-Core

Source:	RAN4

To:	RAN5

Cc:

Contact Person:

#### Name:	Ruixin Wang

#### E-mail Address:	ruixin.wang@vivo.com

#### Name:	Ke Liu

#### E-mail Address:	liu.ke10@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, 3GPPLiaison@etsi.org

Attachments:	None

- Overall Description:

RAN4 has concluded the RF core requirements for Rel-19 LP-WUS/WUR WI in RAN4#116 meeting, and the core part WI is formally closed in RAN#109 meeting.

According to the WID objective, RAN4 also discussed the testability issue of LP-WUS/WUR and reached the following agreements:

- Verify LP-WUR based on 1% MDR of LP-WUS which can be tested based on UE’s response to the NW/TE upon successfully detecting the LP-WUS (e.g. ACK/NACK in CONNECTED state or MSG1/3 in IDLE state or any other methods). RAN4 has assumed a FAR of 1% while discussing the requirements.

  - The test method including any potential test mode can be decided by RAN5

- The following settings can be used for Rx requirements related testing:

  - LP-WUS, LPSS and SSB all share the same EPRE

  - LPSS shares the same chip rate M and same overlaid sequence length as LP-WUS

  - LPSS shares the same length in OFDM symbols as LP-WUS

  - LPSS is transmitted at lowest periodicity (160 ms), or equivalently at highest rate of repetition

  - SSB is transmitted at 20 ms periodicity

- For the case when LR supporting both OOK and OFDM waveforms, only test with OFDM waveform

- LP-WUS RF requirements apply to all three RRC states, but the verification may be limited to only one RRC state

  - For idle-mode only or connected-mode only LP-WUR, the test can be done accordingly.

  - For UE that supports both idle-mode and connected-mode it is up to RAN5 which mode is chosen for verification.

- To verify the LR performance, it is not necessary to test the LR for all possible MR configurations. RAN5 can choose the MR configuration for which LR is tested.

- Testing time reduction of LP-WUS RF requirements can be considered when define the test cases

RAN4 would like to ask RAN5 to take above testability agreements into account when developing conformance test specifications.

2. Actions:

To RAN5.

ACTION: 3GPP RAN4 respectfully asks RAN5 to take the above information into account when developing LP-WUS/WUR RF conformance test specifications.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN4 Meeting#117		17th – 21st November 2025		Dallas, USA

TSG-RAN WG4 Meeting #118	9th – 13th February 2026	       Gothenburg, SE