3GPP TSG-RAN WG4 Meeting #116bis	R4-25xxxxxx

Prague, Czech Republic, 13rd – 17st Oct, 2025

Title: 	           WF on 6MHz

Agenda Item:	7.1.1

Source: 	Samsung, Nokia, Ericsson, Skyworks, Apple, [Huawei, CATT, ZTE, Qualcomm]

Document for:	Approval

1. System parameters

1.1 Spectrum utilization

Agreement:

- For 6MHz (SCS=15KHz), the maximum transmission bandwidth configuration (NRB) is 30RB and guard band is 292.5KHz.

1.2 Channel spacing, channel raster and sync raster

Agreement:

- Channel spacing, channel raster and sync raster are not affected by the introduction of 6 MHz channel bandwidth.

1.3 Optional CBW

Agreement:

- Support of 6 MHz is optional in Rel-20 and in the previous releases. Update TS38.101-1 Table 5.3.5-1 as shown below.

- For single CC operation, 6 MHz is release independent from Rel-15.

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5 | 6 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| n5 | 15 | 34 | 5 | 64 | 10 | 15 | 20 | 253 |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 10 | 15 | 20 | 253 |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n8 | 15 |  | 5 | 64 | 10 | 15 | 20 | 253 | 303 | 353 |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 10 | 15 | 20 | 253 | 303 | 353 |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n12 | 15 | 34 | 5 | 64 | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n29 | 15 |  | 5 | 64 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n85 | 15 | 34 | 5 | 64 | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

1.4 Minimum CBW

Agreement:

- 6 MHz can not be the minimum channel BW in a NR band.

2. UE RF

  - SEM

Agreement:

- SEM requirement for 6MHz channel BW is the same with 7 and 10 MHz channel BW.

  - MPR

Agreement:

- The MPR requirements are not impacted by the introduction of 6MHz.

  - NS_01 and NS_100

Agreement:

- NS_01: 6 MHz shall be added to the list of considered channel BW for the NS.

- NS_100: No update needed.

2.4 NS_06, NS_14, NS_15, NS_43/43U

Agreement:

- Conduct MPR evaluation (if needed) for

- NS_06 for n12 (PC3)

- For information: Current spec states “For power class 3 operation on bands n12, n13, n14, n85 and n110, no A-MPR is applicable.

- NS_06 for n85 (PC3 and PC2)

- For information: Current spec states “For power class 3 operation on bands n12, n13, n14, n85 and n110, no A-MPR is applicable.

- NS_14 for n5 (PC3 and PC2)

- For information: A-MPR is not defined for 7MHz for n5 PC3 and PC2

- NS_15 for n5 (PC3 and PC2)

- [NS_43/43U for n8: A-MPR evaluation not needed as only 5, 10 or 15 MHz channel bandwidths are expected in Japan] → Check with SB

- Additional requirements for NS_06 for 6MHz is defined as

| ΔfOOB (MHz) |  | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- | --- |
|  | 3 | 5 | 6 | 10 | 15 |  |
| ± 0 – 0.1 | -13 | -15 | -15 | -18 | -20 | 30 kHz |
| ± 0.1 – 1 | -13 | -13 | -13 | -13 | -13 | 100 kHz |
| ± 1 – 5 | -13 | -13 | -13 | -13 | -13 | 1 MHz |
| ± 5 – 6 | -25 | -13 | -13 | -13 | -13 | 1 MHz |
| ± 6 – 10 |  | -25 | -25 |  |  |  |
| ± 10 – 11 |  |  |  | -25 |  |  |
| ± 11 – 15 |  |  |  |  |  |  |
| ± 15 – 20 |  |  |  |  | -25 |  |

- FFS whether 6 MHz channel bandwidth needs to be added in the Additional requirements for “NS_15” table

- [6 MHz channel bandwidth shall be updated in the Additional requirements for “NS_43” and “NS_43U” table.]

2.5 Minimum Output power, Transmit Off power, Occupied bandwidth, ACLR, general spurious

Agreement:

- The requirement for 6 MHz is the same requirement with the requirement of 5/7/10MHz. 6 MHz shall be added to:

- Table 6.3.1-1 (minimum output power).

- Table 6.3.2-1(Transmit Off power).

- Table 6.5.1-1 (occupied bandwidth).

- Table 6.5.2.4.1-1 (ACLR).

- Table 6.5.3.1-1 (general spurious).

- Other Tx requirements are not impacted by the introduction of 6 MHz channel BW.

2.6 REFSENS and UL configuration for FDD bands

Agreement:

| Operating Band | SCS kHz | 3 MHz (dBm) | 5 MHz (dBm) | 6 MHz (dBm) | 7 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n5 | 15 | -100.2 | -98.0 | -97.2 | -96.5 | -94.8 | -93.0 | -86.8 | -84.8 |  |  |
|  | 30 |  |  |  |  | -95.1 | -93.1 | -88.6 | -84.9 |  |  |
| n8 | 15 |  | -97.0 | -96.2 |  | -93.8 | -91.4 | -85.8 | -83.6 | -81.3 | -78.4 |
|  | 30 |  |  |  |  | -94.1 | -91.7 | -87.2 | -84.7 | -81.4 | -78.5 |
| n12 | 15 | -99.2 | -97.0 | -96.2 |  | -93.8 | -84.0 |  |  |  |  |
|  | 30 |  |  |  |  | -94.1 | -84.1 |  |  |  |  |
| n85 | 15 | -99.2 | -97.0 | -96.2 |  | -93.8 | -84.0 |  |  |  |  |
|  | 30 |  |  |  |  | -94.1 | -84.1 |  |  |  |  |

| Operating Band | SCS | 3 | 5 | 6 | 7 | 10 | 15 | 20 | 25 | 30 | 35 | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n5 | 15 | 15 | 25 | 251 | 251 | 251 | 201 | 201 | Note 5 |  |  | FDD |
|  | 30 |  |  |  |  | 121 | 101 | 101 | Note 5 |  |  |  |
| n8 | 15 |  | 25 | 251 |  | 251 | 201 | 201 | Note 5 | Note 5 | Note 5 | FDD |
|  | 30 |  |  |  |  | 121 | 101 | 101 | Note 5 | Note 5 | Note 5 |  |
| n12 | 15 | 15 | 201 | 201 |  | 201 | 201 |  |  |  |  | FDD |
|  | 30 |  |  |  |  | 101 | 101 |  |  |  |  |  |
| n85 | 15 | 15 | 201 | 201 |  | 201 | 201 |  |  |  |  | FDD |
|  | 30 |  |  |  |  | 101 | 101 |  |  |  |  |  |

2.7 REFSENS for SDL band n29

Agreement:

- Add 6MHz for n29 in Table 7.3.2-1b

2.8 Maximum input level

Agreement:

- The requirement for 6 MHz is the same requirement than the requirement for 3-20 MHz. 6 MHz shall be added to Table7.4-1.

2.9 ACS, In-band blocking, Out of band blocking, Narrow Band blocking, Intermodulation, Spurious response

Agreement:

- The requirement for 6 MHz is the same requirement with the requirement for 5,7 and10 MHz. 6 MHz shall be added to:

- Tables 7.5-1, 7.5-3 and 7.5-4 (ACS).

- Table 7.6.2-1 (In-band blocking).

- Table 7.6.3-1, and power in transmission bandwidth configuration as REFSENS + 6.0 (Out of band blocking).

- Table 7.6.4-1, and Pw =15 (Narrow band blocking).

- Table 7.7-1(Spurious response).

- Table 7.8.2-1(Intermodulation).

- Other Rx requirements are not impacted.

3. General for BS

3.1 NB-IoT support

Agreement:

- NB-IoT operating in NR in-band is supported for 6 MHz channel bandwidth.

3.2 Multi-carrier and CA

Agreement:

- Muti-carrier (including MSR) and CA related requirements are considered for new added 7MHz WID but no new BCS is specified within this WI.

3.3 IAB and NCR

Agreement:

- 6 MHz should not be supported by IAB and NCR

4. BS RF

4.1 ACLR

Agreement:

- Add 6 MHz to the list of NR channel bandwidths for ACLR/CACLR.

4.2 Total power dynamic range

Agreement:

- The total power dynamic range is 14.7dB for core requirement

4.3 EVM

Agreement:

- Annex B.5.3 should be updated to include EVM window length for 6 MHz channel BW, which is same with 5 and 7MHz channel BW.

4.4 Occupied bandwidth test requirement

Agreement:

- For the Occupied bandwidth test requirement, define Span as 12 MHz, with Minimum number of measurement points as 400.

4.5 RFC for Rx dynamic range

Agreement:

- For RF Rx dynamic range requirement, reuse FRC G-FR1-A2-1

4.6 REFSENS

Agreement:

- REFSENS for 6 MHz channel BW is same with 5, 7, 10, 15MHz, assuming FRC G-FR1-A1-1

4.7 Dynamic range (conducted core)

Agreement:

- Dynamic Range limit and Interfering signal level for 6 MHz channel BW.

- Wanted signal is:

- WA BS: -70.7 dB

- MR BS: -65.7 dB

- LA BS: -62.7 dB

- Interfering signal is:

- WA BS: -81.7 dB (Huawei, Nokia, ZTE) or -81.6 dB (Ericsson, CATT)

- MR BS: -76.7 dB (Huawei, ZTE) or -76.6 dB (Ericsson, CATT)

- LA BS: -73.7 dB (Huawei, ZTE) or -73.6 dB (Ericsson, CATT)

4.8 Dynamic range for NB-IoT operation in NR in-band

Agreement:

- Add 6MHz CBW to Table 6.2.4.2-1, reuse legacy NB-IoT RB power dynamic range for 3, 5, 7, 10MHz CBW.

4.9 Other impacted Rx requirements

Agreement:

- The following Rx requirements should also be updated to include 6 MHz channel BW. FFS the listed options for ACS, Narrowband blocking, Receiver intermodulation and ICS in future meetings.

- ACS. Define the value of Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap as:

- Option 1: ±2.5125 MHz (Ericsson, ZTE, Huawei)

- Option 2: ±2.5050 MHz (Nokia)

- General blocking. Same requirements with 5, 7, 10 MHz channel BW.

- Narrowband blocking.

- Option 1: ±(355+m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 (Nokia)

- Option 2: ±(360+m*180) kHz, with m=0, 1, 2, 3, 4, 9, 14, 19, 24. (Ericsson, ZTE)

- Receiver intermodulation.

Table: Interfering signals for intermodulation requirement

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge (MHz) | Type of interfering signal (Note 3) |
| --- | --- | --- |
| 3 | ±4.5 | CW |
|  | ±10.5 | 3 MHz DFT-s-OFDM NR signal |
| 5 | ±7.5 | CW |
|  | ±17.5 | 5 MHz DFT-s-OFDM NR signal (Note 1) |
| 6 | ±7.475 (Nokia, ZTE); ±7.52 (Ericsson); | CW |
|  | ±17.5 | 5 MHz DFT-s-OFDM NR signal (Note 1) |

Table: Interfering signals for narrowband intermodulation requirement in FR1

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering RB centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (kHz) (Note 3) | Type of interfering signal |
| --- | --- | --- |
| 3 | ±360 | CW |
|  | ±960 | 3 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 5 | ±360 | CW |
|  | ±1420 | 5 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 6 | ±425 (Nokia, ZTE); ±630 (Ericsson); | CW |
|  | ±1240 (Nokia, ZTE); ±1560(Ericsson); | 5 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |

- ICS.

Table: Wide Area BS in-channel selectivity

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 3 | 15 | G-FR1-A1-20 | -102.8 | -83.6 | DFT-s-OFDM NR signal, 15 kHz SCS, 6 RBs |
| 5 | 15 | G-FR1-A1-7 | -100.6 | -81.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs |
| 6 | 15 | G-FR1-A1-7 | -100.6 | Option1: -79.6(Nokia, ZTE, Huawei); Option 2:-80.6(Huawei); Option3: -81.4 (Ericsson,Huawei); | DFT-s-OFDM NR signal, 15 kHz SCS, Option1: 15 RBs(Nokia, ZTE, Huawei); Option 2: 12RBs (Huawei) Option3: 10RBs (Ericsson, Huawei); |

5. Annex: Information for CR splitting

| Technical specification | Companies |
| --- | --- |
| TS 38.104 | Nokia |
| TS 38.141-1 | Ericsson |
| TS 38.141-2 | [Huawei] |
| TS 38.101-1 | [Apple, Qualcomm, T-mobile USA, Skyworks, Huawei] Note: big CR is led by Apple |
| TS 38.101-4 | [Qualcomm] |
| TS 38.133 | [ZTE] |
| TS 38.307 | Nokia |
| TS 37.104 | [Nokia] |
| TS 37.141 | [ZTE] |
| TS 38.106 (*) | Ericsson |
| TS 38.115-1 (*) | [CATT] |
| TS 38.115-2 (*) | [CATT] |
| TS 38.174 (*) | Nokia |
| TS 38.176-1 (*) | Nokia |
| TS 38.176-2 (*) | [Huawei] |
| TS 38.113 | [ZTE] |
| TS 37.113 | Ericsson |
| TS 37.105 | [Huawei] |
| TS 37.145-1 | [Huawei] |
| TS 37.145-2 | [Huawei] |
| (*) The NCR and IAB specs would need to be updated to clarify 6 MHz channel BW is not supported. |  |