3GPP TSG-RAN WG4 Meeting #118R                                                              R4-2601783

Gothenburg, Sweden, 9 – 13 February 2026

Source:	Ericsson, Verizon

Title:	TP for 38.720-03-01 to add CA_n5-n13-n66

Agenda item:			5.6.4

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for 38.720-03-01 to add CA_n5-n13-n66 as defined in WID [1].

# 2. Text Proposal

# ---Start of changes---

## 5.x	CA_n5-n13-n66

### 5.x.1	Common for 1 band UL and 2 bands UL CA

#### 5.x.1.1	Operating bands for CA

Table 5.x.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n5 | 824 MHz – 849 MHz | 869 MHz – 894 MHz | FDD |
| n13 | 777 MHz – 787 MHz | 746 MHz – 756 MHz | FDD |
| n66 | 1710 MHz – 1780 MHz | 2110 MHz – 2200 MHz | FDD |

#### 5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination

| NR CA configuration | Uplink CA configuration or single uplink carrier6 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n5A-n13A-n66A | CA_n5A-n66A CA_n13A-n66A | n5 | n5 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n13 | n13 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n5B-n13A-n66A | CA_n5B CA_n5A-n66A CA_n13A-n66A | n5 | CA_n5B_BCS 4 and 5 | 4 and 5 |
|  |  | n13 | n13 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n5A-n13A-n66(2A) | CA_n5A-n66A CA_n13A-n66A | n5 | n5 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n13 | n13 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |

#### 5.x.1.3	∆TIB,c and ∆RIB,c values

For CA_n5-n13-n66, the TIB,c and RIB,c values are given in the tables below.

Table 5.x.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n5-n13-n66 | 0.3 | 0.3 | 0.3 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.x.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n5-n13-n66 | - | - | - |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

### 5.x.2	Specific for 2 bands UL CA

#### 5.x.2.1	UE co-existence studies

##### 5.x.2.1.1	Co-existence studies for 2UL band with 1CC per band

.

Table 5.x.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n5A-n13A-n66A with UL CA_n5A-n66A.

Table 5.x.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | 824 | 849 | 1710 | 1780 |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 861 – 956 |  | 2534 – 2629 |  |
| Two-tone 3rd order IMD products | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 12 – 132 |  | 2571 – 2736 |  |
| Two-tone 3rd order IMD products | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3358 – 3478 |  | 4244 – 4409 |  |
| Two-tone 4th order IMD products | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 692 – 837 |  | 4281 – 4516 |  |
| Two-tone 4th order IMD products | ¦2*fx_high –2* fy_low¦ |  | ¦2*fx_high –2* fy_low¦ |  |
| IMD frequency limits (MHz) | 1722 – 1912 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4182 – 4327 |  | 5954 – 6189 |  |
| Two-tone 4th order IMD products | ¦2*fx_high +2* fy_high¦ |  | ¦2*fx_high +2* fy_high¦ |  |
| IMD frequency limits (MHz) | 5068 – 5258 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5991 – 6296 |  | 1516 – 1686 |  |
| Two-tone 5th order IMD products | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3432 – 3692 |  | 873 – 1088 |  |
| Two-tone 5th order IMD products | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7664 – 7969 |  | 5006 – 5176 |  |
| Two-tone 5th order IMD products | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6778 – 7038 |  | 5892 – 6107 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above analysis, 4th order IMD generated by UL CA_n5A-n66A may fall into the Rx of band n13.

Table 5.x.2.1.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n5A-n13A-n77A with UL CA_n13A-n66A.

Table 5.x.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | 777 | 787 | 1710 | 1780 |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 923 – 1003 |  | 2587 – 2567 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 136 – 226 |  | 2633 – 2783 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3264 – 3354 |  | 4197 – 4347 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 551 – 651 |  | 4343 – 4563 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1846 – 2006 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4041 – 4141 |  | 5907 – 6127 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 4974 – 5134 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6053 – 6343 |  | 1328 – 1438 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3556 – 3786 |  | 1059 – 1229 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7617 – 7907 |  | 4818 – 4928 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6684 – 6914 |  | 5751 – 5921 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above analysis, no IMD generated by UL CA_n13A-n66A would fall into the Rx of band n5.

##### 5.x.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)

#### 5.x.2.2	REFSENS requirements

Based on the co-existence studies, the MSD value for CA_n5-n29-n66 is reused and is defined in Table 5.x4.2.2-1:

Table 5.x.2.2-1: MSD for the CA configuration

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n13-n66 | n5 | 827 | 5 | 25 | 872 | N/A | FDD | N/A |
|  | n13 | N/A | 5 | N/A | 751 | 9.4 | FDD | IMD4 |
|  | n66 | 1730 | 5 | 25 | 2130 | N/A | FDD | N/A |

x

---End of changes---

# Reference

[1] 		RP-253849, Rel-20 NR Carrier Aggregation (CA)/Dual Connectivity (DC) for x bands DL with y bands UL (x<7, y<3) and Supplementary Uplink (SUL) band combinations/CA band combinations with a single SUL or two SUL cells; Ericsson, ZTE, Huawei