3GPP TSG-RAN WG4 Meeting #116	R4-2509746

# Bangalore, India, 25th – 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 5784 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_redcap-Perf) CR to TS 38.133: Corrections to RedCap RRM test cases (Rel 19) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap-Perf |  |  |  |  |  | Date: |  |  | 2025-08-29 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Rel-19 mirror CR of the Rel-17 CR agreed in R4-2512323 According to Table A.3.9.2.1-1 and Table A.3.9.3.1-1, the BWP BW shall be same as the RMSI CORESET (CORESET#0) definied in the test. Implicitely it is also expected that the SSBs – which the UE needs to read – are included in that BW as well. However for certain bands (e.g. n70) there is an additional offset (kSSB) for the starting ressource of SSB, which pushes the SSB-end beyond the BW of the CORESET#0 (though just some subcarriers). Since the reading of the SSB is required, the BW of the BWP needs to be adopted accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In Table A.3.9.2.1-1 and Table A.3.9.3.1-1 added the following note: Note 2: In case CORESET#0 and SSBs defined in the test do not completely fit into the bandwidth of the reference BWP bandwidth, the latter one is extended to the next smallest size which can fully accommodate the respective CORESET#0 and SSBs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE will not be able to read the SSB for some bands and fail the test unfairly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Table A.3.9.2.1-1, Table A.3.9.3.1-1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

### A.3.9.2	Downlink BWP configurations

#### A.3.9.2.1	Initial BWP

Table A.3.9.2.1-1: Downlink BWP patterns for initial BWP configuration

| BWP Parameters | Unit | Values |  |  |
| --- | --- | --- | --- | --- |
| Reference BWP |  | DLBWP.0.1 | DLBWP.0.2 |  |
| Starting PRB index |  | 0 | RBc Note 1 |  |
| Bandwidth | PRB | Same as RF channel defined in each test | same as RMSI CORESET (CORESET #0) defined in each test Note 2 |  |
| NOTE 1: RBc is the lowest PRB index to guarantee the BWP including CORESET #0 which is defined in clause A.3.1.2. Note 2: In case CORESET#0 and SSBs defined in the test do not completely fit into the bandwidth of the reference BWP bandwidth, the latter one is extended to the next smallest size which can fully accommodate the respective CORESET#0 and SSBs.. |  |  |  |  |

# < Unchanged sections omitted >

### A.3.9.3	Uplink BWP configurations

#### A.3.9.3.1	Initial BWP

Table A.3.9.3.1-1: Uplink BWP patterns for initial BWP configuration

| BWP Parameters | Unit | Values |  |  |
| --- | --- | --- | --- | --- |
| Reference BWP |  | ULBWP.0.1 | ULBWP.0.2 |  |
| Starting PRB index |  | 0 | RBc Note 1 |  |
| Bandwidth | PRB | Same as RF channel defined in each test | same as RMSI CORESET (CORESET #0) defined in each test Note 2 |  |
| NOTE 1: RBc is same as PRBc for DLBWP.0.2 as defined in table A.3.9.2.1-1. Note 2: In case CORESET#0 and SSBs defined in the test do not completely fit into the bandwidth of the reference BWP bandwidth, the latter one is extended to the next smallest size which can fully accommodate the respective CORESET#0 and SSBs. |  |  |  |  |

< End of changes >