3GPP TSG RAN WG4 Meeting #116		R4-2511851

Bengaluru, India, August 25th – 29th , 2025

Title: 	TP to TR 38.793 to introduce UL CA_n5-n8

Agenda Item:	6.11.2

Source: 	CATT

Document for:	Approval

1 Introduction

This contribution provides text proposal on the NR band combination UL CA_n5-n8.

2 Text Proposal

--- < Start of change> ---

5.x	CA_n5-n8

5.x.1	Common for 1 band UL and 2 bands UL CA

5.x.1.1	Operating bands for CA

Table 5.x.1.1-1:  CA band combination of band nX+nY

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n5 | 824 MHz – 849 MHz | 869 MHz – 894 MHz | FDD |
| n8 | 880 MHz – 915 MHz | 925 MHz – 960 MHz | FDD |

5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination of band nX+nY

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n5A-n8A15 | CA_n5A-n8A | n5 | 5, 10 | 0 |
|  |  | n8 | 5, 10 |  |

NOTE 15: For CA_n5-n8, two uplinks are supported for UEs [that support IE supportOfnon-ConcurrentInterbandULDL-r19 capability] with non-concurrent n5 downlink and n8 uplink, otherwise uplink is only in n5.

5.x.2	Specific for 2 bands UL CA

5.x.2.1	Maximum output power for inter-band CA

Table 5.x.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Power Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n5A-n8A | 23 | +2/-3 |

5.x.2.2	UE co-existence studies for 2 bands UL

Table 5.x.2.2-1 lists Band n5 + Band n8 2 bands UL CA(2CC) 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.x.2.2-1: Band n5 and Band n8 for 2CC UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 91 | 69 | 1728 | 1750 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 973 | 1006 | 733 | 766 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2632 | 2665 | 2552 | 2585 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1877 | 1921 | 1557 | 1601 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 138 | 182 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3536 | 3580 | 3376 | 3420 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 3456 | 3500 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2436 | 2381 | 2836 | 2781 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 697 | 642 | 1097 | 1042 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4200 | 4255 | 4440 | 4495 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 4280 | 4335 | 4360 | 4415 |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on Table 5.x.2.2-1, there is no IMD issue for CA_n5-n8 with the following frequency range restriction.

n5	UL: 824 MHz – 835 MHz	DL: 869 MHz – 880 MHz;

n8	UL: 904 MHz – 915 MHz	DL: 949 MHz – 960 MHz.

However, IMD3 may fall into the DL band n8 if both band n5 and n8 transmit the signal without frequency restriction.

For CA_n5-n8, although PHS frequency range 1884.5-1915.7MHz is as protected band of band n5, due to this combination will not be deployed in the PHS region, there is no need to add PHS frequency range as protected band for CA_n5-n8. Thus no protected bands are needed.

5.x.2.3	REFSENS requirements

Referring to the IMD3 MSD value from CA_n8-n20, 25dB MSD can be specified for UL CA_n5-n8 without frequency range restriction by implementing non-concurrent n5 DL and n8 UL solution.

Table 5.x.2.3-1: MSD test parameters due to the two UL IMD3

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n81 | n5 | 837.5 | 5 | 25 | 882.5 | N/A | FDD | N/A |
|  | n8 | 882.5 | 5 | 25 | 927.5 | 25 | FDD | IMD3 |
| NOTE 1: This test point is applicable to UEs [that support IE supportOfnon-ConcurrentInterbandULDL-r19 capability] with non-concurrent n5 downlink and n8 uplink.. |  |  |  |  |  |  |  |  |

5.x.3	The solution with non-concurrent n5 DL and n8 UL in Rel-19

5.x.3.1	RF implementation assumption:

Generally, the RF implementation could be achieved for different RRC configuration as below.

|  | RRC configuration | RFFE configuration |
| --- | --- | --- |
| Case 1 | 2UL/2DL | - #A: n5-UL: Tx, n5-DL:Rx, n8-DL:Rx - #B: n5-UL:Tx, n8-UL:Tx, n8-DL:Rx |
| Case 2 | 1UL/2DL (like SDL) | - #A: n5-UL: Tx, n5-DL:Rx, n8-DL:Rx |

Based on companies’ contributions, there are two assumed RF implementations for 2UL/2DL RRC configuration at least.

RF Implementation 1: the RFFE configuration can be switched between RFFE#A and RFFE#B under RRC configuration 2UL/2DL.

RF Implementation 2: the RFFE configuration can be configured as RFFE#B under RRC configuration 2UL/2DL.

5.x.3.2	The investigation about the impacts from other working groups

Investigation on RAN2’s spec:

According to the RAN2’s LS R2-2501572, CA_n5-n8 with non-concurrent n5 downlink and n8 uplink (option 2 in TR 38.872) is feasible without RAN2’s spec impacts beyond the introduction of UE capability.

Investigation on RAN1’s spec:

According to the paper R4-2510468, some companies declare that network products and chipsets after Rel-18, can support single-DCI feature. The feature is to enable single DCI scheduling multiple PDSCHs or multiple PUSCHs over multi-carriers, specially for this case to enable UL CA_n5-n8 by single DCI in DL band n8.

According to the paper R4-2511054, assuming n5 and n8 are collocated, power control does not seem to be an issue as n5 UL can use the n8 DL for pathloss reference in power control through the configuration of pathlossReferenceLinking.

It’s observed that there is no RAN1/RAN2’s spec impacts beyond the introduction of UE capability for the proposed solution with non-concurrent n5 DL and n8 UL for both RF implementation 1 and RF implementation 2.

Investigation on RRM’s spec:

For RF implementation 1:

Further discuss whether there is an RRM impact based on an example illustrated as follows:

For RF implementation 2:

It may have some RRM spec impacts.

--- < End of change> ---

Reference

[1] RP‑251762, “TR 38.768 v1.0.0 on Low NR band carrier aggregation via switching”, Apple

[2] RP-251315, “Revised WID on Low NR band carrier aggregation via switching”, TELUS

[3] R1- 2505061, “Introduction of low NR band carrier aggregation via switching for NR”, Samsung

[4] R4-2508118, “WF on UE RF requirements for LB CA via switching”, Apple