3GPP TSG-RAN WG4 Meeting #116	             R4-2512146

Bengaluru, India, Aug 25th – 29th, 2025

Agenda Item:	7.27.1

Source: 	Moderator (CATT)

Title: 	WF on RRM requirements for NR_NTN_Ph3_Part1

Document for:	Approval

0 Notes:

In this document,

- < Agreement > represents the decisions made by in this meeting

- < Way forward > represents the next step in later meetings

  - “FFS” does not mean RAN4 will make a down-selection for the item. More other options can be proposed.

- < Tentative Agreement > will be changed to < Agreement > if no comments are received.

Topic #1: RRM core requirements

    - Sub-topic 1-1 What RRM requirements are defined for (e)RedCap UE with FR1-NTN

Issue 1-1-1: MDT requirements

Online agreement

Agreement:

- RAN4 to define the MDT requirements in RRC_IDLE and RRC_INACTIVE state for (e)Redcap over NTN. The legacy requirements for NTN in clauses 4.3C and 5.3C can be reused.

  - Reuse legacy NTN MDT requirements for Redcap UE over NTN, including measurement requirements and relative time stamp requirements for 1Rx and 2Rx RedCap UEs in NTN.

    - Sub-topic 1-2 SMTCs and 'in parallel' related features

Issue 1-2-1: SMTC adjustment in idle/inactive

Agreement:

- RAN4 to maintain the same principle used in NR NTN for RedCap devices and there is no additional RRM spec impact.

    - Sub-topic 1-3 The impact of HD-FDD

Issue 1-3-1: The impact of HD-FDD on scheduling availability of UE related to CSI-RS and PRS Background

Online agreement

Agreement:

- When UE indicates CollisionHandlingOfHDFDDOperation, RAN4 to clarify that measurement requirements for CSI-RS, scheduling restriction of PDSCH, and PRACH apply provided that no collision between CSI-RS/PDSCH and PRACH

Issue 1-3-2: The impact on NR Handover due to HD-FDD

Online agreement

Agreement:

- No spec impact

    - Sub-topic 1-4 UE capability

Issue 1-4-1: Whether and how to design the UE capabilities for (e)RedCap UE with NTN

Online agreement

Agreement:

- RAN4 to introduce new capability to indicate the support over RedCap over NTN.

| Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Support of (e)RedCap UE with FR1-NTN | Indicates whether the UE supports (e)RedCap UE with FR1-NTN. | Layer-1 FG: 28-1, 48-1 Layer-2 and Layer-3 FG: 34-1 | Y | N/A | The requirements defined for (e)RedCap UE with FR1-NTN in Rel-19 may not apply. | Per UE | No | FR1 only | N/A | If the UE indicates this capability, the UE shall support (e)RedCap UE with FR1-NTN. | Optional with capability signalling |

Topic #2: RRM perf requirements

    - Sub-topic 2-1: (e)RedCap UE with FR1-NTN

Issue 2-1-1: Measurement accuracy requirement

- Background

The agreement in the previous meeting:

RAN4#114-bis: #### Issue 2-2-1: L1/L3 measurement accuracy for 1Rx/2Rx (e)RedCap UEs with FR1-NTN Agreement: - For 2Rx (e)RedCap UEs with FR1-NTN bands, RAN4 to reuse the measurement accuracy requirements for normal UEs defined in NTN. - For 1Rx (e)RedCap UEs with FR1-NTN bands, RAN4 to relax the L1/L3 measurement accuracy for SS-RSRP, SS-RSRQ, SS-SINR and L1-RSRP compared to those defined for 2Rx (e)RedCap UEs. - The same relaxation defined in TN for 1Rx (e)RedCap UEs can be reused. - SS-RSRP, SS-RSRQ, SS-SINR: relax by 1dB. - L1-RSRP: relax by 3dB. RAN4#114: #### Issue 2-3-2: Network verified UE location for 1Rx/2Rx (e)RedCap UEs with FR1-NTN Agreement: - For 2Rx (e)RedCap UEs with FR1-NTN bands, the measurement period and measurement accuracy requirements for Network verified UE location defined in NTN can be reused. - For 1Rx (e)RedCap UEs with FR1-NTN bands, the measurement accuracy requirements defined for 1Rx RedCap positioning can be referred, and the same relaxation (as the measurement accuracy requirements defined for 1Rx TN-RedCap ) for accuracy should be reused.

Agreement:

- FFS: RAN4 to define accuracy requirements for following measurements for RedCap in NTN.

  - Intra- and inter-frequency RSRP

  - Intra- and inter-frequency RSRQ

  - Intra- and inter-frequency SINR

  - L1-RSRP

  - Rx-Tx for NW verified location

- For L1/L3 measurement accuracy, RAN4 to follow the agreements reached in RAN4 #114-bis meeting.

- For measurement accuracy for NW verified location, RAN4 to follow the agreements reached in RAN4 #114 meeting.

- RAN4 to define new sections for measurement accuracy requirements with suffix ‘10.X’ for (e)RedCap UE with NTN.

- Further to discuss the side conditions.

Issue 2-1-2: Test principle and setups

<Way Forward>:

- Proposal 1: For test principle and setups, the testing related to satellite access and RedCap defined in A.3.35 and A.3.36 can be a reference.

- Proposal 2: Take the NTN test setups as the reference, except both FDD and HD-FDD shall be involved in test configuration.

Issue 2-1-3: How and where to capture the test cases for (e)RedCap UE with FR1-NTN

<Way Forward>:

- Proposal 1: The existing test cases defined for FR1 NTN in A.14 can be reused as baseline for RedCap UEs with FR1-NTN.

  - Proposal 1a: RAN4 will further adapt the test cases for RedCap UE, e.g. test configuration, 1Rx/2Rx and so on.

  - Proposal 1b: Adapt the test case for reduced BW, e.g. test configuration, some RMC and Io values

- Proposal 2: RAN4 to use the ‘refer to’ method when drafting the performance spec to avoid duplicated contents, as in core requirements.

  - Proposal 2a: RAN4 only clarify the procedure and parameters which can be reused from NTN, and then define new parameters applicable to RedCap UEs.

- Proposal 3:

  - RAN4 to define new sections for test cases with suffix ‘A.X’.

- Proposal 4: For RedCap UEs in NTN scenario,

  - To incorporate clauses for RedCap in NTN within Clause A.14,

  - Some new test cases will be added in the existing NTN test cases, which will also be defined in Clause A.14.

Issue 2-1-4: Test configuration and parameters for (e)RedCap UE with FR1-NTN

<Way Forward>:

- Proposal 1:

  - Both FDD and HD-FDD duplex mode should be considered in test configuration for (e)RedCap UE with FR1-NTN.

  - 15 kHz SCS and 10MHz bandwidth can be configured in test cases.

  - Both GSO and NGSO are configured in test cases for RedCap UE with FR1-NTN.

  - If UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases.

  - Proposal 1a: Take the NTN test setups as the reference, except both FDD and HD-FDD shall be involved in test configuration.

  - Proposal 1b: RAN4 to use following supported test configurations for (e)RedCap with FR1-NTN.

Table A.14.5.x.y.z: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, 15 kHz SSB SCS, 10 MHz BW |
| 2 | NGSO, NR FDD, 15 kHz SSB SCS, 10 MHz BW |
| 3 | GSO, NR HD-FDD, 15 kHz SSB SCS, 10 MHz BW |
| 4 | NGSO, NR HD-FDD, 15 kHz SSB SCS, 10 MHz BW |
| NOTE1: If (e)RedCap UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. NOTE2: If (e)RedCap UE supports both FDD and HD-FDD operation, the UE is only required to be tested in one of both. |  |

  - Proposal 1c: RAN4 shall consider the following general test configurations as follows:

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | GSO, NR HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 4 | NGSO, NR HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

- Proposal 2:

  - For the proposed test configuration, the existing RMC related parameters, Io values, SSB configuration and SMTC configurations can be reused for RedCap UE with FR1-NTN.

  - RAN4 not to consider NCD-SSB related configuration in test cases for (e)RedCap UE with NTN.

  - RAN4 not to consider RedCap specific UL/DL BWP configuration for (e)RedCap UE with NTN.

Issue 2-1-5: Test scope

Issue 2-1-5-1: Whether to define separate test cases for 1Rx and 2Rx (e)RedCap UE with NTN

<Way Forward>:

- Proposal 1: RAN4 to define the separate test cases for 1Rx and 2Rx (e)RedCap UE with NTN.

  - Proposal 1a: Following the same approach as for 1Rx and 2Rx RedCap in TN, apply separate test cases for 1Rx and 2Rx RedCap in NTN.

- Proposal 2: Further discuss whether to define separate test cases for 1Rx and 2Rx RedCap UEs

- Proposal 3: Discuss case by case.

Issue 2-1-5-2: Test case list

<Way Forward>:

  - Proposal 1: RAN4 to define new test cases without differentiating 1Rx and 2Rx (e)RedCap UEs with NTN for the following requirements:

    - Enhanced cell reselection measurement

    - UE transmit timing

    - Timing Advance

    - Active BWP switch delay

    - UE specific CBW change requirements

    - Pathloss reference signal switching delay

  - Proposal 2: RAN4 to define new test cases for the following requirements:

    - Cell Re-selection

    - Configured Grant based Small Data Transmissions (CG-SDT)

    - Handover/CHO/ satellite switching with re-synchronization

    - RRC Connection Mobility Control

    - Radio Link Monitoring

    - Link Recovery Procedures

    - NR intra-frequency measurements

    - NR inter-frequency measurements

    - L1-RSRP measurements for Reporting

    - NR measurements for positioning

  - Proposal 3: The test cases listed above can be a baseline, and RAN4 can further discuss which test cases can be removed to reduce the redundancy of test cases.

    - FFS part can be considered to be removed to reduce test effort.

  - Proposal 4: Consider the test case list as below:

| Test category |  | Comments |
| --- | --- | --- |
| RRC_IDLE state mobility | Cell reselection | Only test the delta part |
| RRC_IDLE state mobility | Configured Grant based Small Data Transmissions (CG-SDT) | Not needed, due to similar core requirements |
| RRC_CONNECTED state mobility | Handover - RACH-based - RACH-less | Not needed, due to similar core requirements |
|  | CHO - time-based - location-based | Choose one |
|  | satellite switching with re-synchronization |  |
|  | RRC Connection Mobility Control | Only test the delta part |
| Timing | UE transmit timing | Only test the delta part |
|  | UE timer accuracy |  |
|  | Timing Advance |  |
| Signalling characteristics | Radio Link Monitoring | Only test the delta part |
|  | Beam Failure Detection and Link recovery | Only test the delta part |
|  | Active BWP switch delay |  |
|  | Active TCI state switching delay |  |
|  | UE-specific CBW change |  |
|  | Pathloss reference signal switching delay |  |
| Measurement procedure | NR intra-frequency measurements | Only test the delta part |
|  | NR inter-frequency measurements | Only test the delta part |
|  | L1-RSRP measurements for Reporting | Only test the delta part |
|  | NR measurements for positioning |  |
| Measurement Performance | SS-RSRP/SS-RSRQ/SS-SINR Measurement Performance | Only test the delta part |
|  | L1-RSRP measurement Performance for beam reporting | Only test the delta part |

  - Proposal 5: Other than the TC in A.14, RAN4 to add new tests for 1Rx (e)RedCap UEs and add HD-FDD duplex mode.

  - Proposal 6:

    - RAN4 to adopt separate test cases for 1Rx and 2Rx (e)RedCap with FR1-NTN as baseline and evaluate thereafter, if test cases can be merged to reduce the extent of changes to spec.

    - RAN4 to adopt the test case scope as listed in the Way Forward under issue 2-1-4-2.

    - RAN4 to discuss the scope of the test case on supported NR measurements for positioning for (e)RedCap UE and preferably align it to normal UE in FR1-NTN.

  - Proposal 7: Define the test cases for RedCap UE over NTN as listed above.

  - Proposal 8:

    - RAN4 shall define the following test cases for redcap UEs in NTN scenario:

| Cell Re-selection | TC 1: Cell re-selection to FR1 intra-frequency NR cell for UE configured with the feature for enhanced requirements. |
| --- | --- |
|  | TC 2: Cell re-selection to FR1 intra-frequency NR case for 1Rx UE. |
|  | TC 3: Cell re-selection to FR1 inter-frequency NR case for 1Rx UE. |
| Handover | TC 4: Intra-frequency SAN RACH-less handover from FR1-FR1 |
|  | TC 5: Intra-frequency SAN handover from FR1-FR1 for 1Rx UE: unknown cell |
|  | TC 6: Inter-frequency SAN handover from FR1-FR1 for 1Rx UE: unknown cell |
| CHO | TC 7: Intra-frequency SAN time-based conditional handover without L3 measurement from FR1 to FR1 for 1Rx UEs |
|  | TC 8: Inter-frequency SAN time-based conditional handover without L3 measurement from FR1 to FR1 for 1Rx UEs |
|  | TC 9: Intra-frequency SAN time-based conditional handover from FR1-FR1 for 1Rx UE |
|  | TC 10: Inter-frequency SAN time-based conditional handover from FR1-FR1 for 1Rx UE |
| Satellite switching | TC 11: RACH-based hard satellite switching with re-sync from FR1 to FR1 for 1Rx UEs |
|  | TC 12: RACH-less soft satellite switching with re-sync from FR1 to FR1 for 1Rx UEs |
| RRC Re-establishment | TC 13: Intra-frequency RRC Re-establishment in FR1 for 1Rx UEs: unknown cell |
|  | [TC 14: Intra-frequency RRC Re-establishment in FR1 for 2Rx UEs: unknown cell] |
| RRC Connection Release with redirection | TC 15: Redirection from NR in FR1 to NR in FR1 for 1Rx UE |
|  | [TC 16: Redirection from NR in FR1 to NR in FR1 for 2Rx UE] |
| Random Access | TC 17: 4-step RA type CBRA test in FR1 for NR standalone for 1Rx UEs |
|  | TC 18: 4-step RA type CFRA test in FR1 for NR standalone for 1Rx UEs |
| RLM/BFD/CBD | TC 19: Intra-frequency RLM/BFD/CBD SSB-based measurement in FR1 for 1Rx UEs |
|  | [TC 20: Intra-frequency RLM/BFD/CBD SSB-based measurement in FR1 for 2Rx UEs] |
| L1/L3 measurement procedure | TC 21: SA event triggered reporting tests without gap under non-DRX with SSB index reading in FR1 for 1Rx UEs |
|  | [TC 22: SA event triggered reporting tests without gap under non-DRX with SSB index reading in FR1 for 2Rx UEs] |

- Try to summarize based on the input from companies and RAN4 need to further discuss which specific test cases with configuration can be deleted to reduce test effort..

| Test categories/configurations |  | Company comments | Test cases (as baseline) |
| --- | --- | --- | --- |
| RRC_IDLE state mobility | 4.2E Cell Re-selection for NR RedCap UE with Satellite Access - Intra/inter-f - 1Rx/2Rx - with the feature for enhanced requirements (without differentiating 1Rx and 2Rx) - time-based/location-based - fulfilling low mobility/ not-at-cell edge relaxed measurement criterion - inter-RAT - with TN carrier | Support: CATT, OPPO (only test the delta part), Samsung, Ericsson, ZTE, CMCC | - Cell reselection to FR1 intra-frequency NR case for 1Rx/2Rx RedCap UE with NTN - Cell reselection to FR1 intra-frequency NR cell for UE configured with the feature for enhanced requirements - Time-based measurement initiation to FR1 intra-frequency NR cell reselection for 1Rx/2Rx RedCap UE with NTN - FFS: Location-based measurement initiation to FR1 intra-frequency NR cell reselection for 1Rx/2Rx RedCap UE with NTN - Cell reselection to FR1 inter-frequency NR case for 1Rx/2Rx RedCap UE with NTN - FFS: Cell re-selection to FR1 inter-frequency NR cell for UE configured with feature for enhanced requirements - FFS: Time-based measurement initiation to FR1 inter-frequency cell reselection for 1Rx/2Rx RedCap UE with NTN - Location-based measurement initiation to FR1 inter-frequency NR cell reselection for 1Rx/2Rx RedCap UE with NTN - Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion for 1Rx/2Rx RedCap UE with NTN - Cell reselection to FR1 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion for 1Rx/2Rx RedCap UE with NTN - Cell reselection to FR1 inter-RAT for NR NTN carrier for 1Rx/2Rx RedCap UE with NTN - Cell re-selection to FR1 inter-frequency NR case with TN carrier for 1Rx/2Rx RedCap UE with NTN |
| RRC_IDLE state mobility | 5.5E Configured Grant based Small Data Transmissions (CG-SDT) for RedCap UE with satellite access - 1Rx/2Rx | Support: CATT Not support: OPPO | - NR UE CG-SDT Test in FR1 for 1Rx/2Rx RedCap UE with NTN |
| RRC_CONNECTED state mobility | 6.1F.1 NR SAN Handover - Intra/inter-f - 1Rx/2Rx - RACH-based/RACH-less | Support: CATT, Samsung, Ericsson, ZTE, CMCC Not support: OPPO | - Intra-frequency SAN Handover from FR1 to FR1 for 1Rx/2Rx RedCap UE with NTN - Inter-frequency SAN Handover from FR1 to FR1 for 1Rx/2Rx RedCap UE with NTN - Intra-frequency SAN RACH-less Handover from FR1 to FR1 for 1Rx/2Rx RedCap UE with NTN |
|  | 6.1F.2 NR SAN Conditional Handover - Intra/inter-f - 1Rx/2Rx - time-based/distance-based CHO - CHO without L3 measurement criteria | Support: CATT, OPPO (choose one from time-based and distance-based CHO), Samsung, Ericsson, ZTE, CMCC | - Intra-frequency SAN time-based conditional Handover from FR1 to FR1 for 1Rx/2Rx RedCap UE with NTN - FFS: Inter-frequency SAN time-based conditional Handover from FR1 to FR1 for 1Rx/2Rx RedCap UE with NTN - FFS: Intra-frequency SAN distance-based conditional Handover from FR1 to FR1 for 1Rx/2Rx RedCap UE with NTN - Inter-frequency SAN distance-based conditional Handover from FR1 to FR1 for 1Rx/2Rx RedCap UE with NTN - Intra-frequency SAN time-based conditional Handover without L3 measurement criteria from FR1 to FR1 for 1Rx/2Rx RedCap UE with NTN - FFS: Inter-frequency SAN time-based conditional Handover without L3 measurement criteria from FR1 to FR1 for 1Rx/2Rx RedCap UE with NTN |
|  | 6.1F.3 NR SAN Satellite switching with re-synchronization - 1Rx/2Rx - RACH-based/RACH-less - Hard/soft | Support: CATT, Samsung, Ericsson, ZTE, CMCC Not support: OPPO | - RACH-based Hard Satellite switching with re-synchronization from FR1 to FR1 for 1Rx/2Rx RedCap UE with NTN for 1Rx/2Rx RedCap UE with NTN - RACH-less Soft Satellite switching with re-synchronization from FR1 to FR1 for 1Rx/2Rx RedCap UE with NTN for 1Rx/2Rx RedCap UE with NTN |
|  | 6.2E RRC Connection Mobility Control for RedCap UE with Satellite Access - Intra/inter-f - contention based/ non-contention based - only 4-step RA type (follow NTN) | Support: CATT, OPPO (only test the delta part), Samsung, Ericsson, ZTE, CMCC | SA: RRC Re-establishment for SAN - Intra-frequency RRC Re-establishment in FR1 for 1Rx/2Rx RedCap UE with NTN - Inter-frequency RRC Re-establishment in FR1 for 1Rx/2Rx RedCap UE with NTN Random Access - 4-step RA type contention based random access test in FR1 for NR standalone for 1Rx/2Rx RedCap UE with NTN - 4-step RA type non-contention based random access test in FR1 for NR standalone for 1Rx/2Rx RedCap UE with NTN RRC Connection Release with Redirection - Redirection from NR in FR1 to NR in FR1 for 1Rx/2Rx RedCap UE with NTN |
| Timing | 7.1E UE transmit timing for Redcap with Satellite Access - FFS: without differentiating 1Rx and 2Rx | Support: CATT, OPPO (only test the delta part), Samsung, Ericsson, CMCC | - NR UE Transmit Timing Test for FR1 |
|  | 7.3E Timing advance for Redcap with Satellite Access - FFS: without differentiating 1Rx and 2Rx | Support: CATT, Samsung, Ericsson, CMCC Not support: OPPO | - SA FR1 timing advance adjustment accuracy |
| Signalling characteristics | 8.1E Radio Link Monitoring for RedCap UE with Satellite Access - 1Rx/2Rx - Other aspect will follow legacy NTN test cases | Support: CATT, OPPO (only test the delta part), Samsung, Ericsson, ZTE, CMCC | - Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in non-DRX mode for 1Rx/2Rx RedCap UE with NTN - FFS: Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in non-DRX mode for 1Rx/2Rx RedCap UE with NTN - FFS: Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in DRX mode for 1Rx/2Rx RedCap UE with NTN - Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in DRX mode for 1Rx/2Rx RedCap UE with NTN - Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in non-DRX mode for 1Rx/2Rx RedCap UE with NTN - FFS: Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in non-DRX mode for 1Rx/2Rx RedCap UE with NTN - FFS: Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in DRX mode for 1Rx/2Rx RedCap UE with NTN - Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with CSI-RS based RLM in DRX mode for 1Rx/2Rx RedCap UE with NTN |
|  | 8.5E Link Recovery Procedures for RedCap UE with Satellite Access - 1Rx/2Rx - Other aspect will follow legacy NTN test cases | Support: CATT, OPPO (only test the delta part), Samsung, Ericsson, ZTE, CMCC | - Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with SSB-based BFD and LR in non-DRX mode for 1Rx/2Rx RedCap UE with NTN - Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with SSB-based BFD and LR in DRX mode for 1Rx/2Rx RedCap UE with NTN - Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with CSI-RS based BFD and LR in non-DRX mode for 1Rx/2Rx RedCap UE with NTN - Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with CSI-RS based BFD and LR in DRX mode for 1Rx/2Rx RedCap UE with NTN |
|  | 8.6E Active BWP switch delay for RedCap UE with satellite access - FFS: without differentiating 1Rx and 2Rx | Support: CATT, Samsung, Ericsson, CMCC Not support: OPPO | - DCI-based and Timer-based Active BWP Switch - RRC-based Active BWP Switch |
|  | 8.13E UE-specific CBW change for RedCap UE with satellite access - FFS: without differentiating 1Rx and 2Rx | Support: CATT, Samsung, Ericsson, CMCC Not support: OPPO | - UE specific CBW change on PCell in FR1 in non-DRX |
|  | 8.14E Pathloss reference signal switching delay for RedCap UE with satellite access - FFS: without differentiating 1Rx and 2Rx | Support: CATT, Samsung, Ericsson Not support: OPPO | - MAC-CE based pathloss reference signal switch delay |
| Measurement procedure | 9.2E NR intra-frequency measurements for RedCap with satellite access - 1Rx/2Rx - Other aspect will follow legacy NTN test cases | Support: CATT, OPPO (only test the delta part), Samsung, Ericsson, ZTE, CMCC | - SA event triggered reporting tests without gap under non-DRX for 1Rx/2Rx RedCap UE with NTN - SA event triggered reporting tests without gap under DRX for 1Rx/2Rx RedCap UE with NTN - SA event triggered reporting tests without gap under non-DRX with SSB index reading for 1Rx/2Rx RedCap UE with NTN - SA event triggered reporting tests with single measurement gap under non-DRX for 1Rx/2Rx RedCap UE with NTN - SA event triggered reporting tests with FNO concurrent gaps under DRX for 1Rx/2Rx RedCap UE with NTN - SA event triggered reporting tests with PPO concurrent gaps under non-DRX with SSB index reading for 1Rx/2Rx RedCap UE with NTN |
|  | 9.3E NR inter-frequency measurements for Redcap UEs with satellite access - 1Rx/2Rx - Other aspect will follow legacy NTN test cases | Support: CATT, OPPO (only test the delta part), Samsung, Ericsson, ZTE, CMCC | - SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with single gap for 1Rx/2Rx RedCap UE with NTN - SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used with single gap for 1Rx/2Rx RedCap UE with NTN - SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used with single gap for 1Rx/2Rx RedCap UE with NTN - SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with two gaps in fully non-overlapped for 1Rx/2Rx RedCap UE with NTN - SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with two gaps in partially partial overlapping for 1Rx/2Rx RedCap UE with NTN - Event triggered reporting test without gap under non-DRX for 1Rx/2Rx RedCap UE with NTN - Event triggered reporting tests without gap under DRX for 1Rx/2Rx RedCap UE with NTN |
|  | 9.5E L1-RSRP measurements for Reporting for RedCap UE with satellite access - SSB/ CSI-RS based - DRX/non-DRX - 1Rx/2Rx | Support: CATT, OPPO (only test the delta part), Samsung, Ericsson, ZTE, CMCC | - SSB based L1-RSRP measurement for satellite access when DRX is not used for 1Rx/2Rx RedCap UE with NTN - SSB based L1-RSRP measurement for satellite access when DRX is used for 1Rx/2Rx RedCap UE with NTN - CSI-RS based L1-RSRP measurement for satellite access when DRX is not used for 1Rx/2Rx RedCap UE with NTN - CSI-RS based L1-RSRP measurement for satellite access when DRX is used for 1Rx/2Rx RedCap UE with NTN |
|  | 9.9E NR measurements for positioning | FFS: Nokia Not support: OPPO |  |
| Measurement Performance | SS-RSRP/SS-RSRQ/SS-SINR Measurement Performance - Intra/inter-f - 1Rx/2Rx | Support: CATT, OPPO (only test the delta part), Ericsson, CMCC | SS-RSRP - SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 1Rx/2Rx RedCap UE with NTN - SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 1Rx/2Rx RedCap UE with NTN SS-RSRQ - SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1Rx/2Rx RedCap UE with NTN - SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1Rx/2Rx RedCap UE with NTN SS-SINR - SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1Rx/2Rx RedCap UE with NTN - SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1Rx/2Rx RedCap UE with NTN |
|  | L1-RSRP measurement Performance for beam reporting - SSB/ CSI-RS based - 1Rx/2Rx | Support: CATT, OPPO (only test the delta part), Ericsson, CMCC | - SSB based L1-RSRP measurement for 1Rx/2Rx RedCap UE with NTN - CSI-RS based L1-RSRP measurement on resource set with repetition off for 1Rx/2Rx RedCap UE with NTN |

    - Sub-topic 2-2 Other parts for NR NTN

Issue 2-2-1: Test scope for downlink coverage enhancement

<Way Forward>:

- Regarding RRC Re-establishment for the UE capable of coverage enhancement,

  - Proposal 1: For default SMTC core requirements updates for SSB periodicity extension, RAN4 not to introduce new TC in NTN.

  - Proposal 2: For the case of SSB Ês/Iot < -8dB and an unknown NR cell core requirements,

    - Option 1: RAN4 to consider adding a new test case.

      - Further discuss how to define the test case.

- Proposal 1a: Define RRC Re-establishment test case for Serving cell SSB Ês/Iot (dB)<-8 and SSB periodicity 160ms case.

- Proposal 1b: Downselect the intra-frequency RRC re-establishment test case to the scenario with SSB Ês/Iot < -8 dB and unknown NR cell.

- Proposal 1c: Define test case for RRC Re-establishment with 160ms SSB periodicity and without serving cell timing based on existing test case in A.6.3.2.1.3.

    - Option 2: RAN4 not to define test case.

- Regarding SMTC enhancement related test cases for the UE capable of coverage enhancement, RAN4 to wait for the core requirement progress firstly.

- Except for the above, RAN4 not to define other new test cases for the UE capable of coverage enhancement.

Issue 2-2-2: Test configuration for downlink coverage enhancement

<Way Forward>:

- Proposal 1: RAN4 need to discuss whether to introduced new SSB patterns for DL enhancement NTN UE.

  - Proposal 1a: Introduce new 160ms periodicity SSB pattern for DL CE.