3GPP TSG-RAN WG4 Meeting #116                                                  R4-2512529

Bengaluru, India, 25th – 29th August 2025

Agenda item:			7.21.1

Source:	Charter Communications, Inc., Nokia, Cable Labs, Samsung

Title:	Way Forward for UE-UE CLI

Document for:	Approval

# Overview

This document provides a way forward of the analysis on Intra-operator adjacent-channel inter-cell UE-to UE CLI and solutions in thread [306] on Rel-19 work item on evolution of NR duplex operation (SBFD) for general aspect and for SBFD in RAN4#116.

Coexistence assumptions and analysis for UE-to-UE CLI problem from WF in RAN4 #115

- [Background] UE-to-UE CLI problem statement: - The scenario to be considered: - Intra-operator adjacent-channel inter-cell (i.e., both channels belong to a single operator) - Use Urban Hotspot -> Urban Hotspot Scenario 2 as reference - FFS non-located assumption is always valid or not - detailed description provided in TR38.858 - The case to be considered: - SBFD-aware UE (aggressor) UL transmission to NR TDD UE (victim) DL reception

- WF-1: Companies are encouraged to provided analysis on the above UE-to-UE CLI problem: - The purpose is to identify whether the above-mentioned UE-to-UE CLI problem exists or not in the practical deployment - One evaluation method is to re-perform RAN4 Rel-18 co-existence study on the above scenario/case - Parameters agreed in Rel-18 will be reused unless difference identified - FFS impact from different grid shift values - FFS impact from ACLR model

- WF-2: Companies are encouraged to identify potential solution(s) to the above UE-to-UE CLI problem: - Solution-1: Reduced p-max for SBFD-aware UE TX power by X dB. - the reduced p-max is configurable per-UE - FFS X dB is a single value or a configurable value or a configurable range - Solution-2: SBFD configuration is set by using DUD (40%, 20%, 40%), or DU (by putting UL subband away from victim channel) - Solution-3: Avoid scheduling the aggressor UE, up to BS implementation. - Solution-4: Configure the aggressor UE fallback to TDD operation, up to BS implementation. - Other new solutions (including the combination of the above-mentioned solutions) are encouraged to be provided, if any.

- WF-3: Companies are encouraged to provide analysis on solutions, by at least considering the following aspects: - Benefit(s): - At least for Solution-1, one evaluation method is to perform RAN4 co-existence study on the above scenario/case, by using XdB lower maximum UE power for aggressor UE(s) - FFS how to determine one SBFD-aware UE is aggressor UE - The value of X can be chosen from the range of [1, 5] and other values may not be precluded. - How to determine the value of X or the range can be FFS - Baseline to be compared to is Rel-18 coexistence study re-performed in WF-1 - Detailed parameters can be further discussed by conference call and/or email discussion before RAN4#116. - Other methods are not precluded - Limitation(s), e.g., decreased coverage for certain SBFD-aware UE, etc. - Expected RAN4 requirement impact, including - the impact to TS 38.104 - the impact to TS 38.101 series, e.g., Pcmax, etc. - Expected RAN2 requirement impact: - whether new or changed RAN2 signaling(s) is required: - Expected RAN3 requirement impact: - whether new or changed RAN3 signaling(s) is required:

# Urban Hotspot inter-UE CLI simulation assumptions

Besides the FFS items described in the WF in section 1, the remaining assumptions of the Urban Hotspot scenario reuses most parameter assumption as Urban Macro, except that Urban Macro assumes random deployment of UEs while Urban Hotspot assumes a clustered-based approach with different outdoor-to-indoor UE ratios. This cluster-based dropping method is given in Table E.2.1-3 in TR 38.858, which is copied below.

Table E.2.1-3: Cluster-based UE dropping parameters

| Parameter | Urban Hotspot |
| --- | --- |
| Cluster number per macro cell | 1 (Note 1) |
| Cluster area | Circular area with 25 m radius |
| UE distribution | UEs dropped within a cluster are indoor and UEs dropped outside a cluster are outdoor |
| Indoor and outdoor ratio | 80% indoor and 20% outdoor (Note 2) |
| Minimum (2D) distance | UE-UE: 1 m Cluster center – cluster center: 50 m Macro BS – cluster center: 60 m (Note 3) |
| UE dropping | Evenly random dropping into the cluster with minimum UE-UE distance limitation |
| Note 1: Cells fully or partly overlapping between the 2 networks share the same cluster. Note 2: O-to-I penetration losses are not calculated for pairs of indoor UEs deployed in the same cluster area. For pairs of indoor UEs in different cluster areas, a single penetration loss component is calculated. Note 3: Consider the hexagonal grid of one of the two operators as the reference when dropping the cluster. The minimum distance between macro BS to cluster centre should be respected also for BSs belonging to the other operator |  |

# Collection of Urban Hotspot inter-UE CLI simulation results

Table 1 below collects the simulation results in Urban Hotspot scenario from different companies in RAN4#116.

Table 1 Collection simulation results for inter-UE CLI analysis

| Company | Simulation Assumptions | Equivalent ACLR & ACS in simulation | Throughput degradation at cell-edge |
| --- | --- | --- | --- |
| Charter Comm./CableLabs | Flat ACLR and ACS model | ACLR = 30 dB ACS = 33 dB | 27.67% |
|  | Stepwise ACLR and ACS model | ACLR = 43 dB ACS = 33.6 dB | 11.58% |
| Samsung | Stepwise ACLR and ACS model | ACLR = 50 dB ACS = 33.6 dB | 11.63% |
|  | Stepwise ACLR and ACS model w/ Optional inter-UE penetration loss (wooden or concrete wall or floor) with probability | ACLR = 50 dB ACS = 33.6 dB | 7.6% (wooden) 2.92% (concrete) |
| Ericsson | Flat ACLR and stepwise ACS model | ACLR = 30 dB ACS = 33.6 dB | 31.49% |
|  | Stepwise ACLR and ACS model | ACLR = 43 dB ACS = 33.6 dB | 19.6% |
| Nokia | Flat ACLR and ACS model | ACLR = 30 dB ACS = 33 dB | 50.84% |
|  | Stepwise ACLR and ACS model | ACLR = 50 dB ACS = 33.6 dB | 38.64% |
| Qualcomm | Flat ACLR and stepwise ACS model | ACLR = 30 dB ACS = 33.6 dB | Observed cell edge degradation |
|  | Stepwise ACLR and ACS model | ACLR = 49 dB ACS = 33.6 dB |  |

Observations on the simulation results given in Table 1:

- Flat ACLR and ACS model (Rel-18 SBFD co-ex study baseline assumptions) present worst-case scenarios in terms of adjacent-channel interference power modelling.

- Stepwise ACLR and/or ACS models considers the 2nd and/or 3rd regions of ACLR and ACS performance. This is based on the model provided in TR 36.942 along with different companies’ understandings of this model. The agreed DUD SBFD configuration considers the SBFD-aware UE transmitting within a 20MHz UL subband, with the presence of a 40MHz downlink subband, acting as a guard between to the legacy TDD UE receiving in DL.

- Stepwise ACLR and/or ACS model improves the performance but still shows TP degradation at cell edge. Note that baseline assumptions are based on flat ACLR and ACS model with minimum RAN4 requirements for BS and UE ACLR and ACS.

- Beside Rel-18 baseline assumptions (i.e., Annex E in TR 38.858), some companies reported the throughput (TP) degradation is negligible with presence of a probabilistic inter-UE penetration loss (wooden or concrete wall or floor). Note that baseline assumptions assumed all indoor UEs in the same cluster are dropped on the same floor, and no additional interior wall or floor slab penetration loss between victim and aggressor UEs.

# RAN4 investigated solutions

## 4.1	Solution-1: Reduced Pmax for SBFD-aware UE TX power by X dB

Table 2 below collects the simulation results from different companies in RAN4#116 assuming Pmax power reduction.

Table 2: Collection simulation results for inter-UE CLI analysis with Pmax power reduction

| Company | Simulation Assumptions | Equivalent ACLR & ACS | TP degradation w/o Pmax reduction | TP degradation w/ Pmax reduction |
| --- | --- | --- | --- | --- |
| Charter Comm./CableLabs | Flat ACLR and ACS model | ACLR = 30 dB ACS = 33 dB | 27.67% | 5% w/ 5.5 dB Pmax reduction |
|  | Stepwise ACLR and ACS model | ACLR = 43 dB ACS = 33.6 dB | 11.58% | 5% w/ 3.5 dB Pmax reduction |
| Samsung | Stepwise ACLR and ACS model | ACLR = 50 dB ACS = 33.6 dB | 11.63% | 2.92% w/ 3dB Pmax reduction |
| Ericsson | Flat ACLR and stepwise ACS model | ACLR = 30 dB ACS = 33.6 dB | 31.49% | 13% w/ 8 dB Pmax reduction |
|  | Stepwise ACLR and ACS model | ACLR = 43 dB ACS = 33.6 dB | 19.6% | 9 % w/ 8 dB Pmax reduction |
| Nokia | Flat ACLR and ACS model | ACLR = 30 dB ACS = 33 dB | 50.84% | 35 % w/ 6 dB Pmax reduction |
|  | Stepwise ACLR and ACS model | ACLR = 50 dB ACS = 33.6 dB | 38.64% | 22.4 % w/ 6 dB Pmax reduction |
| Qualcomm | Flat ACLR and ACS model | ACLR = 30 dB ACS = 33 dB | Observed cell edge degradation | At least 10 dB Pmax reduction needed |
|  | Stepwise ACLR and ACS model | ACLR = 49 dB ACS = 33.6 dB |  |  |

Observations on Solution-1:

- Pmax reduction for SBFD-aware UE improves the throughput performance of legacy TDD DL UE at cell edge.

- Pmax reduction solution needs to be UE specific and not applied to all SBFD-aware UEs. A mechanism to identify SBFD-aware UEs that need to reduce their Pmax is not yet specified in 3GPP till Rel-19.

- Some companies’ results reported the Pmax reduction applied on SBFD-aware UE would lead to performance degradation on SBFD UL carrier, and one company’s results reported performance degradation to overall throughput of two adjacent carriers.

## 4.2	Solution-2: SBFD configuration enforcement

|  | Expected benefit(s) | Expected limitation(s) | Expected RAN4 requirement impact | Expected RAN2 requirement impact | Expected RAN3 requirement impact |
| --- | --- | --- | --- | --- | --- |
| Solution-2 (SBFD configuration is set by using DUD (40%, 20%, 40%), or DU (by putting UL subband away from victim channel) | CLI mitigated | Deployment restriction by not allowing certain SBFD configurations. On its own, this solution might be insufficient to completely solve the UE-to-UE adjacent CLI problem | No impact (Rel-19 RAN4 specification would already include the requirements for these configurations), if it is deployment-based restriction. RAN4 specification impact is expected if it is required to mandate a given SBFD configuration in RAN4 specs to consider the DL subband frequency locations. | No impact | No impact |

Observations on Solution-2:

- Possible inter-UE CLI degradation reduction since downlink subband would act as a guard band to the victim TDD UE receiving in the DL from the interfering UL subband transmissions.

## 4.3	Solution 3: Scheduling-based solutions

|  | Expected benefit(s) | Expected limitation(s) | Expected RAN4 requirement impact | Expected RAN2 requirement impact | Expected RAN3 requirement impact |
| --- | --- | --- | --- | --- | --- |
| Solution-3 (Avoid scheduling the aggressor UE) | CLI mitigated | (1) Scheduling restriction for not scheduling aggressor UE in the concerned SBFD symbols (2) Additional information exchange between BSs to avoid inter-UE CLI. | No impact | FFS if additional signaling is needed between legacy and SBFD-capable BSs belonging to same/ different operator. | Impact expected, because Rel-19 UE-to-UE CLI measurement is only for intra-cell, and measurement reports can’t be shared over Xn interface |

Observations on Solution-3:

- Possible inter-UE CLI degradation reduction since scheduling of geographically close victim and aggressor UEs would be avoided.

- Scheduling UEs between networks can avoid TP degradation because of scheduling SBFD-aware UE’s away from legacy TDD UE’s.

## 4.4	Solution-4: TDD fallback configuration

|  | Expected benefit(s) | Expected limitation(s) | Expected RAN4 requirement impact | Expected RAN2 requirement impact | Expected RAN3 requirement impact |
| --- | --- | --- | --- | --- | --- |
| Solution-4 (Configure the aggressor UE fallback to TDD operation) | Avoids completely UE to UE CLI | no SBFD UL coverage benefit (i.e. same or worse performance than legacy TDD) | None | None | Exchange of information across cells of UE-to-UE CLI measurement results |

Observations on Solution-4:

- The aggressor UE fallback to TDD operation fixes TP degradation.

## 4.5	Solution-5: Scheduling victim UEs further apart in frequency domain

Table 4 below collects the simulation results from one company in RAN4#116 assuming Scheduling victim UEs further apart in frequency domain.

Table 4: Collection simulation results for inter-UE CLI analysis with Scheduling victim UEs further apart in frequency domain

| Company | Simulation Assumptions | TP degradation w/o this scheduling method | TP degradation w/ this scheduling method |
| --- | --- | --- | --- |
| Samsung | Stepwise ACLR and ACS model | 11.63% | 1.74% |

Observations on Solution-5:

- The throughput degradation on legacy TDD DL at cell-edge is mitigated and becomes negligible with this solution by scheduling victim UEs further apart in frequency domain.

# WF Summary

- Based on simulations, RAN4 acknowledges the issue of UE-to-UE CLI in Urban Hotspot scenario at cell-edge following Rel-18 baseline assumptions given in Annex E, TR 38.858 for the following scenario

  - Urban Hotspot -> Urban Hotspot Scenario 2, Case 1 as reference.

  - Intra-operator adjacent-channel inter-cell UE-to UE CLI where the victim UE is legacy TDD, and the aggressor UE is SBFD-aware UE causing TDD DL TP degradation at cell-edge.

  - Flat ACLR and ACS model based on minimum RAN4 BS and UE ACLR and ACS values.

- RAN4 has investigated five solutions to the above problem with the following observations

  - Solution 1: Reducing the power of the SBFD-aware UE reduces the TP degradation and can be considered as part of a solution for future releases, and the detailed observations can be found in section 4.1.

  - Solution 2, 3 and 4 could mitigate UE-to-UE CLI. and can be considered as part of a solution for future releases. The detailed observations can be found in section 4.2 to 4.4.

  - Solution-5: Scheduling victim UEs further apart in frequency domain could mitigate UE-to-UE CLI and can be considered as part of a solution for future releases. The detailed observations can be found in section 4.5.