3GPP TSG-RAN WG4 Meeting #116	R4-2512662

Bangaluru, India, 25th – 29th August 2025                                      (revision of R4-2511148)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.108 | CR | - | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS38.108 – for transmit power and output power in NTN Ku bands |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | THALES |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ku_bands-Core |  |  |  |  |  | Date: |  |  | 2025-07-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of NTN Ku bands. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add Radiated transmit power requirements, OTA Satellite Access Node output power, OTA output power dynamics requirements for NTN Ku bands. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Radiated transmit power requirements, OTA Satellite Access Node output power, OTA output power dynamics for NTN Ku bands not updated correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2, 9.3, 9.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## << Start of change >>

# 9	Radiated transmitter characteristics

## 9.1	General

Radiated transmitter characteristics requirements apply on the SAN type 1-H or SAN type 1-O including all its functional components active and for all foreseen modes of operation of the SAN unless otherwise stated.

NOTE:	For NB-IoT operation in NTN NR in-band, the NR carrier and NB-IoT carrier shall be seen as a single carrier occupied NR channel bandwidth.  The radiated transmitter characteristics in this section apply to all such carriers which may have NB-IoT operating in band.

## 9.2	Radiated transmit power

### 9.2.1	General

SAN type 1-H, SAN type 1-O and SAN type 2-O are declared to support one or more beams, as per manufacturer's declarations specified in TS 38.181 [3]. Radiated transmit power is defined as the EIRP level for a declared beam at a specific beam peak direction.

For each beam, the requirement is based on declaration of a beam identity, reference beam direction pair, beamwidth, rated beam EIRP, OTA peak directions set, the beam direction pairs at the maximum steering directions and their associated rated beam EIRP and beamwidth(s).

For a declared beam and beam direction pair, the rated beam EIRP level is the maximum power that the SAN is declared to radiate at the associated beam peak direction during the transmitter ON period.

For each beam peak direction associated with a beam direction pair within the OTA peak directions set, a specific rated beam EIRP level may be claimed. Any claimed value shall be met within the accuracy requirement as described below. Rated beam EIRP is only required to be declared for the beam direction pairs subject to conformance testing as detailed in TS 38.181 [3].

NOTE 1:	OTA peak directions set is set of beam peak directions for which the EIRP accuracy requirement is intended to be met. The beam peak directions are related to a corresponding contiguous range or discrete list of beam centre directions by the beam direction pairs included in the set.

NOTE 2:	A beam direction pair is data set consisting of the beam centre direction and the related beam peak direction.

NOTE 3:	A declared EIRP value is a value provided by the manufacturer for verification according to the conformance specification declaration requirements, whereas a claimed EIRP value is provided by the manufacturer to the equipment user for normal operation of the equipment and is not subject to formal conformance testing.

### 9.2.2	Minimum requirement for SAN type 1-H and SAN type 1-O operating below 10 GHz

For each declared beam, in normal conditions, for any specific beam peak direction associated with a beam direction pair within the OTA peak directions set, a manufacturer claimed EIRP level in the corresponding beam peak direction shall be achievable to within ±2.2 dB of the claimed value.

Normal conditions are defined in TS 38.181, annex B [3].

In certain regions, the minimum requirement for normal conditions may apply also for some conditions outside the range of conditions defined as normal.

NOTE:	For NB-IoT operation in NTN NR in-band, the NR carrier and NB-IoT carrier shall be seen as a single carrier occupied NR channel bandwidth, the output power over this carrier is shared between NR and NB-IoT. This note shall apply for Pmax,c,TRP and Prated,c,TRP.

### 9.2.2a	Minimum requirement for SAN type 1-O operating above 10 GHz

For each declared beam, in normal conditions, for any specific beam peak direction associated with a beam direction pair within the OTA peak directions set, a manufacturer claimed EIRP level in the corresponding beam peak direction shall be achievable to within ±2.5 dB of the claimed value.

Normal conditions are defined in TS 38.181, annex B [3].

In certain regions, the minimum requirement for normal conditions may apply also for some conditions outside the range of conditions defined as normal.

### 9.2.3	Minimum requirement for SAN type 2-O

For each declared beam, in normal conditions, for any specific beam peak direction associated with a beam direction pair within the OTA peak directions set, a manufacturer claimed EIRP level in the corresponding beam peak direction shall be achievable to within ± 3.4 dB of the claimed value.

Normal conditions are defined in TS 38.181, annex B [3].

In certain regions, the minimum requirement for normal conditions may apply also for some conditions outside the range of conditions defined as normal.

## 9.3	OTA Satellite Access Node output power

### 9.3.1	General

OTA SAN output power is declared as the TRP radiated requirement, with the output power accuracy requirement defined at the RIB. TRP does not change with beamforming settings as long as the beam peak direction is within the OTA peak directions set. Thus the TRP accuracy requirement must be met for any beamforming setting for which the beam peak direction is within the OTA peak directions set.

The SAN rated carrier TRP output power for SAN type 1-O and SAN type 2-O shall be based on manufacturer declaration.

Despite the general requirements for the SAN output power described in clause 9.3.2, additional regional requirements might be applicable.

NOTE:	For NB-IoT operation in NTN NR in-band, the NR carrier and NB-IoT carrier shall be seen as a single carrier occupied NR channel bandwidth, the output power over this carrier is shared between NR and NB-IoT. This note shall apply for Pmax,c,TRP and Prated,c,TRP.

### 9.3.2	Minimum requirement for SAN type 1-O operating below 10 GHz

In normal conditions, the SAN type 1-O maximum carrier TRP output power, Pmax,c,TRP measured at the RIB shall remain within ±2 dB of the rated carrier TRP output power Prated,c,TRP, as declared by the manufacturer.

Normal conditions are defined in TS 38.181 [3], annex B.

### 9.3.2a	Minimum requirement for SAN type 1-O operating above 10 GHz

In normal conditions, the SAN type 1-O maximum carrier TRP output power, Pmax,c,TRP measured at the RIB shall remain within ±2.2 dB of the rated carrier TRP output power Prated,c,TRP, as declared by the manufacturer.

Normal conditions are defined in TS 38.181 [3], annex B.

### 9.3.3	Minimum requirement for SAN type 2-O

In normal conditions, the SAN type 2-O maximum carrier TRP output power, Pmax,c,TRP measured at the RIB shall remain within ±3 dB of the rated carrier TRP output power Prated,c,TRP, as declared by the manufacturer.

Normal conditions are defined in TS 38.181, annex B [3].

## 9.4	OTA output power dynamics

### 9.4.1	General

Transmit signal quality (as specified in clause 9.6) shall be maintained for the output power dynamics requirements.

The OTA output power requirements are directional requirements and apply to the beam peak directions over the OTA peak directions set.

### 9.4.2	OTA RE power control dynamic range

#### 9.4.2.1	General

The OTA RE power control dynamic range is the difference between the power of an RE and the average RE power for a SAN at maximum output power (Pmax,c,EIRP) for a specified reference condition.

This requirement shall apply at each RIB supporting transmission in the operating band.

#### 9.4.2.2	Minimum requirement for SAN type 1-O

The OTA RE power control dynamic range is specified the same as the conducted RE power control dynamic range requirement for SAN type 1-H in table 6.3.2.2-1.

### 9.4.3	OTA total power dynamic range

#### 9.4.3.1	General

The OTA total power dynamic range is the difference between the maximum and the minimum transmit power of an OFDM symbol for a specified reference condition.

This requirement shall apply at each RIB supporting transmission in the operating band.

NOTE 1:	The upper limit of the OTA total power dynamic range is the SAN maximum carrier EIRP (Pmax,c,EIRP) when transmitting on all RBs. The lower limit of the OTA total power dynamic range is the average EIRP for single RB transmission in the same direction using the same beam. The OFDM symbol carries PDSCH and not contain RS or SSB.

#### 9.4.3.2	Minimum requirement for SAN type 1-O operating below 10 GHz

OTA total power dynamic range minimum requirement for SAN type 1-O is specified such as for each NR carrier it shall be larger than or equal to the levels specified for the conducted requirement for SAN type 1-H in table 6.3.3.2-1.

#### 9.4.3.2a	Minimum requirement for SAN type 1-O operating above 10 GHz

OTA total power dynamic range minimum requirement for SAN type 1-O operating above 10 GHz is specified such as for each NR carrier it shall be larger than or equal to the levels specified in table 9.4.3.2a-1 in FR1-NTN.

Table 9.4.3.2a-1: Minimum requirement for SAN type 1-O total power dynamic range in FR1-NTN

| SAN Operating Band | SCS | OTA total power dynamic range (dB) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (kHz) | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 35 MHz | 50 MHz | 70 MHz | 100 MHz |
| n248, n247 | 15 | 17.1 | 18.9 | N/A | 21.2 | 22.7 | 24.3 | N/A | N/A |
|  | 30 | 13.8 | N/A | 17 | 18.1 | 19.6 | 21.2 | 22.7 | 24.3 |

#### 9.4.3.3	Minimum requirement for SAN type 2-O

OTA total power dynamic range minimum requirement for SAN type 2-O is specified such as for each NR carrier it shall be larger than or equal to the levels specified in table 9.4.3.3-1 in FR2-NTN.

Table 9.4.3.3-1: Minimum requirement for SAN type 2-O total power dynamic range in FR2-NTN

| SAN Operating Band | SCS | OTA total power dynamic range (dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | (kHz) | 50 MHz |  | 100 MHz |  | 200 MHz | 400 MHz |
| n512, n511, n510 | 60 | 18.1 |  | 21.2 |  | 24.2 | N/A |
|  | 120 | 15.0 |  | 18.1 |  | 21.2 | 24.2 |
| n509, n508 | 120 | 15.0 |  | 18.1 |  | 21.2 | 24.2 |

### 9.4.4	NB-IoT RB power dynamic range for NB-IoT operation in NTN NR in-band

#### 9.4.4.1	General

The NB-IoT RB power dynamic range (or NB-IoT power boosting) is the difference between the average power of NB-IoT REs (which occupy certain REs within a NR transmission bandwidth configuration plus 15 kHz at each edge but not within the NR minimum guard band GBChannel) and the average power over all REs (from both NB-IoT and the NR carrier containing the NB-IoT REs).

#### 9.4.4.2	Minimum Requirement

NB-IoT RB power dynamic range for NB-IoT operation in NTN NR in-band shall be larger than or equal to the level specified in Table 6.3.4.2-1. This power dynamic range level is only required for one NB-IoT RB.

Table 9.4.4.2-1: NB-IoT RB power dynamic range for NB-IoT operation in NTN NR in-band

| BS channel bandwidth (MHz) | NB-IoT RB frequency position | NB-IoT RB power dynamic range (dB) |
| --- | --- | --- |
| 5, 10 | Any | +6 |
| 15 | Within center 77*180kHz+15kHz at each edge | +6 |
|  | Other | +3 |
| 20 | Within center 102*180kHz+15kHz at each edge | +6 |
|  | Other | +3 |
| 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | Within center 90% of BS channel bandwidth | +6 |
|  | Other | +3 |

## << End change >>