3GPP TSG-RAN WG4 Meeting #116bis                                                                                                      R4-2513155

Prague, CZ, 13rd –17th October, 2025

Source: 	CATT

Title: 	Discussion on SAN conformance tests for NR NTN Ku-band

Agenda item:	5.10.4

Document for:	Approval

- Introduction

In this contribution, we provide our views on SAN conformance tests for Ku-band for NR NTN.

- Discussion

Measurement uncertainties

The operating band for FR1-NTN Ku-band and FR2-NTN Ku-band are defined in Table 5.2-1 and Table 5.2-2 of TS 38.108 copied as below:

Table 5.2-1: Satellite operating bands in FR1-NTN

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n248 | 14000 MHz - 14500 MHz | 10700 MHz – 127503,4 MHz | FDD |
| n247 | 13750 MHz - 14000 MHz | 10700 MHz – 127503,4 MHz | FDD |

Table 5.2-2: Satellite operating bands in FR2-NTN

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n509 | 14000 MHz - 14500 MHz | 10700 MHz – 12750 MHz4,5 | FDD |
| n508 | 13750 MHz - 14000 MHz | 10700 MHz – 12750 MHz4,5 | FDD |

The MU for 13.75GHz-14.5GHz for FR1-NTN Ku-band and FR2-NTN Ku-band OTA receiver tests, and The MU for 10.7GHz-12.75GHz for FR1-NTN Ku-band and FR2-NTN Ku-band OTA transmitter tests need inputs from TE vendors.

Proposal 1: The MU for 13.75GHz-14.5GHz for FR1-NTN Ku-band and FR2-NTN Ku-band OTA receiver tests, and the MU for 10.7GHz-12.75GHz for FR1-NTN Ku-band and FR2-NTN Ku-band OTA transmitter tests need inputs from TE vendors.

Manufacturer declarations

Due to introduce the requirement for SAN type 1-O operating above 10GHz to support FR1-NTN Ku-band, in this case, some requirements are applicable for SAN type 1-O operating below 10GHz, but they are not applicable for SAN type 1-O operating above 10GHz. For example, the OTA sensitivity is applicable for SAN type 1-O operating below 10GHz, but it is not applicable for SAN type 1-O operating above 10GHz. So the manufacturer declaration related OSDD is not applicable for for SAN type 1-O operating above 10GHz, and is just applicable for SAN type 1-O operating below 10GHz. So manufacturer declaration need to differentiate SAN type 1-O operating below 10GHz and SAN type 1-O operating above 10GHz as below example table.

| Declaration identifier | Declaration | Description | Applicability (Note 1) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SAN type 1-H (Note 2) | SAN type 1-O operating below 10GHz | SAN type 1-O operating above 10GHz | SAN type 2-O |
| D.1 | Coordinate system reference point | Location of coordinated system reference point in reference to an identifiable physical feature of the SAN enclosure. | x | x | x | x |
| D.2 | Coordinate system orientation | Orientation of the coordinate system in reference to an identifiable physical feature of the SAN enclosure. | x | x | x | x |
| … | … | … | .. | … | … | … |
| D.19 | OSDD identifier | A unique identifier for the OSDD. | x | x | n/a | n/a |
| D.20 | OSDD operating band support | Operating band supported by the OSDD, declared for every OSDD (D.19). (Note 5) | x | x | n/a | n/a |
| … | … | … | .. | … | … | … |

Proposal 2: To define manufacturer declaration for SAN type 1-O operating below 10GHz and SAN type 1-O operating above 10GHz as following example table.

| Declaration identifier | Declaration | Description | Applicability (Note 1) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SAN type 1-H (Note 2) | SAN type 1-O operating below 10GHz | SAN type 1-O operating above 10GHz | SAN type 2-O |
| D.1 | Coordinate system reference point | Location of coordinated system reference point in reference to an identifiable physical feature of the SAN enclosure. | x | x | x | x |
| D.2 | Coordinate system orientation | Orientation of the coordinate system in reference to an identifiable physical feature of the SAN enclosure. | x | x | x | x |
| … | … | … | .. | … | … | … |
| D.19 | OSDD identifier | A unique identifier for the OSDD. | x | x | n/a | n/a |
| D.20 | OSDD operating band support | Operating band supported by the OSDD, declared for every OSDD (D.19). (Note 5) | x | x | n/a | n/a |
| … | … | … | .. | … | … | … |

Test Signal l used to build Test Configurations

In TS 38.181, the test signal used to build Test configurations for SAN type 1-H and SAN type 1-O defined in Table 4.7.2-1 of TS 38.181 is copied as below:

Table 4.7.2-1: Signal to be used to build NR TCs for SAN type 1-H and SAN type 1-O

| Operating Band characteristics |  | FDL_high – FDL_low <100 MHz |
| --- | --- | --- |
| TC signal | BWchannel | 5 MHz (Note 1, 2 and 3) |
| characteristics | Subcarrier spacing | Smallest supported subcarrier spacing |
| NOTE 1: If this channel bandwidth is not supported, the narrowest supported channel bandwidth shall be used. NOTE 2: For NRTC1, if NB-IoT is not supported, the signal's channel bandwidth shall be the narrowest supported one. NOTE 3: It should use 3MHz channel bandwidth, if supported. |  |  |

For Operating band in FR1-NTN Ku-band, maximum FDL_high – FDL_low is 12750-10700MHz = 2050MHz, so 2050MHz does not meet FDL_high – FDL_low <100 MHz, so to support FR1-NTN Ku-band, it is needed to update it FDL_high – FDL_low ≤2050MHz as following table.

Table 4.7.2-1: Signal to be used to build NR TCs for SAN type 1-H and SAN type 1-O

| Operating Band characteristics |  | FDL_high – FDL_low ≤2050 MHz |
| --- | --- | --- |
| TC signal | BWchannel | 5 MHz (Note 1, 2 and 3) |
| characteristics | Subcarrier spacing | Smallest supported subcarrier spacing |
| NOTE 1: If this channel bandwidth is not supported, the narrowest supported channel bandwidth shall be used. NOTE 2: For NRTC1, if NB-IoT is not supported, the signal's channel bandwidth shall be the narrowest supported one. NOTE 3: It should use 3MHz channel bandwidth, if supported. |  |  |

Proposal 3: Update FDL_high – FDL_low <100 MHz  to FDL_high – FDL_low ≤2050MHz for test signal used to build Test configurations for SAN type 1-H and SAN type 1-O as following table.

Table 4.7.2-1: Signal to be used to build NR TCs for SAN type 1-H and SAN type 1-O

| Operating Band characteristics |  | FDL_high – FDL_low ≤2050 MHz |
| --- | --- | --- |
| TC signal | BWchannel | 5 MHz (Note 1, 2 and 3) |
| characteristics | Subcarrier spacing | Smallest supported subcarrier spacing |
| NOTE 1: If this channel bandwidth is not supported, the narrowest supported channel bandwidth shall be used. NOTE 2: For NRTC1, if NB-IoT is not supported, the signal's channel bandwidth shall be the narrowest supported one. NOTE 3: It should use 3MHz channel bandwidth, if supported. |  |  |

- Conclusion

This contribution provides analysis on SAN conformance tests for NR NTN Ku-band, and the following proposal is concluded as follows:

Proposal 1: The MU for 13.75GHz-14.5GHz for FR1-NTN Ku-band and FR2-NTN Ku-band OTA receiver tests, and the MU for 10.7GHz-12.75GHz for FR1-NTN Ku-band and FR2-NTN Ku-band OTA transmitter tests need inputs from TE vendors.

Proposal 2: To define manufacturer declaration for SAN type 1-O operating below 10GHz and SAN type 1-O operating above 10GHz as following example table.

| Declaration identifier | Declaration | Description | Applicability (Note 1) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SAN type 1-H (Note 2) | SAN type 1-O operating below 10GHz | SAN type 1-O operating above 10GHz | SAN type 2-O |
| D.1 | Coordinate system reference point | Location of coordinated system reference point in reference to an identifiable physical feature of the SAN enclosure. | x | x | x | x |
| D.2 | Coordinate system orientation | Orientation of the coordinate system in reference to an identifiable physical feature of the SAN enclosure. | x | x | x | x |
| … | … | … | .. | … | … | … |
| D.19 | OSDD identifier | A unique identifier for the OSDD. | x | x | n/a | n/a |
| D.20 | OSDD operating band support | Operating band supported by the OSDD, declared for every OSDD (D.19). (Note 5) | x | x | n/a | n/a |
| … | … | … | .. | … | … | … |

Proposal 3: Update FDL_high – FDL_low <100 MHz  to FDL_high – FDL_low ≤2050MHz  for test signal used to build Test configurations for SAN type 1-H and SAN type 1-O as following table.

Table 4.7.2-1: Signal to be used to build NR TCs for SAN type 1-H and SAN type 1-O

| Operating Band characteristics |  | FDL_high – FDL_low ≤2050 MHz |
| --- | --- | --- |
| TC signal | BWchannel | 5 MHz (Note 1, 2 and 3) |
| characteristics | Subcarrier spacing | Smallest supported subcarrier spacing |
| NOTE 1: If this channel bandwidth is not supported, the narrowest supported channel bandwidth shall be used. NOTE 2: For NRTC1, if NB-IoT is not supported, the signal's channel bandwidth shall be the narrowest supported one. NOTE 3: It should use 3MHz channel bandwidth, if supported. |  |  |

- References

- RP-250799, Revised WID: Introduction of Ku bands for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, Airbus, Sharp, Mitre, JSAT, RAN#107