3GPP TSG-RAN WG4 Meeting #116bis	R4-2513157

Prague, Czech Republic, Oct 13th – 17th , 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-2 | CR | 0667 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_BS_RF_req_evo-Perf)CR for TS 38.141-2, On conformance testing requirement for EEIRP for upper 6GHz |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_BS_RF_req_evo-Perf |  |  |  |  |  | Date: |  |  | 2025-09-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Delaration of Θmin, Θmax, Φmax, Φmin for test beam directions set for EEIRP is unclear. The angle and coordinate system in current test procedure and averaging processes is unclear. This CR is on the top of agreed CR R4-2511672 in RAN4#116, |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Update the manufacturer declaration for EEIRP. 2) Update the angle and coordinate system used in current test procedure and averaging processes. 3) Add “mechanical tilts are defined with respect to the horizon with elevation angles of boresight direction of BS enclosure above the horizon negative and below the horizon positive.” In Table 6.9.5.1-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testing requirement for EEIRP for upper 6GHz would be unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6, 4.9.3.2, 6.9.4.2, 6.9.5.1, N.2.1, N.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

4.6	Manufacturer's declarations

The following BS manufacturer's declarations listed in table 4.6-1, when applicable to the BS under test, are required to be provided by the manufacturer for radiated requirements testing for BS type 1-H, BS type 1-O and BS type 2-O.

For the BS type 1-H declarations required for the conducted requirements testing, refer to TS 38.141-1 [3], clause 4.6.

Table 4.6-1 Manufacturers declarations for BS type 1-H, BS type 1-O and BS type 2-O radiated test requirements

| Declaration identifier | Declaration | Description | Applicability (Note 1) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | BS type 1-H (Note 2) | BS type 1-O | BS type 2-O |
| D.1 | Coordinate system reference point | Location of coordinated system reference point in reference to an identifiable physical feature of the BS enclosure. | x | x | x |
| D.2 | Coordinate system orientation | Orientation of the coordinate system in reference to an identifiable physical feature of the BS enclosure. | x | x | x |
| D.3 | Beam identifier | A unique title to identify a beam, e.g. a, b, c or 1, 2, 3. The vendor may declare any number of beams with unique identifiers. The minimum set to declare for conformance, corresponds to the beams at the reference beam direction with the highest intended EIRP, and covering the properties listed below: 1) A beam with the narrowest intended BeWθ and narrowest intended BeWϕ possible when narrowest intended BeWθ is used. 2) A beam with the narrowest intended BeWϕ and narrowest intended BeWθ possible when narrowest intended BeWϕ is used. 3) A beam with the widest intended BeWθ and widest intended BeWϕ possible when widest intended BeWθ is used. 4) A beam with the widest intended BeWϕ and widest intended BeWθ possible when widest intended BeWϕ is used. 5) A beam which provides the highest intended EIRP of all possible beams. When selecting the above five beam widths for declaration, all beams that the BS is intended to produce shall be considered, including beams that during operation may be identified by any kind of cell or UE specific reference signals, with the exception of any type of beam that is created from a group of transmitters that are not all phase synchronised. (Note 3) | x | x | x |
| D.4 | Operating bands and frequency ranges | List of NR operating band(s) supported by the BS and if applicable, frequency range(s) within the operating band(s) that the BS can operate in. Supported bands declared for every beam (D.3). (Note 4) | c | x | x |
| D.5 | BS requirements set | Declaration of one of the NR base station requirement's set as defined for BS type 1-H, BS type 1-O, or BS type 2-O. | c | x | x |
| D.6 | BS class | Declared as Wide Area BS, Medium Range BS, or Local Area BS. | c | x | x |
| D.7 | BS channel band width and SCS support | BS supported SCS and channel bandwidth per supported SCS. Declared for each beam (D.3) and each operating band (D.4). | c | x | x |
| D.8 | OTA peak directions set reference beam direction pair | The beam direction pair, describing the reference beam peak direction and the reference beam centre direction. Declared for every beam (D.3). | x | x | x |
| D.9 | OTA peak directions set | The OTA peak directions set for each beam. Declared for every beam (D.3). | x | x | x |
| D.10 | OTA peak directions set maximum steering direction(s) | The beam direction pair(s) corresponding to the following points: 1) Φmax: The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive Φ direction, while the θ value being the closest possible to the reference beam centre direction. 2) Φmin: The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative Φ direction, while the θ value being the closest possible to the reference beam centre direction. 3) θmax: The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive θ direction, while the Φ value being the closest possible to the reference beam centre direction. 4) θmin: The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative θ direction, while the Φ value being the closest possible to the reference beam centre direction. The maximum steering direction(s) may coincide with the reference beam centre direction. Declared for every beam (D.3). When θmax, θmin, Φmax, Φmin are used for the EEIRP requirement in subclause 6.9, manufacturer shall re-declare θmax, θmin, Φmax, Φmin for an electrical down tilt, where an electrical down tilt is decided by combination of mechanical down tilt and electrical down tilt (D.64). | x | x | x |
| D.11 | Rated beam EIRP | The rated EIRP level per carrier (Prated,c,EIRP) at the beam peak direction associated with a particular beam direction pair for each of the declared maximum steering directions (D.10), as well as the reference beam direction pair (D.8). Declared for every beam (D.3). (Note 12, 14, 18, 20) | x | x | x |
| D.12 | Beamwidth | The beamwidth for the reference beam direction pair and the four maximum steering directions. Declared for every beam (D.3). | x | x | x |
| D.13 | Equivalent beams | List of beams which are declared to be equivalent. Equivalent beams imply that the beams are expected to have identical OTA peak directions sets and intended to have identical spatial properties at all steering directions within the OTA peak directions set when presented with identical signals. All declarations (D.4 – D.12) made for the beams are identical and the transmitter unit, RDN and antenna array responsible for generating the beam are of identical design. | x | x | x |
| D.14 | Parallel beams | List of beams which have been declared equivalent (D.13) and can be generated in parallel using independent RF power resources. Independent power resources mean that the beams are transmitted from mutually exclusive transmitter units. | x | x | x |
| D.15 | Number of carriers at maximum TRP | The number of carriers per operating band the BS is capable of generating at maximum TRP declared for every beam (D.3). | n/a | x | x |
| D.16 | Operating bands with multi-band dependencies | List of operating bands which are generated using transceiver units supporting operation in multiple operating bands through common active RF components. Declared for each operating band for which multi-band transceiver is used. | c | x | n/a |
| D.17 | Maximum radiated Base Station RF Bandwidth | Maximum Base Station RF Bandwidth in the operating band, declared for each supported operating band (D.4). (Note 15) | c | x | x |
| D.18 | Maximum Radio Bandwidth of the operating band with multi-band dependencies | Largest Radio Bandwidth that can be supported by the operating bands with multi-band dependencies. Declared for each supported operating band which has multi-band dependencies (D.16). | c | x | n/a |
| D.19 | Total RF bandwidth (BWtot) | Total RF bandwidth BWtot of transmitter and receiver, declared per the band combinations (D.52). | c | x | x |
| D.20 | CA-only operation | Declared of CA-only (with equal power spectral density among carriers) but not multiple carriers operation, declared per operating band (D.4) and per beam (D.3). | c | x | x |
| D.21 | Maximum number of supported carriers per operating band in multi-band operations | Maximum number of supported carriers per supported operating band declared to have multi-band dependencies (D.16). | c | x | n/a |
| D.22 | Contiguous or non-contiguous spectrum operation support | Ability of BS to support contiguous or non-contiguous (or both) frequency distribution of carriers when operating multi-carrier in an operating band. | c | x | x |
| D.23 | OSDD identifier | A unique identifier for the OSDD. | x | x | n/a |
| D.24 | OSDD operating band support | Operating band supported by the OSDD, declared for every OSDD (D.23). (Note 5) | x | x | n/a |
| D.25 | OTA sensitivity supported BS channel bandwidth and SCS | The BS supported SCS and channel bandwidth per supported SCS by each OSDD. | x | x | n/a |
| D.26 | Redirection of receiver target support | Ability to redirect the receiver target related to the OSDD. | x | x | n/a |
| D.27 | Minimum EIS for FR1 (EISminSENS) | The minimum EISminSENS requirement (i.e. maximum allowable EIS value) applicable to all sensitivity RoAoA per OSDD. Declared per NR supported channel BW for the OSDD (D.30). The lowest EIS value for all the declared OSDD's is called minSENS, while its related range of angles of arrival is called minSENS RoAoA. (Note 6) | x | x | n/a |
| D.28 | EIS REFSENS for FR2 (EISREFSENS_50M) | The EISREFSENS_50M level applicable in the OTA REFSENS RoAoA, (used as a basis for the derivation of the FR2 EISREFSENS for other channel bandwidths supported by BS). (Note 7) | n/a | n/a | x |
| D.29 | Receiver target reference direction Sensitivity Range of Angle of Arrival | The sensitivity RoAoA associated with the receiver target reference direction (D.31) for each OSDD. | x | x | n/a |
| D.30 | Receiver target redirection range | For each OSDD the associated union of all the sensitivity RoAoA achievable through redirecting the receiver target related to the OSDD. (Note 8) | x | x | n/a |
| D.31 | Receiver target reference direction | For each OSDD an associated direction inside the receiver target redirection range (D.30). (Note 9) | x | x | n/a |
| D.32 | Conformance test directions sensitivity RoAoA | For each OSDD that includes a receiver target redirection range, four sensitivity RoAoA comprising the conformance test directions (D.33). | x | x | n/a |
| D.33 | Conformance test directions | For each OSDD four conformance test directions. If the OSDD includes a receiver target redirection range the following four directions shall be declared: 1) The direction determined by the maximum φ value achievable inside the receiver target redirection range, while θ value being the closest possible to the receiver target reference direction. 2) The direction determined by the minimum φ value achievable inside the receiver target redirection range, while θ value being the closest possible to the receiver target reference direction. 3) The direction determined by the maximum θ value achievable inside the receiver target redirection range, while φ value being the closest possible to the receiver target reference direction. 4) The direction determined by the minimum θ value achievable inside the receiver target redirection range, while φ value being the closest possible to the receiver target reference direction. If an OSDD does not include a receiver target redirection range the following 4 directions shall be declared: 1) The direction determined by the maximum φ value achievable inside the sensitivity RoAoA, while θ value being the closest possible to the receiver target reference direction. 2) The direction determined by the minimum φ value achievable inside the sensitivity RoAoA, while θ value being the closest possible to the receiver target reference direction. 3) The direction determined by the maximum θ value achievable inside the sensitivity RoAoA, while φ value being the closest possible to the receiver target reference direction. 4) The direction determined by the minimum θ value achievable inside the sensitivity RoAoA, while φ value being the closest possible to the receiver target reference direction. | x | x | n/a |
| D.34 | OTA coverage range | Declared as a single range of directions within which selected TX OTA requirements are intended to be met. (Note 10) | x | x | x |
| D.35 | OTA coverage range reference direction | The direction describing the reference direction of the OTA converge range (D.34). (Note 11) | x | x | x |
| D.36 | OTA coverage range maximum directions | The directions corresponding to the following points: 1) The direction determined by the maximum φ value achievable inside the OTA coverage range, while θ value being the closest possible to the OTA coverage range reference direction. 2) The direction determined by the minimum φ value achievable inside the OTA coverage range, while θ value being the closest possible to the OTA coverage range reference direction. 3) The direction determined by the maximum θ value achievable inside the OTA coverage range, while φ value being the closest possible to the OTA coverage range reference direction. 4) The direction determined by the minimum θ value achievable inside the OTA coverage range, while φ value being the closest possible to the OTA coverage range reference direction. | x | x | x |
| D.37 | The rated carrier OTA BS power, Prated,c,TRP | Prated,c,TRP is declared as TRP OTA power per carrier, declared per supported operating band. (Note 12, 14, 18, 20) | n/a | x | x |
| D.38 | Rated transmitter TRP, Prated,t,TRP | Rated total radiated output power. Declared per supported operating band. (Note 12,14, 18, 20) | n/a | x | x |
| D.39 | CLTA placement for co-location test | The manufacturer shall declare the side of EUT where radiating elements are placed closest to the edge of EUT when applicable. The CLTA shall be placed at the EUT side where radiating elements are placed closest. | n/a | x | n/a |
| D.40 | Spurious emission category | Declare the BS spurious emission category as either category A or B with respect to the limits for spurious emissions, as defined in Recommendation ITU-R SM.329 [5]. | c | x | x |
| D.41 | Additional operating band unwanted emissions | The manufacturer shall declare whether the BS under test is intended to operate in geographic areas where the additional operating band unwanted emission limits defined in clause 6.7.4 apply. (Note 16, Note 19) | c | x | x |
| D.42 | Co-existence with other systems | The manufacturer shall declare whether the BS under test is intended to operate in geographic areas where one or more of the systems GSM850, GSM900, DCS1800, PCS1900, UTRA FDD, UTRA TDD, E-UTRA, PHS and/or NR operating in another operating band are deployed. | c | x | x |
| D.43 | Co-location with other base stations | The manufacturer shall declare whether the BS under test is intended to operate co-located with Base Stations of one or more of the systems GSM850, GSM900, DCS1800, PCS1900, UTRA FDD, UTRA TDD, E-UTRA and/or NR operating in another operating band. | c | x | n/a |
| D.44 | Single-band RIB or multi-band RIB | List of single-band RIB and/or multi-band RIB for the supported operating bands (D.4). | c | x | n/a |
| D.45 | Single or multiple carrier | BS capability to operate with a single carrier (only) or multiple carriers. Declared per supported operating band, per RIB. (Note 17) | c | x | x |
| D.46 | Maximum number of supported carriers per operating band | Maximum number of supported carriers. Declared per supported operating band, per RIB. (Note 15) | c | x | x |
| D.47 | Total maximum number of supported carriers | Maximum number of supported carriers for all supported operating bands. Declared per RIB. | c | x | x |
| D.48 | Other band combination multi-band restrictions | Declare any other limitation under simultaneous operation in the declared band combinations (D.16), which have any impact on the test configuration generation. | c | x | n/a |
| D.49 | Ncells | Number corresponding to the minimum number of cells that can be transmitted by a BS in a particular operating band. Declared per operating band (D.4). | c | n/a | n/a |
| D.50 | Maximum supported power difference between carriers | Maximum supported power difference between carriers in each supported operating band. Declared per operating band (D.4). | c | x | x |
| D.51 | Maximum supported power difference between carriers is different operating bands | Maximum supported power difference between any two carriers in any two different supported operating bands. Declared per operating bands combination (D.52). | c | x | n/a |
| D.52 | Operating band combination support | List of operating bands combinations supported by single-band RIB(s) and/or multi-band RIB(s) of the BS. | c | x | n/a |
| D.53 | OTA REFSENS RoAoA | Range of angles of arrival associated with the OTA REFSENS. | n/a | x | x |
| D.54 | OTA REFSENS receiver target reference direction | Reference direction inside the OTA REFSENS RoAoA (D.53). | n/a | x | x |
| D.55 | OTA REFSENS conformance test directions | The following four OTA REFSENS conformance test directions shall be declared: 1) The direction determined by the maximum φ value achievable inside the OTA REFSENS RoAoA, while θ value being the closest possible to the OTA REFSENS receiver target reference direction. 2) The direction determined by the minimum φ value achievable inside the OTA REFSENS RoAoA, while θ value being the closest possible to the OTA REFSENS receiver target reference direction. 3) The direction determined by the maximum θ value achievable inside the OTA REFSENS RoAoA, while φ value being the closest possible to the OTA REFSENS receiver target reference direction. 4) The direction determined by the minimum θ value achievable inside the OTA REFSENS RoAoA, while φ value being the closest possible to the OTA REFSENS receiver target reference direction. | n/a | x | x |
| D.56 | Supported frequency range of the NR operating band | List of supported frequency ranges representing fractional bandwidths (FBW) of operating bands with FBW larger than 6%. | x | x | x |
| D.57 | Rated beam EIRP at lower end of the fractional bandwidth (Prated,c,FBWlow) | The rated EIRP level per carrier at lower frequency range of the fractional bandwidth (Prated,c,FBWlow), at the beam peak direction associated with a particular beam direction pair for each of the declared maximum steering directions (D.10), as well as the reference beam direction pair (D.8). Declared per beam for all supported frequency ranges (D.56). (Note 12, 13, 14, 15, 18, 20) | x | x | x |
| D.58 | Rated beam EIRP at higher frequency range of the fractional bandwidth (Prated,c,FBWhigh) | The rated EIRP level per carrier at higher frequency range of the fractional bandwidth (Prated,c,FBWhigh), at the beam peak direction associated with a particular beam direction pair for each of the declared maximum steering directions (D.10), as well as the reference beam direction pair (D.8). Declared per beam for all supported frequency ranges in (D.56). (Note 12, 13, 14 ,15, 18, 20) | x | x | x |
| D.59 | Relation between supported maximum RF bandwidth, number of carriers and Rated maximum TRP | If the rated transmitter TRP and total number of supported carriers are not simultaneously supported, the manufacturer shall declare the following additional parameters: - The reduced number of supported carriers at the rated transmitter TRP; - The reduced total output power at the maximum number of supported carriers. | n/a | x | x |
| D.60 | Inter-band CA | Declaration of operating band(s) combinations supporting inter-band CA. Declared per operating band combination (D.52). | c | x | x |
| D.61 | Intra-band contiguous CA | Declaration of operating band(s) supporting intra-band contiguous CA. Declared per operating band with CA support. | c | x | x |
| D.62 | Intra-band non-contiguous CA | Declaration of operating band(s) supporting intra-band non-contiguous CA. Declared per operating band with CA support. | c | x | x |
| D.63 | Total maximum number of supported carriers in multi-band operation | Maximum number of supported carriers for all supported operating bands declared to have multi-band dependencies (D.16). | c | x | n/a |
| D.64 | The mechanical down-tilt | For EEIRP the declaration of combination of mechanical down tilt and electrical down tilt. And mechanical tilts are defined with respect to the horizon with elevation angles of boresight direction of BS enclosure above the horizon negative and below the horizon positive.(NOTE 21) | x | x | n/a |
| D.100 | PUSCH mapping type | Declaration of the supported PUSCH mapping type for FR1 as specified in TS 38.211 [20], i.e., type A, type B or both. | c | x | n/a |
| D.101 | PUSCH additional DM-RS positions | Declaration of the supported additional DM-RS position(s) for FR2, i.e., pos0, pos1, or both. | n/a | n/a | x |
| D.102 | PUCCH format | Declaration of the supported PUCCH format(s) as specified in TS 38.211 [20], i.e., format 0, format 1, format 2, format 3, format 4. | c | x | x |
| D.103 | PRACH format and SCS | Declaration of the supported PRACH format(s) as specified in TS 38.211 [20], i.e., format: 0, A1, A2, A3, B4, C0, C2. Declaration of the supported SCS(s) per supported PRACH format with short sequence, as specified in TS 38.211 [20], i.e.: - For BS type 1-O: 15 kHz, 30 kHz or both. - For BS type 2-O: 60 kHz, 120 kHz or both. | c | x | x |
| D.104 | Additional DM-RS for PUCCH format 3 | Declaration of the supported additional DM-RS for PUCCH format 3: without additional DM-RS, with additional DM-RS or both. | c | x | x |
| D.105 | Additional DM-RS for PUCCH format 4 | Declaration of the supported additional DM-RS for PUCCH format 4: without additional DM-RS, with additional DM-RS or both. | c | x | x |
| D.106 | PUSCH PT-RS | Declaration of PT-RS in PUSCH support: without PT-RS, with PT-RS or both. | n/a | n/a | x |
| D.107 | PUCCH multi-slot | Declaration of multi-slot PUCCH support. | c | x | n/a |
| D.108 | UL CA | For the highest supported SCS, declaration of the carrier combination with the largest aggregated bandwidth. If there is more than one combination, the carrier combination with the largest number of carriers shall be declared. | c | x | x |
| D.109 | High speed train | Declaration of high speed train scenario support, i.e. HST support or no HST support | c | x | x |
| D.110 | Maximum speed of high speed train for PUSCH | Declaration of supported maximum speed for high speed train scenario, i.e. 350 km/h or 500 km/h. This declaration is applicable to PUSCH for high speed train and UL timing adjustment only if BS declares to support high speed train in D.109. | c | x | n/a |
| D.111 | PRACH format for high speed train | Declaration of supported PRACH format(s) for high speed train scenario, i.e. format 0 restricted set type A, format 0 restricted set type B, format A2, format B4, format C2. This declaration is applicable to PRACH for high speed train only if BS declares to support high speed train in D.109. | c | x | x |
| D.112 | Interlaced formats | Declaration of support of interlaced PUSCH and PUCCH formats, i.e. interlaced format support or no interlaced format support. | c | x | n/a |
| D.113 | PRACH format with LRA = 1151 for 15 kHz SCS and LRA = 571 for 30 kHz SCS | Declaration of the supported PRACH format(s) as specified in TS 38.211 [17], i.e., format: A2, B4, C2. Declaration of the supported SCS(s) per supported PRACH format as specified in TS 38.211 [17], i.e., 15 kHz, 30 kHz or both. | c | x | n/a |
| D.114 | CG-UCI | Declaration of support of GC-UCI multiplexed on PUSCH as specified in TS 38.211 [17]. | c | x | n/a |
| D.115 | 2-step RA | Declaration of support of 2-step RA type. | c | x | x |
| D.116 | PUSCH 256QAM | Declaration of PUSCH 256QAM support | c | x | x |
| D.117 | Additional DM-RS for FR2 high speed train | Declaration of supported additional DM-RS position(s) for FR2 high speed train scenario for PUSCH and UL timing adjustment, i.e., pos0, pos1, pos2, or any combination | n/a | n/a | x |
| D.118 | PUCCH sub-slot based repetition formats | Declaration of PUCCH sub-slot based repetition formats. | c | x | n/a |
| D.119 | PUSCH TB over multi-slots | BS support TBoMS over physical consecutive UL slots | c | x | x |
| D.120 | PUSCH TB over multi-slots | BS support TBoMS over physical non-consecutive UL slots | c | x | x |
| D.121 | Supported SCS for TDD PUSCH DM-RS bundling and PUCCH DM-RS bundling | Declaration of supported SCS for TDD PUSCH DM-RS bundling and and PUCCH DM-RS bundling and, i.e. {15 kHz, 30 kHz, 60 kHz 120 kHz} | c | x | x |
| D.122 | Supported FDD PUSCH DM-RS bundling and and PUCCH DM-RS bundling and | Declaration of supporting FDD PUSCH DM-RS bundling and PUCCH DM-RS bundling | c | x | x |
| D.123 | MCS index table 3 | Declaration of support MCS index table 3 as specified in TS 38.214 [18]. | c | x | n/a |
| D.124 | PUSCH repetition type A | Declaration of support PUSCH repetition type A | c | x | x |
| D.125 | Air-to-ground scenario | Declaration of air-to-ground scenario support, i.e. ATG support or no ATG support | c | x | n/a |
| D.126 | PRACH format and SCS for Multiple PRACH transmission | Declaration of the supported PRACH format(s) as specified in TS 38.211 [20] for Multiple PRACH transmission, i.e., format: A2, B4, C2. Declaration of the supported SCS(s) per supported PRACH format with short sequence for Multiple PRACH transmission, as specified in TS 38.211 [20], i.e.: - For BS type 2-O: 120 kHz. | c | n/a | x |
| D.127 | PUSCH with enhanced DM-RS type | Declaration of support enhanced DM-RS type as specified in TS 38.211 [17]. | c | x | n/a |
| D.128 | PRACH format for HAPS scenario | Declaration of supported PRACH format(s) for HAPS scenario, i.e., format 1. | c | x | n/a |
| NOTE 1: Manufacturer declarations applicable per BS requirement set were marked as "x". Manufacturer declarations not applicable per BS requirement set were marked as "n/a". NOTE 2: For BS type 1-H, the only radiated declarations are related to EIRP and EIS requirements. For BS type 1-H declarations required for the conducted requirements testing, refer to TS 38.141-1 [3]. For declarations marked as 'c', related conducted declarations in TS 38.141-1 [3] apply. When separately declared, they shall still use the same declaration identifier. NOTE 3: Depending on the capability of the system some of these beams may be the same. For those same beams, testing is not repeated. NOTE 4: These operating bands are related to their respective single-band RIBs. NOTE 5: As each identified OSDD has a declared minimum EIS value (D.27), multiple operating band can only be declared if they have the same minimum EIS declaration. NOTE 6: If the BS type 1-H or BS type 1-O is not capable of redirecting the receiver target related to the OSDD then there is only one RoAoA applicable to the OSDD. NOTE 7: Although EISREFSENS_50M level is based on a reference measurement channel with BWChannel = 50 MHz, it does not imply that BS has to support 50 MHz channel bandwidth. NOTE 8: Not applicable for BS type 2-O. NOTE 9: For an OSDD without receiver target redirection range, this is a direction inside the sensitivity RoAoA. NOTE 10: OTA coverage range is used for conformance testing of such TX OTA requirements as occupied bandwidth, frequency error, TAE or EVM. NOTE 11: The OTA coverage reference direction may be the same as the Reference beam direction pair (D.8) but does not have to be. NOTE 12: If a BS type 2-O is capable of 64QAM DL operation but not capable of 256QAM DL operation, then up to two rated output power declarations may be made. One declaration is applicable when configured for 64QAM transmissions and the other declaration is applicable when not configured for 64QAM transmissions. NOTE 13: If D.57 and D.58 are declared for certain frequency range (D.56), there shall be no "Rated beam EIRP" declaration (D.11) for the operating band containing that particular frequency range. NOTE 14: If a BS is capable of 1024QAM DL operation then up to three rated output power declarations may be made. One declaration is applicable when configured for 1024QAM transmissions, a different declaration is applicable when configured 256QAM transmissions and the other declaration is applicable when configured neither for 256QAM nor 1024QAM transmissions. NOTE 15: Parameters for contiguous or non-contiguous spectrum operation in the operating band are assumed to be the same unless they are separately declared. NOTE 16: If BS is declared to support Band n20 (D.4), the manufacturer shall declare if the BS may operate in geographical areas allocated to broadcasting (DTT). Additionally, related declarations of the emission levels and maximum output power shall be declared. NOTE 17: In case of BS type 1-H, this declaration applies per TAB connector. NOTE 18: If a BS type 2-O is capable of 256QAM DL operation, then up to three rated output power declarations may be made. One declaration is applicable when configured for 256QAM transmissions, a different declaration is applicable when configured for 64QAM transmissions and the other declaration is applicable when not configured neither for 256QAM nor 64QAM transmissions. NOTE 19: If BS is declared to support Band n24 (D.4), the manufacturer shall declare if the BS may operate in geographical areas where FCC regulations apply. Additionally, related declarations of the emission levels and maximum output power shall be declared. NOTE 20: If a BS is capable of 256QAM DL operation but not capable of 1024QAM DL operation then up to two rated output power declarations may be made. One declaration is applicable when configured for 256QAM transmissions, and the other declaration is applicable when not configured for 256QAM transmissions NOTE 21: The combination of maximum mechanical down tilt and the maximum electrical down tilt shall be declared. If the maximum electrical down tilt and the maximum mechanical down tilt are not simultaneously supported in application, the manufacturer shall declare and test the following two instances: One instance is combination of maximum mechanical down tilt and the reduced electrical down tilt, and the other instance is combination of maximum electrical down tilt and reduced mechanical down tilt. |  |  |  |  |  |

<End of Change 1>

<Start of Change 2>

4.9.3	Test beam direction set

4.9.3.1	General

Conformance testing of OTA spatial emission requirement is based on a test beam direction set.

4.9.3.2	Test beam direction set for protection of FSS UL

The following beam direction set is utilized for conformance testing of requirements in clause 6.9.5.1 for BS type 1-H and BS type 1-O operating in band n104. The test beam directions are defined in table 4.9.3.2-1.

Table 4.9.3.2-1: Test beam direction set for protection of FSS UL

| Test Beam direction index | Elevation angle | Azimuth angle | Weighting factor |
| --- | --- | --- | --- |
| 1 | Θmin+2.5(Θmax-Θmin)/25 | Φmin+7.5(Φmax-Φmin)/25 | 1/21 |
| 2 | Θmin+2.5(Θmax-Θmin)/25 | Φmin+12.5(Φmax-Φmin)/25 | 1/21 |
| 3 | Θmin+2.5(Θmax-Θmin)/25 | Φmin+17.5(Φmax-Φmin)/25 | 1/21 |
| 4 | Θmin+7.5(Θmax-Θmin)/25 | Φmin+2.5(Φmax-Φmin)/25 | 1/21 |
| 5 | Θmin+7.5(Θmax-Θmin)/25 | Φmin+7.5(Φmax-Φmin)/25 | 1/21 |
| 6 | Θmin+7.5(Θmax-Θmin)/25 | Φmin+12.5(Φmax-Φmin)/25 | 1/21 |
| 7 | Θmin+7.5(Θmax-Θmin)/25 | Φmin+17.5(Φmax-Φmin)/25 | 1/21 |
| 8 | Θmin+7.5(Θmax-Θmin)/25 | Φmin+22.5(Φmax-Φmin)/25 | 1/21 |
| 9 | Θmin+12.5(Θmax-Θmin)/25 | Φmin+2.5(Φmax-Φmin)/25 | 1/21 |
| 10 | Θmin+12.5(Θmax-Θmin)/25 | Φmin+7.5(Φmax-Φmin)/25 | 1/21 |
| 11 | Θmin+12.5(Θmax-Θmin)/25 | Φmin+12.5(Φmax-Φmin)/25 | 1/21 |
| 12 | Θmin+12.5(Θmax-Θmin)/25 | Φmin+17.5(Φmax-Φmin)/25 | 1/21 |
| 13 | Θmin+12.5(Θmax-Θmin)/25 | Φmin+22.5(Φmax-Φmin)/25 | 1/21 |
| 14 | Θmin+17.5(Θmax-Θmin)/25 | Φmin+2.5(Φmax-Φmin)/25 | 1/21 |
| 15 | Θmin+17.5(Θmax-Θmin)/25 | Φmin+7.5(Φmax-Φmin)/25 | 1/21 |
| 16 | Θmin+17.5(Θmax-Θmin)/25 | Φmin+12.5(Φmax-Φmin)/25 | 1/21 |
| 17 | Θmin+17.5(Θmax-Θmin)/25 | Φmin+17.5(Φmax-Φmin)/25 | 1/21 |
| 18 | Θmin+17.5(Θmax-Θmin)/25 | Φmin+22.5(Φmax-Φmin)/25 | 1/21 |
| 19 | Θmin+22.5(Θmax-Θmin)/25 | Φmin+7.5(Φmax-Φmin)/25 | 1/21 |
| 20 | Θmin+22.5(Θmax-Θmin)/25 | Φmin+12.5(Φmax-Φmin)/25 | 1/21 |
| 21 | Θmin+22.5(Θmax-Θmin)/25 | Φmin+17.5(Φmax-Φmin)/25 | 1/21 |
| NOTE 1: Θmin, Θmax, Φmax, Φmin is defined in the declaration D.10. |  |  |  |

The test beam directions relative to Θmin, Θmax, Φmax, Φmin are visualised in Figure 4.9.3.2-1.

Figure 4.9.3.2-1 Test beam direction set for conformance testing

<End of Change 2>

<Start of Change 3>

6.9	OTA spatial emission

6.9.1	Definition and applicability

OTA spatial emission requirements are defined to set upper limits on radiated power in specific directions.

6.9.2	Minimum requirement

The minimum requirement for BS type 1-H and BS type 1-O operation is defined in TS 38.104 [2], clause 9.9.2.

6.9.3	Test purpose

The test purpose is to verify the ability of the transmitter units associated with the RIB under test to radiated power in specified elevation angle ranges with respect to the horizon.

6.9.4	Method of test

6.9.4.1	Initial conditions

Test environment: normal; see annex B.2.

RF channels to be tested for single carrier: M; see clause 4.9.1. SC carrier, the one characterised by all of requirements below:

-	Highest supported Power Spectral Density (PSD) level with the corresponding BW.

In the test range, the EUT is mounted at the positioner with mechanical down-tilt set to zero.

6.9.4.2	Procedure

When calibrated and operated within the guidance of 3GPP TR 37.941 [29] the measurement methods are applicable and selected depending on availability at the test facility.

The test range shall be calibrated according to calibration method described in TR 37.941, clause 8.3.

1. Place the BS at the positioner with mechanical down-tilt set to zero, such that blocking effect in the vertical domain is minimized.

2. Align the manufacturer declared coordinate system orientation (D.2) of the BS for test beam direction(s) in test beam direction set in sub-clause 4.9.3.2 with the coordinate system used by the test system.

3. The measurement device characteristics shall be: Detection mode set to True RMS.

4. Set the BS to transmit on both polarizations according to applicable test configuration in clause 4.8 using the corresponding test model NR-FR1-TM1.1 described in TS 38.141-1 [3], clause 4.9.2. The configured EIRP for each of the test beams shall be in direct relation to the intended use of the BS.

5. Orient the positioner (and the BS) to the positioner angle for measurement point(Θl,Φq) for l=1..L and q=1.. Q, -1800<Φq≤1800 , -900≤Θl≤900, and Θl , Φq are defined in the reference coordinate system defined in sub-clause 4.14(θn,φm)m=1..Mn=1.. . Spatial sampling grid ∆θ for Θl and ∆φ for  Φq is defined in Annex N, clause N.3.

6. Configure test beam direction(s) in test beam direction set and test equipment for measuring test beam k for k=1..K, as defined in clause 4.9.3.2.

7. Measure EIRP as a sum of two orthogonal polarizations:

EIRPkθn,φmEIRP0kΘl,Φq=EIRPp1+EIRPp2 where p1 and p2 denote two orthogonal polarizations.

Follow either the order of steps 8a/9a or 8b/9b

8a. Repeat step 6 and 7 for all K test beam directions for each positioner angle(Θl,Φq).

8b. Repeat step 5 and 7 for all M and N positioner angles for each test beam direction k.

9a. Repeat step 5, and 8a for all M and N positioner angles.

9b. Repeat step 6 and 8b for all K test beam directions.

10. Calculate EEIRPi for mechanical tilt (D.64) from measured EIRP0kΘl,Φq EIRPkθn,φm for bins i=1..7 as described in Annex N.

6.9.5	Test requirements

6.9.5.1	Requirement for BS type 1-H and BS type 1-O

For BS type 1-H and BS type 1-O operating in band n104, the Expected EIRP (EEIRP) in the frequency range 6425 – 7075 MHz, the power measured EEIRP shall not exceed the limits specified in table 6.9.5.1-1.

θ

θ

Table 6.9.5.1-1: EEIRP limits as function of elevation above horizon

| Bin number i | Elevation angular range θHLi ≤ θ < θHHi (Degrees) | EEIRP limit (dBm/MHz) |
| --- | --- | --- |
| 1 | 0≤<5 | 27 |
| 2 | 5≤<10 | 23 |
| 3 | 10≤<15 | 19 |
| 4 | 15≤<20 | 18 |
| 5 | 20≤<<30 | 16 |
| 6 | 30≤<<60 | 15 |
| 7 | 60≤<<90 | 15 |
| Note 1: The requirement shall apply to all supported mechanical tilts. And mechanical tilts are defined with respect to the horizon with elevation angles of boresight direction of BS enclosure above the horizon negative and below the horizon positive. |  |  |

Figure 6.9.5.1-1: Coordinate system for EEIRP and definitions of θHLi and θHHi angles.

<End of Change 3>

<Start of Change 4>

Annex N (normative):
Expected EIRP calculation and spatial sampling grid

N.1	General

The Expected EIRP calculation is based on two averaging processes:

-	Averaging over the test beam directions.

-	Averaging over horizontal and vertical angles within specified vertical angles bins

Clause N.2 describes the details of calculation of the EEIRP from the measured EIRP samples.

The required sampling resolution for the EIRP measurement per test beam direction is described in annex N.3

N.2	Averaging processes

N.2.1	Averaging over beam directions

The first averaging process involves averaging over different test beam directions. For a given vertical angle θ and horizontal angle φ with a sampling of K beamforming directions, the equation can be written as:

EIRPθ,φEIRP0Θl,Φq=1Kk=1KwkEIRPk(θ,φ)EIRP0k(Θl,Φq)

The power measured is the sum of the EIRP in both polarisations. The EIRP0Θl,Φq EIRPθ,φ  is the average EIRP0Θl,Φq EIRPk(θ,φ)over the K number of test beam directions for the positioner angle (Θl,Φq), which is defined  based on reference coordinate system defined in sub-clause 4.14(θ,φ).

The second process is calculating EIRPθn,φm for each mechanical tilt θMT from EIRP0Θl,Φq with zero mechanical tilt using transformation from global angle θn,φm in global coordinate system defined with respect to the horizon in Figure 6.9.5.1-1 to local angle Θl,Φq in local (zero mechanical tilt) reference coordinate system defined in sub-clause 4.14.

For each mechanical tilt θMT (D.64), the EIRPθn,φm can calculate as following:

EIRPθn,φm=EIRP0Θl,Φq

ΘlθH=90-arccoscosφmφsinθnθ+90sin-θMT+cosθnθ+90cos-θMT

ΦqφH=argcosφmφsinθnθ+90cos-θMT-cosθnθ+90sin-θMT+jsinφmφsinθnθ+90

, where

Θlθ	The angle in the reference coordinate system defined in clause 4.14 between the projection of the x/y plane and the radiation vector defined between -90° and 90°. 0° represents the direction perpendicular to the y/z plane. The angle is aligned with the down-tilt angle.

Φqφ 	is the angle in the reference coordinate system defined in clause 4.14 between the x-axis and the projection of the radiation vector onto the x/y plane defined between -180° and 180°.

θnθH 	is the angle with respect to the horizon defined between +90° and -90°, above the horizon is positive below the horizon is negative in Figure 6.9.5.1-1. Note the orientation is opposite to 

θMT	is the down-tilt angle representing the mechanical tilt defined between -90° and +90° (positive towards the ground).

φmφH	  is the horizontal angle in the global coordinate system, defined with respect to the horizon in Figure 6.9.5.1-1.

N.2.2	Averaging over horizontal and vertical angles

The third averaging process is over a spherical strip. This EEIRP for a spherical strip is therefore:

EIRPEEIRPθHLi , θHHi=(θHHi-θHLi)×π180°MNsinθHHi-sinθHLin=nLnHm=1MEIRPθn,φm cosθn    ,                       (2)

, where

(n, m) represents discrete spatial points for EEIRP. The sample resolution is defined in clause N.3.

θHLi , θHHi	is the elevation angular range bounded by θHLi and θHHi in degrees, e.g. for bin 1, this will be 0⁰           and 5⁰ respectively.

θn	is the corresponding angle of the nth midpoint between the elevation intervals, referenced from the horizon. Midpoint is the middle point between the intervals, e.g. for the interval 1°-2°, the midpoint is 1.5°.

φmis the corresponding angle of the mth midpoint between the azimuth intervals

nLis the lowest elevation index of sampling points within the θHLi , θHHi range

nHis the highest elevation index of sampling points within the θHLi , θHHi range

Mis the number of azimuth points within the (-180°, 180°) range

Nis the number of elevation points within the θHLi , θHHi range, and 00≤θHLi ≤900, and 00≤θHHi ≤900

Here,  the θn and  φm are defined based on in Figure 6.9.5.1-1.

NOTE:	All angles are in degrees.

N.3	Spatial sampling grid

The maximum EIRP sampling resolution in degrees required for the EIRP pattern measurement is given by:

∆θ=1∆φ=180πλD

, where  is the wavelength and D is the physical length of the array antenna diagonal. The physical length of the array antenna diagonal can be determined by following expression:

D=w2+h2

, where w is the array antenna width, and h is the array antenna height.

<End of Change 4>