3GPP TSG-RAN WG4 Meeting #116bis                                                                                                      R4-2513158

Prague, CZ, 13rd –17th October, 2025

Source: 	CATT

Title: 	Further discussion on framework simplification for co-location/co-existence requirements

Agenda item:	4.30.3

Document for:	Approval

- Introduction

In RAN4#116 meeting, A Way forward for RAN Task on specification improvements [1] was agreed and remaining issues in WF need further discussion.

In this contribution, we provide our views on framework simplification for co-location/co-existence requirements.

- Discussion

Note 1 for frequency range not applicable

As per WF[1], the remaining issue concerning frequency range not applicable for co-existence/co-location requirements is copied as below:

- At next meetings further check and if needed update notes below the requirement tables. E.g. update the NOTE1 for co-existence requirement as the following to solve the deleting note in note column of original table. NOTE 1: As defined in the scope for spurious emissions in this clause, except for the cases where the noted requirements apply to a BS operating in Band n28, the co-existence requirements in table 6.6.5.5.1.3-1 do not apply for the frequency range extending ΔfOBUE immediately outside the downlink operating band (see TS 38.104 [2], table 5.2-1). Emission limits for this excluded frequency range may be covered by local or regional requirements.

The Note 1(s) for co-existence/co-location requirement in TS 38.104 and TS 36.104 are copied as in below table, the frequency range not applicable are summarized in last column.

| TS | Requirement | Note 1 | Frequency range not applicable. |
| --- | --- | --- | --- |
| 38.104 | Note 1 for co-existencee requirment | NOTE 1: As defined in the scope for spurious emissions in this clause, except for the cases where the noted requirements apply to a BS operating in Band n28, the co-existence requirements in table 6.6.5.2.3 -1 do not apply for the ΔfOBUE frequency range immediately outside the downlink operating band (see table 5.2-1). Emission limits for this excluded frequency range may be covered by local or regional requirements. | FDL_low-ΔfOBUE ~ FDL_low, and FDL_high ~ FDL_high+ΔfOBUE |
| 38.104 | Note 1 for co-location requirement | NOTE 1: As defined in the scope for spurious emissions in this clause, the co-location requirements in table 6.6.5.2.4-1 do not apply for the frequency range extending ΔfOBUE immediately outside the BS transmit frequency range of a downlink operating band (see table 5.2-1). The current state-of-the-art technology does not allow a single generic solution for co-location with other system on adjacent frequencies for 30dB BS-BS minimum coupling loss. However, there are certain site-engineering solutions that can be used. These techniques are addressed in TR 25.942 [4]. | FDL_low-ΔfOBUE ~ FDL_high+ΔfOBUE |
| 36.104 | Note 1 for co-existence requirement | NOTE 1: As defined in the scope for spurious emissions in this clause, except for the cases where the noted requirements apply to a BS operating in Band 25, Band 27, Band 28 or Band 29, the co-existence requirements in Table 6.6.4.3.1-1 do not apply for the 10 MHz frequency range immediately outside the downlink operating band (see Table 5.5-1). Emission limits for this excluded frequency range may be covered by local or regional requirements. | For LTE, 10 MHz is ΔfOBUE. FDL_low-ΔfOBUE ~ FDL_low, and FDL_high ~ FDL_high+ΔfOBUE |
| 36.104 | Note 1 for co-location requirement | NOTE 1: As defined in the scope for spurious emissions in this clause, the co-location requirements in Table 6.6.4.4.1-1 to Table 6.6.4.4.1-3 do not apply for the 10 MHz frequency range immediately outside the BS transmit frequency range of a downlink operating band (see Table 5.5-1). The current state-of-the-art technology does not allow a single generic solution for co-location with other system on adjacent frequencies for 30dB BS-BS minimum coupling loss. However, there are certain site-engineering solutions that can be used. These techniques are addressed in TR 25.942 [8]. | For LTE, 10 MHz is ΔfOBUE. FDL_low-ΔfOBUE ~ FDL_low, and FDL_high ~ FDL_high+ΔfOBUE |

As above, in TS 38.104, the frequency range not applicable for co-existence requirement is FDL_low-ΔfOBUE ~ FDL_low and FDL_high ~ FDL_high+ΔfOBUE, and the frequency range not applicable for co-location requirement is FDL_low-ΔfOBUE ~ FDL_high+ΔfOBUE, they are different.  And in TS 36.104, the frequency range not applicable for co-existence requirement is FDL_low-ΔfOBUE ~ FDL_low and FDL_high ~ FDL_high+ΔfOBUE, and the frequency range not applicable for co-location requirement is also FDL_low-ΔfOBUE ~ FDL_low and FDL_high ~ FDL_high+ΔfOBUE, they are same.

The text “The transmitter spurious emission limits shall apply from 9 kHz to 12.75 GHz, excluding the frequency range from ΔfOBUE below the lowest frequency of each supported downlink operating band, up to ΔfOBUE above the highest frequency of each supported downlink operating band, where the ΔfOBUE is defined in table 6.6.1-1.” in TS 38.104 indicates that frequency range FDL_low-ΔfOBUE ~ FDL_high+ΔfOBUE is not applicable for spurious requirement, so it is also not applicable for co-existence/co-location spurious requirement.

So, we think that “do not apply for the frequency range extending ΔfOBUE immediately outside the BS transmit frequency range of a downlink operating band” in Note 1 for co-location requirement in TS 38.104 is reasonable for co-existence/co-location requirement. At same time, the deleting notes in note column of origin table can be full covered by “requirements do not apply for the frequency range extending ΔfOBUE immediately outside the BS transmit frequency range of a downlink operating band”.

Based on above analysis, we propose,

Proposal 1: Update the NOTE1 for co-existence requirement as the following to solve the deleting note in note column of original table.

NOTE 1:	As defined in the scope for spurious emissions in this clause, except for the cases where the noted requirements apply to a BS operating in Band n28, the co-existence requirements in table 6.6.5.5.1.3-1 do not apply for the frequency range extending ΔfOBUE immediately outside the downlink operating band (see TS 38.104 [2], table 5.2-1). Emission limits for this excluded frequency range may be covered by local or regional requirements.

Missed co-location requirement in exemplified CR for TS 381.41-1

Referring to exemplified CR[2], the simplified co-location requirement table in exemplified CR for TS 381.41-1 is copied as below:

| Frequency range of uplink operating band of the | System type to co-locate with | Basic limits (dBm/100kHz) |  |  | Notes |
| --- | --- | --- | --- | --- | --- |
| co-located BS |  | WA BS | MR BS | LA BS |  |
| 824 - 849 MHz | GSM850 or CDMA850 | -98 | -91 | -70 | NOTE 1 |
| 876 - 915 MHz | GSM900 | -98 | -91 | -70 |  |
| 1710 - 1785 MHz | DCS1800 | -98 | -91 | -80 |  |
| 1850 - 1910 MHz | PCS1900 | -98 | -91 | -80 |  |
| UL frequency range of n51, n91, n93 | NR | N/A | N/A | -88 |  |
| UL frequency range of n53 | NR | N/A | -91 | -88 |  |
| UL frequency range of n100, n101 | NR | -96 | N/A | N/A |  |
| UL frequency range of n96, n102 | NR | N/A | -90 | -87 |  |
| UL frequency range of n104 | NR | -95 | -90 | -87 |  |
| Frequency range of uplink operating band of the co-located BS | Other (NOTE 4, 5) | -96 | -91 | -88 |  |

The typical co-location requirements in Table 6.6.5.5.1.4-1 of TS 38.104 are summarized in the following table:

| Type of co-located BS | Frequency range for | Basic limit |  |  | Measurement | Note |
| --- | --- | --- | --- | --- | --- | --- |
|  | co-location requirement | WA BS | MR BS | LA BS | bandwidth |  |
| GSM900 | 876-915 MHz | -98 dBm | -91 dBm | -70 dBm | 100 kHz |  |
| DCS1800 | 1710 – 1785 MHz | -98 dBm | -91 dBm | -80 dBm | 100 kHz |  |
| PCS1900 | 1850 – 1910 MHz | -98 dBm | -91 dBm | -80 dBm | 100 kHz |  |
| GSM850 or CDMA850 | 824 – 849 MHz | -98 dBm | -91 dBm | -70 dBm | 100 kHz |  |
| UTRA FDD Band I or E-UTRA Band 1 or NR Band n1 | 1920 – 1980 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| E-UTRA Band 46 or NR Band n46 | 5150 – 5925 MHz | N/A | -91 dBm | -88 dBm | 100 kHz |  |
| E-UTRA Band 51 or NR Band n51 | 1427 – 1432 MHz | N/A | N/A | -88 dBm | 100 kHz |  |
| E-UTRA Band 53 or NR Band n53 | 2483.5 – 2495 MHz | N/A | -91 dBm | -88 dBm | 100 kHz |  |
| NR Band n91 | 832 – 862 MHz | N/A | N/A | -88 dBm | 100 kHz |  |
| NR Band n93 | 880 – 915 MHz | N/A | N/A | -88 dBm | 100 kHz |  |
| NR Band n96 | 5925 – 7125 MHz | N/A | -90 dBm | -87 dBm | 100 kHz |  |
| NR Band n100 | 874.4 – 880 MHz | -96 dBm | N/A | N/A | 100 kHz |  |
| NR Band n101 | 1900 – 1910 MHz | -96 dBm | N/A | N/A | 100 kHz |  |
| NR Band n102 | 5925 – 6425 MHz | N/A | -90 dBm | -87 dBm | 100 kHz |  |
| NR Band n104 | 6425 – 7125 MHz | -95 dBm | -90 dBm | -87 dBm | 100 kHz |  |

Comparing the above two tables for TS 38.141-1, we can find that typical co-location requirement for E-UTRA Band 46 or NR Band n46, E-UTRA Band 51, E-UTRA Band 53 was not captured in simplified co-location requirement table in TS 38.141-1. RAN4 need to discuss whether to add them as shown in the table below.

| Frequency range of uplink operating band of the | System type to co-locate with | Basic limits (dBm/100kHz) |  |  | Notes |
| --- | --- | --- | --- | --- | --- |
| co-located BS |  | WA BS | MR BS | LA BS |  |
| 824 - 849 MHz | GSM850 or CDMA850 | -98 | -91 | -70 | NOTE 1 |
| 876 - 915 MHz | GSM900 | -98 | -91 | -70 |  |
| 1710 - 1785 MHz | DCS1800 | -98 | -91 | -80 |  |
| 1850 - 1910 MHz | PCS1900 | -98 | -91 | -80 |  |
| UL frequency range of n51, n91, n93, or UL frequency range of 51 | NR or E-UTRA | N/A | N/A | -88 |  |
| UL frequency range of n46, n53, or UL frequency range of 46, 53 | NR or E-UTRA | N/A | -91 | -88 |  |
| UL frequency range of n100, n101 | NR | -96 | N/A | N/A |  |
| UL frequency range of n96, n102 | NR | N/A | -90 | -87 |  |
| UL frequency range of n104 | NR | -95 | -90 | -87 |  |
| Frequency range of uplink operating band of the co-located BS | Other (NOTE 4, 5) | -96 | -91 | -88 |  |

So, we propose,

Proposal 2: RAN4 to discuss whether to update the simplified table for co-location requirement in endorsed draft CR for TS 38,141-1 as below:

| Frequency range of uplink operating band of the | System type to co-locate with | Basic limits (dBm/100kHz) |  |  | Notes |
| --- | --- | --- | --- | --- | --- |
| co-located BS |  | WA BS | MR BS | LA BS |  |
| 824 - 849 MHz | GSM850 or CDMA850 | -98 | -91 | -70 | NOTE 1 |
| 876 - 915 MHz | GSM900 | -98 | -91 | -70 |  |
| 1710 - 1785 MHz | DCS1800 | -98 | -91 | -80 |  |
| 1850 - 1910 MHz | PCS1900 | -98 | -91 | -80 |  |
| UL frequency range of n51, n91, n93, or UL frequency range of 51 | NR or E-UTRA | N/A | N/A | -88 |  |
| UL frequency range of n46, n53, or UL frequency range of 46, 53 | NR or E-UTRA | N/A | -91 | -88 |  |
| UL frequency range of n100, n101 | NR | -96 | N/A | N/A |  |
| UL frequency range of n96, n102 | NR | N/A | -90 | -87 |  |
| UL frequency range of n104 | NR | -95 | -90 | -87 |  |
| Frequency range of uplink operating band of the co-located BS | Other (NOTE 4, 5) | -96 | -91 | -88 |  |

Missed co-location requirement in exemplified CR for TS 37.104

Referring to exemplified CR[3], the simplified co-location requirement table in exemplified CR for TS 37.104 is copied as below:

| Frequency range of uplink operating band of the | System type to co-locate with | Basic limits (dBm/100kHz) |  |  | Notes |
| --- | --- | --- | --- | --- | --- |
| co-located BS |  | WA BS | MR BS | LA BS |  |
| 824 - 849 MHz | GSM850 or CDMA850 | -98 | -91 | -70 | NOTE 1 |
| 876 - 915 MHz | GSM900 | -98 | -91 | -70 |  |
| 1710 - 1785 MHz | DCS1800 | -98 | -91 | -80 |  |
| 1850 - 1910 MHz | PCS1900 | -98 | -91 | -80 |  |
| UL frequency range of n51, n91, n93 | NR | N/A | N/A | -88 |  |
| UL frequency range of n53 | NR | N/A | -91 | -88 |  |
| UL frequency range of n100, n101 | NR | -96 | N/A | N/A |  |
| UL frequency range of n96, n102 | NR | N/A | -90 | -87 |  |
| UL frequency range of n104 | NR | -95 | -90 | -87 |  |
| Frequency range of uplink operating band of the co-located BS | Other (NOTE 4, 5) | -96 | -91 | -88 |  |

The typical co-location requirements in Table 6.6.1.4.1-1 of TS 37.104 are summarized in the following table:

| Type of co-located BS | Frequency range for co-location requirement | Maximum Level (WA-BS) | Maximum Level (MR-BS) | Maximum Level (LA-BS) | Measurement Bandwidth | Note |
| --- | --- | --- | --- | --- | --- | --- |
| GSM900 | 876-915 MHz | -98 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| DCS1800 | 1710 - 1785 MHz | -98 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| PCS1900 | 1850 - 1910 MHz | -98 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| GSM850 or CDMA850 | 824 - 849 MHz | -98 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA FDD Band I or E-UTRA Band 1 or NR Band n1 | 1920 - 1980 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| E-UTRA Band 46 or NR Band n46 | 5150 – 5925 MHz | N/A | -91 dBm | -88 dBm | 100 kHz |  |
| E-UTRA Band 49 | 3550 – 3700 MHz | N/A | N/A | -88 dBm | 100 kHz |  |
| E-UTRA Band 51 or NR Band n51 | 1427 – 1432 MHz | N/A | N/A | -88 dBm | 100 kHz |  |
| E-UTRA Band 53 or NR Band n53 | 2483.5 – 2495 MHz | N/A | -91 dBm | -88 dBm | 100 kHz |  |
| NR Band n91 | 832 – 862 MHz | N/A | N/A | -88 dBm | 100 kHz |  |
| NR Band n93 | 880 – 915 MHz | N/A | N/A | -88 dBm | 100 kHz |  |
| NR Band n96 | 5925 - 7125 MHz | N/A | -90dBm | -87 dBm | 100 kHz |  |
| NR Band n100 | 874.4 – 880 MHz | -96 dBm | N/A | N/A | 100 kHz |  |
| NR Band n101 | 1900 – 1910 MHz | -96 dBm | N/A | N/A | 100 kHz |  |
| NR Band n102 | 5925 – 6425 MHz | N/A | -90dBm | -87 dBm | 100 kHz |  |
| NR Band n104 | 6425 – 7125 MHz | -95 dBm | -90 dBm | -87 dBm | 100 kHz |  |

Comparing the above two tables for TS 37.104, we can find that typical co-location requirement for E-UTRA Band 46 or NR Band n46, E-UTRA Band 49, E-UTRA Band 51, E-UTRA Band 53 was not captured in simplified co-location requirement table in draft CR for 37.104. RAN4 need to discuss whether to add them as shown in the table below.

| Frequency range of uplink operating band of the | System type to co-locate with | Basic limits (dBm/100kHz) |  |  | Notes |
| --- | --- | --- | --- | --- | --- |
| co-located BS |  | WA BS | MR BS | LA BS |  |
| 824 - 849 MHz | GSM850 or CDMA850 | -98 | -91 | -70 | NOTE 1 |
| 876 - 915 MHz | GSM900 | -98 | -91 | -70 |  |
| 1710 - 1785 MHz | DCS1800 | -98 | -91 | -80 |  |
| 1850 - 1910 MHz | PCS1900 | -98 | -91 | -80 |  |
| UL frequency range of n51, n91, n93, or UL frequency range of 49, 51 | NR, or E-UTRA | N/A | N/A | -88 |  |
| UL frequency range of n46, n53, or UL frequency range of 46, 53 | NR, or E-UTRA | N/A | -91 | -88 |  |
| UL frequency range of n100, n101 | NR | -96 | N/A | N/A |  |
| UL frequency range of n96, n102 | NR | N/A | -90 | -87 |  |
| UL frequency range of n104 | NR | -95 | -90 | -87 |  |
| Frequency range of uplink operating band of the co-located BS | Other (NOTE 4, 5) | -96 | -91 | -88 |  |

So, we propose,

Proposal 3: RAN4 to discuss whether to update the simplified table for co-location requirement in endorsed draft CR for TS 37.107 as below:

| Frequency range of uplink operating band of the | System type to co-locate with | Basic limits (dBm/100kHz) |  |  | Notes |
| --- | --- | --- | --- | --- | --- |
| co-located BS |  | WA BS | MR BS | LA BS |  |
| 824 - 849 MHz | GSM850 or CDMA850 | -98 | -91 | -70 | NOTE 1 |
| 876 - 915 MHz | GSM900 | -98 | -91 | -70 |  |
| 1710 - 1785 MHz | DCS1800 | -98 | -91 | -80 |  |
| 1850 - 1910 MHz | PCS1900 | -98 | -91 | -80 |  |
| UL frequency range of n51, n91, n93, or UL frequency range of 49, 51 | NR, or E-UTRA | N/A | N/A | -88 |  |
| UL frequency range of n46, n53, or UL frequency range of 46, 53 | NR, or E-UTRA | N/A | -91 | -88 |  |
| UL frequency range of n100, n101 | NR | -96 | N/A | N/A |  |
| UL frequency range of n96, n102 | NR | N/A | -90 | -87 |  |
| UL frequency range of n104 | NR | -95 | -90 | -87 |  |
| Frequency range of uplink operating band of the co-located BS | Other (NOTE 4, 5) | -96 | -91 | -88 |  |

- Conclusion

This contribution provides analysis on framework simplification for co-location/co-existence requirements, and the following proposal is concluded as follows:

Proposal 1: Update the NOTE1 for co-existence requirement as the following to solve the deleting note in note column of original table.

NOTE 1:	As defined in the scope for spurious emissions in this clause, except for the cases where the noted requirements apply to a BS operating in Band n28, the co-existence requirements in table 6.6.5.5.1.3-1 do not apply for the frequency range extending ΔfOBUE immediately outside the downlink operating band (see TS 38.104 [2], table 5.2-1). Emission limits for this excluded frequency range may be covered by local or regional requirements.

Proposal 2: RAN4 to discuss whether to update the simplified table for co-location requirement in endorsed draft CR for TS 38,141-1 as below:

| Frequency range of uplink operating band of the | System type to co-locate with | Basic limits (dBm/100kHz) |  |  | Notes |
| --- | --- | --- | --- | --- | --- |
| co-located BS |  | WA BS | MR BS | LA BS |  |
| 824 - 849 MHz | GSM850 or CDMA850 | -98 | -91 | -70 | NOTE 1 |
| 876 - 915 MHz | GSM900 | -98 | -91 | -70 |  |
| 1710 - 1785 MHz | DCS1800 | -98 | -91 | -80 |  |
| 1850 - 1910 MHz | PCS1900 | -98 | -91 | -80 |  |
| UL frequency range of n51, n91, n93, or UL frequency range of 51 | NR or E-UTRA | N/A | N/A | -88 |  |
| UL frequency range of n46, n53, or UL frequency range of 46, 53 | NR or E-UTRA | N/A | -91 | -88 |  |
| UL frequency range of n100, n101 | NR | -96 | N/A | N/A |  |
| UL frequency range of n96, n102 | NR | N/A | -90 | -87 |  |
| UL frequency range of n104 | NR | -95 | -90 | -87 |  |
| Frequency range of uplink operating band of the co-located BS | Other (NOTE 4, 5) | -96 | -91 | -88 |  |

Proposal 3: RAN4 to discuss whether to update the simplified table for co-location requirement in endorsed draft CR for TS 37.107 as below:

| Frequency range of uplink operating band of the | System type to co-locate with | Basic limits (dBm/100kHz) |  |  | Notes |
| --- | --- | --- | --- | --- | --- |
| co-located BS |  | WA BS | MR BS | LA BS |  |
| 824 - 849 MHz | GSM850 or CDMA850 | -98 | -91 | -70 | NOTE 1 |
| 876 - 915 MHz | GSM900 | -98 | -91 | -70 |  |
| 1710 - 1785 MHz | DCS1800 | -98 | -91 | -80 |  |
| 1850 - 1910 MHz | PCS1900 | -98 | -91 | -80 |  |
| UL frequency range of n51, n91, n93, or UL frequency range of 49, 51 | NR, or E-UTRA | N/A | N/A | -88 |  |
| UL frequency range of n46, n53, or UL frequency range of 46, 53 | NR, or E-UTRA | N/A | -91 | -88 |  |
| UL frequency range of n100, n101 | NR | -96 | N/A | N/A |  |
| UL frequency range of n96, n102 | NR | N/A | -90 | -87 |  |
| UL frequency range of n104 | NR | -95 | -90 | -87 |  |
| Frequency range of uplink operating band of the co-located BS | Other (NOTE 4, 5) | -96 | -91 | -88 |  |

- References

- R4-2512581, Way-Forward for RAN Task on specification improvements, Ericsson, RAN4#116

- R4-2511664, (TEI19-BDaT_simp_improvement) CR to 38.141-1 on framework simplification for co-location/co-existence requirements, Nokia, RAN4#116

- R4-2511663, (TEI19-BDaT_simp_improvement) CR to 37.104 on framework simplification for co-location/co-existence requirements, Nokia, RAN4#116

- R4-2508664, Way forward for [115][340] BSRF_Simplify_CoLo_Coex, Moderator (Ericsson), RAN4#115

- R4-2504741, Way forward for [114bis][331] BSRF_Simplify_CoLo_Coex, Moderator (Ericsson), RAN4#114-bis

- R4-2502306, Way forward for [114][331] BSRF_Simplify_CoLo_Coex, Moderator (Ericsson), RAN4#114

- RP-243288, Moderator's summary of discussion about R19-RAN4-BSRF, RAN4 Chair (Huawei), RAN#106

- R4-2500102, On RAN task to work on framework simplification for co-location/co-existence requirements, Nokia

- R4-2500880, On the topic of BS RF specification structure improvements, Ericsson

- RP-242767, On new objective under NR_BS_RF_req_evo, Nokia, Ericsson, ZTE, Huawei, HiSilicon, RAN#106