# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2514824

# Prague, Czech Republic, October 13 – 17, 2025

Agenda item:			7.8.1

Source:	Moderator (Samsung)

Title:	WF for [116bis][224] NR_LBCA_Sw_enh_RRM

Document for:	Approval

Agreements

Sub-topic 1-1 General

Issue 1-1-1: RRM work scope

Thursday online Agreement: - It is RAN4 common understanding that the following RRM requirements are defined in Rel-19 LB-CA switching SDL SCell related RRM requirement; Index; Requirements. 1; Deactivated SDL SCell measurement. 2; Interruption due to deactivated SCell measurement. 3; Activated SDL SCell L3 measurement. 4; BFD/CBD for SDL SCell. 5; L1-RSRP/SINR for SDL SCell. 6; SDL SCell activation/deactivation. 7; TCI state switch delay. FDD PCell related RRM requirement; 8; PCell L3 measurement. 9; BFD/CBD for PCell. 10; L1-RSRP/SINR for PCell. 11; RLM for PCell. 12; TCI state switch delay. 13; PL-RS switch delay for PCell. - For Rel-20, following the WID, RAN4 to Reuse Rel-19 LB-CA RAN4 requirements as much as possible. Session Chair: - Based on companies’ comments, the following difference between Rel-19 and Rel-20 can be further discussed in RAN4, for example and for information: - Potential requirement impact considering the DL reception in FDD PCell in Rel-20 (HW, OPPO) - Need of Tx timing requirement for PCell (ZTE) - Need of Interruption or scheduling restriction requirement for CC switching (QC) - Need of switching gap (Nokia)

Sub-topic 1-2 Carrier switching between CC/Switching pattern applicability

Issue 1-2-1: Switching pattern applicability

Thursday online Agreement: - Switching pattern applicability for RRM requirement in Rel-20 LB-CA via switching: - For the SCell which has been activated (i.e., after UE reporting valid CQI for SCell activation in section 8.3.2 TS38.133), - The switching pattern for SDL SCell is applied. - For the SCell which is in activation procedure, - The switching pattern for SDL SCell is applied. - For the SCell which is deactivated, - The switching pattern for SDL SCell is not applied.

Sub-topic 1-3 Applicability of Rel-19 LB-CA RRM requirements

Issue 1-3-2: SDL SCell related requirement

Thursday online Agreement: - For L3 measurement and L1 measurement on activated SDL SCell, reuse the Rel-19 requirement as starting point. - For TCI state switch delay requirements, reuse the Rel-19 requirement as starting point. - For SDL SCell activation/deactivation delay requirements, reuse the Rel-19 requirement as starting point. - No new RRM requirement is expected for deactivated SCell measurement period requirements

Topic #1: RRM requirements for Rel-20 LB-CA via switching (7.8.4)

Sub-topic 1-1 General

Issue 1-1-2: MRTD assumption

Way forward:

  - FFS: Options in topic summary R4-2513530.

Issue 1-1-4: SSB assumption

Way forward:

  - FFS: Options in topic summary R4-2513530.

Sub-topic 1-2 Carrier switching between CC/Switching pattern applicability

Issue 1-2-2: Interruption or scheduling restriction requirement for CC switching

Way forward:

  - FFS: Options in topic summary R4-2513530.

Sub-topic 1-3 Applicability of Rel-19 LB-CA RRM requirements

Issue 1-3-1: Requirement design for gap-less based L3 measurement and L1 measurement of activated SDL SCC

Way forward:

  - FFS: Options in topic summary R4-2513530.

Issue 1-3-3: Tx timing requirement for FDD PCell

Way forward:

  - FFS: Options in topic summary R4-2513530.

Sub-topic 1-4 Potential RRM requirement impact for Rel-20 LB-CA

Sub-topic description: To discuss new potential RRM requirements identified for Rel-20 LB-CA

Issue 1-4-1: SDL SCell related RRM requirement

Way forward:

  - FFS: Options in topic summary R4-2513530.

Issue 1-4-2: FDD PCell related RRM requirement

Way forward:

  - FFS: Options in topic summary R4-2513530.

Issue 1-4-3: Active BWP switching for PCell and SCell

Way forward:

  - FFS: Options in topic summary R4-2513530.

Sub-topic 1-5 Potential RAN1 impacts

Issue 1-5-1: Interpretation on switching pattern

Way forward:

  - FFS: Options in topic summary R4-2513530.