# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2515102

Prague, Czech Republic, Oct. 13-17, 2025

Agenda item:			7.7.1

Source:	Qualcomm Incorporated, CMCC

Title:	Way Forward for [116bis][319] NR_UAV

Document for:	Approval

# Introduction

This document is for Way Forward for Rel-20 WI on enhancement of NR RF and RRM requirements for uncrewed aerial vehicle (UAV).

# Coexistence

Issue 2-1-1: Coexistence scenarios

- Agreements:

  - RAN4 to consider the following synchronized coexistence scenarios in Table 2.2.1-1 as the starting point.

  - Further down-selection for the simulation scenario is not precluded.

Table 2.2.1-1: Co-existence scenarios between UAV and TN network

|  | Aggressor |  | Victim |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| No. | deployment scenario UL/DL | System BW duplex mode | deployment scenario UL/DL | System BW duplex mode | Simulation frequency | ACLR/ACS requirements |
| 1 | TN UL | 100MHz TDD | UAV UL | 100MHz TDD | 4 GHz | UAV BS ACS |
| 2 | UAV DL | 100MHz TDD | TN DL | 100MHz TDD | 4 GHz | UAV BS ACLR |
| 3 | TN DL | 100MHz TDD | UAV DL | 100MHz TDD | 4 GHz | UAV UE ACS |
| 4 | UAV UL | 100MHz TDD | TN UL | 100MHz TDD | 4 GHz | UAV UE ACLR |
| 5 | TN UL | [20MHz] FDD | UAV UL | [20MHz] FDD | 2 GHz | UAV BS ACS |
| 6 | UAV DL | [20MHz] FDD | TN DL | [20MHz] FDD | 2 GHz | UAV BS ACLR |
| 7 | TN DL | [20MHz] FDD | UAV DL | [20MHz] FDD | 2 GHz | UAV UE ACS |
| 8 | UAV UL | [20MHz] FDD | TN UL | [20MHz] FDD | 2 GHz | UAV UE ACLR |

Issue 2-1-2: Coexistence analysis vs coexistence simulation

- Agreements:

  - Companies are encouraged to provide coexistence analysis to the co-existence scenarios listed in Table 2.2.1-1, either through simulation or link budget analysis with considering aggregated interference at the next meeting.

  - RAN4 will conduct coexistence simulation for the scenarios where coexistence issues are identified.

Issue 2-2-1: Deployment scenario

- Agreements:

  - RAN4 needs to further discuss deployment scenarios for coexistence analysis

    - Option 1: UMa

    - Option 2: RMa

    - Option 3: UMa and RMa

Issue 2-2-2: Network layout

- Agreements:

  - RAN4 needs to further discuss network layout used for coexistence analysis

    - Option 1: co-located

    - Option 2: co-located and non-collocated

Figure 2.1.1-1: Illustration of co-located network layout

Figure 2.1.1-2: Illustration of non-collocated network layout

    - Other network layouts such as only covering UAV routes are not precluded.

  - ISDUAV =X*ISDNR, FFS on X

    - ISD for UAV: 2-4km for UAV ISD

Issue 2-2-3: Deployment parameters

- Agreements:

  - Take the assumptions in Table 2.2.3-1 and Table 2.2.3-2 as a starting point for future coexistence analysis.

Table 2.2.3-1: Assumptions for UAV deployment scenarios

| Parameter | Descriptions |
| --- | --- |
| UAV BS deployment | Hexagonal grid, 3 sectors per site with wrap around, [19 BS sites, 57 sectors] all the UAV BSs are activated in one snapshot (NOT applicable for only covering UAV route scenario) Inter-UAV BS distance: [2-4km] [The number of UAV BS according to ISD of UAV BS] |
| Carrier frequency | 2GHz, 4GHz |
| UAV CBW | [20MHz] for 2GHz, 100MHz for 4GHz |
| UAV UE UL power control | [Yes, UL power control model in section 6.2.6.3, TR 38.876 is reused]. |
| Traffic model | Full buffer |
| Frequency reuse | 1 |
| UAV UE dropping | For UAV UE, the minimum height for UAV UE is [50m-150m], FFS on range of uniform distribution for vertical and horizontal distance, minimum horizontal distance [20m]. UAV UE number: - DL active UE: 1 UE per cell - UL active UE: 1 UE per cell |
| Pathloss model | [UAV BS to UE: Propagation model between TN UE and ATG UE in TR 38.876] |
| UAV UE max Tx power for both antenna types | up to 33dBm for conducted power, e.g., 31dBm, 29dBm, 26dBm. |

Table 2.2.3-2: Assumptions for TN deployment scenarios

| Parameter | Descriptions |
| --- | --- |
| TN BS deployment | Hexagonal grid, 3 sectors per site with wraparound, 19 BS sites, 57 sectors all NR BS are activated in one snapshot [BS height: 30m] Inter-NR BS distance: [500m/750m for UMa] or [1732m for RMa] |
| Carrier frequency | 2GHz, 4GHz |
| TN CBW | [20MHz] for 2GHz, 100MHz for 4GHz |
| UE UL power control | [Yes, UL power control model in section 6.2.6.1, TR 38.876 is reused]. |
| Traffic model | Full buffer |
| Frequency reuse | 1 |
| UAV UE dropping | Uniformly distributed TN UE number: - DL active UE: 1 UE per cell - UL active UE: 1 UE per cell |
| BS Inter-site distance | MCL of 70dB for NR BS |
| Pathloss model | NR BS to UE: NLOS and LOS UMa or RMa in TR38.901 |
| NR UE max Tx power | 23dBm |

Issue 2-2-4: UAV and TN BS antenna assumptions

- Agreements:

  - For AAS BS model, adopt the same 4400 to 4800 MHz sub-array model for UAV and TN AAS BS antenna configuration, as captured in Clause 4.4.1, TR 38.922.

  - For non-AAS model, adopt the same antenna parameters in ITU-R M.2292 for both UAV and TN BS

  - RAN4 needs to discuss the uptilt/ downtilt for UAV and TN BS

    - Option 1: Separate antenna with uptilt for UAV BS and downtilt TN BS

      - Option 1 is with high priority

    - Option 2: Same antenna for TN and UAV BS with same downtilt serving both UAV UE and TN UE

Issue 2-2-6: UAV UE antenna assumptions

- Agreements:

  - As the starting point, omni-directional is assumed for 2GHz, and phase array is assumed for 4GHz.

  - The antenna parameters for antenna array in the table below are used as the starting point.

| Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 90º for V |
| --- | --- |
| Element gain (dBi) | 5 dBi |
| Horizontal/vertical front-to-back ratio (dB) | 30dBc |
| Antenna polarization | Linear ±90º |
| Antenna array configuration (Row × Column x Polarization) | [(2x2x2)] |
| Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H, 0.5 of wavelength for V |

Issue 2-2-7: TN UE antenna assumptions

- Agreements:

  - The TN UE can be considered an omni-directional UE antenna configuration with 0 dBi gain.

# UAV UE RF requirements

### Sub-topic 3-1: Tx RF requirements

Issue 3-1-1: General aspects

- Agreements:

  - The high-power UAV UE applies for the listed example bands in Rel-20 WID.

  - RF requirements need to be defined on both omni-directional antenna connectors and transceiver array boundary (TAB) connectors.

    - The same UE can support either omni-directional antenna and phase array antenna.

### Sub-topic 3-2: Tx RF requirements

Issue 3-2-1: Maximum output power

- Agreements:

  - RAN4 to specify maximum output power for UAV higher power UE considering the following two options:

    - Option 1: Introduce the new capability for high power UAV to indicate the rated maximum output power (Similar as ATG)

    - Option 2: Introduce a new power class for UAV UE for up to 33dBm max total conductive output power for the example bands (Similar as legacy TN UE power class definition)

    - Option 3: Other options are not precluded.

Issue 3-2-3: Output dynamic range

- Agreements:

  - FFS on RAN4 needs to evaluate the minimum output power for high power UAV equipped with omni-directional antenna and directional antenna array considering the different center frequency, pass loss and antenna gain, etc.

Issue 3-2-4: Transmit signal quality

- Agreements:

  - As a starting point, the legacy frequency error and transmit modulation quality requirements can be reused for high power UAV.

Issue 3-2-5: Output RF spectrum emissions

- Agreements:

  - Take the following conclusions for output RF spectrum emissions as the starting point.

| Requirements | Conclusions |
| --- | --- |
| Occupied bandwidth | Legacy requirements can apply to high power UAV UE. |
| SEM requirements | The discussion on SEM can be held until ACLR is concluded. |
| ACLR requirements | RAN4 needs to evaluate ACLR for high power UAV based on co-existence analysis. |
| Spurious emissions | Legacy requirements can apply to high power UAV UE. |
| Spurious emissions for UE co-existence | Legacy requirements can apply to high power UAV UE. |
| Transmit intermodulation | FFS on whether this requirement is needed or not for high power UAV UE. |

### Sub-topic 3-3: Rx RF requirements

Issue 3-3-2: ACS

- Agreements:

  - FFS on how to specify ACS requirements