3GPP TSG-RAN4 Meeting #117	R4-2522766

# Dallas, USA, 17 – 21 Novermber, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 6192 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on test case for less than 5MHz Ph2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR1_lessthan_5MHz_BW_Ph2-Perf |  |  |  |  |  | Date: |  |  | 2025-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This is a re-submission of draftCR R4-2514155 endorsed in RAN4#116-bis. There are some issues with TC in A.6.6.1.15: - The power levels for cell 3 (SCell) is missing, and the Io values are wrong (which is based on 10MHz BW). - There are some redundant rows in Table A.6.6.1.15.2-3 as the test has only 1 test configurations. - The test requirements of 1120ms only accounts for the SBI reading delay, but the event triggered reporting delay should also include cell detection delay and measurement delay. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the the TC in A.6.6.1.15: - Add power levels for cell 3 (SCell) and correct the Io values. - Remove the redundant rows. - Update the test requirements to 2720 = (800+1120+800) ms |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TC in A.6.6.1.15 is not fully correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.6.1.15 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | It is noted that the table is replaced with a new one because there are some redudant rows in the orignal table and removal of them is not shown as change mark. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

A.6.6.1.15	SA event triggered reporting test without gap under non-DRX with SSB index reading and 12 PRB SSB for a deactivated SCell

A.6.6.1.15.1	Test purpose and Environment

The purpose of this test is to verify that the UE supporting support-3MHz-ChannelBW-r18 makes correct reporting of an event on an intra-frequency layer measurement performed without measurement gaps when the UE is also configured deactivated SCell. This test will partly verify the intra-frequency cell search requirements in clauses 9.2.5.1 and 9.2.5.2.

A.6.6.1.15.2	Test parameters

Three cells are deployed in the test, which are FR1 PCell (Cell 1) on Carrier 1, FR1 Neighbouring cell (Cell 2) and a FR1 SCell (Cell 3) on Carrier 2. Carrier 2 is on a different frequency than the PCell.

The test parameters for PCell, SCell and neighbour cell are given in table A.6.6.1.15.2-1, A.6.6.1.15.2-2 and A.6.6.1.15.2-3 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and the SCell and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

Table A.6.6.1.15.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | PCell: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode; Neighbouring cell: 15 kHz SSB SCS, 3 MHz bandwidth, FDD duplex mode Scell: 15 kHz SSB SCS, 3 MHz bandwidth, FDD duplex mode |

Table A.6.6.1.15.2-2: General test parameters for SA intra-frequency event triggered reporting without gap for FR1 with 12 PRB SSB for SCell

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | PCell |
| Neighbour cell |  |  | Cell 2 | Cell to be identified. |
| Deactivated SCell |  |  | Cell 3 | Deactivated throughout the test. |
| RF Channel Number |  |  | 1: Cell 1 2: Cell 2 and Cell 3 | 1: Cell 1 is PCell 2: Cell 2 is neighbouring cell to be detected |
| SSB configuration |  |  | SSB.1 FR1 | Cell 1 |
|  |  |  | SSB.13 FR1 | 12 PRB Cell 2 and Cell 3 |
| SMTC configuration |  |  | SMTC.2 | Cell 1 |
|  |  |  | SMTC.1 | Cell 2 and Cell 3 |
| A3-Offset | dB | 1 | -4.5 |  |
| CP length |  |  | Normal |  |
| Hysteresis | dB |  | 0 |  |
| Time To Trigger | s |  | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| DRX |  |  |  | OFF |
| measCycleScell | ms |  | 160ms |  |
| Time offset between serving and neighbour cells |  |  | 3 ms | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
|  |  |  | 3 s | Synchronous cells |
| T1 | s |  | 5 |  |
| T2 | s |  | 5 |  |

Table A.6.6.1.15.2-3: NR Cell specific test parameters for SA intra-frequency event triggered reporting without gap for FR1 with 12 PRB SSB for SCell

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| PDSCH RMC configuration |  |  | SR.1.1 FDD |  | SR.1.3 FDD |  |  |  |
| RMSI CORESET RMC configuration |  |  | CCR.1.1 FDD |  | CCR.1.7 FDD |  |  |  |
| Dedicated CORESET RMC configuration |  |  | CCR.1.1 FDD |  | CCR.1.7 FDD |  |  |  |
| OCNG Patterns |  |  |  |  | OP.1 |  |  |  |
| TRS Configuration |  |  |  |  | TRS.1.1 FDD |  |  |  |
| IInitial BWP configuration |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |  |  |  |
| Active DL BWP configuration |  |  | DLBWP.1.1 |  |  |  |  |  |
| Active UL BWP configuration |  | 1 | ULBWP.1.1 |  |  |  |  |  |
| RLM-RS |  |  | SSB |  |  |  |  |  |
| Note 2 | dBm/SCS |  | -98 |  |  |  |  |  |
| Note 2 | dBm/15 kHz |  | -98 |  |  |  |  |  |
|  | dB |  | 4 | -1.46 | -Infinity | -1.46 | 4 | -1.46 |
|  | dB |  | 4 | 4 | -Infinity | 4 | 4 | 4 |
| SS-RSRP Note 3 | dBm/SCS kHz |  | -94 | -94 | -Infinity | -94 | -94 | -94 |
| Io | dBm/2.7 MHz |  | -70.00 | -70.00 | -75.44 | -67.65 | -70.00 | -67.65 |
| Propagation Condition |  |  | AWGN |  |  |  |  |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

A.6.6.1.15.3	Test requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 2720 ms from the beginning of time period T2. The UE is required to read the neighbour cell SSB index in this test. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

<End of Change 1>