3GPP TSG-RAN WG4 Meeting #116	rev R4-2509893

Bengaluru, India, 25th -29th August, 2025

Source:	Huawei, Hisilicon

Title:	Draft TP to TR 38.719-02-01 include CA_n41-n75

Agenda item:			6.3.3(NR_CADC_R19_2BDL_yBUL)

Document for:	Approval

# Introduction

The revised basket WID was approved in RAN #108 meeting [1]. This contribution provides a text proposal for TR 38.719-02-01 to include CA_n41-n75.

# References

[1] RP-251806, Revised WID: Rel-19 NR Carrier Aggregation (CA)/Dual Connectivity (DC) for x bands DL with y bands UL (x<7, y<3) and Supplementary Uplink (SUL) band combinations/CA band combinations with a single SUL or two SUL cells, Ericsson, ZTE, Huawei, HiSilicon, June,2025, RAN#108

# Text Proposal

<Start of Text Proposal>

## 5.x	CA_n41-n75

### 5.x.1	Common for 1 band UL and 2 bands UL CA

#### 5.x.1.1	Operating bands for CA

Table 5.x.1.1-1:  CA band combination of band n41+n75

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n41 | 2496 MHz – 2690 MHz | 2496 MHz – 2690 MHz | TDD |
| n75 | N/A | 1432 MHz – 1517 MHz | SDL |

#### 5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination of band n41+n75

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41A-n75A | - | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n75 | 5,10, 15, 20, 25,30,40,50 |  |

#### 5.x.1.3	UE co-existence studies for 1 band UL

Table 5.x.1.3-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n41-n75.

Table 5.x.1.3-1 UL/DL harmonics/harmonic mixing analysis

| UL/DL harmonics |  | n41 | UL12 | UL2 | UL33 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | fLow | 2496 | 4992 | 7488 | 9984 | 12480 |  |
| n75 | fLow | fHigh | 2690 | 5380 | 8070 | 10760 | 13450 |  |
| DL1 | 1432 | 1517 | N/A | - | - | - | - | UL harmonic |
| DL23 | 2864 | 3034 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 4296 | 4551 | - | - | N/A | - | N/A |  |
| DL4 | 5728 | 6068 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 7160 | 7585 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | There is no collision detected with both harmonic and harmonic mixing. |  |  |  |  |  |

Based on Table 5.x.1.3-1, there is no collision detected with both harmonic and harmonic mixing.

Table 5.x.1.3-2 summarizes frequency ranges where cross band isolation may occur for CA_n41A-n75A.

Table 5.x.1.3-2: Cross-band isolation analysis

| Bands | n41 |  |
| --- | --- | --- |
| Frequency limit | flow | fhigh |
| fUL (MHz) | 2496 | 2690 |
| fDL (MHz) | 2496 | 2690 |
| UL CBW (MHz)2 | Minimum CBW | Maximum CBW |
|  | 10 | 100 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx |
| ACLR1 (MHz) | 2396 | 2790 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx |
| ACLR2 (MHz) | 2296 | 2890 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx |
| ACLR3 (MHz) | 2196 | 2990 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx |
| ACLR4 (MHz) | 2096 | 3090 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx |
| ACLR5 (MHz) | 1996 | 3190 |
| Analysis | There is no overlap of the band n8 ACLR range with the band n50 DL up to order 5. The sufficient rejection of the transmitter noise floor should be at band n50 so no cross-band MSD is needed. |  |

Based on Table 5.x.1.3-2:

There is no uplink band overlapping with the downlink in ACLR range. The existing diplexer isolation should provide sufficient rejection.

#### 5.x.1.4	∆TIB,c and ∆RIB,c values

For CA_n41-n75, the TIB,c and RIB,c of CA_n7-n75 can be reused, and values are given in the tables below.

Table 5.x.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n41-n75 | 0.7 | N/A |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.x.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n41-n75 | - | - |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

#### 5.x.1.5	REFSENS requirements

There is no need for new REFSENS requirements.

#### 5.x.1.6	OOB blocking exception requirements

There is no need for OOB blocking exception.

<End of Text Proposal>