3GPP TSG-RAN WG4 Meeting #116	rev R4-2509891

Bengaluru, India, 25th -29th August, 2025

Source:	Huawei, Hisilicon

Title:	Draft TP to TR 38.719-02-01 include CA_n50-n77

Agenda item:			6.3.4(NR_CADC_R19_2BDL_yBUL)

Document for:	Approval

# Introduction

The revised basket WID was approved in RAN #108 meeting [1]. This contribution provides a text proposal for TR 38.719-02-01 to include CA_n50-n77.

# References

[1] RP-251806, Revised WID: Rel-19 NR Carrier Aggregation (CA)/Dual Connectivity (DC) for x bands DL with y bands UL (x<7, y<3) and Supplementary Uplink (SUL) band combinations/CA band combinations with a single SUL or two SUL cells, Ericsson, ZTE, Huawei, HiSilicon, June,2025, RAN#108

# Text Proposal

<Start of Text Proposal>

## 5.x	CA_n50-n77

### 5.x.1	Common for 1 band UL and 2 bands UL CA

#### 5.x.1.1	Operating bands for CA

Table 5.x.1.1-1:  CA band combination of band n50+n77

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n50 | 1432 MHz – 1517 MHz | 1432 MHz – 1517 MHz | TDD |
| n77 | 3300 MHz – 4200 MHz | 3300 MHz – 4200 MHz | TDD |
| NOTE X: Editor's note: Whether or not supporting simultaneous Rx/Tx capability should be identified for TDD-TDD or FDD-TDD band combination Copy from R4-2507910: RAN4 has concluded simultaneous Rx/Tx requirements for different types of inter-band CA and SUL configurations: TDD + TDD -> Conditional mandatory support for simultaneous RxTx with signaling |  |  |  |

If the band combination is TDD/TDD, is SimRx/Tx supported (YES/NO/N-A)? ==> YES.

#### 5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination of band n50+n77

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n50A-n77A | CA_n50A-n77A | n50 | n50 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |

#### 5.x.1.3	UE co-existence studies for 1 band UL

Table 5.x.1.3-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_ n50-n77.

Table 5.x.1.3-1 UL/DL harmonics/harmonic mixing analysis

| UL/DL harmonics |  | n50 | UL12 | UL2 | UL33 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | fLow | 1432 | 2864 | 4296 | 5728 | 7160 |  |
| n77 | fLow | fHigh | 1517 | 3034 | 4551 | 6068 | 7585 |  |
| DL1 | 3300 | 4200 | N/A | - | - | - | - | UL harmonic |
| DL23 | 6600 | 8400 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 9900 | 12600 | - | - | N/A | - | N/A |  |
| DL4 | 13200 | 16800 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 16500 | 21000 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | There is no collision detected with both harmonic and harmonic mixing. |  |  |  |  |  |
| UL/DL harmonics |  | n77 | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
|  |  | fLow | 3300 | 6600 | 9900 | 13200 | 16500 |  |
| n50 | fLow | fHigh | 4200 | 8400 | 12600 | 16800 | 21000 |  |
| DL1 | 1432 | 1517 | N/A | - | - | - | - | UL harmonic |
| DL23 | 2864 | 3034 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 4296 | 4551 | - | - | N/A | - | N/A |  |
| DL4 | 5728 | 6068 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 7160 | 7585 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | There is no collision detected with both harmonic and harmonic mixing. |  |  |  |  |  |

Based on Table 5.x.1.3-1:

-	There is no harmonic or harmonic mixing collision detected for this band combination.

Table 5.x.1.3-2 summarizes frequency ranges where cross band isolation may occur for CA_n50A-n77A.

Table 5.x.1.3-2: Cross-band isolation analysis

| Bands | n50 |  | n77 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | flow | fhigh | flow | fhigh |
| fUL (MHz) | 1432 | 1517 | 3300 | 4200 |
| fDL (MHz) | 1432 | 1517 | 3300 | 4200 |
| UL CBW (MHz)2 | Minimum CBW | Maximum CBW | Minimum CBW | Maximum CBW |
|  | 5 | 60 | 10 | 100 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 1372 | 1577 | 3200 | 4300 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 1312 | 1637 | 3100 | 4400 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 1252 | 1697 | 3000 | 4500 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 1192 | 1757 | 2900 | 4600 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 1132 | 1817 | 2800 | 4700 |
| Analysis | There is no overlap of the band n50 ACLR range with the band n77 DL up to order 5. The sufficient rejection of the transmitter noise floor should be at band n77 so no cross-band MSD is needed. |  | There is no overlap of the band n77 ACLR range with the band n50 DL up to order 5. The sufficient rejection of the transmitter noise floor should be at band n50 so no cross-band MSD is needed. |  |

Based on Table 5.x.1.3-2:

There is no uplink band overlapping with the downlink in ACLR range. The existing diplexer isolation should provide sufficient rejection.

#### 5.x.1.4	∆TIB,c and ∆RIB,c values

For CA_n50-n77, the TIB,c and RIB,c of CA_n74-n77 can be reused, and values are given in the tables below.

Table 5.x.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n50-n77 | 0.4 | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.x.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n50-n77 | - | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

#### 5.x.1.5	REFSENS requirements

There is no need for new REFSENS requirements.

#### 5.x.1.6	OOB blocking exception requirements

There is no need for OOB blocking exception.

### 5.x.2	Specific for 2 bands UL CA

There is no need for OOB blocking exception.

#### 5.x.2.1	Maximum output power for inter-band CA

Table 5.x.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Power Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n50A-n77A | 23 | +2/-3 |

#### 5.x.2.2	UE co-existence studies for 2 bands UL

Since this is a TDD/TDD combination, co-existence studies for 2 band UL analysis is not relevant. When the two UL are active there is no active DL.

For CA_n50-n77, although PHS frequency range 1884.5-1915.7MHz is as pretected band of band n77, due to this combination will not be deployed in the PHS region, there is no need to add PHS frequency range as protected band for CA_n50-n77. Thus no protected bands are needed.

#### 5.x.2.3	REFSENS requirements

There are no IMD co-existence issue, so no need to defined additional REFSEN requirements (MSD).

<End of Text Proposal>