3GPP TSG-RAN4 Meeting #116	R4-2511758

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

Title: 	WF on 7 MHz channel BW

Agenda item:	7.4.1

Source:	Ericsson

Document for:	Approval

# 1- 	General

## Minimum / Maximum channel BW

Agreement:

- In single carrier, 7 MHz can’t be the lowest channel BW in a band.

- In CA, 7MHz can be min/max CBW of one band in the combination supporting BCS5.

- In MSD, 7MHz cannot be a minimum channel BW in CA

# 2-	BS RF

## 2.1	EVM window length for 7 MHz channel BW

Agreement:

| Channel bandwidth (MHz) | FFT size | CP length for symbols 1-6 and 8-13 in FFT samples | EVM window length W | Ratio of W to total CP length for symbols 1-6 and 8-13 (Note) (%) |
| --- | --- | --- | --- | --- |
| 7 | 512 | 36 | 14 | 40 |

# 3- UE A-MPR

## 3.1	UE RF A-MPR tables

Agreement:

- Remove the texts “Outer/Inner”, “Outer” and “Edge/Inner” from the NS_12, NS_13, NS_14 and NS_15 A-MPR tables

- RAN4 can further discuss on the clarification on the general statement on RB allocation for the aforementioned NS values.

## 3.2	NS_12: A-MPR for PC3

Agreement: The following NS_12 A-MPR regions definition was agreed.

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | 817.5 ≤ Fc ≤ 820.5 | ≤1.8 | ≤0.72 | A3 |
|  |  | ≤1.8 | >0.72 | A4 |
|  |  | >1.8 | >1.44 | A5 |

| Modulation/Waveform | A3 | A4 | A5 |
| --- | --- | --- | --- |
| DFT-s-OFDM PI/2 BPSK | ≤ 3.5 | ≤ 4.5 | 4 |
| DFT-s-OFDM QPSK | ≤ 4 | ≤ 4.5 | 4 |
| DFT-s-OFDM 16 QAM | ≤ 4 | ≤ 5 | 4.5 |
| DFT-s-OFDM 64 QAM | ≤ 4.5 | ≤ 5 | 5 |
| DFT-s-OFDM 256 QAM | ≤ 5 | ≤ 5.5 | 5.5 |
| CP-OFDM QPSK | ≤ 5.5 | ≤ 6.5 | 6.5 |
| CP-OFDM 16 QAM | ≤ 5.5 | ≤ 6.5 | 6.5 |
| CP-OFDM 64 QAM | ≤ 6 | ≤ 7 | 6.5 |
| CP-OFDM 256 QAM |  | ≤ 7 | 6.5 |

## 3.3	NS_13: A-MPR for PC3

Agreement: The following NS_13 A-MPR regions definition was agreed.

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz |  | ≤1.8 | <1.44 | A5 |
|  | 820.5 = Fc | ≤1.8 | ≥1.44 | A6 |
|  |  | >1.8 | ≥1.8 | A7 |

| Modulation/Waveform | A5 | A6 | A7 |
| --- | --- | --- | --- |
| DFT-s-OFDM PI/2 BPSK | ≤ 3.5 | ≤ 4 | ≤ 3 |
| DFT-s-OFDM QPSK | ≤ 3.5 | ≤ 4 | ≤ 3 |
| DFT-s-OFDM 16 QAM | ≤ 3.5 | ≤ 4.5 | ≤ 3.5 |
| DFT-s-OFDM 64 QAM | ≤ 4.5 | ≤ 4.5 | ≤ 4 |
| DFT-s-OFDM 256 QAM | ≤ 5 | ≤ 6 | ≤ 4.5 |
| CP-OFDM QPSK | ≤ 5.5 | ≤ 6 | ≤ 4.5 |
| CP-OFDM 16 QAM | ≤ 5.5 | ≤ 6 | ≤ 4.5 |
| CP-OFDM 64 QAM | ≤ 6 | ≤ 6 | ≤ 5 |
| CP-OFDM 256 QAM | ≤ 8 | ≤ 8 | ≤ 6.5 |

## 3.4	NS_15: A-MPR for PC3

Agreement: The following NS_15 A-MPR regions definition was agreed.

| Channel BW | Carrier Frequency, Fc, MHz | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7 MHz | 841.5 < Fc ≤ 845.5 | ≥ 4.5 | > 0 | A7 |
|  |  | < 4.5, ≥ 2.16 | ≥ 1.8 | A2 |
|  |  | ≤ 1.44 | ≤ 0.36 | A8 |
|  | 838 < Fc ≤ 841.5 | > 0 | > 2.7 | A9 |
|  |  | ≥ 5.94 | ≤ 2.7 | A10 |

| Modulation/Waveform | A7 | A2 | A8 | A9 | A10 |
| --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM PI/2 BPSK | ≤ 8.5 | ≤ 5 | ≤ 5 | ≤ 4 | ≤ 2 |
| DFT-s-OFDM QPSK | ≤ 10 | ≤ 5 | ≤ 6.5 | ≤ 4.5 | ≤ 2 |
| DFT-s-OFDM 16 QAM | ≤ 10 | ≤ 5 | ≤ 6.5 | ≤ 4.5 | ≤ 2.5 |
| DFT-s-OFDM 64 QAM | ≤ 10 | ≤ 5 | ≤ 6.5 | ≤ 4.5 | ≤ 2.5 |
| DFT-s-OFDM 256 QAM | ≤ 10 | ≤ 5 | ≤ 9 | ≤ 5 | ≤ 3.5 |
| CP-OFDM QPSK | ≤ 11 | ≤ 6.5 | ≤ 6.5 | ≤ 6 | ≤ 3 |
| CP-OFDM 16 QAM | ≤ 11 | ≤ 6.5 | ≤ 6.5 | ≤ 6 | ≤ 3 |
| CP-OFDM 64 QAM | ≤ 11 | ≤ 6.5 | ≤ 6.5 | ≤ 6 | ≤ 4 |
| CP-OFDM 256 QAM | ≤ 11 | ≤ 6.5 | ≤ 9.5 | ≤ 6.5 | ≤ 5 |

## 3.5	NS_12: A-MPR for PC2

Agreement: The following PC2 NS_12 A-MPR regions definition was agreed.

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | 817.5 ≤ Fc < 820.5 | ≤1.8 | ≤0.72 | A4 |
|  |  | ≤1.8 | >0.72 | A5 |
|  |  | >1.8 | >1.44 | A6 |

Table 3: A-MPR for NS_12 (PC2), from [3]

| Modulation/Waveform | A4 | A5 | A6 |
| --- | --- | --- | --- |
| DFT-s-OFDM PI/2 BPSK | ≤ 4 | ≤ 6 | ≤ 5 |
| DFT-s-OFDM QPSK | ≤ 4.5 | ≤ 6 | ≤ 5 |
| DFT-s-OFDM 16 QAM | ≤ 4.5 | ≤ 6.5 | ≤ 5 |
| DFT-s-OFDM 64 QAM | ≤ 5 | ≤ 6.5 | ≤ 5 |
| DFT-s-OFDM 256 QAM | ≤ 5.5 | ≤ 7.5 | ≤ 6.5 |
| CP-OFDM QPSK | ≤ 6 | ≤ 7.5 | ≤ 6.5 |
| CP-OFDM 16 QAM | ≤ 6 | ≤ 7.5 | ≤ 6.5 |
| CP-OFDM 64 QAM | ≤ 6 | ≤ 7.5 | ≤ 6.5 |
| CP-OFDM 256 QAM | ≤ 7 | ≤ 8 | ≤ 7.5 |

## 3.6	NS_13: A-MPR for PC2

Agreement: The following PC2 NS_13 A-MPR regions definition was agreed.

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz |  | ≤1.8 | <1.44 | A5 |
|  | 820.5 ≤ Fc | ≤1.8 | ≥1.44 | A6 |
|  |  | >1.8 | ≥1.8 | A7 |

Table 6: A-MPR for NS_13 (Power Class 2), from [3]

| Modulation/Waveform | A1 | A2 | A3 |
| --- | --- | --- | --- |
| DFT-s-OFDM PI/2 BPSK | ≤ 3.5 | ≤ 4 | ≤ 3 |
| DFT-s-OFDM QPSK | ≤ 3.5 | ≤ 4 | ≤ 3 |
| DFT-s-OFDM 16 QAM | ≤ 3.5 | ≤ 4.5 | ≤ 3.5 |
| DFT-s-OFDM 64 QAM | ≤ 4.5 | ≤ 4.5 | ≤ 4 |
| DFT-s-OFDM 256 QAM | ≤ 5 | ≤ 6 | ≤ 4.5 |
| CP-OFDM QPSK | ≤ 5.5 | ≤ 6 | ≤ 4.5 |
| CP-OFDM 16 QAM | ≤ 5.5 | ≤ 6 | ≤ 4.5 |
| CP-OFDM 64 QAM | ≤ 6 | ≤ 6 | ≤ 5 |
| CP-OFDM 256 QAM | ≤ 8 | ≤ 8 | ≤ 6.5 |

## 3.7	NS_15: A-MPR for PC2

Agreement: The following PC2 NS_15 A-MPR regions definition was agreed.

| Channel BW | Carrier Frequency, Fc, MHz | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | 841.5 < Fc ≤ 845.5 | ≥ 4.5 | > 0 | A7 |
|  |  | < 4.5, ≥ 2.16 | ≥ 1.8 | A8 |
|  |  | < 4.5, ≥ 2.16 | <1,8 ≥1.44 | A9 |
|  |  | ≤ 1.44 | ≤ 0.36 | A10 |
|  | 837 < Fc ≤ 841.5 | > 0 | > 2.7 | A11 |
|  |  | ≥ 5.94 | ≤ 2.7 | A12 |

Table 10: A-MPR for NS_15 (Power Class 2), from [3]

| Modulation/Waveform | A7 | A8 | A9 | A10 | A11 | A12 |
| --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM PI/2 BPSK | ≤ 11 | ≤ 6 | ≤ 4.5 | ≤ 7.5 | ≤ 5.5 | ≤ 3 |
| DFT-s-OFDM QPSK | ≤ 11.5 | ≤ 6.5 | ≤ 4.5 | ≤ 7.5 | ≤ 5.5 | ≤ 3 |
| DFT-s-OFDM 16 QAM | ≤ 11.5 | ≤ 6.5 | ≤ 4.5 | ≤ 8 | ≤ 5.5 | ≤ 3.5 |
| DFT-s-OFDM 64 QAM | ≤ 11.5 | ≤ 6.5 | ≤ 4.5 | ≤ 8 | ≤ 5.5 | ≤ 4 |
| DFT-s-OFDM 256 QAM | ≤ 12.5 | ≤ 6.5 | ≤ 5 | ≤ 9 | ≤ 5.5 | ≤ 5.5 |
| CP-OFDM QPSK | ≤ 13 | ≤ 8 | ≤ 5.5 | ≤ 10 | ≤ 7 | ≤ 4.5 |
| CP-OFDM 16 QAM | ≤ 13 | ≤ 8.5 | ≤ 5.5 | ≤ 10 | ≤ 7 | ≤ 4.5 |
| CP-OFDM 64 QAM | ≤ 13 | ≤ 8.5 | ≤ 5.5 | ≤ 10 | ≤ 7 | ≤ 5 |
| CP-OFDM 256 QAM | ≤ 13.5 | ≤ 8.5 | ≤ 7 | ≤ 10.5 | ≤ 7.5 | ≤ 7 |