# 3GPP TSG-RAN WG4 Meeting #116	R4-2512135

Bengaluru, India, Aug 25th – 29th, 2025

Agenda item:			7.23.1

Source:	Moderator (Ericsson)

Title:	Ad-hoc minutes for Netw_Energy_NR_enh

Document for:	Approval

# Topic #1: SSB adaptation (AI 7.27.3.3)

### Issue 1-1-1: Applicable scenario for SSB adaption

Background:

TS 38.321 1> if the SCell is deactivated: 2> not transmit SRS on the SCell; 2> not report CSI for the SCell; 2> not transmit on UL-SCH on the SCell; 2> not transmit on RACH on the SCell; 2> not monitor the PDCCH on the SCell; 2> not monitor the PDCCH for the SCell; 2> not transmit PUCCH on the SCell.

Tentative Agreement (to be confirmed by Thursday):

RAN4 not to define requirement for DCI based SSB adaption for deactivated SCell measurement and SCell activation.

- Note: this RAN4 agreement does not intend to impact the discussion and decision in other WGs.

Discussion

# Topic #2: On-demand SSB(OD-SSB) (AI 7.27.3.1)

### Issue 1-5-1: Requirement for Alt scenario 3 (OD-SSB and AO-SSB are within different frequencies)

- Background

  - In RAN1 #120 meeting, RAN1 agreed to introduce the scenario 3(OD-SSB and AO-SSB are within different frequencies).

RAN1 #120 Agreement Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB, - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - On-demand SSB is not on sync raster - AO-SSB and OD-SSB are located in the same BWP - FFS: Additional conditions - Subject to separate UE capability - Note: UE is not required to measure both AO-SSB and OD-SSB

  - In RAN2 #130 meeting, RAN2 made the following agreements.

RAN2 confirms the working consumption that when AO-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB, as agreement. When OD-SSB is activated, UE uses servingCellMO-OD to measure serving cell; when OD-SSB is deactivated, UE uses servingCellMO-AO (i.e., legacy servingCellMO) to measure serving cell.

- Tentative agreement

  - RAN4 NOT to define the requirements when OD-SSB and AO-SSB are within different frequencies.

    - Issue 1-5-1(Requirement for Alt scenario 3), Issue 1-5-2(Neighbour cell measurement for scenario 3), Issue 1-5-3(Serving cell measurement for scenario 3), Issue 1-5-4(SCell activation for scenario 3), Issue 1-5-5(Measurement reporting for scenario 3), Issue 1-5-6(AO-SSB and OD-SSB CA combination) are closed

Discussion

### Issue 1-2-1: OD-SSB based L3 measurement when SCell is activated(scenario 3B)

- Background

  - In RAN2 #130 meeting, an LS has sent to RAN4 as follow.

For the case where AO-SSB and OD-SSB have the same center frequency, RAN2 had made the following agreement: - The UE applies the OD-SSB specific SMTC when the OD-SSB is activated and SCell is activated. This decision does not impact RAN4 discussion whether both OD-SSB and AO-SSB can be measured.

- Tentative agreement

For SCell measurement, UE shall follow the OD-SSB specific SMTC when OD-SSB is activated; For neighbor cell measurement, UE follows legacy SMTC regardless of status of OD-SSB.

Discussion

### Issue 1-1-4: Requirement for RAN1 Alt Time-C2

- Proposals

  - Option 1: CATT, China Telecom, CMCC, Ericsson, Nokia, ZTE, Huawei, Samsung

    - RAN4 to define the requirement based on the OD-SSB periodicity.

  - Option 1a: CATT, Nokia, Ericsson

    - No scheduling restriction is expected for AO-SSB when OD-SSB is activated.

  - Option 1b: Nokia

    - L1 measurement requirement is based on OD-SSB and DRX cycle if OD-SSB is activated, otherwise based on AO-SSB and DRX cycle.

  - Option 1c (Qualcomm)

    - RAN4 to define the delay requirement only based on the OD-SSB periodicity only if the periodicity of AO-SSB and OD-SSB are same

  - Option 2: LG

    - RAN4 to define the L3 and L1 measurement requirement only based on the OD-SSB periodicity only if the periodicity of AO-SSB and OD-SSB are same.

      - For L3 measurement requirement, RAN4 to take into account only OD-SSB

      - For L1 measurement requirement, RAN4 to take into account union of AO-SSB and OD-SSB

  - Option 3: OPPO

    - RAN4 to define the requirement following legacy AO-SSB within SMTC.

- Recommended WF

  - Moderator suggests the group to check whether the following proposal can be agreed.

    - RAN4 to define the requirement for Alt Time-C2 only based on the OD-SSB periodicity.

      - It applies to both deactivated SCell L3 measurement and L1 measurement.

      - No scheduling restriction is expected for AO-SSB in Alt Time-C2.

Discussion

### Issue 1-4-3: Additional Processing time for joint OD-SSB and SCell activation

- Background

In RAN4 #114bis meeting, RAN4 achieved agreements as follow.

Agreement UE applies the additional processing timing only in deactivated SCell measurement. - Note: The additional UE processing timing is not applied to SCell activation procedure. The additional processing timing is T_processing, the value of T_processing: - Introduce different values with UE capability: [2 or 3]ms and 5ms.

- Proposal

  - Option 1: Qualcomm, LGE, Apple

    - A UE capability for OD-SSB processing should be introduced with the same values as for MAC CE based OD-SSB activation for the deactivated SCell.

      - The candidate values can be 0ms, 2 ms and 5ms.

      - Concern: HW, Nokia, ZTE, Samsung, E///

- Tentative Agreement:

  - No consensus to introduce a UE capability for OD-SSB processing with the same values as for MAC CE based OD-SSB activation for the deactivated SCell.

Discussion