# 3GPP TSG-RAN WG4 Meeting #116                                R4-251xxxx

# Bangalore, India, 25th – 29th August 2025

# Title: 	TP for 38.753 introduction of spatial channel models for SU-MIMO PMI cases

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.12.3

# Document for:	Approval

# 1 Background

This draft TP captures comparison of PMI performance under SU-MIMO scenario for different spatial channel model parts.

# 2 Draft TP

# 6  Comparison of Spatial Channel Models

6.2  SU-MIMO (PMI Aspects)

This section provides comparison results for different spatial channel modes with simulation assumptions captured in Table 6.2-1.

Table 6.2-1: Common simulation assumptions for PMI

| Parameter |  |  | Unit | 8Tx |
| --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 40 |
| Subcarrier spacing |  |  | kHz | 30 |
| Duplex Mode |  |  |  | TDD |
| TDD DL-UL configurations |  |  |  | 7D1S2U S=6D+4G+4U |
| Antenna configuration |  |  |  | 8 x 4 (N1,N2) = (4,1) |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 8 |
|  | CDM Type |  |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  |  | Row 8, (4,6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | Row 8, (5) |
|  | CSI-RS interval and offset |  | slot | 10, 1 |
| cqi-FormatIndicator |  |  |  | Wideband |
| pmi-FormatIndicator |  |  |  | Not configured for eType II Wideband for Type I |
| Sub-band Size |  |  | RB | 8 |
| csi-ReportingBand |  |  |  | 11111111111111 |
| Codebook configuration | Codebook Type |  |  | - typeII-r16 - typeI-SP |
|  | eType II CB config | paramCombination-r16 |  | 6 (L =4, pν =1/2, β=1/2 ) |
|  |  | R(numberOfPMISubbandsPerCQISubband-r16) |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  |  | (4,1) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  |  | (4,1) |
|  | CodebookSubsetRestriction |  |  | 0x FFFF |
|  | RI Restriction (typeII-RI-Restriction-r16) |  |  | Rank 2: 0010 Rank 4: 1000 |
| Physical channel for CSI report |  |  |  | PUSCH |
| CQI/RI/PMI delay |  |  | ms | 7 |
| Maximum number of HARQ transmission |  |  |  | 4 |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  |  | Type I: Random and Follow PMI. eType II: Folow PMI |
| Note: Use DM-RS based FOE and compensation. |  |  |  |  |

Table 6.2-2: Simulation assumptions for CDL channel

| Parameter |  | Value |
| --- | --- | --- |
| FR / Carrier frequency |  | FR1,3.5GHz |
| UE speed and movement direction |  | 3km/h, (65°,90°) |
| Channel Geometry | LCS UE | α = 180°, β=0°, γ = 0° |
|  | LCS gNodeB | α = 0°, β=10°, γ = 0° |
|  | GCS UE | Height = 1.5 m; Azimuth = 0; X Coordinate = 100 m |
|  | GCS gNodeB | Height = 25 m; Azimuth = 0; X Coordinate = 0 m |
|  | BS Antenna Polarisation | Cross Polarized antenna elements with +/-45 degrees polarization slant angles |
|  | BS Radiation Pattern | Defined Table 7.3-1 in TS 38.901 |
|  | UE Antenna Polarisation | cross-polarized antenna elements with +90/0 degrees polarization slant angles |
|  | UE Antenna Radiation Pattern | Omnidirectional |
|  | Antenna Panel Placement | YZ Plane |

The BS antenna configuration for CDL comparison is one antenna element per subarray.

- 8Tx : (M, N, P, Ms, Ns) = (1, 4, 2, 1, 1).

The following comparison test cases are included this chapter:

- FR1 SU-MIMO PMI 8Tx 4Rx 4 layers with Type I (PMI Follow and PMI Random).

- FR1 SU-MIMO PMI 8Tx 4Rx 4 layers with eType II (PMI Follow).

- FR1 SU-MIMO PMI 8Tx 4Rx 2 layers with Type I (PMI Follow and PMI Random).

- FR1 SU-MIMO PMI 8Tx 4Rx 4 layers with eType II (PMI Follow).

Table 6.2-3: Simulation result summary for FR1 SU-MIMO PMI 8Tx 4Rx with 4 layers

|  | Source #1 | Source #2 | Source #3 | Source #4 | Source #5 | Source #6 | Average | Span |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Gamma | 1.8 | 1.8 |  |  |  |  |  |  |

- Table 6.2-4: Simulation result summary for FR1 SU-MIMO PMI 8Tx 4Rx with 4 layers

| SNR at Max Throughput [dB] |  | Source #1 | Source #2 | Source #3 | Source #4 | Source #5 | Source #6 | Average | Span |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type I (Follow) | 70% | 9.7 | 9.8 |  |  |  |  |  |  |
| Type I (Random) | 70% | 17.5 | 18.1 |  |  |  |  |  |  |

- Table 6.2-5: Simulation result summary for FR1 SU-MIMO PMI 8Tx 4Rx with 4 layers

| SNR at Max Throughput [dB] |  | Source #1 | Source #2 | Source #3 | Source #4 | Source #5 | Source #6 | Average | Span |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| eType II (Follow) | 70% | 8.5 | 10.8 |  |  |  |  |  |  |

- Table 6.2-6: Simulation result summary for FR1 SU-MIMO PMI 8Tx 4Rx with 2 layers

|  | Source #1 | Source #2 | Source #3 | Source #4 | Source #5 | Source #6 | Average | Span |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Gamma | 2.6 | 2.3 |  |  |  |  |  |  |

- Table 6.2-7: Simulation result summary for FR1 SU-MIMO PMI 8Tx 4Rx with 2 layers

| SNR at Max Throughput [dB] |  | Source #1 | Source #2 | Source #3 | Source #4 | Source #5 | Source #6 | Average | Span |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type I (Follow | 70% | 2.6 | 0.9 |  |  |  |  |  |  |
| Type I (Random) | 70% | 8.8 | 9.2 |  |  |  |  |  |  |

- Table 6.2-8: Simulation result summary for FR1 SU-MIMO PMI 8Tx 4Rx with 2 layers

| SNR at Max Throughput [dB] |  | Source #1 | Source #2 | Source #3 | Source #4 | Source #5 | Source #6 | Average | Span |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| eType II (Follow) | 70% | -0.5 | 1.3 |  |  |  |  |  |  |

Observation

- 2 companies said that the performance difference between Type I and eType II codebook can be observed in CDL channel model. But the Type I and eType II deployment scenarios are not currently captured in the TR.

- 1 company mentioned that the differences are observed for two different MMSE implementations.

- 1 company said that RAN4 to capture the qualitative observations related to the autocorrelation function of the CDL clusters. In detail, long duration and its high value between I/Q components are observed.

- 1 company said that no consensus on matches deployment observations on advanced receivers in MU adjacent setups.

- 1 company said that using CDL will align RAN4 with RAN1.

# 3 Conclusion

This draft TP captures comparison of PMI performance under SU-MIMO scenario for different spatial channel model part.