# 3GPP TSG-RAN WG4 Meeting #116                                R4-251xxxx

# Bangalore, India, 25th – 29th August 2025

# Title: 	TP for 38.753 introduction of spatial channel models for SU-MIMO PMI cases

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.12.3

# Document for:	Approval

# 1 Background

This draft TP captures comparison of PMI performance under SU-MIMO scenario for different spatial channel model parts.

# 2 Draft TP

# 6  Comparison of Spatial Channel Models

6.2  SU-MIMO (PMI Aspects)

This section provides comparison results for different spatial channel modes with simulation assumptions captured in Table 6.2-1.

Table 6.2-1: Common simulation assumptions for PMI

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 40 |
| Subcarrier spacing |  |  | kHz | 30 |
| Duplex Mode |  |  |  | TDD |
| TDD DL-UL configurations |  |  |  | 7D1S2U S=6D+4G+4U |
| Antenna configuration |  |  |  | 8 x 4 (M, N, P, Ms, Ns) = (1, 4, 2, 1, 1) |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 8 |
|  | CDM Type |  |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  |  | Row 8, (4,6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | Row 8, (5) |
|  | CSI-RS interval and offset |  | slot | 10, 1 |
| cqi-FormatIndicator |  |  |  | Wideband |
| pmi-FormatIndicator |  |  |  | Not configured for eType II Wideband for Type I |
| Sub-band Size |  |  | RB | 8 |
| csi-ReportingBand |  |  |  | 11111111111111 |
| Codebook configuration | Codebook Type |  |  | - typeII-r16 - typeI-SP |
|  | eType II CB config | paramCombination-r16 |  | 6 (L =4, pν =1/2, β=1/2 ) |
|  |  | R(numberOfPMISubbandsPerCQISubband-r16) |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  |  | (4,1) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  |  | (4,1) |
|  | CodebookSubsetRestriction |  |  | 0x FFFF |
|  | RI Restriction (typeII-RI-Restriction-r16) |  |  | Rank 2: 0010 Rank 4: 1000 |
| Physical channel for CSI report |  |  |  | PUSCH |
| CQI/RI/PMI delay |  |  | ms | 7 |
| Maximum number of HARQ transmission |  |  |  | 4 |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  |  | Type I: Random and Follow PMI. eType II: Folow PMI |
| Note: Use DM-RS based FOE and compensation. |  |  |  |  |

Table 6.2-2: Simulation assumptions for CDL channel

| Parameter |  | Value |
| --- | --- | --- |
| FR / Carrier frequency |  | FR1,3.5GHz |
| UE speed and movement direction |  | 3km/h, (65°,90°) |
| Channel Geometry | LCS UE | α = 180°, β=0°, γ = 0° |
|  | LCS gNodeB | α = 0°, β=10°, γ = 0° |
|  | GCS UE | Height = 1.5 m; Azimuth = 0; X Coordinate = 100 m |
|  | GCS gNodeB | Height = 25 m; Azimuth = 0; X Coordinate = 0 m |
|  | BS Antenna Polarisation | Cross Polarized antenna elements with +/-45 degrees polarization slant angles |
|  | BS Radiation Pattern | Defined Table 7.3-1 in TS 38.901 |
|  | UE Antenna Polarisation | cross-polarized antenna elements with +90/0 degrees polarization slant angles |
|  | UE Antenna Radiation Pattern | Omnidirectional |
|  | Antenna Panel Placement | YZ Plane |

The BS antenna configuration for CDL comparison is one antenna element per subarray.

- 8Tx : (M, N, P, Ms, Ns) = (1, 4, 2, 1, 1).

The following comparison test cases are included this chapter:

- FR1 SU-MIMO PMI 8Tx 4Rx 4 layers with Type I.

- FR1 SU-MIMO PMI 8Tx 4Rx 4 layers with eType II.

- FR1 SU-MIMO PMI 8Tx 4Rx 2 layers with Type I.

- FR1 SU-MIMO PMI 8Tx 4Rx 2 layers with eType II.

- Table 6.2-3: Simulation result summary for FR1 SU-MIMO PMI 8Tx 4Rx with 4 layers

| Channel Model | Doppler/Speed | PMI | Gamma at 90% Norm. Throughput |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Source #1 | Source #2 | Source #3 | Source #4 | Source #5 | Source #6 | Source #7 | Source #8 | Source #9 |
| rCDL-C | 3km/h | Type I | 1.8 |  |  |  |  | 1.8 |  | 1.7 |  |
|  |  | eType II | 1.8 |  |  |  |  | 1.7 |  | 1.4 |  |
| xTDL-C | 10Hz | Type I |  |  |  |  |  |  |  | 1.4 |  |
|  |  | eType II |  |  |  |  |  |  |  | 1.3 |  |
| TDLC-300 low | 10Hz | Type I | 1.3 |  |  |  |  | 1.1 |  |  |  |
|  |  | eType II | 1.3 |  |  |  |  | 0.9 |  |  |  |
| TDLC-300 Med | 10Hz | Type I |  |  |  |  |  |  |  |  |  |
|  |  | eType II |  |  |  |  |  |  |  |  |  |
| TDLC-300 XP Med | 10Hz | Type I | 1.9 |  |  |  |  | 1.8 |  |  |  |
|  |  | eType II | 1.8 |  |  |  |  | 1.2 |  |  |  |
| TDLC-300 XP High | 10Hz | Type I | 2.0 |  |  |  |  |  |  | 2.1 |  |
|  |  | eType II | 2.0 |  |  |  |  |  |  | 2.0 |  |

Based on current simulation results are provided by companies, we have the following observation.

- The results show differences between the various channel models.

- All companies' results are very similar under the same channel model.

- Table 6.2-4: Simulation result summary for FR1 SU-MIMO PMI 8Tx 4Rx with 4 layers

| Channel Model | Doppler/Speed | PMI | SNR at 70% Norm. Throughput |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Source #1 | Source #2 | Source #3 | Source #4 | Source #5 | Source #6 | Source #7 | Source #8 | Source #9 |
| rCDL-C | 3km/h | Type I | 12.4 | 11.0 | 10.4 | 10.3 | 10.2 | 9.7 | 9.5 | 10.2 | 9.8 |
|  |  | eType II | 14.0 | 14.3 | 10.3 | 11.6 | 10.1 | 10.8 | 8.2 | 13.8 | 10.8 |
| xTDL-C | 10Hz | Type I |  |  |  | 12.3 | 13.0 |  |  | 10.8 |  |
|  |  | eType II |  |  |  | 13.5 | 14.7 |  |  | 11.6 |  |
| TDLC-300 low | 10Hz | Type I | 12.6 |  |  |  |  | 13.2 |  |  |  |
|  |  | eType II | 12.8 |  |  |  |  | 15.7 |  |  |  |
| TDLC-300 Med | 10Hz | Type I |  |  |  |  |  |  |  |  |  |
|  |  | eType II |  |  |  |  |  |  |  |  |  |
| TDLC-300 XP Med | 10Hz | Type I | 12.7 |  |  |  |  | 18.9 |  |  |  |
|  |  | eType II | 12.8 |  |  |  |  | 24.5 |  |  |  |
| TDLC-300 XP High | 10Hz | Type I | 18.6 |  |  |  |  |  |  | 23.4 |  |
|  |  | eType II | 17.4 |  |  |  |  |  |  | 24.6 |  |

Based on current simulation results are provided by companies, we have the following observation.

- The results show differences between the various channel models, especially for rCDL-C and TDLC-300 XP High.

- All companies' results are very similar under the same channel model.

- Table 6.2-5: Simulation result summary for FR1 SU-MIMO PMI 8Tx 4Rx with 2

- layers

| Channel Model | Doppler/Speed | PMI | Gamma at 90% Norm. Throughput |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Source #1 | Source #2 | Source #3 | Source #4 | Source #5 | Source #6 | Source #7 | Source #8 | Source #9 |
| rCDL-C | 3km/h | Type I | 2.0 |  |  |  |  | 2.3 |  | 1.7 |  |
|  |  | eType II | 1.9 |  |  |  |  | 2.4 |  | 1.6 |  |
| xTDL-C | 10Hz | Type I |  |  |  |  |  |  |  | 1.5 |  |
|  |  | eType II |  |  |  |  |  |  |  | 1.6 |  |
| TDLC-300 low | 10Hz | Type I | 1.5 |  |  |  |  |  |  |  |  |
|  |  | eType II | 1.8 |  |  |  |  |  |  |  |  |
| TDLC-300 Med | 10Hz | Type I |  |  |  |  |  | 1.05 |  |  |  |
|  |  | eType II |  |  |  |  |  | 1.4 |  |  |  |
| TDLC-300 XP Med | 10Hz | Type I | 2.8 |  |  |  |  | 2.1 |  |  |  |
|  |  | eType II | 2.6 |  |  |  |  | 2 |  |  |  |
| TDLC-300 XP High | 10Hz | Type I | 6.1 |  |  |  |  |  |  | 3.7 |  |
|  |  | eType II | 3.3 |  |  |  |  |  |  | 2.3 |  |

Based on current simulation results are provided by companies, we have the following observation.

- The results show differences between the various channel models, especially for rCDL-C and TDLC-300 XP High.

- Companies' results are have a big difference under the TDLC-300 XP High.

- Table 6.2-6: Simulation result summary for FR1 SU-MIMO PMI 8Tx 4Rx with 2 layers

| Channel Model | Doppler/Speed | PMI | SNR at 70% Norm. Throughput |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Source #1 | Source #2 | Source #3 | Source #4 | Source #5 | Source #6 | Source #7 | Source #8 | Source #9 |
| rCDL-C | 3km/h | Type I | 4.7 |  |  |  |  | 0.9 |  | 2.2 |  |
|  |  | eType II | 5.3 |  |  |  |  | 1.5 |  | 3.3 |  |
| xTDL-C | 10Hz | Type I |  |  |  |  |  |  |  | 4.3 |  |
|  |  | eType II |  |  |  |  |  |  |  | 3.5 |  |
| TDLC-300 low | 10Hz | Type I | 5.1 |  |  |  |  |  |  |  |  |
|  |  | eType II | 4.4 |  |  |  |  |  |  |  |  |
| TDLC-300 Med | 10Hz | Type I |  |  |  |  |  | 1.5 |  |  |  |
|  |  | eType II |  |  |  |  |  | 1.3 |  |  |  |
| TDLC-300 XP Med | 10Hz | Type I | 2.1 |  |  |  |  | 3.8 |  |  |  |
|  |  | eType II | 2.7 |  |  |  |  | 5.4 |  |  |  |
| TDLC-300 XP High | 10Hz | Type I | 2.1 |  |  |  |  |  |  | 1.1 |  |
|  |  | eType II | 2.9 |  |  |  |  |  |  | 4.5 |  |

Based on current simulation results are provided by companies, we have the following observation.

- The results show differences between the various channel models, especially for rCDL-C and TDLC-300 XP High.

- Companies' results are have a big difference under rCDL-C channel model.

# 3 Conclusion

This draft TP captures comparison of PMI performance under SU-MIMO scenario for different spatial channel model part.