3GPP TSG-RAN WG4 Meeting #116	                                                                     R4-2510887

Bengaluru, India, August 25th – 29th, 2025

Title: 	Draft TP introduction of comparison of spatial channel models for SU-MIMO cases

Source: 	Huawei, HiSilicon

Agenda item:       7.12.2

Document for:	Approval

# Background

This draft TP captures comparison of PDSCH performance under SU-MIMO scenario for different spatial channel model part.

- Draft TP

# 6.  Comparison of spatial channel Models

## 6.1  PDSCH performance under SU-MIMO scenario

This section provides comparison for different spatial channel models with common simulation assumptions captured in Table 6.1-1.

Table 6.1-1: Common Simulation assumptions

| Parameter |  |  | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| TDD Slot Configuration Pattern |  |  | 7D1S2U (PDSCH is not scheduled in S slot for Rank8) |
| Channel Bandwidth/SCS |  |  | 40MHz/30kHz |
| Rank |  |  | 4, 8 |
| Antenna configuration |  |  | Rank4: 4T4R Rank8: 8T8R |
| MCS |  |  | 13 (64 QAM table) |
| Channel model |  |  | rCDL-C1 xTDL-C1 TDLC300-Low TDLC300-ULA Med TDLC300-ULA High TDLC300-XP Med TDLC300-XP High |
| Codebook configuration for PDSCH and DMRS |  | CodebookType | Single Panel Type I; Randomized precoder selection for every REG bundle and updated per slot with equal probability of each applicable i1/i2 combination or codebook . |
|  |  | Codebook configuration | (N1,N2,O1,O2) = (4,1,4,1) |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | Resource allocation type |  | Type 0 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 2 |
| Number of HARQ Processes |  |  | 8 |
| Maximum HARQ transmissions |  |  | 4 |
| UE receiver type |  |  | MMSE-IRC |
| Test metric |  |  | Rank 4: SNR@70% of maximum throughput Rank8: SNR(dB) @ 30% and 70% of max throughput for each codeword |

Simulation assumptions for rCDL-C1 channel specifically are captured in Table 6.1-2:

Table 6.1-2: Simulation assumptions for rCDL-C1 channel

| Parameter |  | Value |  |
| --- | --- | --- | --- |
|  |  | Rank8 |  |
| FR / Carrier frequency |  | FR1, 3.5GHz |  |
| UE speed and movement direction |  | 3km/h, (65°,90°) |  |
| AAV assumptions |  | (M,N,P,Ms,Ns) = (1,2,2,1,1) | (M,N,P,Ms,Ns) = (1,4,2,1,1) |
| Channel Geometry | LCS UE | α = 180°, β=0°, γ = 0° |  |
|  | LCS gNodeB | α = 0°, β=10°, γ = 0° |  |
|  | GCS UE | Height = 1.5 m; Azimuth = 0; X Coordinate = 100 m |  |
|  | GCS gNodeB | Height = 25 m; Azimuth = 0; X Coordinate = 0 m |  |
|  | BS Antenna Polarisation | Cross Polarized antenna elements with +/-45 degrees polarization slant angles |  |
|  | BS Radiation Pattern | Defined Table 7.3-1 in TS 38.901 |  |
|  | UE Antenna Polarisation | cross-polarized antenna elements with +90/0 degrees polarization slant angles |  |
|  | UE Antenna Radiation Pattern | Omnidirectional |  |
|  | Antenna Panel Placement | YZ Plane |  |

The following comparison test cases are included

- FR1 SU-MIMO PMI 4Tx 4Rx 4 layers

- FR1 SU-MIMO PMI 8Tx 8Rx 8 layers

Table 6.2-3: Simulation result summary of SNR@70% of maximum throughput for FR1 SU-MIMO PMI 4Tx 4Rx with 4 layers

| Channel model | UE speed/Doppler | Source #1 | Source #2 | Source #3 | Source #4 | Source #5 | Source #6 | Source #7 | Source #8 | Source #9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| rCDL-C1 | 3 kph | 15.5 | 14.3 | 16.48 | 15.6 | 15.9 | 16.6 | 14.7 | 15.6 |  |
| xTDL-C1 | 10 Hz |  |  |  | 13.5 | 15.9 |  | 14 | 15.4 | 15.8 |
| TDLC300-Low | 10 Hz | 14 |  | 13.21 |  | 14.7 | 14.3 | 13 | 13.6 |  |
| TDLC300-ULA Med | 10 Hz | N/A |  |  |  | 49.2 | N/A |  | N/A |  |
| TDLC300-ULA High | 10 Hz |  |  |  |  |  |  |  | N/A |  |
| TDLC300-XP Med | 10 Hz | 15.3 |  | 14.95 |  | 16.3 |  |  | 15.5 |  |
| TDLC300-XP High | 10 Hz |  |  |  |  |  |  |  | 27.6 |  |
| rCDL-C1 | 30 kph | 16.3 | 15.34 | 17.17 |  |  | 17.9 | 15.2 | 17.5 |  |
| xTDL-C1 | 100 Hz |  |  |  |  |  |  | 14.5 |  |  |
| TDLC300-Low | 100 Hz | 14.5 |  | 14.05 |  | 15.6 |  | 13.3 |  |  |
| TDLC300-ULA Med | 100 Hz | N/A |  |  |  | N/A |  |  |  |  |
| TDLC300-ULA High | 100 Hz |  |  |  |  |  |  |  |  |  |
| TDLC300-XP Med | 100 Hz | 16.1 |  | 14.95 |  | 17.4 |  |  |  |  |
| TDLC300-XP High | 100 Hz |  |  |  |  | 15.9 | 16.6 | 14.7 | 15.6 |  |

Table 6.2-4: Simulation result summary of SNR@70% of maximum throughput for FR1 SU-MIMO PMI 8Tx 8Rx with 8 layers (sources #1-#5)

| Channel model | UE speed/Doppler | Source #1 |  | Source #2 |  | Source #3 |  | Source #4 |  | Source #5 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | CW1 | CW2 | CW1 | CW2 | CW1 | CW2 | CW1 | CW2 | CW1 | CW2 |
| rCDL -C 1 | 3 kph |  |  |  |  |  | 16.9 |  | 23.7 |  | 14.2 |  | 26 |  | 16.5 |  | 24.4 |  | 16.7 |  | 24 |  |
| xTDL-C1 | 10 Hz |  |  |  |  |  |  |  |  |  |  |  |  |  | 14.5 |  | 21.2 |  | 11.1 |  | 19.7 |  |
| TDLC300-Low | 10 Hz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 15.7 |  | 15.8 |  |
| TDLC300-ULA Med | 10 Hz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  | N/A |  |
| TDLC300-ULA High | 10 Hz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TDLC300-XP Med | 10 Hz |  |  |  |  |  |  |  |  |  | N/A |  | N/A |  |  |  |  |  | N/A |  | 35.1 |  |
| TDLC300-XP High | 10 Hz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| rCDL -C 1 | 30 kph |  |  |  |  |  | 18.13 |  | 23.91 |  | 15.32 |  | 28.54 |  |  |  |  |  |  |  |  |  |
| xTDL-C1 | 100 Hz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TDLC300-Low | 100 Hz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 16.4 |  | 16.4 |  |
| TDLC300-ULA Med | 100 Hz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  | N/A |  |
| TDLC300-ULA High | 100 Hz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TDLC300-XP Med | 100 Hz |  |  |  |  |  |  |  |  |  | N/A |  | N/A |  |  |  |  |  | N/A |  | 39.1 |  |
| TDLC300-XP High | 100 Hz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2-5: Simulation result summary of SNR@70% of maximum throughput for FR1 SU-MIMO PMI 8Tx 8Rx with 8 layers (sources #6-#9)

| Channel model | UE speed/Doppler |  | Source #6 |  |  | Source #7 |  |  |  | Source #8 |  |  |  | Source #9 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | CW1 |  | CW2 | CW1 |  | CW2 |  | CW1 |  | CW2 |  | CW1 |  | CW2 |
| rCDL -C 1 |  | 3 kph |  | 17.5 |  |  | 24.7 |  | 17.2 |  | 23.4 |  | 18.5 |  | 24.4 |  |  |  |  |
| xTDL-C1 | 10 Hz |  |  |  |  | 15.4 |  | 16.6 |  | 14.8 |  | 20.4 |  |  |  |  |
| TDLC300-Low | 10 Hz |  | 15.9 |  | 15.9 | 15.5 |  | 15.7 |  | 14.4 |  | 14.3 |  |  |  |  |
| TDLC300-ULA Med | 10 Hz |  | N/A |  | N/A |  |  |  |  | N/A |  | N/A |  |  |  |  |
| TDLC300-ULA High | 10 Hz |  |  |  |  |  |  |  |  | N/A |  | N/A |  |  |  |  |
| TDLC300-XP Med | 10 Hz |  |  |  |  |  |  |  |  | 27.4 |  | N/A |  |  |  |  |
| TDLC300-XP High | 10 Hz |  |  |  |  |  |  |  |  | N/A |  | N/A |  |  |  |  |
| rCDL -C 1 | 30 kph |  | 18.6 |  | 25.5 | 17.4 |  | 23.8 |  | 21.1 |  | 26.4 |  |  |  |  |
| xTDL-C1 | 100 Hz |  |  |  |  | 16 |  | 17.4 |  |  |  |  |  |  |  |  |
| TDLC300-Low | 100 Hz |  |  |  |  | 16.4 |  | 16.6 |  |  |  |  |  |  |  |  |
| TDLC300-ULA Med | 100 Hz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TDLC300-ULA High | 100 Hz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TDLC300-XP Med | 100 Hz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TDLC300-XP High | 100 Hz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

For more results with full curves refer to [R4-2509413].

Regarding legacy TDL the following observations can be drawn:

[See conclusion section]

Regarding rCDL-C1 the following observations can be drawn:

[See conclusion section]

Regarding xTDL-C1 the following observations can be drawn:

[See conclusion section]

- Conclusion

This draft TP captures comparison of PDSCH performance under SU-MIMO scenario for different spatial channel model part.