3GPP TSG-RAN WG4 Meeting # 117  		                                    R4-2522645

Dallas, USA, Nov. 17-21, 2025

Agenda item:			6.16.1

Source:	Moderator (vivo)

Title:	WF on NR_LPWUS_RRM

Document for:	Approval

Topic #1: Agreement on RRM core requirement maintenance for LP-WUS/WUR

Issue 1-1-1: LP-WUR operation with Rel-18 EMR

Agreement:

RAN4 confirms that no spec change is needed for R18 EMR

Issue 1-1-2: LP-WUR operation with SDT

Agreement:

No RRM spec impact. This issue is closed.

Issue 1-1-3: UE behavior when both low mobility for Rel-16 relaxation(or stationary for Rel-17) and Rel-19 LP-WUR offloading/relaxation are configured; and Rel-19 LP-WUR offloading/relaxation are satisfied however Rel-16 low-mobility/Rel-17 stationary is not satisfied

Agreement:

Close the issue based on latest RAN2 agreement

Issue 1-1-5: On whether to consider MR wake up delay after MR exits from case 3 due to relaxation conditions cannot be met

Agreement:

When the UE is in RRM relaxation (case 3) state, and the UE exits from case 3 due to the configured LR conditions of RRM relaxation is not met, MR reuse the MR wake up delay.

Issue 1-1-6: On MR measurement behaviour after exiting case 1 due to corresponding conditions cannot be met

Agreement:

After exiting case 1 due to corresponding conditions cannot be met, UE follows legacy measurement and meet the corresponding requirements.

Issue 1-1-7: MR measurement at RRM relaxation mode

Agreement:

In RRM relaxation mode, the 2 samples used for filtering should be spaced by at least 8 DRX cycles.

Topic #2: Agreement on LP-WUR performance

Issue 3-2-1: Test case design for entry conditions

Agreement:

No consensus to define test case for no-entry. This issue is closed.

Issue 3-2-2: Test case design for LP-WUS monitoring

Agreement:

Do not define dedicated test case for LP-WUS monitoring.

Issue 3-2-3: Test case design for higher priority frequency layer search

Agreement:

Do not define test case for higher priority frequency layer search.

Issue 3-2-4: On {M, L} for TC1 and TC3

Agreement: TC1 {M, L} = {1, 8}, TC3 {M, L} = {4, 32}

Topic #3: Recommendation for issues

On maintenance issues

For remaining issues in [1], contribution driven

On issues for performance part

For remaining issues in [1], contribution driven

Topic #4: CRs for test case list and test configuration for information

Issue 4-1: Test case list phase 1 based on existing agreements:

| Test case list | Company |
| --- | --- |
|  TC1: UE exit Case 1 to legacy where only LP-SS signal is used in the test case for OOK based LR | vivo |
|  TC2: UE exit Case 3 to legacy where only PSS/SSS signal is used in the test case for OFDM based LR | ZTE |
|  TC3: UE exit Case 3 to legacy where only LP-SS signal is used in the test case for OOK based LR | Xiaomi |
|  TC4: Cell reselection to FR1 intra-frequency NR case for UE fulfilling Rel-19 LP-WUR RRM relaxation criterion | OPPO |
|  TC5: UE exit Case 3 to legacy where only PSS/SSS signal is used in the test case for OFDM based LR for FR2 | Qualcomm |

|  Test configuration for LP-SS | Huawei |
| --- | --- |

Reference

[1] R4-2520839, Topic summary for [116bis][210] NR_LPWUS_RRM, vivo, RAN4 117

[2] R4-2522646,  Ad-hoc minutes for NR_LPWUS_RRM, vivo, RAN4 117