3GPP TSG-RAN5 Meeting #108	R5-253795

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 5047 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of LTM test case 7.1.1.1.28 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc.,MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | All cells were configured with a TA value of 0 by default.If varying TA values were to be utilized, it was imperative to specify the TA value for each individual cell within the pre-test conditions. Table 7.1.1.1.28.3.2-1/2 is executed in time order, T1 has been done at step 3, new Time instances need to be created for subsequent steps. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The TA value of the NR Cell 1 and NR Cell 2 were indicated. Table 7.1.1.1.28.3.2-1/2 were updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case will contain inaccuracies. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.1.28 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

7.1.1.1.28	Random access procedure / Successful / LTM cell switch / LBT failure

7.1.1.1.28.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state with Random Access Procedure triggered by a PDCCH order for an LTM candidate cell }

ensure that {

when { LBT failure indication is received from lower layers }

then { UE considers this Random Access procedure completed }

}

7.1.1.1.28.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.321 clauses 5.1.3 and 5.21.2. Unless otherwise stated these are Rel-18 requirements.

[TS 38.321, clause 5.1.3]

The MAC entity shall, for each Random Access Preamble:

1>	if PREAMBLE_TRANSMISSION_COUNTER is greater than one; and

1>	if the notification of suspending power ramping counter has not been received from lower layers; and

1>	if LBT failure indication was not received from lower layers for the last Random Access Preamble transmission; and

1>	if SSB or CSI-RS selected is not changed from the selection in the last Random Access Preamble transmission; and

1>	if the Random Access procedure is not initiated by the PDCCH order for an LTM candidate cell:

2>	increment PREAMBLE_POWER_RAMPING_COUNTER by 1.

1>	if the Random Access procedure is initiated by the PDCCH order for an LTM candidate cell as preamble re-transmission; and

1>	if the PDCCH order indicates the same LTM candidate cell and the same SSB as the last Random Access Preamble transmission:

2>	increment PREAMBLE_POWER_RAMPING_COUNTER by 1.

1>	select the value of DELTA_PREAMBLE according to clause 7.3;

1>	set PREAMBLE_RECEIVED_TARGET_POWER to preambleReceivedTargetPower + DELTA_PREAMBLE + (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA;

1>	except for contention-free Random Access Preamble for beam failure recovery request and contention-free Random Access Preamble triggered by a PDCCH order for an LTM candidate cell, compute the RA-RNTI associated with the PRACH occasion in which the Random Access Preamble is transmitted;

1>	instruct the physical layer to transmit the Random Access Preamble using the selected PRACH occasion, corresponding RA-RNTI (if available), PREAMBLE_INDEX, and PREAMBLE_RECEIVED_TARGET_POWER.

1>	if the Random Access Procedure is triggered by a PDCCH order for an LTM candidate cell:

2>	consider this Random Access procedure completed.

[TS 38.321, clause 5.21.2]

The MAC entity shall:

1>	if consistent LBT failure has been triggered, and not cancelled, in the SpCell; and

1>	if UL-SCH resources are available for a new transmission in the SpCell and these UL-SCH resources can accommodate the LBT failure MAC CE plus its subheader as a result of logical channel prioritization:

2>	instruct the Multiplexing and Assembly procedure to generate the LBT failure MAC CE.

1>	else if consistent LBT failure has been triggered, and not cancelled, in at least one SCell:

2>	if UL-SCH resources are available for a new transmission in a Serving Cell for which consistent LBT failure has not been triggered and these UL-SCH resources can accommodate the LBT failure MAC CE plus its subheader as a result of logical channel prioritization:

3>	instruct the Multiplexing and Assembly procedure to generate the LBT failure MAC CE.

2>	else:

3>	trigger a Scheduling Request for LBT failure MAC CE.

1>	if a MAC PDU is transmitted and LBT failure indication is not received from lower layers and this PDU includes the LBT failure MAC CE:

2>	cancel all the triggered consistent LBT failure(s) in SCell(s) for which consistent LBT failure was indicated in the transmitted LBT failure MAC CE.

1>	if consistent LBT failure is triggered and not cancelled in the SpCell; and

1>	if the Random Access procedure is considered successfully completed (see clause 5.1) in the SpCell:

2>	cancel all the triggered consistent LBT failure(s) in the SpCell.

1>	if lbt-FailureRecoveryConfig is reconfigured by upper layers for a Serving Cell:

2>	cancel all the triggered consistent LBT failure(s) in this Serving Cell.

7.1.1.1.28.3	Test description

7.1.1.1.28.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except the following:

-	Test loop function (Off)

-	2 NR cells (NR Cell 1 and NR Cell 2) are configured. The TA value of NR Cell 1 is set to 0 and the TA value of NR Cell 2 is set to 1.

7.1.1.1.28.3.2	Test procedure sequence

Tables 7.1.1.1.28.3.2-1 and 7.1.1.1.28.3.2-2 illustrate the downlink power levels to be applied for NR Cell 1 and NR Cell 2 at various time instants of the test execution. Row marked "T0" denotes the conditions after the preamble, while the configuration marked "T1" is applied at the point indicated in the Main behaviour description in Table 7.1.1.1.28.3.2-3.

Table 7.1.1.1.28.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -85 | -91 | Power levels are such that entry condition for event A3 is not satisfied for the neighbour NR cell |
| T1 |  | SS/PBCH SSS EPRE |  | dBm/SCS |  | -85 |  | -79 |  | Power levels are such that entry condition for event A3 is satisfied for intra-frequency neighbour NR cell |  |  |
| T2 |  | SS/PBCH SSS EPRE |  | dBm/SCS |  | -85 |  | -79 |  |  |  |  |
|  |  | Noc |  | dBm/SCS |  | -85 |  | - |  | Noc is applied for NR Cell 1 |  |  |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | -85 | -79 |  |

Table 7.1.1.1.28.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -91 | -100 | Power levels are such that entry condition for event A3 is not satisfied for the neighbour NR cell |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -91 | -82 | Power levels are such that entry condition for event A3 is satisfied for intra-frequency neighbour NR cell |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -91 | -82 |  |
|  | Noc | dBm/SCS | -91 | - | Noc is applied for NR Cell 1 |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | -91 | -82 |  |

Table 7.1.1.1.28.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message including MeasConfig to setup NR measurement and reporting for intra-frequency event A3 on NR Cell 1. | <-- | RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message. | --> | RRCReconfigurationComplete | - | - |
| 3 | SS re-adjusts the cell-specific reference signal level according to row "T1" in table 7.1.1.1.28.3.2-1/2. | - | - | - | - |
| 4 | The UE transmits a MeasurementReport message to report event A3 with the measured RSRP value for NR Cell 2 in NR Cell 1. | --> | MeasurementReport | - | - |
| 5 | The SS transmits an RRCReconfiguration message with LTM-Candidate-r18 for NR Cell 2 on NR Cell 1. | <-- | RRCReconfiguration | - | - |
| 6 | The UE transmits RRCReconfigurationComplete message. | --> | RRCReconfigurationComplete | - | - |
| 7 | The SS transmits the candidate cell TCI activation MAC CE with Candidate Cell ID set to 0 for NR Cell 2 on NR Cell 1. | <-- | MAC PDU (Candidate Cell TCI state activation/deactivation MAC CE) | - | - |
| 8 | The SS transmits DCI 1_0 PDCCH order on NR Cell 1 to trigger the UE PRACH transmission. | <-- | (PDCCH Order) | - | - |
| 9 | The SS adjusts the SS/PBCH EPRE and Noc levels according to row "T2" in table 7.1.1.1.28.3.2-1/2. | - | - | - | - |
| 10 | Check: Does UE detect consistent LBT failures while attempting to transmits the PRACH preamble and transmit a MAC PDU containing LBT failure MAC CE on NR Cell 1? | --> | MAC PDU (LBT failure MAC CE of one octet (C1=1)) | 1 | P |
| 11 | The SS adjusts the SS/PBCH EPRE according to row "T3" in table 7.1.1.1.28.3.2-1/2. | - | - | - | - |
| 12 | The SS transmits DCI 1_0 PDCCH order on NR Cell 1 to trigger the UE PRACH transmission. | <-- | (PDCCH Order) | - | - |
| 13 | The UE transmits the PRACH preamble on NR Cell 2. | --> | (PRACH Preamble) | - | - |
| 14 | The SS transmits LTM cell switch command MAC CE with Timing Advance Command value set to 1 and Target Configuration ID value set to 0 to the UE. | <-- | MAC PDU (LTM Cell Switch Command MAC CE) | - | - |
| 15 | The SS transmits an UL Grant, allowing the UE to transmit the RRCReconfigurationComplete message on NR Cell 2. | <-- | (UL Grant (C-RNTI)) | - | - |
| 16 | The UE transmits RRCReconfigurationComplete message on NR Cell 2. | --> | RRCReconfigurationComplete | - | - |

<End of change>