3GPP TSG-RAN WG5 Meeting #108	R5-253842

Bangaluru, India, 25th – 29th Aug, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3522 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of SIB23 and related scheduling information for SLPP test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | SIB23 contains NR sidelink Positioning configuration for dedicated SL-PRS resource pool. SIB23 need to be introduced for SLPP test cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added system information combination NR-35 and related scheduling information. - Added default information elements contents for SIB23. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The release 18 positioning work item will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.3, 4.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

4.4.3.1.2	Combinations of system information blocks for NR standalone and NE-DC

The combination of system information blocks required by a test case depends on the test case scenario. In this clause, several combinations of system information blocks are defined.

Regardless of the combination of system information blocks indicated as being used by a test case, the SS shall broadcast only the NR MIB on the NR Cell(s) configured on an SDL band and on NR SCell(s) configured on a band having no uplink frequency specified.

For NE-DC network scenario the SS shall in addition to broadcasting the NR system information blocks also broadcast the E-UTRA system information combination 1 on the E-UTRA cell(s) as specified in TS 36.508 [2].

Combination NR-1 is the default combination which applies to the following test case scenarios:

-	NR FDD single cell scenario except RRM test case scenarios

-	NR TDD single cell scenario except RRM test case scenarios

-	NR FDD inter-band DC component carriers cell scenario

-	NR TDD inter-band DC component carriers cell scenario

-	NR FDD and NR TDD inter-band DC component carriers cell scenario

Combination NR-2 applies to the following test case scenarios:

-	NR FDD intra-frequency multi cell scenario

-	NR TDD intra-frequency multi cell scenario

-	NR FDD and NR TDD dual mode multi cell roaming scenario

-	NR FDD single cell RRM test case scenario

-	NR TDD single cell RRM test case scenario

Combination NR-3 applies to the following test case scenarios:

-	NR FDD intra-frequency multi cell scenario with neighbouring cell related information

-	NR TDD intra-frequency multi cell scenario with neighbouring cell related information

Combination NR-4 applies to the following test case scenarios:

-	NR FDD inter-frequency multi cell scenario

-	NR TDD inter-frequency multi cell scenario

-	NR FDD inter-band multi cell scenario

-	NR TDD inter-band multi cell scenario

-	NR FDD and NR TDD dual mode multi cell non-roaming scenario

-	NR FDD intra-band carrier aggregation component carriers cell scenario

-	NR FDD inter-band carrier aggregation component carriers cell scenario

-	NR TDD intra-band carrier aggregation component carriers cell scenario

-	NR FDD and NR TDD inter-band carrier aggregation component carriers cell scenario

Combination NR-5 applies to the following test case scenarios:

-	NR FDD intra-band carrier aggregation component carriers cell scenario + NR FDD intra-frequency neighbour.

-	NR FDD inter-band carrier aggregation component carriers cell scenario+ NR FDD intra-frequency neighbour.

-	NR TDD intra-band carrier aggregation component carriers cell scenario+ NR FDD intra-frequency neighbour.

-	NR FDD and NR TDD inter-band carrier aggregation component carriers cell scenario+ NR FDD intra-frequency neighbour.

Combination NR-6 applies to the following test case scenarios:

-	3GPP inter-RAT NR FDD + E-UTRA FDD multi cell scenario

-	3GPP inter-RAT NR TDD + E-UTRA TDD multi cell scenario

-	3GPP inter-RAT NR TDD + E-UTRA FDD multi cell scenario

Combination NR-7 applies to the following test case scenarios:

-	NR FDD inter-frequency + 3GPP inter-RAT E-UTRA multi-cell scenario

-	NR TDD inter-frequency + 3GPP inter-RAT E-UTRA multi-cell scenario

Combination NR-8 applies to the following test case scenarios:

-	NR FDD ETWS single cell scenario

-	NR TDD ETWS single cell scenario

Combination NR-9 applies to the following test case scenarios:

-	3GPP NR FDD + CMAS single cell scenario

-	3GPP NR TDD + CMAS single cell scenario

Combination NR-10 applies to the following test case scenarios:

-	3GPP NR FDD + ETWS primary notification single cell scenario

-	3GPP NR TDD + ETWS primary notification single cell scenario

Combination NR-11 applies to the following test case scenarios:

-	3GPP NR FDD + ETWS secondary notification single cell scenario

-	3GPP NR TDD + ETWS secondary notification single cell scenario

Combination NR-12 applies to the following test case scenarios:

-	3GPP NR FDD + SNPN only single cell scenario

-	3GPP NR TDD + SNPN only single cell scenario

Combination NR-13 applies to the following test case scenarios:

-	3GPP NR FDD + CAG cell  multi cell scenario

-	3GPP NR TDD + CAG cell  multi cell scenario

Combination NR-14 applies to the following test case scenarios:

-	3GPP NR FDD single cell scenario + NR sidelink communication with or without network scheduling.

-	3GPP NR TDD single cell scenario + NR sidelink communication with or without network scheduling.

-	3GPP NR FDD intra-frequency multi cell scenario + NR sidelink communication with or without network scheduling.

-	3GPP NR TDD intra-frequency multi cell scenario + NR sidelink communication with or without network scheduling.

-	3GPP NR FDD inter-frequency multi cell scenario + NR sidelink communication with or without network scheduling.

-	3GPP NR TDD inter-frequency multi cell scenario + NR sidelink communication with or without network scheduling.

-	3GPP NR FDD inter-band multi cell scenario + NR sidelink communication with or without network scheduling.

-	3GPP NR TDD inter-band multi cell scenario + NR sidelink communication with or without network scheduling.

Combination NR-15 applies to the following test case scenarios:

-	3GPP NR FDD single cell scenario + positioning test case which require posSIBs.

-	3GPP NR TDD single cell scenario + positioning test case which require posSIBs.

Combination NR-16 applies to the following test case scenarios:

-	3GPP NR FDD intra-frequency multi cell scenario + positioning test case which require posSIBs.

-	3GPP NR TDD intra-frequency multi cell scenario + positioning test case which require posSIBs.

Combination NR-17 applies to the following test case scenarios:

-	3GPP NR FDD inter-frequency multi cell scenario + positioning test case which require posSIBs.

-	3GPP NR TDD inter-frequency multi cell scenario + positioning test case which require posSIBs.

Combination NR-18 applies to the following test case scenarios:

-	3GPP NR FDD inter-frequency multi cell scenario + idle/inactive measurements.

-	3GPP NR TDD inter-frequency multi cell scenario + idle/inactive measurements.

Combination NR-19 applies to the following test case scenarios:

-	3GPP NR FDD + Extended field in SI scenario

-	3GPP NR TDD + Extended field in SI scenario

Combination NR-20 applies to the following test case scenarios:

-	3GPP NR FDD single cell scenario + MBS data.

-	3GPP NR TDD single cell scenario + MBS data.

-	3GPP NR FDD intra-frequency multi cell scenario + MBS data.

-	3GPP NR TDD intra-frequency multi cell scenario + MBS data.

Combination NR-21 applies to the following test case scenarios:

-	3GPP NR FDD intra-frequency multi cell scenario + MBS Service Continuity.

-	3GPP NR TDD intra-frequency multi cell scenario + MBS Service Continuity.

Combination NR-22 applies to the following test case scenarios:

-	3GPP NR FDD intra-frequency multi cell scenario + MBS data + MBS Service Continuity.

-	3GPP NR TDD intra-frequency multi cell scenario + MBS data + MBS Service Continuity.

Combination NR-23 applies to the following test case scenarios:

-	3GPP NR FDD inter-frequency multi cell scenario + MBS data

-	3GPP NR TDD inter-frequency multi cell scenario + MBS data.

Combination NR-24 applies to the following test case scenarios:

-	3GPP NR FDD inter-frequency multi cell scenario + MBS Service Continuity.

-	3GPP NR TDD inter-frequency multi cell scenario + MBS Service Continuity.

Combination NR-25 applies to the following test case scenarios:

-	3GPP NR FDD inter-frequency multi cell scenario + MBS data + MBS Service Continuity.

-	3GPP NR TDD inter-frequency multi cell scenario + MBS data + MBS Service Continuity.

Combination NR-26 applies to the following test case scenarios:

-	3GPP NR FDD + eNPN single cell scenario

-	3GPP NR TDD + eNPN single cell scenario

Combination NR-27 applies to the following test case scenarios:

-	3GPP NR FDD + eNPN multi cell scenario

-	3GPP NR TDD + eNPN multi cell scenario

Combination NR-28 is void

Combination NR-29 applies to the following test case scenarios:

-	3GPP NR FDD single NTN cell scenario

-	3GPP NR FDD intra-frequency multi NTN cell scenario

Combination NR-30 applies to the following test case scenarios:

-	3GPP NR FDD inter-frequency multi cell scenario + slice-based cell reselection in idle/inactive state

-	3GPP NR TDD inter-frequency multi cell scenario + slice-based cell reselection in idle/inactive state

Combination NR-31 applies to the following test case scenarios:

-	3GPP NR FDD single cell scenario + NR sidelink communication.

-	3GPP NR TDD single cell scenario + NR sidelink communication.

Combination NR-32 applies to the following test case scenarios:

-	3GPP NR FDD inter-frequency multi NTN cell scenario

Combination NR-33 applies to the following test case scenarios:

-	3GPP NR FDD single ATG cell scenario

-	3GPP NR TDD single ATG cell scenario

Combination NR-34 applies to the following test case scenarios:

-	3GPP NR FDD intra-frequency multi ATG cell scenario

-	3GPP NR TDD intra-frequency multi ATG cell scenario

Combination NR-35 applies to the following test case scenarios:

-	3GPP NR FDD single cell scenario + sidelink positioning test case

-	3GPP NR TDD single cell scenario + sidelink positioning test case

Table 4.4.3.1.2-1: Combinations of system information blocks, first set

| Combination No. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| System information block type | NR-1 | NR-2 | NR-3 | NR-4 | NR-5 | NR-6 | NR-7 | NR-8 | NR-9 | NR-10 | NR-11 | NR-12 | NR-13 |
| SIB1 | X | X | X | X | X | X | X | X | X | X | X | X | X |
| SIB2 |  | X | X | X | X | X | X |  |  |  |  |  | X |
| SIB3 |  |  | X |  | X |  |  |  |  |  |  |  |  |
| SIB4 |  |  |  | X | X |  | X |  |  |  |  |  |  |
| SIB5 |  |  |  |  |  | X | X |  |  |  |  |  |  |
| SIB6 |  |  |  |  |  |  |  | X |  | X |  |  |  |
| SIB7 |  |  |  |  |  |  |  | X |  |  | X |  |  |
| SIB8 |  |  |  |  |  |  |  |  | X |  |  |  |  |
| SIB9 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SIB10 |  |  |  |  |  |  |  |  |  |  |  | X | X |
| SIB11 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SIB12 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SIB13 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SIB14 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SIB16 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SIB18 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SIB19 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SIB20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SIB21 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SIB22 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SIB23 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| posSIBs |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.4.3.1.2-2: Combinations of system information blocks, second set

| Combination No. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| System information block type | NR-14 | NR-15 | NR-16 | NR-17 | NR-18 | NR-19 | NR-20 | NR-21 | NR-22 | NR-23 | NR-24 | NR-25 | NR-26 |
| SIB1 | X | X | X | X | X | X | X | X | X | X | X | X | X |
| SIB2 | X |  | X | X |  | X | X | X | X | X | X | X |  |
| SIB3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SIB4 | X |  |  | X | X |  |  |  |  | X | X | X |  |
| SIB5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SIB6 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SIB7 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SIB8 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SIB9 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SIB10 |  |  |  |  |  |  |  |  |  |  |  |  | X |
| SIB11 |  |  |  |  | X | X |  |  |  |  |  |  |  |
| SIB12 | X |  |  |  |  |  |  |  |  |  |  |  |  |
| SIB13 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SIB14 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SIB16 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SIB18 |  |  |  |  |  |  |  |  |  |  |  |  | X |
| SIB19 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SIB20 |  |  |  |  |  |  | X |  | X | X |  | X |  |
| SIB21 |  |  |  |  |  |  |  | X | X |  | X | X |  |
| SIB22 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SIB23 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| posSIBs |  | X | X | X |  |  |  |  |  |  |  |  |  |

Table 4.4.3.1.2-3: Combinations of system information blocks, third set

| Combination No. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| System information block type | NR-27 | Void | NR-29 | NR-30 | NR-31 | NR-32 | NR-33 | NR-34 | NR-35 |  |  |  |  |
| SIB1 | X |  | X | X | X | X | X | X | X |  |  |  |  |
| SIB2 | X |  | X | X |  | X |  | X |  |  |  |  |  |
| SIB3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SIB4 |  |  |  | X |  | X |  |  |  |  |  |  |  |
| SIB5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SIB6 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SIB7 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SIB8 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SIB9 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SIB10 | X |  |  |  |  |  |  |  |  |  |  |  |  |
| SIB11 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SIB12 |  |  |  |  | X |  |  |  | X |  |  |  |  |
| SIB13 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SIB14 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SIB16 |  |  |  | X |  |  |  |  |  |  |  |  |  |
| SIB18 | X |  |  |  |  |  |  |  |  |  |  |  |  |
| SIB19 |  |  | X |  |  | X |  |  |  |  |  |  |  |
| SIB20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SIB21 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SIB22 |  |  |  |  |  |  | X | X |  |  |  |  |  |
| SIB23 |  |  |  |  |  |  |  |  | X |  |  |  |  |
| posSIBs |  |  |  |  |  |  |  |  |  |  |  |  |  |

4.4.3.1.3	Scheduling of system information blocks

The scheduling configurations for combinations of system information blocks are defined in the following tables. There is no scheduling information for combination NR-1.

< Unchanged sections omitted >

Table 4.4.3.1.3-34: Scheduling for combination NR-34

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB2 |
| 2 | 64 | SIB22 |

Table 4.4.3.1.3-35: Scheduling for combination NR-35

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 64 | SIB12 |
| 2 | 64 | SIB23 |

< Unchanged sections omitted >

–	SIB23

Table 4.6.2-22: SIB23

| Derivation Path: TS 38.331 [6], clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB23-r18 ::= SEQUENCE { |  |  |  |
| segmentNumber-r18 | 0 |  |  |
| segmentType-r18 | lastSegment |  |  |
| segmentContainer-r18 | OCTET STRING (CONTAINING SIB23-IEs) | SIB23-IEs |  |
| } |  |  |  |

Table 4.6.2-22A: SIB23-IEs

| Derivation Path: TS 38.331 [6], clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB23-IEs-r18 ::= SEQUENCE { |  |  |  |
| sl-PosConfigCommonNR-r18 SEQUENCE { |  |  |  |
| sl-PosFreqInfoList-r18 SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-FreqConfigCommon-r16 { | 1 entry |  |  |
| SL-FreqConfigCommon-r16[1] | SL-FreqConfigCommon | entry 1 |  |
| } |  |  |  |
| sl-PosUE-SelectedConfig-r18 | SL-UE-SelectedConfig |  |  |
| sl-PosNR-AnchorCarrierFreqList-r18 | Not present |  |  |
| sl-PosMeasConfigCommon-r18 | SL-MeasConfigCommon |  |  |
| sl-PosOffsetDFN-r18 | Not present |  |  |
| sl-PosSSB-PriorityNR-r18 | 1 |  |  |
| } |  |  |  |
| lateNonCriticalExtension |  |  |  |
| } |  |  |  |