3GPP TSG-RAN5 Meeting #108	R5-254014

# Bangalore, India, 25th - 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0682 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of test case 7.35 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test |  |  |  |  |  | Date: |  |  | 2025-08-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - "text" feature tag shall be specified consistently in default message contents and so that there is no need anymore to specify it in specific message contents. - An SDP message needs to have a c= line in the session description or in every media description. - In contrast to the 4G sister test case 17.19 and to 4G/5G video test cases 17.1 and 7.20 the preconditions for the RTT stream in the 1st SDP offer restrict the strength of the desired remote status to "optional" (instead of "optional" or "mandatory"). - In contrast to the 183 Session Progress at step 3 of Annex A.4.1a in the SDP answer of Table 7.35.3.3-2 the preconditions for audio shall not have an a=conf attribute. - Message content of step 6 (UPDATE) of A.4.1a specifies the requirements for the UPDATE sent by the UE and therefore cannot be used for specification of the 200 OK to be sent to the UE. - Editorial issues. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - "text" feature tag removed from specific message contents. - c= line added to the media description for RTT in the SDP offer at step 2 and step 8. - Restriction of strength of desired remote status lowered "optional" or "mandatory" in SDP offer at step 2 (Table 7.35.3.3-1). - Precondition attributes for audio fully specified in Table 7.35.3.3-2. - Specification of 200 OK for UPDATE (Table 7.35.3.3-4) changed to refer to the received UPDATE as done at other places (A.15.1a, 5G sister test case 17.19); derivation path changed to 34.229-1 A.3.1; Require header field and Content-Type added. - Editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect and inconsistent test specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.35 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 34.229-1 CR 1622 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | NOTE 1: There are further inconsistencies in 4G test case 17.19 and 5G test case 7.35 and between 5G test cases 7.35 and 7.36 mainly in the 1st SDP answer due to Table 7.35.3.3-3 referring to SDP body of Step 3 in Annex A.4.1a (183 Session Progress) which specifies security and ECN attributes and hard-codes the AS bandwidth for audio to 65. In addition the readability is difficult. the SDP messages of 7.35 should be specified in the same way as in 7.36 (and as in 4G sister test case 17.19) i.e. to avoid the references to A.4.1a. NOTE 2: It is not clear whether the format specific parameter cps is needed in the SDP answers at steps 4 and 9 as it is not specified in the 4G/5G sister test case 17.19 and 7.36 and the value of 30 is the default according to RFC 4103. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 7.35	MTSI MO Voice Call / add RTT and remove RTT / with preconditions at both originating UE and terminating UE / 5GS

7.35.1	Test Purpose (TP)

(1)

with { UE is configured to use preconditions and has set up a voice call with preconditions }

ensure that {

when { UE is made to add real-time text to the voice call }

then { UE sends re-INVITE with SDP media for both voice and real-time text }

}

(2)

with { UE having sent re-INVITE }

ensure that {

when { UE receives 100 Trying and 183 Session Progress with SDP answer }

then { UE acks with PRACK }

}

(3)

with { UE having sent PRACK }

ensure that {

when { UE receives 200 OK for PRACK and indication that resources are reserved }

then { UE sends UPDATE with SDP offer indicating reserved resources }

}

(4)

with { UE having sent UPDATE }

ensure that {

when { UE receives 200 OK for UPDATE followed by 200 OK for re-INVITE }

then { UE sends ACK }

}

(5)

with { UE having successfully added real-time text }

ensure that {

when { UE is made to remove real-time text }

then { UE sends re-INVITE with SDP indicating that real-time text is being removed from the call }

}

(6)

with { UE having sent re-INVITE }

ensure that {

when { UE receives 200 OK for re-INVITE and indication that real-time text has been removed from the call }

then { UE sends ACK }

}

7.35.2	Conformance Requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-15 requirements.

[TS 24.173, clause 5.2]:

The IMS multimedia telephony communication service can support different types of media, including media types listed in 3GPP TS 22.173 [2]. The session control procedures for the different media types shall be in accordance with 3GPP TS 24.229 [13] and 3GPP TS 24.247 [14], with the following additions:

a)	Multimedia telephony is an IMS communication service and the P-Preferred-Service and P-Asserted-Service headers shall be treated as described in 3GPP TS 24.229 [13]. The coding of the ICSI value in the P-Preferred-Service and P-Asserted-Service headers shall be according to subclause 5.1.

[TS 24.229, clause 5.1.2A.1]:

If this is a request within an existing dialog, and the request includes a Contact header field, then the UE should insert the previously used Contact header field.

...

After the dialog is established the UE may change the dialog capabilities (e.g. add a media or request a supplementary service) if defined for the IMS communication service as identified by the ICSI value using the same dialog. Otherwise, the UE shall initiate a new initial request to the other user.

[TS 24.229, clause 5.1.3]:

The "integration of resource management and SIP" extension is hereafter in this subclause referred to as "the precondition mechanism" and is defined in RFC 3312 [30] as updated by RFC 4032 [64].

The preconditions mechanism should be supported by the originating UE.

If the precondition mechanism is disabled as specified in subclause 5.1.5A, the UE shall not use the precondition mechanism.

The UE may initiate a session without the precondition mechanism if the originating UE does not require local resource reservation.

NOTE 1:	The originating UE can decide if local resource reservation is required based on e.g. application requirements, current access network capabilities, local configuration, etc.

In order to allow the peer entity to reserve its required resources, if the precondition mechanism is enabled as specified in subclause 5.1.5A; the originating UE supporting the precondition mechanism should make use of the precondition mechanism, even if it does not require local resource reservation.

Upon generating an initial INVITE request using the precondition mechanism, the UE shall:

-	indicate the support for reliable provisional responses and specify it using the Supported header field; and

-	indicate the support for the preconditions mechanism and specify it using the Supported header field.

Upon generating an initial INVITE request using the precondition mechanism, the UE shall not indicate the requirement for the precondition mechanism by using the Require header field.

During the session initiation, if the originating UE indicated the support for the precondition mechanism in the initial INVITE request and:

a)	the received response with an SDP body includes a Require header field with "precondition" option-tag, the originating UE shall include a Require header field with the "precondition" option-tag:

-	in subsequent requests that include an SDP body, that the originating UE sends in the same dialog as the response is received from; and

-	in responses with an SDP body to subsequent requests that include an SDP body and include "precondition" option-tag in Supported header field or Require header field received in-dialog; or

b)	the received response with an SDP body does not include the "precondition" option-tag in the Require header field,

-	in subsequent requests that include an SDP body, the originating UE shall not include a Require or Supported header field with "precondition" option-tag in the same dialog;

-	in responses with an SDP body to subsequent requests with an SDP body but without "precondition" option-tag in the Require or Supported header field, the originating UE shall not include a Require or Supported header field with "precondition" option-tag in the same dialog; and

-	in responses with an SDP body to subsequent requests with an SDP body and with "precondition" option-tag in the Require or Supported header field, the originating UE shall include a Require header field with "precondition" option-tag in the same dialog.

NOTE 2:	Table A.4 specifies that UE support of forking is required in accordance with RFC 3261 [26]. The UE can accept or reject any of the forked responses, for example, if the UE is capable of supporting a limited number of simultaneous transactions or early dialogs.

Upon successful reservation of local resources the UE shall confirm the successful resource reservation (see subclause 6.1.2) within the next SIP request.

NOTE 3:	In case of the precondition mechanism being used on both sides, this confirmation will be sent in either a PRACK request or an UPDATE request. In case of the precondition mechanism not being supported on one or both sides, alternatively a reINVITE request can be used for this confirmation after a 200 (OK) response has been received for the initial INVITE request, in case the terminating UE does not support the PRACK request (as described in RFC 3262 [27]) and does not support the UPDATE request (as described in RFC 3311 [29]).

NOTE 4:	The UE can receive a P-Early-Media header field authorizing an early-media flow while the required preconditions, if any, are not met and/or the flow direction is not enabled by the SDP direction parameter. According to RFC 5009 [109], an authorized early-media flow can be established only if the necessary conditions related to the SDP negotiation are met. These conditions can evolve during the session establishment.

NOTE 5:	When the UE is confirming the successful resource reservation using an UPDATE request (or a PRACK request) and the UE receives a 180 (Ringing) response or a 200 (OK) response to the initial INVITE request before receiving a 200 (OK) response to the UPDATE request (or a 200 (OK) response to the PRACK request), the UE does not treat this as an error case and does not release the session.

NOTE 6:	The UE procedures for rendering of the received early media and of the locally generated communication progress information are specified in 3GPP TS 24.628 [8ZF].

[TS 24.229, clause 5.1.4A.1]:

If the precondition mechanism was used during the session establishment, as described in subclause 5.1.3.1 or 5.1.4.1, the UE shall indicate support of the precondition mechanism during a session modification. If the precondition mechanism was not used during the session establishment, the UE shall not indicate support of the precondition mechanism during a session modification.

In order to indicate support of the precondition mechanism during a session modification, upon generating a reINVITE request, an UPDATE request with an SDP body, or a PRACK request with an SDP body, the UE shall:

a)	indicate the support for the precondition mechanism using the Supported header field;

b)	not indicate the requirement for the precondition mechanism using the Require header field; and

c)	if a re-INVITE request is being generated, indicate the support for reliable provisional responses using the Supported header field;

[TS 24.229, clause 5.1.4A.2]:

Upon receiving a reINVITE request, an UPDATE request, or a PRACK request that indicates support for the precondition mechanism by using the Supported header field or requires use of the precondition mechanism by using the Require header field, the UE shall:

a)	if the precondition mechanism was used during the session establishment, as described in subclause 5.1.3.1 or 5.1.4.1, use the precondition mechanism for the session modification; and

…

If the precondition mechanism is used for the session modification, the UE shall indicate support for the preconditions mechanism, using the Require header field, in responses that include an SDP body, to the session modification request.

[TS 24.229, clause 6.1]:

The "integration of resource management and SIP" extension is hereafter in this subclause referred to as "the precondition mechanism" and is defined in RFC 3312 [30] as updated by RFC 4032 [64].

In order to authorize the media streams, the P-CSCF and S-CSCF have to be able to inspect SDP message bodies. Hence, the UE shall not encrypt SDP message bodies.

During the session establishment procedure, and during session modification procedures, SIP messages shall only contain an SDP message body if that is intended to modify the session description, or when the SDP message body is included in the message because of SIP rules described in RFC 3261 [26].

...

For "video" and "audio" media types that use the RTP/RTCP and where the port number is not zero, the UE shall specify the proposed bandwidth for each media stream using the "b=" media descriptor and the "AS" bandwidth modifier in the SDP.

...

If the media line in the SDP message body indicates the usage of RTP/RTCP, and if the UE is configured to request an RTCP bandwidth level for the session is different than the default RTCP bandwidth as specified in RFC 3556 [56], then in addition to the "AS" bandwidth modifier in the media-level "b=" line, the UE shall include two media-level "b=" lines, one with the "RS" bandwidth modifier and the other with the "RR" bandwidth modifier as described in RFC 3556 [56] to specify the required bandwidth allocation for RTCP. The bandwidth-value in the b=RS: and b=RR: lines may include transport overhead as described in subclause 6.1 of RFC 3890 [152].

For other media streams the "b=" media descriptor may be included. The value or absence of the "b=" parameter will affect the assigned QoS which is defined in or 3GPP 29.213 [13C].

NOTE 3:	In a two-party session where both participants are active, the RTCP receiver reports are not sent, therefore, the RR bandwidth modifier will typically get the value of zero.

[TS 24.229, clause 6.1.2]:

An INVITE request generated by a UE shall contain a SDP offer and at least one media description. This SDP offer shall reflect the calling user's terminal capabilities and user preferences for the session.

If the desired QoS resources for one or more media streams have not been reserved at the UE when constructing the SDP offer, the UE:

shall indicate the related local preconditions for QoS as not met, using the segmented status type, as defined in RFC 3312 [30] and RFC 4032 [64], as well as the strength-tag value "mandatory" for the local segment and the strength-tag value either "optional" or as specified in RFC 3312 [30] and RFC 4032 [64] for the remote segment, if the UE uses the precondition mechanism (see subclause 5.1.3.1); and

-	if the UE uses the precondition mechanism (see subclause 5.1.3.1), shall not request confirmation for the result of the resource reservation (as defined in RFC 3312 [30]) at the terminating UE.

NOTE 1:	Previous versions of this document mandated the use of the SDP inactive attribute. This document does not prohibit specific services from using direction attributes to implement their service-specific behaviours.

If the UE uses the precondition mechanism (see subclause 5.1.3.1), and the desired QoS resources for one or more media streams are available at the UE when the SDP offer is sent, the UE shall indicate the related local preconditions as met, using the segmented status type, as defined in RFC 3312 [30] and RFC 4032 [64], as well as the strength-tag value "mandatory" for the local segment and the strength-tag value either "optional" or as specified in RFC 3312 [30] and RFC 4032 [64] for the remote segment and shall not request confirmation for the result of the resource reservation (as defined in RFC 3312 [30]) at the terminating UE.

NOTE 2:	If the originating UE does not use the precondition mechanism (see subclause 5.1.3.1), it will not include any precondition information in the SDP message body.

...

Upon confirming successful local resource reservation, the UE shall create an SDP offer in which the related local preconditions are set to met, using the segmented status type, as defined in RFC 3312 [30] and RFC 4032 [64].

Upon receiving an SDP answer, which includes more than one codec per media stream, excluding the in-band DTMF codec, as described in subclause 6.1.1, the UE shall:

send an SDP offer at the first possible time, selecting only one codec per media stream; or

-	if the UE is participant in a multi-stream multiparty multimedia conference session using simulcast (indicated by the presence of "a=simulcast" SDP attribute(s) in the SDP answer, as defined in RFC 8853 [249]), apply the procedures defined in 3GPP TS 26.114 [9B] annex S.

If the UE sends an initial INVITE request that includes only an IPv6 address in the SDP offer, and receives an error response (e.g., 488 (Not Acceptable Here) with 301 Warning header field) indicating "incompatible network address format", the UE shall send an ACK as per standard SIP procedures. Subsequently, the UE may acquire an IPv4 address or use an existing IPv4 address, and send a new initial INVITE request to the same destination containing only the IPv4 address in the SDP offer.

[TS 26.114, clause 6.3]:

During session renegotiation for adding or removing media components, the SDP offeror should continue to use the same media (m=) line(s) from the previously negotiated SDP for the media components that are not being added or removed.

An MTSI client in terminal may support multiple media components including media components of the same media type. An MTSI client in terminal may support adding one or more media components to an on-going session which already contains a media component of the same media type. If an MTSI client in terminal needs to have multiple media components of the same media type in a single MTSI session, then the MTSI client in terminal should use the SDP content attributes as defined in [81] for identifying different media components.

[TS 26.114, clause 7.3.1]

The RS and RR values included in the SDP answer should be treated as the negotiated values for the session and should be used to calculate the total RTCP bandwidth for all terminals in the session.

[TS 26.114, clause 7.4.4]:

The following RTP payload format shall be used:

-	T.140 text conversation RTP payload format according to RFC 4103 [31] including the updates from RFC 9071 [185] when the negotiation for support of multiparty real-time text is successful.

Real-time text shall be the only payload type in its RTP stream because the RTP sequence numbers are used for loss detection and recovery. The redundant transmission format shall be used for keeping the effect of packet loss low.

Media type signalling for usage in SDP is specified in section 10 of RFC 4103 [31], section 3 of RFC 4102 [49] and section 2.3 of RFC 9071 [185].

[TS 26.114, clause 7.5.2.3]:

Real-time text is intended for human conversation applications. Text shall not be transferred with higher rate than 30 characters per second (as defined for cps in section 6 of RFC 4103 [31]). A text-capable MTSI client shall be able to receive text with cps set up to 30 from an MTSI client in terminal. A text-capable MTSI client may be able to receive text with cps set up to 90 in order to be prepared to handle text received in a multiparty session, with multiple parties sending at a rate of 30 characters per second. This shall be indicated by the cps parameter in the sdp fmtp attribute for the "t140" format.

7.35.3	Test description

7.35.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC.

-	UE is configured to register for IMS after switch on.

-	UE is configured to use preconditions.

Preamble:

-	UE has registered to IMS services and set up the MO call, by executing annex A.4.1a.

7.35.3.2	Test procedure sequence

Table 7.35.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make UE add real-time text to the voice call. | - | - | - | - |
| 2 | Check: Does the UE send re-INVITE with an SDP offer containing media lines for both voice and real-time text? | -> | INVITE | 1 | P |
| 3 | The SS responds with a 100 Trying provisional response. | <- | 100 Trying | - | - |
| 4 | SS responds with an SDP answer indicating that SS has not reserved its resources for real-time text. | <- | 183 Session Progress | - | - |
| 5 | Check: Does the UE acknowledge the receipt of 183 response with PRACK? | -> | PRACK | 2 | P |
| 6 | The SS responds PRACK with 200 OK. | <- | 200 OK | - | - |
| 7 | SS reserves resources for real-time text by executing TS 38.508-1 cl 4.9.46. | - | - | - | - |
| 8 | Check: Does the UE send an UPDATE after resources for real-time text are reserved and meeting the preconditions indicated in step 4? | -> | UPDATE | 3 | P |
| 9 | The SS responds to UPDATE with 200 OK and indicates having reserved the resources. | <- | 200 OK | - | - |
| 10 | The SS responds to re-INVITE with 200 OK. | <- | 200 OK | - | - |
| 11 | Check: Does the UE acknowledge the receipt of 200 OK for re-INVITE? | -> | ACK | 4 | P |
| 12 | Make UE remove real-time text from the media call. | - | - | - | - |
| 13 | Check: Does the UE send re-INVITE with a SDP offer indicating that the real-time text component is removed from the call? | -> | INVITE | 5 | P |
| 14 | The SS responds with a 100 Trying provisional response | <- | 100 Trying | - | - |
| 15 | SS releases the QoS flow for real-time text by executing TS 38.508-1 cl 4.9.48. | - | - | - | - |
| 16 | The SS responds re-INVITE with 200 OK. | <- | 200 OK | - | - |
| 17 | Check: Does the UE acknowledge the receipt of 200 OK for re-INVITE? | -> | ACK | 6 | P |
| 18 | Make the UE release the call. | - | - | - | - |
| 19-20 | UE releases the call. (Annex A.7) | - | - | - | - |

7.35.3.3	Specific message contents

Table 7.35.3.3-1: INVITE (step 2, table 7.35.3.2-1)

| Derivation Path: TS 34.229-1 [2], Annex A.2.1, conditions A5 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Supported |  |  |  |  |
| option-tag |  | precondition |  |  |
| Message-body |  | Same SDP body as in Step 6 (UPDATE) of Annex A.4.1a, with following differences: Attributes for preconditions for audio media: a=curr:qos remote sendrecv Media description for Real-time text: m=text (transport port) RTP/AVP (fmt) c=IN (addrtype) (connection-address for UE) [Note 1] b=AS: (bandwidth-value) b=RS: (bandwidth-value) b=RR: (bandwidth-value) Attributes for media: a=rtpmap: (payload type) t140/1000 a=rtpmap: (payload type) red/1000 a=fmtp: (format) (format specific parameters) Attributes for preconditions: a=curr:qos local none a=curr:qos remote none a=des:qos mandatory local sendrecv a=des:qos optional remote sendrecv or a=des:qos mandatory remote sendrecv Note 1: "c=" field shall be present when there is no "c=" field in the session description. |  | TS 26.114 [33] |

Table 7.35.3.3-2: 183 Session Progress (step 4, table 7.35.3.2-1)

| Derivation Path: TS 34.229-1 [2], Annex A.2.3, Condition A15 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Require |  |  |  |  |
| option-tag |  | precondition |  |  |
| Message-body |  | Same SDP body as in Step 3 (183 Session Progress) of Annex A.4.1a, with following exceptions and preconditions for audio as shown below: Session description: "o=" line identical to previous SDP sent by SS except that sess-version is incremented by one. b=AS: (bandwidth-value) [Note 1] Attributes for preconditions for audio media: a=curr:qos local sendrecv a=curr:qos remote sendrecv a=des:qos mandatory local sendrecv a=des:qos mandatory remote sendrecv Media description for real-time text: m=text (transport port) RTP/AVP (fmt) [Note 1] b=AS: (bandwidth-value) [Note 1] b=RS: (bandwidth-value) [Note 1] b=RR: (bandwidth-value) [Note 1] Attributes for real-time text media: a=rtpmap: (payload type) t140/1000 [Note 1] a=fmtp: (format) cps=30 [Note 1] a=rtpmap: (payload type) red/1000 [Note 1] a=fmtp: (format) (format specific parameters) [Note 1] Attributes for preconditions: a=curr:qos local none a=curr:qos remote none a=des:qos mandatory local sendrecv a=des:qos mandatory remote sendrecv a=conf:qos remote sendrecv Note 1: The values for fmt, payload type, bandwidth-value, format specific parameters and format are copied from step 2. |  | TS 24.229 [7] TS 26.114 [33] |

Table 7.35.3.3-3: UPDATE (step 8, table 7.35.3.2-1)

| Derivation Path: TS 34.229-1 [2], Annex A.2.5 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Supported |  |  |  |  |
| option-tag |  | precondition |  |  |
| Message-body |  | Same contents as specified in step 6 (UPDATE) of A.4.1a with the following exceptions: Attributes for preconditions for audio media: a=curr:qos remote sendrecv Media description for real-time text: m=text (transport port) RTP/AVP (fmt) [Note 1] c=IN (addrtype) (connection-address for UE) [Note 3] b=AS: (bandwidth-value) b=RS: (bandwidth-value) b=RR: (bandwidth-value) Attributes for real time text media: a=rtpmap: (payload type) t140/1000 [Note 1] a=rtpmap: (payload type) red/1000 [Note 1] a=fmtp: (format) (format specific parameters) [Note 2] Attributes for preconditions: a=curr:qos local sendrecv a=curr:qos remote none a=des:qos mandatory local sendrecv a=des:qos optional remote sendrecv or a=des:qos mandatory remote sendrecv Note 1: The value for fmt, payload type and format are not checked. Note 2: Parameters are not checked. Note 3: "c=" field shall be present when there is no "c=" field in the session description. |  | TS 24.229 [7] TS 26.114 [33] |

Table 7.35.3.3-4: 200 OK for UPDATE (step 9, table 7.35.3.2-1)

| Derivation Path: TS 34.229-1 [2], Annex A.3.1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Require |  |  |  |  |
| option-tag |  | precondition |  |  |
| Content-Type |  |  |  |  |
| media-type |  | application/sdp |  |  |
| Message-body |  | SDP body of the 200 response copied from the received UPDATE and modified as follows: - IP address on "c=" lines and transport port on "m=" lines changed to indicate to which IP address and port the UE should start sending the media; - "o=" line identical to previous SDP sent by SS except that sess-version is incremented; - Attributes for preconditions (RTT): a=curr:qos remote sendrecv - Attributes for real time text media: a=fmtp: (format) cps=30 [Note 1] Note 1: The value for format is copied from UPDATE in step 7. |  | TS 24.229 [7] TS 26.114 [33] |

Table 7.35.3.3-5: 200 OK for INVITE (step 10, table 7.35.3.2-1)

| Derivation Path: TS 34.229-1 [2], Annex A.3.1, condition A23 |  |  |  |  |
| --- | --- | --- | --- | --- |

Table 7.35.3.3-6: ACK for re-INVITE (step 11, table 7.35.3.2-1)

Derivation Path: TS 34.229-1 [2], Annex A.2.7, condition A5

Table 7.35.3.3-7: INVITE (step 13, table 7.35.3.2-1)

| Derivation Path: TS 34.229-1 [2], Annex A.2.1, condition A5. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Supported |  |  |  |  |
| option-tag |  | precondition |  |  |
| Message-body |  | Same SDP body as in Step 2 (re-INVITE), with following exceptions and no requirements on b-, c- or a-lines on the RTT stream: Media description for real-time text: m=text 0 RTP/AVP (fmt) |  | TS 24.229 [7] TS 26.114 [33] |

Table 7.35.3.3-8: 200 OK for INVITE (step 16, table 7.35.3.2-1)

| Derivation Path: TS 34.229-1 [2], Annex A.3.1, Condition A23 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Require |  |  |  |  |
| option-tag |  | precondition |  |  |
| Content-Type |  |  |  |  |
| media-type |  | application/sdp |  |  |
| Content-Length |  |  |  |  |
| value |  | length of message-body |  |  |
| Message-body |  | SDP body copied from the received re-INVITE in step 13 and modified as follows: - IP address on "c=" lines and transport port on "m=" lines changed to indicate to which IP address and port the UE should start sending the media; - "o=" line identical to previous SDP sent by SS except that sess-version is incremented |  |  |

Table 7.35.3.3-9: ACK (step 17, table 7.35.3.2-1)

Derivation Path: TS 34.229-1 [2], Annex A.2.7, condition A5