3GPP TSG-RAN5 Meeting #108	R5-254031

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 5072 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of MA PDU session establishment / ATSSS / UE establishing a PDN connection over untrusted non-3GPP access network as a user-plane resource of an already established MA PDU session test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS_Ph3-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | MA PDU session establishment / ATSSS / UE establishing a PDN connection over untrusted non-3GPP access network as a user-plane resource of an already established MA PDU session test case has been missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | The above test case has been added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case would be missing spec |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 38.523-2 CR 0605 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

---- Start of change 1 ----

#### 10.6.1.2	UE-requested MA PDU session establishment / ATSSS / UE establishing a PDN connection over untrusted non-3GPP access network as a user-plane resource of an already established MA PDU session

10.6.1.2.1	Test Purpose (TP)

(1)

with { UE is registered over 3GPP access to 5GC, 5GMM idle mode over 3GPP access, no coverage of untrusted non-3GPP connected to EPC }

ensure that {

when { UE is configured to request a new MA PDU session }

then { UE sends a PDU Session Establishment Request over 5GC access and sets the request type to "MA PDU request" in the UL NAS TRANSPORT message and in the Extended protocol configuration options IE of the PDU SESSION ESTABLISHMENT REQUEST message to indicate its support of establishing a PDN connection  over untrusted non-3GPP access network as the user plane resource of an MA PDU session }

(2)

with { UE is registered over 3GPP access to 5GC, and an MA PDU session has been established successfully on this access }

ensure that {

when { UE detects a good non-3gpp coverage connected to EPC }

then { UE sends a IKE_AUTH request message including an indication about Attach Type indicating handover Attach and ATSSS_REQUEST Notify payload including PDU session ID of the already established MA PDU session to the ePDG }

10.6.1.2.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 23.501 clause 5.32.2, TS 24.193, clause 5.3.2. Unless otherwise stated these are Rel-17 requirements.

In order to establish a PDN connection over untrusted non-3GPP access network as a user-plane resource of an already established MA PDU session, the UE shall initiate the IPsec tunnel establishment procedure using the IKEv2 protocol according to 3GPP TS 24.302 [17].

In the IKE_AUTH request message to the ePDG:

a)	the CFG_REQUEST configuration payload shall include the INTERNAL_IP4_ADDRESS attribute, the INTERNAL_IP6_ADDRESS attribute or both, indicating the PDU session address of the established MA PDU session;

b)	the IDr payload shall be set to the APN mapped from the DNN of the established MA PDU session;

c)	the PDU Session ID field of the N1_MODE_CAPABILITY Notify payload shall be set to the PDU session identity of the establish MA PDU session;

d)	the S-NSSAI Value field of the N1_MODE INFORMATION Notify payload shall be set to the S-NSSAI of the established MA PDU session; and

e)	the ATSSS_REQUEST Notify payload shall be included as specified in clause 8.2.9.20 of 3GPP TS 24.302 [17].

Upon receipt of the IKE_AUTH response as specified in clause 7.2.2.1 of 3GPP TS 24.302 [17], the UE shall consider that the PDN connection over untrusted non-3GPP access network is established as a user-plane resource of the MA PDU session.

Upon receipt of an IKE_AUTH response message including a Notify payload with a Private Notify Message Type as specified in clause 7.2.2.2 of 3GPP TS 24.302 [17], the UE shall consider that the PDN connection over non-3GPP access network is not established as a user-plane resource of the MA PDU session.

10.6.1.2.3	Test description

10.6.1.2.3.1	Pre-test conditions

System Simulator:

-	WLAN Cell 27 is configured according to TS 38.508-1 [4], clause 4.4.1.3.1.

-	NR Cell 1 is configured according to TS 38.508-1 [4].

-	NR Cell 1 is set to '' Serving cell'' and WLAN Cell is set to “Off Cell”.

UE:

-	UE is registered over 3GPP access to the 5GC.

Preamble:

-	The UE is brought to state 3N-A on NR Cell 1 according to TS 38.508-1 [4].

10.6.1.2.3.2	Test procedure sequence

Table 10.6.1.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Cause the UE to request to establish an MA PDU session by MMI or AT command on NR Cell 1. | - | - | - | - |
| 2 | Check: Does the UE transmit a PDU SESSION ESTABLISHMENT REQUEST message and sets the request type to "MA PDU request" in the UL NAS TRANSPORT message and sets the ATSSS-ST bits in the 5GSM capability IE of the PDU SESSION ESTABLISHMENT REQUEST message according to the support of establishing a PDN connection over untrusted non-3GPP access network on NR Cell 1? | --> | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT REQUEST | 1 | P |
| 3 | The SS transmits an PDU SESSION ESTABLISHMENT ACCEPT on NR Cell 1 | <-- | 5GMM: DL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT ACCEPT | - | - |
| 4 | WLAN Cell 27 is set to “Serving cell”. |  |  |  |  |
| 5 | The UE establishes an IPsec tunnel using the IKEv2 protocol as defined in 3GPP TS 33.402 [55] clause 8.2.2, Figure 8.2.2-1. |  |  |  |  |
| 6-14 | The UE performs the IMS registration procedure according to TS 34.229-1 [43] subclause C.2c (steps 2-9). |  |  |  |  |
| 15 | Check: Does the UE set request type as “handover” and include ATSSS request PCO parameter in the PDN CONNECTIVITY REQUEST message. The PDN CONNECTIVITY REQUEST message is piggybacked in ATTACH REQUEST? | --> | NAS: ATTACH REQUEST NAS: PDN CONNECTIVITY REQUEST | 2 | P |
| 16 | The SS transmits a PDU SESSION ESTABLISHMENT ACCEPT message. |  | 5GMM: DL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT ACCEPT |  |  |
| NOTE 1: The request to establish a MA PDU session may be performed by MMI or AT Command. |  |  |  |  |  |

10.6.1.2.3.3	Specific message contents

-	TBD

---- End of change 1 ----