3GPP TSG-RAN5 Meeting #108	R5-254079

# Bengaluru, India, 25th - 29th August, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3432 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | FR1 ACS CA tests - exceptions for combos including both n48A and n77A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In test case 7.5A.1, exceptions were introduced for band combo CA_n48A-n77A (R5-230211). Due to proximity of the bands, if both carriers are set to Mid a per default configuration, the ACS interferer can overlap with one of the CCs. Similar exceptions are required for the 3DL and 4DL CA combos containing the band combo CA_n48A-n77A. Given that at some point both cells in the metioned combos can be SCCs, the solution in test case 7.5A.1 is not valid. Therefore, it is proposed to resolve the issue by setting n48 CC to Low. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For 3CC DL CA and 4CC DL CA configurations containing the band combination CA_n48A-n77A, set Low frequency in band n48 and Mid frequency in band n77. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification will remain inconsitent and good UEs may fail the test if the interferer overlaps with any of the carriers channel bandwidth. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5A.2, 7.5A.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

### 7.5A.2	Adjacent channel selectivity for CA (3DL CA)

7.5A.2.1	Test Purpose

Adjacent channel selectivity for 3DL CA verifies the receiver's ability to receive a wanted 3DL carrier aggregated at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel.

7.5A.2.2	Test Applicability

This test case applies to all types of NR UE release 15 and forward that support 3DL CA.

7.5A.2.3	Minimum Conformance Requirements

The minimum conformance requirements are defined in clause 7.5A.0.

7.5A.2.4	Test Description

7.5A.2.4.1	Initial Conditions

Same as in clause 7.5A.1.4.1 with following exceptions:

-	Instead of Table 7.5A.1.4.1-1 use Table 7.5A.2.4.1-1.

-	Instead of Table 7.5A.1.4.1-2 use Table 7.5A.2.4.1-1.

-	Instead of Table 7.5A.1.4.1-3 use Table 7.5A.2.4.1-1.

Table 7.5A.2.4.1-1: Test Configuration Table for 3DL CA

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Intra-band contiguous: Mid range for PCC and SCCs Inter-band CA: NOTE 1, NOTE 5 Inter-band + Intra-band contiguous : NOTE 1, NOTE 5 Inter-band + Intra-band non-contiguous, Intra-band non-contiguous, Intra-band contiguous + Intra-band non-contiguous: NOTE 1 with Wgap for intra-band non-contiguous defined in table 7.3A.2.4.1-1(NOTE 5) CA configurations containing the following band combinations: CA_n48A-n77A: Low in band n48 and Mid in band n77 |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in Tables 5.5A.1-1, 5.5A.2-1, or tables in clauses 5.5A.3.x for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | Intra-band contiguous: Lowest NRB_agg, Highest NRB_agg Inter-band: Highest NRB_agg Inter-band + Intra-band contiguous : Highest NRB_agg Inter-band + Intra-band non-contiguous : Highest NRB_agg Intra-band non-contiguous : Highest NRB_agg Intra-band contiguous + Intra-band non-contiguous : Highest NRB_agg NOTE 6 NOTE 7 |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest for PCC and SCCs |  |  |  |
| Network signalling value |  |  | NS_01 Unless given by Table 7.3.2.3-4 for the band with active uplink carrier |  |  |  |
| Test Parameters |  |  |  |  |  |  |
|  | Downlink Configuration |  |  |  | Uplink Configuration |  |
| Test ID | CC Mod'n | PCC RB allocation | SCC1 RB allocation | SCC2 RB allocation | CC Mod'n | PCC RB allocation |
| Default Test Settings for a CA_nXD Configuration (Intra-band contiguous) |  |  |  |  |  |  |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 |  | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nXA-nYA-nZA Configuration (Inter-band) |  |  |  |  |  |  |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 |  | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nXC-nYA, CA_nYA-nXC, CA_nYA-nXB and CA_nXB-nYA Configurations (Intra-band contiguous + Inter-band) |  |  |  |  |  |  |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 |  | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nX(2A)-nYA Configuration (Intra-band non-contiguous + Inter-band) |  |  |  |  |  |  |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 |  | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nX(3A) Configuration (Intra-band non-contiguous) |  |  |  |  |  |  |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 |  | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nX(A-C) and CA_nX(A-B) Configuration (Intra-band contiguous + Intra-band non-contiguous) |  |  |  |  |  |  |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 |  | DFT-s-OFDM QPSK | NOTE 1 |
| NOTE 1: The specific configuration of uplink and downlink are defined in Table 7.3A.2.4.1-1. NOTE 2: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. NOTE 3: Inter-band: X,Y,Z correspond to the different bands in the CA Configuration. E.g. for CA_n1A-n3A-n8A, X=1, Y=3, Z=8; Intra-band contiguous + Inter-band: X,Y correspond to the different bands in the CA Configuration, e.g. for CA_1C-3A, X=1,Y=3; Intra-band non-contiguous + Inter-band: X and Y correspond to the different bands in the CA Configuration. E.g. for CA_n1A-n1A-n8A, X=1, Y =8 NOTE 4: In a band where UE supports 4Rx but not supports 8Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2a and Table 7.3.2.5-2b) is used in the test requirements. In a band where UE supports 8Rx, the test shall be performed only with 8Rx antennas ports connected and 8Rx REFSENS requirement (Table 7.3.2.5-2e and Table 7.3.2.5-2f) is used in the test requirements. Otherwise, the UE shall be verified with two Rx antenna ports. NOTE 5: Test frequency is set to Mid Range for PCC and SCC with exceptions defined in Table 7.3A.2.4.1-1. For NR band n28, 30MHz test channel bandwidth is tested with Low range test frequencies. NOTE 6: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. NOTE 7: For nXA PCC, each of Highest UL and Highest DL shall be selected according to clause 5.5A.3.2. DL channel bandwidth shall be selected first. |  |  |  |  |  |  |

## <<< Skip unchanged sections >>>

### 7.5A.3	Adjacent channel selectivity for CA (4DL CA)

Editor’s note: - part content of Table 7.5A.3.4.2-1 is FFS.

7.5A.3.1	Test Purpose

Adjacent channel selectivity for 4DL CA verifies the receiver's ability to receive a wanted 4DL carrier aggregated at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel.

7.5A.3.2	Test Applicability

This test case applies to all types of NR UE release 15 and forward that support 4DL CA.

7.5A.3.3	Minimum Conformance Requirements

The minimum conformance requirements are defined in clause 7.5A.0.

7.5A.3.4	Test Description

7.5A.3.4.1	Initial Conditions

Same as in clause 7.5A.1.4.1 with following exceptions:

-	Instead of Table 7.5A.1.4.1-1 use Table 7.5A.3.4.1-1.

-	Instead of Table 7.5A.1.4.1-2 use Table 7.5A.3.4.1-2.

-	Instead of Table 7.5A.1.4.1-3 use Table 7.5A.3.4.1-3.

Table 7.5A.3.4.1-1: Test Configuration Table for 4DL CA

| Default Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Intra-band contiguous: Mid range for PCC and SCCs Inter-band CA: NOTE 1, NOTE 5 Inter-band + Intra-band contiguous : NOTE 1, NOTE 5 Inter-band + Intra-band non-contiguous 2CC,3CC : NOTE 1 with Wgap for intra-band non-contiguous defined in table 7.3A.3.4.1-1(NOTE 5) Inter-band non-contiguous + Intra-band non-contiguous : NOTE 1 with Wgap for intra-band non-contiguous defined in table 7.3A.3.4.1-1(NOTE 5) CA configurations containing the following band combinations: CA_n48A-n77A: Low in band n48 and Mid in band n77 |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in Tables 5.5A.1-1, 5.5A.2-1, or tables in clauses 5.5A.3.x for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Intra-band contiguous: Lowest NRB_agg, Highest NRB_agg Inter-band: Highest NRB_agg Inter-band + Intra-band contiguous : Highest NRB_agg Inter-band + Intra-band non-contiguous 2CC,3CC: Highest NRB_agg Inter-band non-contiguous + Intra-band non-contiguous : Highest NRB_agg NOTE 6 NOTE 7 |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | Lowest for PCC and SCCs |  |  |  |  |
| Network signalling value |  |  |  | NS_01 Unless given by Table 7.3.2.3-4 for the band with active uplink carrier |  |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |  |
|  | Downlink Configuration |  |  |  |  |  | Uplink Configuration |  |
| Test ID | CC Mod'n | PCC RB allocation | SCC1 RB allocation |  | SCC2 RB allocation | SCC3 RB allocation | CC Mod'n | PCC RB allocation |
| Default Test Settings for a CA_nXE Configuration (intra-band contiguous) |  |  |  |  |  |  |  |  |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 |  | NOTE 1 | NOTE 1 | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nXA-nYA-nZA-nRA Configuration (Inter-band) |  |  |  |  |  |  |  |  |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 |  | NOTE 1 | NOTE 1 | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nXA-nYB-nZA, CA_nXA-nYA-nZB, CA_nXD-nYA, CA_nXC-nYB, CA_nXC-nYC and CA_nXB-nYB Configurations (Intra-band contiguous + Inter-band) |  |  |  |  |  |  |  |  |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 |  | NOTE 1 | NOTE 1 | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nX(2A)-nYA-nZA and CA_nX(2A)-nY(2A) Configuration (Intra-band non-contiguous 2CC + Inter-band and Intra-band non-contiguous + Intra-band non-contiguous) |  |  |  |  |  |  |  |  |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 |  | NOTE 1 | NOTE 1 | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for CA_nX(3A)-nYA Configuration (Intra-band non-contiguous 3CC + Inter-band) |  |  |  |  |  |  |  |  |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 |  | NOTE 1 | NOTE 1 | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nX(4A) Configuration intra-band non-contiguous) |  |  |  |  |  |  |  |  |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 |  | NOTE 1 | NOTE 1 | DFT-s-OFDM QPSK | NOTE 1 |
| NOTE 1: The specific configuration of uplink and downlink are defined in Table 7.3A.3.4.1-1. NOTE 2: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. NOTE 3: Inter-band: nX, nY, nZ, nR correspond to the different bands in the CA Configuration. E.g. for CA_n1A-n3A-n7A-n28A, nX=n1, nY=n3, nZ=n7,nR=n28; Intra-band contiguous + Inter-band: nX, nY, nZ correspond to the different bands in the CA Configuration, E.g. for CA_n3A-n7B-n28A, nX=n1, nY=n3, nz=n28; Intra-band non-contiguous 2CC + Inter-band: X,Y,Z correspond to the different bands in the CA Configuration. E.g. for CA_n48(2A)-n66A-n70A, nX=n48, nY=n66, nZ=n70; Intra-band non-contiguous 3CC + Inter-band: nX and nY correspond to the different bands in the CA Configuration, e.g., for CA_n66(3A)-n77A, nX=n66, nY=n77. NOTE 4: In a band where UE supports 4Rx but not supports 8Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2a and Table 7.3.2.5-2b) is used in the test requirements. In a band where UE supports 8Rx, the test shall be performed only with 8Rx antennas ports connected and 8Rx REFSENS requirement (Table 7.3.2.5-2e and Table 7.3.2.5-2f) is used in the test requirements. Otherwise, the UE shall be verified with two Rx antenna ports. NOTE 5: Test frequency is set to Mid Range for PCC and SCC with exceptions defined in Table 7.3A.3.4.1-1. For NR band n28, 30MHz test channel bandwidth is tested with Low range test frequencies. NOTE 6: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. NOTE 7: For nXA PCC, each of Highest UL and Highest DL shall be selected according to clause 5.5A.3.3. DL channel bandwidth shall be selected first. |  |  |  |  |  |  |  |  |

## <<< END OF CHANGES >>>