3GPP TSG-RAN5 Meeting #108	R5-254106

# Bengaluru, India, 25th – 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 5083 | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the feMIMO test case 7.1.1.3.17.X - Multi-TRP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_feMIMO-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | CR R5-253188 updats the PICS parameter pc_nrMIMO_CB_PUSCH. “pc_nrMIMO_CB_PUSCH = true” does not indicate UE support more than one layer uplink MIMO. “pc_maxNumberMIMO_LayersCB_PUSCH_twolayer = true” means UE support two layers uplink MIMO and “pc_maxNumberMIMO_LayersCB_PUSCH_twolayer = true” means UE support two layers and four layers uplink MIMO. Therefore, the condition pc_nrMIMO_CB_PUSCH to configure 2 SRS port needs to be changed to “pc_maxNumberMIMO_LayersCB_PUSCH_twolayer OR pc_maxNumberMIMO_LayersCB_PUSCH_twolayer”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the conditon pc_nrMIMO_CB_PUSCH to pc_maxNumberMIMO_LayersCB_PUSCH_twolayer OR pc_maxNumberMIMO_LayersCB_PUSCH_twolayer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The condition in the test cases refers to a wrong pics. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3.17 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

7.1.1.3.17	Correct Handling of UL HARQ process / PUSCH Repetition / Multi-TRP

7.1.1.3.17.1	Correct Handling of UL HARQ process / PUSCH Repetition / Multi-TRP / PUSCH Repetition Type A / dynamic grant

7.1.1.3.17.1.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state and supporting mTRP PUSCH Type A repetitions, and configured with two SRS resource sets with 'usage' set to 'codebook' and PUSCH Aggregation > 2 }

ensure that {

when { UE has data available for transmission, UE receives an UL Grant with toggled NDI which indicates codepoint "10", and the UL grant includes 'Second Precoding information' }

then { UE transmits two new MAC PDUs and repeats sending the MAC PDUs according to 'cyclicMapping pattern' or 'sequentialMapping pattern' configured as per UE's capability, and the layer of transmission is according to ‘Precoding information and number of layers’, and the redundancy versions of PUSCH transmission associated with first and second SRS resource are selected correctly. }

}

(2)

with { UE in RRC_CONNECTED state and supporting mTRP PUSCH Type A repetitions, and configured with two SRS resource sets with 'usage' set to 'codebook' and PUSCH Aggregation > 2, and the MAC entity is configured with twoPHRMode }

ensure that {

when { phr-PeriodicTimer expires }

then { UE transmits an Enhanced Single entry PHR including two values of power headroom. }

}

7.1.1.3.17.1.2	Conformance requirements

<Skip unchanged part>

7.1.1.3.17.1.3	Test description

7.1.1.3.17.1.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except that DRB is configured in RLC AM mode according to Table 7.1.1.3.17.1.3.1-1.

Table 7.1.1.3.17.1.3.1-1: RLC parameters

| t-PollRetransmit | ms80 |
| --- | --- |

7.1.1.3.17.1.3.2	Test procedure sequence

Table 7.1.1.3.17.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits in the indicated downlink assignment an NR RRCReconfiguration with sequential mapping pattern configured as per Table 7.1.1.3.17.1.3.3-5. (Note 1) | <--- | (RRCReconfiguration) | - | - |
| 2 | UE transmits NR RRCReconfigurationComplete message to the SS. (Note 2) | --> | (RRCReconfigurationComplete) | - | - |
| 3 | The SS transmits a valid MAC PDU containing one RLC PDU. | <--- | MAC PDU | - | - |
| 4 | The UE transmits a Scheduling Request. | --> | (SR) | - | - |
| 5 | The SS allocates an UL Grant, sufficient for one RLC SDU to be looped back in a slot n, NDI indicates new transmission, DCI scheduling the PUSCH indicates rvID = 0. | <-- | UL Grant | - | - |
| 6 | Check: Does the UE transmit a MAC PDU including one RLC SDU, in slot n+4 and repeats in following pusch-AggregationFactor-1 slots with same resource allocation but different redundancy version? (Note 3) (Note 4) | --> | MAC PDU | 1 | P |
| 7 | SS transmits a MAC PDU containing an RLC STATUS PDU acknowledging the reception of the AMD PDU in step 6. | <-- | MAC PDU (RLC STATUS PDU) | - | - |
| 8-12 | If UE supports pc_mtrpPuschCyclicMapping_r17, repeat steps 1 to 5 with cyclic mapping pattern configured (in step 1) as per Table 7.1.1.3.17.1.3.3-5. | - | - | - | - |
| 13 | Check: Does the UE transmit a MAC PDU including one RLC SDU, in HARQ process 1 and in slot n+4 and repeats in following pusch-AggregationFactor-1 slots with same resource allocation but different redundancy version? (Note 3) (Note 5) | --> | MAC PDU | 1 | P |
| 14 | SS transmits a MAC PDU containing an RLC STATUS PDU acknowledging the reception of the AMD PDU in step 9. | <-- | MAC PDU (RLC STATUS PDU) | - | - |
| - | Execute steps 15-18 if UE supports pc_mtrpPuschTwoPhrReporting_r17. | - | - | - | - |
| 15 | SS transmits NR RRCReconfiguration message to configure specific Power Headroom parameters for NR Cell. (Note 1) | <-- | (RRCReconfiguration) | - | - |
| 16 | Check: does the UE transmit a MAC PDU containing enhanced Power Headroom MAC Control Element for multiple TRP? (Note 6, 7) | --> | MAC PDU | 2 | P |
| 17 | The UE transmits an NR RRCReconfigurationComplete message to confirm the setup of Power Headroom parameters. (Note 6,2) | --> | (RRCReconfigurationComplete) | - | - |
| 18 | Check: does the UE transmit a MAC PDU containing an enhanced Power Headroom MAC Control Element for multiple TRP 500ms after step 13? (Note 7) | --> | MAC PDU | 2 | P |
| Note 1: For EN-DC the NR RRCReconfiguration message is contained in RRCConnectionReconfiguration 36.508 [7], Table 4.6.1-8 using condition EN-DC_EmbedNR_RRCRecon. Note 2: For EN-DC the NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete. Note 3: Checking the UE uses correct redundancy version and UE decides the number of layer of all transmission according to the ‘Precoding information and number of layers’ implicitly upon correctly decoding by the SS of the UE UL repetitions. Note 4: The redundancy version for the first transmission and all possible repetitions are set in the following order {0, 2, 1, 3} according to TS 38.214 [15] Table 6.1.2.1-2 and Table 6.1.2.1-3, first row. Note 5: The redundancy version for the first transmission and all possible repetitions are set in the following order {0, 1, 2, 3} according to TS 38.214 [15] Table 6.1.2.1-2 and Table 6.1.2.1-3, first row. Note 6: Steps 16 and 18 can happen in any order. Note 7: For NR5GC the received MAC PDU will contain Enhanced Single Entry PHR for multiple TRP MAC CE. For EN-DC/NE-DC the received MAC PDU will contain Enhanced Multiple Entry PHR for multiple TRP MAC CE. |  |  |  |  |  |

7.1.1.3.17.1.3.3	Specific message contents

Table 7.1.1.3.17.1.3.3-0: ServingCellConfigCommon (Pre-test conditions)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-168 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| ssb-PositionsInBurst CHOICE { |  |  |  |
| shortBitmap | 1100 |  | FR1 AND (1.88GHz<FREQ<=3GHz AND (FDD OR (TDD AND SCS15)) OR FREQ<=1.88GHZ) |
| mediumBitmap | 11000000 |  | FR1 AND (1.88GHz<FREQ<=3GHz AND (TDD AND SCS30) OR FREQ>3GHz) |
| longBitmap | 1100000000000000000000000000000000000000000000000000000000000000 |  | FR2 |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.17.1.3.3-1: RRCReconfiguration (steps 1 and 8, Table 7.1.1.3.17.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| secondaryCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) | EN-DC |
| nonCriticalExtension SEQUENCE { |  |  | NR |
| masterCellGroup | CellGroupConfig |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.17.1.3.3-2: CellGroupConfig (Table 7.1.1.3.17.1.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig::= SEQUENCE { |  |  |  |
| cellGroupId | 0 |  |  |
|  | 1 |  | EN-DC |
| spCellConfig SEQUENCE { |  |  |  |
| spCellConfigDedicated | ServingCellConfig |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.17.1.3.3-3: ServingCellConfig (Table 7.1.1.3.17.1.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP | BWP-UplinkDedicated |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.17.1.3.3-4: BWP-UplinkDedicated (Table 7.1.1.3.17.1.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkDedicated ::= SEQUENCE { |  |  |  |
| pusch-Config CHOICE { |  |  |  |
| setup | PUSCH-Config |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.17.1.3.3-5: PUSCH-Config (Table 7.1.1.3.17.1.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-118 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-Config ::= SEQUENCE { |  |  |  |
| dmrs-UplinkForPUSCH-MappingTypeB CHOICE { |  |  |  |
| setup | DMRS-UplinkConfig |  |  |
| } |  |  |  |
| pusch-AggregationFactor | n4 |  |  |
|  | n8 |  | (TDD AND SCS15) OR FR2 |
| pusch-PowerControl | PUSCH-PowerControl |  |  |
| pusch-RepTypeIndicatorDCI-0-1-r16 | pusch-RepTypeA |  |  |
| mappingPattern-r17 | cyclicMapping |  | pc_mtrpPuschCyclicMapping_r17 |
|  | sequentialMapping |  |  |
| secondTPCFieldDCI-0-1-r17 | Not present |  |  |
| secondTPCFieldDCI-0-2-r17 | Not present |  |  |
| sequenceOffsetForRV-r17 | 1 |  |  |
| } |  |  |  |

Table 7.1.1.3.17.1.3.3-6: DMRS-UplinkConfig (Table 7.1.1.3.17.1.3.3-5)

Derivation Path: TS 38.508-1 [4], Table 4.6.3-51

Table 7.1.1.3.17.1.3.3-7: PUSCH-PowerControl (Table 7.1.1.3.17.1.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-120 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-PowerControl ::= SEQUENCE { |  |  |  |
| p0-AlphaSets SEQUENCE (SIZE (1..maxNrofP0-PUSCH-AlphaSets)) OF P0-PUSCH-AlphaSet { | 2 entries |  |  |
| P0-PUSCH-AlphaSet[1] SEQUENCE { |  | entry 1 |  |
| p0-PUSCH-AlphaSetId | 0 |  |  |
| p0 | 0 |  |  |
| alpha | alpha08 |  |  |
| } |  |  |  |
| P0-PUSCH-AlphaSet[2] SEQUENCE { |  | entry 2 |  |
| p0-PUSCH-AlphaSetId | 1 |  |  |
| p0 | -20 |  |  |
| alpha | alpha08 |  |  |
| } |  |  |  |
| } |  |  |  |
| pathlossReferenceRSToAddModList SEQUENCE (SIZE (1..maxNrofPUSCH-PathlossReferenceRSs)) OF PUSCH-PathlossReferenceRS { | 2 entries |  |  |
| PUSCH-PathlossReferenceRS[1] SEQUENCE { |  | entry 1 |  |
| pusch-PathlossReferenceRS-Id | 0 |  |  |
| referenceSignal CHOICE{ |  |  |  |
| ssb-Index | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| PUSCH-PathlossReferenceRS[2] SEQUENCE { |  | entry 2 |  |
| pusch-PathlossReferenceRS-Id | 1 |  |  |
| referenceSignal CHOICE{ |  |  |  |
| ssb-Index | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| sri-PUSCH-MappingToAddModList SEQUENCE (SIZE (1..maxNrofSRI-PUSCH-Mappings)) OF SRI-PUSCH-PowerControl { | 2 entries |  |  |
| SRI-PUSCH-PowerControl[1] SEQUENCE { |  | entry 1 |  |
| sri-PUSCH-PowerControlId | 0 |  |  |
| sri-PUSCH-PathlossReferenceRS-Id | 0 |  |  |
| sri-P0-PUSCH-AlphaSetId | 0 |  |  |
| sri-PUSCH-ClosedLoopIndex | i0 |  |  |
| } |  |  |  |
| SRI-PUSCH-PowerControl[2] SEQUENCE { |  | entry 2 |  |
| sri-PUSCH-PowerControlId | 1 |  |  |
| sri-PUSCH-PathlossReferenceRS-Id | 1 |  |  |
| sri-P0-PUSCH-AlphaSetId | 1 |  |  |
| sri-PUSCH-ClosedLoopIndex | i1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.17.1.3.3-8: SRS-Config (Table 7.1.1.3.17.1.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-182 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SRS-Config ::= SEQUENCE { |  |  |  |
| srs-ResourceSetToReleaseList | Not present |  |  |
| srs-ResourceSetToAddModList SEQUENCE (SIZE(0..maxNrofSRS-ResourceSets)) OF SRS-ResourceSet { | 2 entries |  |  |
| SRS-ResourceSet[1] SEQUENCE { |  | entry 1 |  |
| srs-ResourceSetId | 0 |  |  |
| srs-ResourceIdList SEQUENCE (SIZE(1..maxNrofSRS-ResourcesPerSet)) OF SRS-ResourceId { | 1 entry |  |  |
| SRS-ResourceId[1] | 0 | entry 1 |  |
| } |  |  |  |
| resourceType CHOICE { |  |  |  |
| aperiodic SEQUENCE { |  |  |  |
| aperiodicSRS-ResourceTrigger | 1 |  |  |
| csi-RS | Not present |  |  |
| slotOffset | 7 |  | FR1 |
|  | 4 |  | FR2 |
| } |  |  |  |
| } |  |  |  |
| usage | codebook |  |  |
| alpha | Alpha |  |  |
| p0 | 0 |  |  |
| pathlossReferenceRS CHOICE { |  |  |  |
| ssb-Index | SSB-Index |  |  |
| } |  |  |  |
| srs-PowerControlAdjustmentStates | Not present |  |  |
| } |  |  |  |
| SRS-ResourceSet[2] SEQUENCE { |  | entry 2 |  |
| srs-ResourceSetId | 1 |  |  |
| srs-ResourceIdList SEQUENCE (SIZE(1..maxNrofSRS-ResourcesPerSet)) OF SRS-ResourceId { | 1 entry |  |  |
| SRS-ResourceId[1] | 1 | entry 1 |  |
| } |  |  |  |
| resourceType CHOICE { |  |  |  |
| aperiodic SEQUENCE { |  |  |  |
| aperiodicSRS-ResourceTrigger | 1 |  |  |
| csi-RS | Not present |  |  |
| slotOffset | 7 |  | FR1 |
|  | 4 |  | FR2 |
| } |  |  |  |
| } |  |  |  |
| usage | codebook |  |  |
| alpha | Alpha |  |  |
| p0 | 0 |  |  |
| pathlossReferenceRS CHOICE { |  |  |  |
| ssb-Index | SSB-Index |  |  |
| } |  |  |  |
| srs-PowerControlAdjustmentStates | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| srs-ResourceToReleaseList | Not present |  |  |
| srs-ResourceToAddModList SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SRS-Resource { | 2 entries |  |  |
| SRS-Resource[1] SEQUENCE { |  | entry 1 |  |
| srs-ResourceId | 0 |  |  |
| nrofSRS-Ports | ports2 |  | pc_maxNumberMIMO_LayersCB_PUSCH_fourlayer OR pc_maxNumberMIMO_LayersCB_PUSCH_twolayer |
|  | port1 |  |  |
| ptrs-PortIndex | Not present |  |  |
| transmissionComb CHOICE { |  |  |  |
| n2 SEQUENCE { |  |  |  |
| combOffset-n2 | 0 |  |  |
| cyclicShift-n2 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| resourceMapping SEQUENCE { |  |  |  |
| startPosition | 0 |  |  |
| nrofSymbols | n1 |  |  |
| repetitionFactor | n1 |  |  |
| } |  |  |  |
| freqDomainPosition | 0 |  |  |
| freqDomainShift | 0 |  |  |
| freqHopping SEQUENCE { |  |  |  |
| c-SRS | 0 |  |  |
|  | 63 |  | FR1_100MHz |
|  | 17 |  | FR2_100MHz |
| b-SRS | 0 |  |  |
| b-hop | 0 |  |  |
| } |  |  |  |
| groupOrSequenceHopping | groupHopping |  |  |
| resourceType CHOICE { |  |  |  |
| aperiodic SEQUENCE { |  |  |  |
| } |  |  |  |
| } |  |  |  |
| sequenceId | 0 |  |  |
| spatialRelationInfo SEQUENCE { |  |  |  |
| servingCellId | Not present |  |  |
| referenceSignal CHOICE { |  |  |  |
| ssb-Index | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| SRS-Resource[2] SEQUENCE { |  | entry 2 |  |
| srs-ResourceId | 0 |  |  |
| nrofSRS-Ports | ports2 |  | pc_maxNumberMIMO_LayersCB_PUSCH_fourlayer OR pc_maxNumberMIMO_LayersCB_PUSCH_twolayer |
|  | port1 |  |  |
| ptrs-PortIndex | Not present |  |  |
| transmissionComb CHOICE { |  |  |  |
| n2 SEQUENCE { |  |  |  |
| combOffset-n2 | 1 |  |  |
| cyclicShift-n2 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| resourceMapping SEQUENCE { |  |  |  |
| startPosition | 0 |  |  |
| nrofSymbols | n1 |  |  |
| repetitionFactor | n1 |  |  |
| } |  |  |  |
| freqDomainPosition | 0 |  |  |
| freqDomainShift | 0 |  |  |
| freqHopping SEQUENCE { |  |  |  |
| c-SRS | 0 |  |  |
|  | 63 |  | FR1_100MHz |
|  | 17 |  | FR2_100MHz |
|  | 0 |  | FR2_MBWP |
| b-SRS | 0 |  |  |
| b-hop | 0 |  |  |
| } |  |  |  |
| groupOrSequenceHopping | groupHopping |  |  |
| resourceType CHOICE { |  |  |  |
| aperiodic SEQUENCE { |  |  |  |
| } |  |  |  |
| } |  |  |  |
| sequenceId | 0 |  |  |
| spatialRelationInfo SEQUENCE { |  |  |  |
| servingCellId | Not present |  |  |
| referenceSignal CHOICE { |  |  |  |
| ssb-Index | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| tpc-Accumulation | Not present |  |  |
| } |  |  |  |

Table 7.1.1.3.17.1.3.3-9: RRCReconfiguration (step 15, Table 7.1.1.3.17.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| secondaryCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) | EN-DC |
| nonCriticalExtension SEQUENCE { |  |  | NR NE-DC |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.17.1.3.3-10: CellGroupConfig (Table 7.1.1.3.17.1.3.3-9)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| cellGroupConfig::= SEQUENCE { |  |  |  |
| mac-CellGroupConfig SEQUENCE { |  |  |  |
| phr-Config CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| phr-PeriodicTimer | sf500 |  |  |
| phr-ProhibitTimer | sf1000 |  |  |
| phr-Tx-PowerFactorChange | infinity |  |  |
| multiplePHR | false |  |  |
| multiplePHR | true |  | EN-DC NE-DC |
| phr-Type2PCell | false |  |  |
| phr-Type2OtherCell | false |  |  |
| phr-ModeOtherCG | real |  |  |
| mpe-Reporting-FR2-r17 | Not present |  |  |
| twoPHRMode-r17 | enabled |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.17.1.3.3-11: Physical layer parameters for DCI format 0_1 (Steps 5, 8, Table 7.1.1.3.17.1.3.2-1)

| Derivation path: TS 38.508-1 [4] Table 4.3.6.1.1.2-1 |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| TPC command for scheduled PUSCH | 0 dB (accumulated TPC) as per Table 7.1.1-1 in TS 38.213 [22] | “01” |  |
| Second TPC command for scheduled PUSCH | 0 dB (accumulated TPC) as per Table 7.1.1-1 in TS 38.213 [22] | “01” |  |
| SRS resource set indicator | 2 as per Table 7.3.1.1.2-36 in TS 38.212 [27] | “10” |  |
| SRS resource indicator | 0 | “0” |  |
| Second SRS resource indicator | 1 | “1” |  |
| Precoding information and number of layers | Not present (0 bits for 1 antenna port and if the higher layer parameter txConfig = Codebook as per clause 7.3.1.1.2 TS 38.212 [27]) | - |  |
|  | 2 | “10” | pc_maxNumberMIMO_LayersCB_PUSCH_fourlayer OR pc_maxNumberMIMO_LayersCB_PUSCH_twolayer |
| Second Precoding information | Not present (0 bits for 1 antenna port and if the higher layer parameter txConfig = Codebook as per clause 7.3.1.1.2 TS 38.212 [27]) | - |  |
|  | 0 | “00” | pc_maxNumberMIMO_LayersCB_PUSCH_fourlayer OR pc_maxNumberMIMO_LayersCB_PUSCH_twolayer |

7.1.1.3.17.2	Correct Handling of UL HARQ process / PUSCH Repetition / Multi-TRP / PUSCH Repetition Type B / dynamic grant

7.1.1.3.17.2.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state and supporting mTRP PUSCH Type B repetitions, and configured with two SRS resource sets with 'usage' set to 'codebook' and number of repetition K > 2 }

ensure that {

when { UE has data available for transmission, UE receives an UL Grant with toggled NDI which indicates codepoint "10", and the UL grant includes 'Second Precoding information' }

then { UE transmits two new MAC PDUs and repeats sending the MAC PDUs according to 'cyclicMapping pattern' or 'sequentialMapping pattern' configured as per UE's capability, and the layer of transmission is according to the Precoding information and number of layers, and the redundancy versions of PUSCH transmission associated with first and second SRS resource are selected correctly. }

}

(2)

with { UE in RRC_CONNECTED state and supporting mTRP PUSCH Type B repetitions, and configured with two SRS resource sets with 'usage' set to 'codebook' and number of repetition K > 2, and the MAC entity is configured with twoPHRMode }

ensure that {

when { phr-PeriodicTimer expires }

then { UE transmits an Enhanced Single entry PHR including two values of power headroom. }

}

7.1.1.3.17.2.2	Conformance requirements

<Skip unchanged part>

7.1.1.3.17.2.3	Test description

7.1.1.3.17.2.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except that DRB is configured in RLC AM mode according to Table 7.1.1.3.17.2.3.1-1.

Table 7.1.1.3.17.2.3.1-1: RLC parameters

| t-PollRetransmit | ms80 |
| --- | --- |

7.1.1.3.17.2.3.2	Test procedure sequence

Table 7.1.1.3.17.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits in the indicated downlink assignment an NR RRCReconfiguration with sequential mapping pattern configured as per Table 7.1.1.3.17.2.3.3-5. (Note 1) | <--- | (RRCReconfiguration) | - | - |
| 2 | UE transmits NR RRCReconfigurationComplete message to the SS. (Note 2) | --> | (RRCReconfigurationComplete) | - | - |
| 3 | The SS transmits a valid MAC PDU containing one RLC PDU. | <--- | MAC PDU | - | - |
| 4 | The UE transmits a Scheduling Request. | --> | (SR) | - | - |
| 5 | The SS allocates an UL Grant sufficient for one RLC SDU to be looped back in a slot n, NDI indicates new transmission, DCI scheduling the PUSCH indicates rvID = 0 and invalid symbol pattern indicator set to 1. The slot n is selected such that slot n+k2-r16 is the first UL slot in subframe m satisfying (m mod 20) = 9 for SCS=15kHz (except for HD_FDD) and SCS=120kHz and (m mod 20) = 8 for SCS=30kHz and HD_FDD. (Note 4) (Note 5) (Note 6) | <-- | UL Grant | - | - |
| 6 | Check: Does the UE transmit a MAC PDU including one RLC SDU, in slot n+4 and repeats in actual repetitions with same resource allocation but different redundancy version according to Table 7.1.1.3.17.2.3.2-2 for sequentialMapping, if the slot can be used for uplink transmission? (Note 3) | --> | MAC PDU | 1 | P |
| 7 | SS transmits a MAC PDU containing an RLC STATUS PDU acknowledging the reception of the AMD PDU in step 6. | <-- | MAC PDU (RLC STATUS PDU) | - | - |
| 8-12 | If UE supports pc_ mtrpPuschCyclicMapping_r17, repeat steps 1 to 5 with cyclic mapping pattern configured (in step 1) as per Table 7.1.1.3.17.2.3.3-5. | - | - | - | - |
| 13 | Check: Does the UE transmit a MAC PDU including one RLC SDU, in HARQ process 1 and in slot n+4 and repeats in actual repetitions with same resource allocation but different redundancy version according to Table 7.1.1.3.17.2.3.2-3 for cyclicMapping, if the slot can be used for uplink transmission? (Note 3) | --> | MAC PDU | 1 | P |
| 14 | SS transmits a MAC PDU containing an RLC STATUS PDU acknowledging the reception of the AMD PDU. | <-- | MAC PDU (RLC STATUS PDU) | - | - |
| - | Execute steps 15-18 if UE supports pc_mtrpPuschTwoPhrReporting_r17 | - | - | - | - |
| 15 | SS transmits NR RRCReconfiguration message to configure specific Power Headroom parameters for NR Cell. (Note 1) | <-- | (RRCReconfiguration) | - | - |
| 16 | Check: does the UE transmit a MAC PDU containing enhanced Power Headroom MAC Control Element for multiple TRP? (Note 7, 8) | --> | MAC PDU | 2 | P |
| 17 | The UE transmits an NR RRCReconfigurationComplete message to confirm the setup of Power Headroom parameters. (Note 7,2) | --> | (RRCReconfigurationComplete) | - | - |
| 18 | Check: does the UE transmit a MAC PDU containing an enhanced Power Headroom MAC Control Element for multiple TRP 500ms after step 13? (Note 8) | --> | MAC PDU | 2 | P |
| Note 1: For EN-DC the NR RRCReconfiguration message is contained in RRCConnectionReconfiguration 36.508 [7], Table 4.6.1-8 using condition EN-DC_EmbedNR_RRCRecon. Note 2: For EN-DC the NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete. Note 3: Checking the UE uses correct redundancy version and UE decides the number of layer of all transmission according to the ‘Precoding information and number of layers’ implicitly upon correctly decoding by the SS of the UE UL repetitions. Note 4: The UL grant is set to 384 bits: LRBs = 24 & IMCS = 2 (with PUSCH-duration=4 and PUSCH mapping Type B). Note 5: HD_FDD means pc_halfDuplexFDD_TypeA_RedCap_r17=true (i.e HD_FDD UE are performing test on FDD band). Note 6: k2-r16 is configured in Table 7.1.1.3.17.2.3.3-3. Note 7: Steps 16 and 17 can happen in any order. Note 8: For NR5GC the received MAC PDU will contain Enhanced Single Entry PHR for multiple TRP MAC CE. For EN-DC/NE-DC the received MAC PDU will contain Enhanced Multiple Entry PHR for multiple TRP MAC CE. |  |  |  |  |  |

Table 7.1.1.3.17.2.3.2-2: Transmitted actual repetition and redundancy version in each nominal repetition for sequentialMapping

|  | Nominal repetition 0 (NOTE 1, 8) | Nominal repetition 1 (NOTE 1, 8) | Nominal repetition 2 (NOTE 2, 9) | Nominal repetition 3 (NOTE 3, 9) |
| --- | --- | --- | --- | --- |
| FR1 TDD SCS=15kHz | Actual repetition 0 rvid = 0 | Actual repetition 0 rvid = 2 | N/A (NOTE 4, 5) | N/A (NOTE 6) |
| FR1 FDD SCS=15kHz | Actual repetition 0 rvid = 0 | Actual repetition 0 rvid = 2 | Actual repetition 1 (NOTE 4, 7) rvid = 3 | Actual repetition 0 rvid = 0 |
| FR1 TDD SCS=30kHz | Actual repetition 0 rvid = 0 | Actual repetition 0 rvid = 2 | Actual repetition 1 (NOTE 4) rvid = 3 | Actual repetition 0 rvid = 0 |
| FR2 SCS=120kHz | Actual repetition 0 rvid = 0 | Actual repetition 0 rvid = 2 | N/A (NOTE 4, 5) | N/A (NOTE 6) |
| NOTE 1: The nominal repetition is in slot n+k2-r16 and only one actual repetition expected. NOTE 2: The nominal repetition is split into two actual repetitions in slot n+ k2-r16 and n+ k2-r16+1 respectively. NOTE 3: The nominal repetition is in slot n+ k2-r16+1 and only one actual repetition expected. NOTE 4: The actual repetition 0 is skipped due to only 1 valid symbol left according to invalidSymbolPattern-r16. NOTE 5: The actual repetition 1 is skipped due to being located in DL symbols NOTE 6: The actual repetition 0 is skipped due to being located in DL symbols NOTE 7: The SS may not be able to decode the MAC PDU and may only detect a CRC error as there are less symbols available for transmission of the TBS NOTE 8: Associated with the SRS resource set 0. NOTE 9: Associated with the SRS resource set 1. NOTE 10: The redundancy versions are decided according to TS 38.214 [15] Table 6.1.2.1-2 and Table 6.1.2.1-3. |  |  |  |  |

Table 7.1.1.3.17.2.3.2-3: Transmitted actual repetition and redundancy version in each nominal repetition for cyclicMapping

|  | Nominal repetition 0 (NOTE 1, 8) | Nominal repetition 1 (NOTE 1, 9) | Nominal repetition 2 (NOTE 2, 8) | Nominal repetition 3 (NOTE 3, 9) |
| --- | --- | --- | --- | --- |
| FR1 TDD SCS=15kHz | Actual repetition 0 rvid = 0 | Actual repetition 0 rvid = 1 | N/A (NOTE 4, 5) | N/A (NOTE 6) |
| FR1 FDD SCS=15kHz | Actual repetition 0 rvid = 0 | Actual repetition 0 rvid = 1 | Actual repetition 1 rvid = 3 (NOTE 4, 7) | Actual repetition 0 rvid = 3 |
| FR1 TDD SCS=30kHz | Actual repetition 0 rvid = 0 | Actual repetition 0 rvid = 1 | Actual repetition 1 (NOTE 4) rvid = 3 | Actual repetition 0 rvid = 3 |
| FR2 SCS=120kHz | Actual repetition 0 rvid = 0 | Actual repetition 0 rvid = 1 | N/A (NOTE 4, 5) | N/A (NOTE 6) |
| NOTE 1: The nominal repetition is in slot n+k2-r16 and only one actual repetition expected. NOTE 2: The nominal repetition is split into two actual repetitions in slot n+ k2-r16 and n+ k2-r16+1 respectively. NOTE 3: The nominal repetition is in slot n+ k2-r16+1 and only one actual repetition expected. NOTE 4: The actual repetition 0 is skipped due to only 1 valid symbol left according to invalidSymbolPattern-r16. NOTE 5: The actual repetition 1 is skipped due to being located in DL symbols NOTE 6: The actual repetition 0 is skipped due to being located in DL symbols NOTE 7: The SS may not be able to decode the MAC PDU and may only detect a CRC error as there are less symbols available for transmission of the TBS NOTE 8: Associated with the SRS resource set 0. NOTE 9: Associated with the SRS resource set 1. NOTE 10: The redundancy versions are decided according to TS 38.214 [15] Table 6.1.2.1-2 and Table 6.1.2.1-3. |  |  |  |  |

7.1.1.3.17.2.3.3	Specific message contents

Table 7.1.1.3.17.2.3.3-0: ServingCellConfigCommon (Pre-test conditions)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-168 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| ssb-PositionsInBurst CHOICE { |  |  |  |
| shortBitmap | 1100 |  | FR1 AND (1.88GHz<FREQ<=3GHz AND (FDD OR (TDD AND SCS15)) OR FREQ<=1.88GHZ) |
| mediumBitmap | 11000000 |  | FR1 AND (1.88GHz<FREQ<=3GHz AND (TDD AND SCS30) OR FREQ>3GHz) |
| longBitmap | 1100000000000000000000000000000000000000000000000000000000000000 |  | FR2 |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.17.2.3.3-1: RRCReconfiguration (steps 1 and 8, Table 7.1.1.3.17.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| secondaryCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) | EN-DC |
| nonCriticalExtension SEQUENCE { |  |  | NR |
| masterCellGroup | CellGroupConfig |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.17.2.3.3-2: CellGroupConfig (Table 7.1.1.3.17.2.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig::= SEQUENCE { |  |  |  |
| cellGroupId | 0 |  |  |
|  | 1 |  | EN-DC |
| spCellConfig SEQUENCE { |  |  |  |
| spCellConfigDedicated | ServingCellConfig |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.17.2.3.3-3: ServingCellConfig (Table 7.1.1.3.17.2.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP | BWP-UplinkDedicated |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.17.2.3.3-4: BWP-UplinkDedicated (Table 7.1.1.3.17.2.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkDedicated ::= SEQUENCE { |  |  |  |
| pusch-Config CHOICE { |  |  |  |
| setup | PUSCH-Config |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.17.2.3.3-5 PUSCH-Config (Table 7.1.1.3.17.2.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-118 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-Config ::= SEQUENCE { |  |  |  |
| dmrs-UplinkForPUSCH-MappingTypeB CHOICE { |  |  |  |
| setup | DMRS-UplinkConfig |  |  |
| } |  |  |  |
| pusch-PowerControl | PUSCH-PowerControl |  |  |
| pusch-TimeDomainAllocationListDCI-0-1-r16 CHOICE { |  |  |  |
| setup SEQUENCE (SIZE(1..maxNrofUL-Allocations-r16)) OF SEQUENCE { | 1 entry |  |  |
| k2-r16[1] | 4 |  |  |
|  | 3 |  | HD_FDD |
| puschAllocationList-r16[1] SEQUENCE (SIZE(1..maxNrofMultiplePUSCHs-r16)) OF SEQUENCE { | 1 entry |  |  |
| mappingType-r16[1] | Not present |  |  |
| startSymbolAndLength-r16[1] | Not present |  |  |
| startSymbol-r16[1] | 4 |  |  |
| length-r16[1] | 4 |  |  |
| numberOfRepetitions-r16[1] | n4 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| invalidSymbolPatternIndicatorDCI-0-1-r16 | enabled |  |  |
| pusch-RepTypeIndicatorDCI-0-1-r16 | pusch-RepTypeB |  |  |
| invalidSymbolPattern-r16 SEQUENCE { |  |  |  |
| symbols-r16 CHOICE { |  |  |  |
| oneSlot | 00000000000001 |  |  |
| } |  |  |  |
| periodicityAndPattern-r16 | Not present |  |  |
| } |  |  |  |
| mappingPattern-r17 | cyclicMapping |  | pc_mtrpPuschCyclicMapping_r17 |
|  | sequentialMapping |  |  |
| secondTPCFieldDCI-0-1-r17 | Not present |  |  |
| secondTPCFieldDCI-0-2-r17 | Not present |  |  |
| sequenceOffsetForRV-r17 | 1 |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| HD_FDD | pc_halfDuplexFDD_TypeA_RedCap_r17 (i.e HD_FDD UE are performing test on FDD band) |

Table 7.1.1.3.17.2.3.3-6: DMRS-UplinkConfig (Table 7.1.1.3.17.2.3.3-5)

Derivation Path: TS 38.508-1 [4], Table 4.6.3-51

Table 7.1.1.3.17.2.3.3-7: PUSCH-PowerControl (Table 7.1.1.3.17.2.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-120 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-PowerControl ::= SEQUENCE { |  |  |  |
| p0-AlphaSets SEQUENCE (SIZE (1..maxNrofP0-PUSCH-AlphaSets)) OF P0-PUSCH-AlphaSet { | 2 entries |  |  |
| P0-PUSCH-AlphaSet[1] SEQUENCE { |  | entry 1 |  |
| p0-PUSCH-AlphaSetId | 0 |  |  |
| p0 | 0 |  |  |
| alpha | alpha08 |  |  |
| } |  |  |  |
| P0-PUSCH-AlphaSet[2] SEQUENCE { |  | entry 2 |  |
| p0-PUSCH-AlphaSetId | 1 |  |  |
| p0 | -20 |  |  |
| alpha | alpha08 |  |  |
| } |  |  |  |
| } |  |  |  |
| pathlossReferenceRSToAddModList SEQUENCE (SIZE (1..maxNrofPUSCH-PathlossReferenceRSs)) OF PUSCH-PathlossReferenceRS { | 2 entries |  |  |
| PUSCH-PathlossReferenceRS[1] SEQUENCE { |  | entry 1 |  |
| pusch-PathlossReferenceRS-Id | 0 |  |  |
| referenceSignal CHOICE{ |  |  |  |
| ssb-Index | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| PUSCH-PathlossReferenceRS[2] SEQUENCE { |  | entry 2 |  |
| pusch-PathlossReferenceRS-Id | 1 |  |  |
| referenceSignal CHOICE{ |  |  |  |
| ssb-Index | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| sri-PUSCH-MappingToAddModList SEQUENCE (SIZE (1..maxNrofSRI-PUSCH-Mappings)) OF SRI-PUSCH-PowerControl { | 2 entries |  |  |
| SRI-PUSCH-PowerControl[1] SEQUENCE { |  | entry 1 |  |
| sri-PUSCH-PowerControlId | 0 |  |  |
| sri-PUSCH-PathlossReferenceRS-Id | 0 |  |  |
| sri-P0-PUSCH-AlphaSetId | 0 |  |  |
| sri-PUSCH-ClosedLoopIndex | i0 |  |  |
| } |  |  |  |
| SRI-PUSCH-PowerControl[2] SEQUENCE { |  | entry 2 |  |
| sri-PUSCH-PowerControlId | 1 |  |  |
| sri-PUSCH-PathlossReferenceRS-Id | 1 |  |  |
| sri-P0-PUSCH-AlphaSetId | 1 |  |  |
| sri-PUSCH-ClosedLoopIndex | i1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.17.2.3.3-8: SRS-Config (Table 7.1.1.3.17.2.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-182 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SRS-Config ::= SEQUENCE { |  |  |  |
| srs-ResourceSetToReleaseList | Not present |  |  |
| srs-ResourceSetToAddModList SEQUENCE (SIZE(0..maxNrofSRS-ResourceSets)) OF SRS-ResourceSet { | 2 entries |  |  |
| SRS-ResourceSet[1] SEQUENCE { |  | entry 1 |  |
| srs-ResourceSetId | 0 |  |  |
| srs-ResourceIdList SEQUENCE (SIZE(1..maxNrofSRS-ResourcesPerSet)) OF SRS-ResourceId { | 1 entry |  |  |
| SRS-ResourceId[1] | 0 | entry 1 |  |
| } |  |  |  |
| resourceType CHOICE { |  |  |  |
| aperiodic SEQUENCE { |  |  |  |
| aperiodicSRS-ResourceTrigger | 1 |  |  |
| csi-RS | Not present |  |  |
| slotOffset | 7 |  | FR1 |
|  | 4 |  | FR2 |
| } |  |  |  |
| } |  |  |  |
| usage | codebook |  |  |
| alpha | Alpha |  |  |
| p0 | 0 |  |  |
| pathlossReferenceRS CHOICE { |  |  |  |
| ssb-Index | SSB-Index |  |  |
| } |  |  |  |
| srs-PowerControlAdjustmentStates | Not present |  |  |
| } |  |  |  |
| SRS-ResourceSet[2] SEQUENCE { |  | entry 2 |  |
| srs-ResourceSetId | 1 |  |  |
| srs-ResourceIdList SEQUENCE (SIZE(1..maxNrofSRS-ResourcesPerSet)) OF SRS-ResourceId { | 1 entry |  |  |
| SRS-ResourceId[1] | 1 | entry 1 |  |
| } |  |  |  |
| resourceType CHOICE { |  |  |  |
| aperiodic SEQUENCE { |  |  |  |
| aperiodicSRS-ResourceTrigger | 1 |  |  |
| csi-RS | Not present |  |  |
| slotOffset | 7 |  | FR1 |
|  | 4 |  | FR2 |
| } |  |  |  |
| } |  |  |  |
| usage | codebook |  |  |
| alpha | Alpha |  |  |
| p0 | 0 |  |  |
| pathlossReferenceRS CHOICE { |  |  |  |
| ssb-Index | SSB-Index |  |  |
| } |  |  |  |
| srs-PowerControlAdjustmentStates | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| srs-ResourceToReleaseList | Not present |  |  |
| srs-ResourceToAddModList SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SRS-Resource { | 2 entries |  |  |
| SRS-Resource[1] SEQUENCE { |  | entry 1 |  |
| srs-ResourceId | 0 |  |  |
| nrofSRS-Ports | ports2 |  | pc_maxNumberMIMO_LayersCB_PUSCH_fourlayer OR pc_maxNumberMIMO_LayersCB_PUSCH_twolayer |
|  | port1 |  |  |
| ptrs-PortIndex | Not present |  |  |
| transmissionComb CHOICE { |  |  |  |
| n2 SEQUENCE { |  |  |  |
| combOffset-n2 | 0 |  |  |
| cyclicShift-n2 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| resourceMapping SEQUENCE { |  |  |  |
| startPosition | 0 |  |  |
| nrofSymbols | n1 |  |  |
| repetitionFactor | n1 |  |  |
| } |  |  |  |
| freqDomainPosition | 0 |  |  |
| freqDomainShift | 0 |  |  |
| freqHopping SEQUENCE { |  |  |  |
| c-SRS | 0 |  |  |
|  | 63 |  | FR1_100MHz |
|  | 17 |  | FR2_100MHz |
| b-SRS | 0 |  |  |
| b-hop | 0 |  |  |
| } |  |  |  |
| groupOrSequenceHopping | groupHopping |  |  |
| resourceType CHOICE { |  |  |  |
| aperiodic SEQUENCE { |  |  |  |
| } |  |  |  |
| } |  |  |  |
| sequenceId | 0 |  |  |
| spatialRelationInfo SEQUENCE { |  |  |  |
| servingCellId | Not present |  |  |
| referenceSignal CHOICE { |  |  |  |
| ssb-Index | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| SRS-Resource[2] SEQUENCE { |  | entry 2 |  |
| srs-ResourceId | 0 |  |  |
| nrofSRS-Ports | ports2 |  | pc_maxNumberMIMO_LayersCB_PUSCH_fourlayer OR pc_maxNumberMIMO_LayersCB_PUSCH_twolayer |
|  | port1 |  |  |
| ptrs-PortIndex | Not present |  |  |
| transmissionComb CHOICE { |  |  |  |
| n2 SEQUENCE { |  |  |  |
| combOffset-n2 | 1 |  |  |
| cyclicShift-n2 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| resourceMapping SEQUENCE { |  |  |  |
| startPosition | 0 |  |  |
| nrofSymbols | n1 |  |  |
| repetitionFactor | n1 |  |  |
| } |  |  |  |
| freqDomainPosition | 0 |  |  |
| freqDomainShift | 0 |  |  |
| freqHopping SEQUENCE { |  |  |  |
| c-SRS | 0 |  |  |
|  | 63 |  | FR1_100MHz |
|  | 17 |  | FR2_100MHz |
|  | 0 |  | FR2_MBWP |
| b-SRS | 0 |  |  |
| b-hop | 0 |  |  |
| } |  |  |  |
| groupOrSequenceHopping | groupHopping |  |  |
| resourceType CHOICE { |  |  |  |
| aperiodic SEQUENCE { |  |  |  |
| } |  |  |  |
| } |  |  |  |
| sequenceId | 0 |  |  |
| spatialRelationInfo SEQUENCE { |  |  |  |
| servingCellId | Not present |  |  |
| referenceSignal CHOICE { |  |  |  |
| ssb-Index | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| tpc-Accumulation | Not present |  |  |
| } |  |  |  |

Table 7.1.1.3.17.2.3.3-9: RRCReconfiguration (step 12, Table 7.1.1.3.17.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| secondaryCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) | EN-DC |
| nonCriticalExtension SEQUENCE { |  |  | NR NE-DC |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.17.2.3.3-10: CellGroupConfig (Table 7.1.1.3.17.2.3.3-9)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| cellGroupConfig::= SEQUENCE { |  |  |  |
| mac-CellGroupConfig SEQUENCE { |  |  |  |
| phr-Config CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| phr-PeriodicTimer | sf500 |  |  |
| phr-ProhibitTimer | sf1000 |  |  |
| phr-Tx-PowerFactorChange | infinity |  |  |
| multiplePHR | false |  |  |
| multiplePHR | true |  | EN-DC NE-DC |
| phr-Type2PCell | false |  |  |
| phr-Type2OtherCell | false |  |  |
| phr-ModeOtherCG | real |  |  |
| mpe-Reporting-FR2-r17 | Not present |  |  |
| twoPHRMode-r17 | enabled |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.17.2.3.3-11: Physical layer parameters for DCI format 0_1 (Steps 5, 8, Table 7.1.1.3.17.2.3.2-1)

| Derivation path: TS 38.508-1 [4] Table 4.3.6.1.1.2-1 |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| TPC command for scheduled PUSCH | 0 dB (accumulated TPC) as per Table 7.1.1-1 in TS 38.213 [22] | “01” |  |
| Second TPC command for scheduled PUSCH | 0 dB (accumulated TPC) as per Table 7.1.1-1 in TS 38.213 [22] | “01” |  |
| SRS resource set indicator | 2 as per Table 7.3.1.1.2-36 in TS 38.212 [27] | “10” |  |
| SRS resource indicator | 0 | “0” |  |
| Second SRS resource indicator | 1 | “1” |  |
| Precoding information and number of layers | Not present (0 bits for 1 antenna port and if the higher layer parameter txConfig = Codebook as per clause 7.3.1.1.2 TS 38.212 [27]) | - |  |
|  | 2 | “10” | pc_maxNumberMIMO_LayersCB_PUSCH_fourlayer OR pc_maxNumberMIMO_LayersCB_PUSCH_twolayer |
| Second Precoding information | Not present (0 bits for 1 antenna port and if the higher layer parameter txConfig = Codebook as per clause 7.3.1.1.2 TS 38.212 [27]) | - |  |
|  | 0 | “00” | pc_maxNumberMIMO_LayersCB_PUSCH_fourlayer OR pc_maxNumberMIMO_LayersCB_PUSCH_twolayer |

<End of modified section 1>