3GPP TSG-RAN5 Meeting #108	R5-254500

# Bengaluru, India, 25th Aug 2025 – 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1631 | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to A.2.19 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated , Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCallCEN-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1. During PSAP callback following eCall release, when updated MSD is requested by the SS, UE populates the Request URI of MO INFO from the Contact header of the MT INVITE establishing the dialog. This is currently not handled in A.2.19. 2. During PSAP callback, MO INFO is the first request sent by the UE in the dialog and hence the reference of Cseq being “value sent by the UE within its previous request in the same dialog but increased by one” is incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Added text for Request uri to be same as Contact header value of MT INVITE. 2. Added text that Cseq value is not checked. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may unfairly fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.2.19 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

## A.2.19	MO INFO for eCall over IMS / PSAP callback over IMS

| Header/param |  | Cond |  | Value/remark |  | Rel |  | Reference |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request-Line |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| Method |  |  |  | INFO |  |  |  | RFC 6086 [139] |  |
| Request-URI |  |  |  | MO Call (eCall): same URI as SS sent in its Contact header before when it sent 200 OK for INVITE MT Call (PSAP callback): Same URI as SS sent in its Contact header before within the INVITE creating the dialog |  |  |  |  |  |
| SIP-Version |  |  |  | SIP/2.0 |  |  |  |  |  |
| Via |  |  |  |  |  |  |  | RFC 3261 [15] RFC 3581 [96] |  |
| sent-protocol |  |  |  | SIP/2.0/UDP when using UDP or SIP/2.0/TCP when using TCP |  |  |  |  |  |
| sent-by |  |  |  | IP address or FQDN and protected server port of the UE |  |  |  |  |  |
| via-branch |  |  |  | value starting with ‘z9hG4bK’ |  |  |  |  |  |
| Route |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| route-param |  |  |  | MO Call (eCall): URIs of the Record-Route header field of the dialog creating response in reverse order (200 OK for eCall, 183 Session Progress for test eCall) MT Call (PSAP callback): URIs of the Record-Route header field as specified for the INVITE in A.2.9 |  |  |  |  |  |
| From |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| addr-spec |  |  |  | SIP URI of the UE URI must be the same as used for the endpoint in the earlier requests within the dialog. |  |  |  | TS 24.229 [10] |  |
| tag |  |  |  | local tag of the dialog ID |  |  |  |  |  |
| To |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| addr-spec |  |  |  | SIP URI of the SS. URI must be the same as used for the endpoint in the earlier requests within the dialog. |  |  |  |  |  |
| tag |  |  |  | remote tag of the dialog ID |  |  |  |  |  |
| Call-ID |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| callid |  |  |  | same as value received in INVITE message |  |  |  |  |  |
| CSeq |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| value |  |  |  | value of CSeq sent by the UE within its previous request in the same dialog but increased by one. If this is first request sent by the UE in the dialog, then value is not checked. |  |  |  |  |  |
| method |  |  |  | INFO |  |  |  |  |  |
| Call-Info |  |  |  |  |  |  |  |  |  |
| info |  |  |  |  |  |  |  |  |  |
| absoluteURI |  |  |  | any cid URL |  | Rel-14 |  | RFC 8147 [149] |  |
| info-param |  | A3 |  | purpose=EmergencyCallData.eCall.MSD |  | Rel-14 |  | RFC 8147 [149] |  |
|  |  | A4 |  | purpose=EmergencyCallData.Control |  | Rel-14 |  | RFC 8147 [149] |  |
| Info-Package |  |  |  | EmergencyCallData.eCall.MSD |  | Rel-14 |  | RFC 8147 [149] |  |
| Content-Type |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| media-type |  |  |  | multipart/mixed;boundary=any value |  |  |  | TS 24.229 [10] |  |
| Content-Length |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| value |  |  |  | length of message-body |  |  |  |  |  |
| Content-Disposition |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| disp-type |  |  |  | Info-Package |  | Rel-14 |  | RFC 8147 [149] |  |
| Message-body |  |  |  |  |  |  |  |  |  |
|  |  | A3 |  | --boundary value (as provided in Content-Type) Content-Type: application/EmergencyCallData.eCall.MSD Content-ID: converted cid URL from Call-Info header. Note that this is done by removing the cid: prefix. Content-Disposition: by-reference MSD in ASN.1 PER encoding --boundary value (as provided in Content-Type) |  | Rel-14 |  | RFC 8147 [149] |  |
|  |  | A4 |  | --boundary value (as provided in Content-Type) Content-Type: application/EmergencyCallData.Control+xml Content-ID: converted cid URL from Call-Info header. Note that this is done by removing the cid: prefix.Content-ID: same cid as in Call-Info header Content-Disposition: by-reference <?xml version=”1.0” encoding=”UTF-8”?> <EmergencyCallData.Control xmlns="urn:ietf:params:xml:ns:EmergencyCallData:control"> <ack ref=cid of the body part of corresponding INFO request from SS> <actionResult action=”send-data” success=”false” reason=any value/> </ack> </EmergencyCallData.Control> --boundary value (as provided in Content-Type) |  | Rel-14 |  | RFC 8147 [149] |  |

| Condition | Explanation |
| --- | --- |
| A1 | Void |
| A2 | Void |
| A3 | UE able to provide an updated MSD |
| A4 | UE not able to provide an updated MSD |

<End of modified section>