3GPP TSG-RAN5 Meeting #108	R5-254518

# Bengaluru, India, 25th Aug 2025 – 29th Aug 2025

# Source:	Qualcomm Incorporated

# Title:	Discussion on test coverage for NG eCall alignment CEN cases 6 to 9

# Document for:	Discussion

## 1.	Introduction

Following the completion of NG eCall alignments for CEN cases 1 to 5, CEN/TC 278/WG 15 has requested that 3GPP working groups consider aligning CEN cases 6 to 9 on Next Generation eCall as part of 3GPP Release 19. CT1 has re-analysed the differences between the requirements in the CEN and the 3GPP specifications for these cases and sent LS response in [1] with all agreed changes in [2] of 3GPP CT1 specification [3].

This discussion paper outlines the updates made in [3] and presents the plan for new test cases or updating existing ones for CEN cases 6 to 9 as needed.

## 2.	Discussion

This section presents the details and observations related to CEN cases 6 to 9.

Case 6: CEN has different requirements and procedures for an event where the eCall support flag (ECL) is not set at the moment an eCall is triggered (see CEN/TS 17184:2022 § 7.13.1.2).

CT1 response: For last resort eCall triggered in a network that supports IMS emergency calls without ECL=T (case 6), the usage of a URI without an sos sub-type as specified in CEN specifications might result in emergency call setup failure in some countries (see Note 2 in clause 5.1.6.8.1 and several other clauses in TS 24.229). Therefore, an existing defined URI with an eCall sos sub-type in 3GPP specification is preferred.

Observation 1: Based on Note 0 added in [3], a Request-URI with eCall sub-type is needed to avoid failure in routing IMS emergency calls without ECL=T. This change is already handed in existing eCall capable test cases 11.3.3 from [4] and 11.5.6 from [5].

Proposal 1-1: No updates are needed for eCall capable test cases 11.3.3 from [4] and 11.5.6 from [5].

Proposal 1-2: New eCall only variant test case is needed for case 6 in [4].

Case 7: CEN has different requirements for an event where the eCall support flag (ECL) is set at the moment an eCall is triggered but where the IVS later receives a SIP 200 OK response not containing a positive or negative MSD ACK while using IMS over packet switched networks (see CEN/TS 17184:2022 § 7.6.4).

CT1 response: The detailed requirements for UE sending of MSD using audio media stream (e.g. cases 6 and 7) were not defined in 3GPP specifications previously. To avoid misinterpretation, we clarified this requirement and referred to CEN specifications for further details.

Observation 2: There are no test cases in [4], [5], [6], [7] where the IVS later receives a SIP 200 OK response not containing a positive or negative MSD ACK while using IMS over packet switched networks.

Proposal 2: Add new test cases in [6], [7] for a case where SIP 200 OK response not containing a positive or negative MSD ACK for eCall capable and eCall only UEs.

Case 8: CEN has different requirements for an event where an eCall fails completely while using IMS over packet switched networks (see CEN/TS 17184:2022 § 7.4.8).

Case 9: CEN has different requirements for an event where an eCall drops after successful call establishment (200 OK) but where there was no positive MSD ACK (either in a 200 OK or inband) while using IMS over packet switched networks (see CEN/TS 17184:2022 § 7.13.4.2).

CT1 response:

3GPP specifications do not fully define all possible redial attempts for pending voice, emergency or eCall (case 8). Switching from PS to CS domain as defined in 3GPP specifications is useful for implementation if the CS domain is available. To help facilitate UE redial requirements defined in CEN specifications, a note for implementation is added in 3GPP specification.

For case 9, any redial is left up to implementation – allowing an IVS to follow the CEN requirement as an option.

Observation 3: In the event of call failures redial mechanism is left up to UE implementation.

Proposal 3: No test case coverage planned for cases 8, 9.

## 3.  Proposals/Way Forward

Proposal 1-1: No updates are needed for eCall capable test cases 11.3.3 from [4] and 11.5.6 from [5].

Proposal 1-2: New eCall only variant test case is needed for case 6 in [4].

Proposal 2: Add new test cases in [6], [7] for a case where SIP 200 OK response not containing a positive or negative MSD ACK for eCall capable and eCall only UEs (case 7).

Proposal 3: No test case coverage planned for cases 8, 9.

CRs [8], [9], [10], [11] are submitted at this meeting.

## 4.	References

[1]	C1-253720	Reply LS on Next Generation eCall

[2]	C1-253772	Clarifications to eCall setup, in-band MSD transmission and retry

[3]	TS 24.229	IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3

[4]	TS 36.523-1	Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); User Equipment (UE) conformance specification; Part 1: Protocol conformance specification

[5]	TS 38.523-1	5GS; User Equipment (UE) conformance specification; Part 1: Protocol

[6]	TS 34.229-1	Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 1: Protocol conformance specification

[7]	TS 34.229-5	Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 5: Protocol conformance specification using 5G System (5GS)

[8]	R5-254519	Addition of eCall only mode test case 11.3.9

[9]	R5-254521	Applicability addition for EPS CEN test cases

[10]	R5-254522	Addition of EPS CEN alignments test case 21.32

[11]	R5-254524	Addition of applicability for EPS CEN test cases