3GPP TSG-RAN5 Meeting #108	R5-254557

# Bengaluru, India, Aug.25th – 29rd 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3471 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to connection diagram used in 4T UL MIMO and TxD Rx test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_RF_FR1_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Connection diagrams used in several 4T UL MIMO and TxD RF test cases are incorrect. As a result, connection diagrams used in these TCs need to be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Connection diagrams used in several 4T UL MIMO and TxD Rx TCs mentioned above are corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TCs are incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5D_2, 7.5G.1, 7.6D.2_2, 7.6D.3_2, 7.6D.4_2, 7.6G.1, 7.6G.2, 7.6G.3, 7.7D_2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

7.5D_2	Adjacent channel selectivity for UL MIMO for UE supporting 4Tx

<Unchanged contents are omitted>

7.5D_2.4.1	Initial conditions

Same test description as specified in 7.5D.4.1 with following exceptions:

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.4.4a for TE diagram and clause A.3.2 for UE diagram.

7.5D_2.4.2	Test procedure

Same test procedure as specified in 7.5D.4.2 with the following exception:

-	2: SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.5D.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with condition 4TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

7.5D_2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 ensuring Table 4.6.3-182 with condition 4TX_UL_MIMO.

7.5D_2.5	Test requirement

Same test requirement as defined in Clause 7.5.5.

<End of Change 1>

<Start of Change 2>

7.5G.1	Adjacent channel selectivity for Tx Diversity for UE supporting 4Tx

<Unchanged contents are omitted>

7.5G.1.4.1	Initial conditions

Same as the initial conditions given in clause 7.5.4.1 with following exceptions:

-	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, in Figure A.3.1.4.4a for TE diagram and clause A.3.2 for UE diagram.

7.5G.1.4.2	Test procedure

Same as the test procedure given in clause 7.5.4.2 with following exceptions:

-	Table 7.5.5-2, Table 7.5.5-3, Table 7.5.5-5 and Table 7.5.5-6 are replaced by Table 7.5G.1.5-2, Table 7.5G.1.5-3, Table 7.5G.1.5-5, Table 7.5G.1.5-6, respectively;

-	Step 3 is replaced by following step:

3.	Set the Downlink signal level to the value as defined in Table 7.5G.1.5-2 or Table 7.5G.1.5-5 as appropriate (Case 1). Send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of the target power level in Table 7.5G.1.5-2 or Table 7.5G.1.5-5 for at least the duration of the Throughput measurement, where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency f and the channel bandwidth BW

-	Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.3-1.

-	For UEs supporting Tx diversity, the transmit power is measured as the sum of the output power from all UE antenna connectors.

<Unchanged contents are omitted>

<End of Change 2>

<Start of Change 3>

7.6D.2_2	In-band blocking for UL MIMO for UE supporting 4Tx

<Unchanged contents are omitted>

7.6D.2_2.4.1	Initial conditions

Same test description as specified in clause 7.6D.2.4.1 with following exceptions:

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.4.4a for TE diagram and clause A.3.2 for UE diagram.

7.6D.2_2.4.2	Test procedure

Same test procedure as specified in 7.6D.2.4.2 with following exceptions:

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.6.2.4.1-1. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 4TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

<Unchanged contents are omitted>

7.6D.3_2	Out-of-band blocking for UL MIMO for UE supporting 4Tx

<Unchanged contents are omitted>

7.6D.3_2.4.1	Initial conditions

Same conditions as specified in clause 7.6D.3.4.1 with following exceptions:

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.4.5a for TE diagram and clause A.3.2 for UE diagram.

7.6D.3_2.4.2	Test procedure

Same test procedure as specified in 7.6D.3.4.2 with following exceptions:

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.6.3.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 4TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

<Unchanged contents are omitted>

7.6D.4_2	Narrow band blocking for UL MIMO for UE supporting 4Tx

<Unchanged contents are omitted>

7.6D.4_2.4.1	Initial conditions

Same test procedure as specified in 7.6D.4.4.1 with following exceptions:

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.4.5a for TE diagram and clause A.3.2 for UE diagram.

7.6D.4_2.4.2	Test procedure

Same test procedure as specified in 7.6D.4.4.2 with following exceptions:

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.6.4.4.1-1. Since the UE has no payload and no loopback data to send, the UE transmits uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 4TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

7.6D.4_2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 ensuring Table 4.6.3-182 with condition 4TX_UL_MIMO.

7.6D.4_2.5	Test requirement

Same test requirement as specified in 7.6.4.5.

<End of Change 3>

<Start of Change 4>

7.6G.1	In-band blocking for Tx Diversity for UE supporting 4Tx

<Unchanged contents are omitted>

7.6G.1.4.1	Initial conditions

Same as the initial conditions given in clause 7.6.2.4.1 with following exceptions:

-	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, in Figure A.3.1.4.4a for TE diagram and clause A.3.2 for UE diagram.

7.6G.1.4.2	Test procedure

Same as the test procedure given in clause 7.6.2.4.2 with following exceptions:

-	Table 7.6.2.5-1, Table 7.6.2.5-2, Table 7.6.2.5-3 and Table 7.6.2.5-4 are replaced by Table 7.6G.1.5-1, Table 7.6G.1.5-2, Table 7.6G.1.5-3 and Table 7.6G.1.5-4, resplectively.

-	Step 4 is replaced by following steps:

4.	Set the downlink signal level according to the table 7.6G.1.5-1 or 7.6G.1.5-3 as appropriate. Send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of the target power level in Table 7.6G.1.5-1 or Table 7.6G.1.5-3 for at least the duration of the Throughput measurement, where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency f and the channel bandwidth BW.

-	Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified for test case 6.3.4.3 in Table F.1.2-1.

-	For UEs supporting Tx diversity, the transmit power is measured as the sum of the output power from all UE antenna connectors.

<Unchanged contents are omitted>

7.6G.2	Out-of-band blocking for Tx Diversity for UE supporting 4Tx

<Unchanged contents are omitted>

7.6G.2.4	Test Description

Same as the test description in 7.6.3.4 with following exceptions:

In initial condition:

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, in Figure A.3.1.4.5a for TE diagram and clause A.3.2 for UE diagram.

In test procedure:

4.	Set the downlink signal level according to the table 7.6.3.5-1 or 7.6.3.5-3. Send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of the target power level in Table 7.6.3.5-1 or Table 7.6.3.5-3 for at least the duration of the Throughput measurement, where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency f and the channel bandwidth BW

-	Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified for test case 6.3.4.3 in Table F.1.2-1.

-	For UEs supporting 4Tx Tx diversity, the transmit power is measured as the sum of the output power from all UE antenna connectors.

<Unchanged contents are omitted>

7.6G.3	Narrow band blocking for Tx Diversity for UE supporting 4Tx

7.6G.3.1	Test Purpose

Same test purpose as in 7.6.4.1

7.6G.3.2	Test Applicability

This test applies to all types of NR Power Class 1.5 UE release 18 and forward that support 4Tx Tx diversity.

7.6G.3.3	Minimum Conformance Requirements

Same as the minimum requirements in 7.6.4.3.

7.6G.3.4	Test Description

Same as the test description in 7.6.4.4 with following exceptions:

In initial condition:

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, in Figure A.3.1.4.5a for TE diagram and clause A.3.2 for UE diagram.

In test procedure:

4.	Set the downlink signal level according to the table 7.6G.3.5-1. Send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of the target power level in Table 7.6G.3.5-1 for at least the duration of the Throughput measurement, where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency f and the channel bandwidth BW

-	Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified for test case 6.3.4.3 in Table F.1.2-1.

-	For UEs supporting 4Tx Tx diversity, the transmit power is measured as the sum of the output power from all UE antenna connectors.

<Unchanged contents are omitted>

<End of Change 4>

<Start of Change 5>

7.7D_2	Spurious response for UL MIMO for UE supporting 4Tx

7.7D_2.1	Test Purpose

Same test purpose as in clause 7.7D.1.

7.7D_2.2	Test Applicability

This test applies to all types of NR Power Class 1.5 UE release 18 and forward that support 4-layer codebook based UL MIMO on FDD bands.

7.7D_2.3	Minimum Conformance Requirements

Same minimum conformance requirements as clause 7.7D.3.

7.7D_2.4	Test Description

7.7D_2.4.1	Initial Conditions

The initial conditions shall be the same as in clause 7.7D.4.1 with the following exceptions:

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.4.5a for TE diagram and clause A.3.2 for UE diagram.

7.7D_2.4.2	Test Procedure

The test produce shall be the same as in clause 7.7D.4.2 with the following exceptions:

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Test Configuration Table 7.6D.3.4.1-1 in Clause 7.6D.3. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with condition 4TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

7.7D_2.4.3	Message Contents

Message contents are according to TS 38.508-1 [5] clause 4.6 ensuring Table 4.6.3-182 with condition 4TX_UL_MIMO.

7.7D_2.5	Test Requirement

Same test requirement as specified in 7.7.5.

<End of Change 5>