3GPP TSG-RAN5 Meeting #108	R5-254607

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3586 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of RRC messages for MBS Multicast reception in RRC_INACTIVE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS_enh_5MBS_Ph2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The contents of RRC messages for MBS Multicast reception in RRC_INACTIVE haven’t been added |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the contents of RRC messages for MBS Multicast reception in RRC_INACTIVE according to TS 38.331 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases for MBS Multicast reception in RRC_INACTIVE will be not implementable |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.1, 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changed Section>

4.6.1	Contents of RRC messages

<Unchanged Section skips>

–	MBSMulticastConfiguration

Table 4.6.1-5ABC: MBSMulticastConfiguration

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MBSMulticastConfiguration-r18 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| mbsMulticastConfiguration-r18 SEQUENCE { |  |  |  |
| mbs-SessionInfoListMulticast-r18 | MBS-SessionInfoListMulticast |  |  |
| mbs-NeighbourCellList-r18 | Not present |  |  |
| drx-ConfigPTM-List-r18 | Not present |  |  |
| pdsch-ConfigMTCH-r18 | Not present |  |  |
| mtch-SSB-MappingWindowList-r18 | Not present |  |  |
| thresholdMBS-List-r18 SEQUENCE (SIZE (1..maxNrofThresholdMBS-r18)) OF ThresholdMBS-r18 { | 1 entry |  |  |
| ThresholdMBS-r18[1] SEQUENCE { |  | entry 1 |  |
| rsrp-r18 | 51 |  |  |
| rsrq-r18 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<Unchanged Section skips>

–	Paging

Table 4.6.1-9: Paging

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Paging ::= SEQUENCE { |  |  |  |  |
| pagingRecordList SEQUENCE (SIZE(1..maxNrofPageRec)) OF PagingRecord { |  | 1 entry |  |  |
| PagingRecord[1] SEQUENCE { |  |  | entry 1 |  |
| ue-Identity CHOICE { |  |  |  |  |
| ng-5G-S-TMSI |  | NG-5G-S-TMSI |  |  |
| fullI-RNTI |  | I-RNTI-Value |  | NR_RRC_RESUME |
| } |  |  |  |  |
| accessType |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | TMGI, MT_SDT, MBS_Multicast_Inactive |
| pagingRecordList-v1700 |  | Not present |  |  |
| pagingGroupList-r17 SEQUENCE (SIZE(1..maxNrofPageGroup-r17)) OF TMGI-r17 { |  | 1 entry |  |  |
| TMGI-r17[1] |  | TMGI-r17 | entry 1 |  |
| } |  |  |  |  |
| nonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | MT_SDT, MBS_Multicast_Inactive |
| PagingRecordList-v1800 |  | Not present |  | MBS_Multicast_Inactive |
| PagingRecordList-v1800 SEQUENCE (SIZE(1..maxNrofPageRec)) OF PagingRecord-v1800 { |  | 1 entry |  | MT_SDT |
| PagingRecord-v1800[1] SEQUENCE { |  |  | entry 1 |  |
| mt-SDT |  | true |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| pagingGroupList-v1800 |  | Not present |  | MT_SDT |
| pagingGroupList-v1800 SEQUENCE (SIZE(1..maxNrofPageGroup-r17)) OF GroupPaging-r18 { |  | 1 entry |  | MBS_Multicast_Inactive |
| GroupPaging-r18[1] SEQUENCE { |  |  | entry 1 |  |
| inactiveReceptionAllowed-r18 |  | true |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| nonCriticalExtension |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| NR_RRC_RESUME | To page a UE in RRC_INACTIVE state to request RRC connection resumption |
| TMGI | TMGI is used as paging identity |
| MT_SDT | MT-SDT is used as Mobile Terminated SDT indication |
| MBS_Multicast_Inactive | Used for MBS Multicast reception when UE in RRC_INACTIVE state |

<Unchanged Section skips>

–	RRCRelease

Table 4.6.1-16: RRCRelease

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCRelease ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcRelease SEQUENCE { |  |  |  |  |
| redirectedCarrierInfo |  | Not present |  |  |
| cellReselectionPriorities |  | Not present |  |  |
| suspendConfig |  | Not present |  |  |
| suspendConfig SEQUENCE { |  |  |  | NR_RRC_INACTIVE |
| fullI-RNTI |  | I-RNTI-Value |  |  |
| shortI-RNTI |  | ShortI-RNTI-Value |  |  |
| ran-PagingCycle |  | rf32 |  |  |
| ran-NotificationAreaInfo CHOICE { |  |  |  |  |
| cellList SEQUENCE (SIZE (1.. maxPLMNIdentities)) OF PLMN-RAN-AreaCell { |  | 1 entry |  |  |
| PLMN-RAN-AreaCellList[1] SEQUENCE { |  |  | entry 1 |  |
| plmn-Identity |  | Not present |  |  |
| ran-AreaCells SEQUENCE (SIZE (1..32)) OF CellIdentity { |  | 1 entry |  |  |
| CellIdentity[1] |  | CellIdentity | entry 1 Cellidentity for the used cell. |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| t380 |  | Not present |  |  |
| nextHopChainingCount |  | NextHopChainingCount |  |  |
| sl-UEIdentityRemote-r17 |  | Not present |  |  |
| sl-UEIdentityRemote-r17 |  | RNTI-Value |  | L2RemoteUE |
| sdt-Config-r17 |  | Not present |  |  |
| sdt-Config-r17 CHOICE { |  |  |  | SDT, MT_SDT |
| setup SEQUENCE { |  |  |  |  |
| sdt-DRB-List-r17 SEQUENCE (SIZE (0..maxDRB)) OF DRB-Identity { |  | 1 entry |  |  |
| DRB-Identity |  | DRB-Identity using condition DRB1 |  |  |
| } |  |  |  |  |
| sdt-SRB2-Indication-r17 |  | Not present |  |  |
|  |  | allowed |  | pc_srb_SDT_r17 |
| sdt-MAC-PHY-CG-Config-r17 CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| cg-SDT-ConfigLCH-RestrictionToAddModList-r17 SEQUENCE (SIZE(1..maxLC-ID)) OF CG-SDT-Config-LCH-restriction-r17 { |  | 1 entry |  |  |
| logicalChannelIdentity-r17 |  | LogicalChannelIdentity with condition DRB1 |  |  |
| configuredGrantType1Allowed-r17 |  | true |  |  |
| allowedCG-List-r17 |  | Not present |  |  |
| } |  |  |  |  |
| cg-SDT-ConfigLCH-RestrictionToReleaseList-r17 |  | Not present |  |  |
| cg-SDT-ConfigInitialBWP-NUL-r17 |  | Not present |  |  |
| cg-SDT-ConfigInitialBWP-SUL-r17 |  | Not present |  |  |
| cg-SDT-ConfigInitialBWP-DL-r17 |  | Not present |  |  |
| cg-SDT-TimeAlignmentTimer-r17 |  | ms750 |  |  |
| cg-SDT-RSRP-ThresholdSSB-r17 |  | 66 |  |  |
| cg-SDT-TA-ValiditationConfig-r17 |  | Not present |  |  |
| cg-SDT-CS-RNTI-r17 |  | Not present |  |  |
| cg-SDT-ConfigLCH-RestrictionToAddModListExt-v1800 |  | Not present |  |  |
| cg-SDT-ConfigLCH-RestrictionToAddModListExt-v1800 SEQUENCE (SIZE(1..maxLC-ID)) OF CG-SDT-ConfigLCH-Restriction-v1800 { |  | 1 entry |  | MT_SDT AND pc_cg-SDT-PeriodicityExt-r18 |
| cg-MT-SDT-MaxDurationToNext-CG-Occasion-r18 [1] |  | FFS | Entry 1 |  |
| } |  |  |  |  |
| cg-MT-SDT-MaxDurationToNextCG-Occasion-r18 |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sdt-DRB-ContinueROHC-r17 |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| srs-PosRRC-Inactive-r17 |  | Not present |  |  |
| ran-ExtendedPagingCycle-r17 |  | Not present |  |  |
| multicastConfigInactive-r18 |  | Not present |  |  |
| multicastConfigInactive-r18 CHOICE { |  |  |  | MBS_Multicast_Inactive |
| setup SEQUENCE { |  |  |  |  |
| inactivePTM-Config-r18 |  | MBSMulticastConfiguration |  |  |
| inactiveMCCH-Config-r18 |  | SystemInformation | Only SIB24 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| deprioritisationReq |  | Not present |  |  |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| NR_RRC_INACTIVE | NR RRC state RRC_INACTIVE |
| SDT | For SDT test cases |
| L2RemoteUE | For L2 U2N Remote UE test cases. |
| MT-SDT | For MT-SDT test cases |
| MBS_Multicast_Inactive | Used for MBS Multicast reception when UE in RRC_INACTIVE state |

<Unchanged Section skips>

–	SIB1

Table 4.6.1-28: SIB1

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |  |
| q-RxLevMin |  | -70 | -140 dBm | RF OR RRM |
|  |  | -55 | -110 dBm | SIG AND FR1 |
|  |  | ROUND((-110+Delta(NRfs))/2) | -110+Delta(NRfs) | SIG AND FR2 |
| q-RxLevMinOffset |  | Not present |  |  |
| q-RxLevMinSUL |  | Not Present |  |  |
|  |  | -70 | -140 dBm | SUL AND (RF OR RRM) |
|  |  | -55 | -110 dBm | SUL AND SIG |
| q-QualMin |  | -20 | -20dB | QBASED |
|  |  | Not present |  |  |
| q-QualMinOffset |  | Not present |  |  |
| } |  |  |  |  |
| cellAccessRelatedInfo |  | CellAccessRelatedInfo |  |  |
| connEstFailureControl |  | ConnEstFailureControl |  |  |
| si-SchedulingInfo |  | Not present |  | NR_1 |
|  |  | SI-SchedulingInfo |  |  |
| servingCellConfigCommon |  | ServingCellConfigCommonSIB |  |  |
|  |  | ServingCellConfigCommonSIB with condition PEI |  | PEI |
|  |  | ServingCellConfigCommonSIB with condition SDT |  | SDT |
|  |  | ServingCellConfigCommonSIB with condition MBS_Multicast_Inactive |  | MBS_Multicast_Inactive |
| ims-EmergencySupport |  | Not present |  |  |
|  |  | true | Indicates the cell supports IMS emergency bearer services for UEs in limited service mode. | SIG |
| eCallOverIMS-Support |  | Not present |  |  |
|  |  | true | Support of eCall over IMS services | eCalloverIMSforNR |
| ue-TimersAndConstants |  | UE-TimersAndConstants |  |  |
| uac-BarringInfo |  | Not present |  |  |
| useFullResumeID |  | Not present |  |  |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | EMR_EUTRA, EMR_NR, posSIB, pc_supportOfRedCap_r17, SDT, MT_SDT, ATG, MUSIM r18, pc_supportOfERedCap_r18, SI_Msg1Repetition |
| idleModeMeasurementsEUTRA-r16 |  | Not present |  |  |
|  |  | true |  | EMR_EUTRA |
| idleModeMeasurementsNR-r16 |  | Not present |  |  |
|  |  | true |  | EMR_NR |
| posSI-SchedulingInfo-r16 |  | PosSI-SchedulingInfo-r16 |  |  |
|  |  | PosSI-SchedulingInfo-r16 |  | posSIB |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| uac-BarringInfo-v1630 |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| hsdn-Cell-r17 |  | Not present |  |  |
| uac-BarringInfo-v1700 |  | Not present |  |  |
| sdt-ConfigCommon-r17 |  | Not present |  |  |
| sdt-ConfigCommon-r17 SEQUENCE { |  |  |  | SDT |
| sdt-RSRP-Threshold-r17 |  | 66 |  |  |
| sdt-LogicalChannelSR-DelayTimer-r17 |  | Not present |  |  |
| sdt-DataVolumeThreshold-r17 |  | byte1000 |  |  |
| t319a-r17 |  | ms1000 |  |  |
| } |  |  |  |  |
| redCap-ConfigCommon-r17 |  | Not present |  |  |
| redCap-ConfigCommon-r17 SEQUENCE { |  |  |  | pc_halfDuplexFDD_TypeA_RedCap_r17 AND FDD |
| halfDuplexRedCapAllowed-r17 |  | true |  |  |
| cellBarredRedCap-r17 |  | Not present |  |  |
| } |  |  |  |  |
| featurePriorities-r17 |  | Not present |  |  |
| si-SchedulingInfo-v1700 |  | Not present |  |  |
|  |  | SI-SchedulingInfo-v1700 |  | SIBs_r17 |
| hyperSFN-r17 |  | Not present |  |  |
| eDRX-AllowedIdle-r17 |  | Not present |  |  |
| eDRX-AllowedInactive-r17 |  | Not present |  |  |
| intraFreqReselectionRedCap-r17 |  | Not present |  |  |
|  |  | allowed |  | pc_supportOfRedCap_r17 |
| cellBarredNTN-r17 |  | Not present |  |  |
|  |  | notBarred |  | NTN |
| nonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | MUSIM r18, MT-SDT, SI_Msg1Repetition, ATG, pc_supportOfERedCap_r18 |
| si-SchedulingInfo-v1740 |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| ncr-Support-r18 |  | Not present |  |  |
| mt-SDT-ConfigCommonSIB-r18 |  | Not present |  |  |
| mt-SDT-ConfigCommonSIB-r18 SEQUENCE { |  |  |  | MT-SDT |
| mt-SDT-RSRP-Threshold-r18 |  | FFS |  |  |
| sdt-LogicalChannelSR-DelayTimer-r18 |  | FFS |  |  |
| t319a-r18 |  | FFS |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| musim-CapRestrictionAllowed-r18 |  | Not present |  |  |
|  |  | true |  | MUSIM_r18 |
| featurePriorities-v1800 |  | Not present |  |  |
| si-SchedulingInfo-v1800 |  | Not present |  |  |
|  |  | SI-SchedulingInfo-v1800 |  | SI_Msg1Repetition |
| cellBarredATG-r18 |  | Not present |  |  |
|  |  | notBarred |  | ATG |
| cellBarredNES-r18 |  | Not present |  |  |
| mobileIAB-Cell-r18 |  | Not present |  |  |
| eDRX-AllowedInactive-r18 |  | Not present |  |  |
| intraFreqReselection-eRedCap-r18 |  | Not present |  |  |
|  |  | allowed |  | pc_supportOfERedCap_r18 |
| nonServingCellMII-r18 |  | Not present |  |  |
| sdt-BeamFailureRecoveryProhibitTimer-r18 |  | Not present |  |  |
| eRedCap-ConfigCommon-r18 |  | Not present |  |  |
| cellBarredFixedVSAT-r18 |  | Not present |  |  |
| cellBarredMobileVSAT-r18 |  | Not present |  |  |
| reselectionMeasurementsNR-r18 |  | Not present |  |  |
| cellBarred2RxXR-r18 |  | Not present |  |  |
| intraFreqReselection2RxXR-r18 |  | Not present |  |  |
| barringExemptEmergencyCall-r18 |  | Not present |  |  |
| n3c-Support-r18 |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| NOTE 1: Delta(NRfs) is derived based on calibration procedure defined in the clause 6.1.3.3. NRfs is NR frequency on which SIB1 is broadcasted. NOTE 2: ROUND is rounded off to the nearest integer. As an example, '1 to 1.49' set to '1' while '1.5 to 2' to '2' and '-2.0 to 1.5' set to '-2' while '-1.49 to -1' set to '-1'. |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| SUL | For test cases using SUL frequency for the serving cell, Qrxlevmin is obtained from q-RxLevMinSUL. |
| QBASED | This condition applies to Quality based signalling test cases. |
| NR_1 | System information combination NR-1 according table 4.4.3.1.2-1 is applied. |
| SIG | Used for signalling test cases. |
| eCalloverIMSforNR | Used for eCall over IMS test cases (TS 38.523-1[12], TS 34.229-5[47]) |
| posSIB | For test cases using posSIBs in system information. |
| EMR_EUTRA | For E-UTRA idle/inactive measurement test cases. |
| EMR_NR | For NR idle/inactive measurement test cases. |
| SDT | For SDT test cases |
| NTN | For NTN test cases |
| PEI | Paging Early Indication is configured in the cell. |
| SIBs_r17 | For test cases using sibTypes defined in SIB-TypeInfo-v1700. |
| MT-SDT | For MT-SDT test cases |
| ATG | For ATG test cases |
| MUSIM_r18 | For Rel-18 MUSIM test cases |
| SI_Msg1Repetition | For test cases using SI request with Msg1 repetition. |
| MBS_Multicast_Inactive | Used for MBS Multicast reception when UE in RRC_INACTIVE state |

<Unchanged Section skips>

4.6.3	Radio resource control information elements

<Unchanged Section skips>

–	BWP-DownlinkCommon

Table 4.6.3-10: BWP-DownlinkCommon

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkCommon ::= SEQUENCE { |  |  |  |
| genericParameters | BWP |  |  |
| pdcch-ConfigCommon CHOICE { |  |  |  |
| setup | PDCCH-ConfigCommon |  |  |
|  | PDCCH-ConfigCommon with condition SCell_add |  | SCell_add |
|  | PDCCH-ConfigCommon with condition InitialBWP_SIB |  | InitialBWP_SIB |
|  | PDCCH-ConfigCommon with condition BWP-Id1 |  | BWP-Id1 |
|  | PDCCH-ConfigCommon with condition PEI |  | PEI |
|  | PDCCH-ConfigCommon with condition SDT |  | SDT |
|  | PDCCH-ConfigCommon with condition MBS_Broadcast |  | MBS_Broadcast |
|  | PDCCH-ConfigCommon with condition MBS_Multicast_Inactive |  | MBS_Multicast_Inactive |
| } |  |  |  |
| pdsch-ConfigCommon CHOICE { |  |  |  |
| setup | PDSCH-ConfigCommon |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| InitialBWP_SIB | Configured via DownlinkConfigCommonSIB |
| BWP-Id1 | Additional BWP 1 |
| PEI | Paging Early Indication is configured in the cell. |
| SCell_add | Add SCell |
| SDT | For SDT test cases |
| MBS_Broadcast | NR Cell provide MBS Broadcast service |
| MBS_Multicast_Inactive | Used for MBS Multicast reception when UE in RRC_INACTIVE state |

<Unchanged Section skips>

–	DownlinkConfigCommonSIB

Table 4.6.3-53: DownlinkConfigCommonSIB

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DownlinkConfigCommonSIB ::= SEQUENCE { |  |  |  |
| frequencyInfoDL | FrequencyInfoDL-SIB |  |  |
| initialDownlinkBWP | BWP-DownlinkCommon with condition InitialBWP_SIB |  |  |
|  | BWP-DownlinkCommon with condition PEI |  | PEI |
|  | BWP-DownlinkCommon with condition SDT |  | SDT |
|  | BWP-DownlinkCommon with condition MBS_Broadcast |  | MBS_Broadcast |
|  | BWP-DownlinkCommon with condition MBS_Multicast_Inactive |  | MBS_Multicast_Inactive |
| bcch-Config SEQUENCE { |  |  |  |
| modificationPeriodCoeff | n4 |  |  |
| } |  |  |  |
| pcch-Config SEQUENCE { |  |  |  |
| defaultPagingCycle | rf128 |  |  |
| nAndPagingFrameOffset CHOICE { |  |  |  |
| halfT | 0 |  |  |
|  | 1 |  | PEI |
| } |  |  |  |
| ns | one |  |  |
|  | two |  | PEI |
| firstPDCCH-MonitoringOccasionOfPO | Not present |  |  |
| nrofPDCCH-MonitoringOccasionPerSSB-InPO-r16 | Not present |  |  |
| } |  |  |  |
| pei-Config-r17 | Not present |  |  |
| pei-Config-r17 SEQUENCE { |  |  | PEI |
| po-NumPerPEI-r17 | po2 |  |  |
| payloadSizeDCI-2-7-r17 | 22 |  |  |
| pei-FrameOffset-r17 | 3 |  |  |
| subgroupConfig-r17 SEQUENCE { |  |  |  |
| subgroupsNumPerPO-r17 | 2 |  |  |
| subgroupsNumForUEID-r17 | 2 |  |  |
| } |  |  |  |
| lastUsedCellOnly-r17 | Not present |  |  |
| } |  |  |  |
| initialDownlinkBWP-RedCap-r17 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| PEI | Paging Early Indication is configured in the cell. |
| SDT | For SDT test cases. |
| MBS_Broadcast | NR Cell provide MBS Broadcast service |
| MBS_Multicast_Inactive | Used for MBS Multicast reception when UE in RRC_INACTIVE state |

<Unchanged Section skips>

–	PDCCH-ConfigCommon

Table 4.6.3-96: PDCCH-ConfigCommon

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| controlResourceSetZero | ControlResourceSetZero | Initial BWP |  |
|  | Not present |  | SCell_add InitialBWP_SIB, BWP-Id1 |
| commonControlResourceSet | ControlResourceSet |  | EN-DC |
|  | Not present |  |  |
| searchSpaceZero | SearchSpaceZero | Initial BWP |  |
|  | Not present |  | SCell_add InitialBWP_SIB, BWP-Id1 |
| commonSearchSpaceList SEQUENCE (SIZE (1..4)) OF SearchSpace { | 2 entries |  |  |
|  | 3 entries |  | PEI |
| SearchSpace[1] | SearchSpace with condition CSS | entry 1 |  |
| SearchSpace[2] | SearchSpace with condition SISS | entry 2 |  |
| SearchSpace[3] | SearchSpace with condition PEI | entry 3 | PEI |
| } |  |  |  |
| commonSearchSpaceList SEQUENCE (SIZE (1..4)) OF SearchSpace { | 1 entry |  | EN-DC |
| SearchSpace[1] | SearchSpace with condition CSS | entry 1 |  |
| } |  |  |  |
| commonSearchSpaceList | Not present |  | SCell_add |
| searchSpaceSIB1 | 0 |  |  |
|  | Not present |  | EN-DC, SCell_add |
| searchSpaceOtherSystemInformation | SearchSpaceId with condition SISS |  |  |
|  | Not present |  | EN-DC, SCell_add |
| pagingSearchSpace | 0 |  |  |
|  | Not present |  | EN-DC, SCell_add |
| ra-SearchSpace | SearchSpaceId with condition CSS |  |  |
|  | Not present |  | SCell_add |
| firstPDCCH-MonitoringOccasionOfPO | Not present |  |  |
| commonSearchSpaceListExt-r16 | Not present |  |  |
| sdt-SearchSpace-r17 | Not present |  |  |
| sdt-SearchSpace-r17 CHOICE { |  |  | SDT |
| existingSearchSpace | SearchSpaceId with condition CSS |  |  |
| } |  |  |  |
| searchSpaceMCCH-r17 | Not present |  |  |
| searchSpaceMCCH-r17 | SearchSpaceId with condition CSS |  | MBS_Broadcast |
| searchSpaceMTCH-r17 | Not present |  |  |
| commonSearchSpaceListExt2-r17 | Not present |  |  |
| commonSearchSpaceListExt2-r17 SEQUENCE(SIZE (1..4)) OF SearchSpaceExt-v1700 { | 2 entries |  | MBS_Broadcast |
|  | 3 entries |  | PEI |
| SearchSpaceExt-v1700[1] SEQUENCE { |  | entry 1 | MBS_Broadcast, PEI |
| monitoringSlotPeriodicityAndOffset-v1710 | Not present |  |  |
| monitoringSlotsWithinSlotGroup-r17 | Not present |  |  |
| duration-r17 | Not present |  |  |
| searchSpaceType-r17 | Not present |  | PEI |
| searchSpaceType-r17 SEQUENCE { |  |  | MBS_Broadcast |
| common-r17 SEQUENCE { |  |  |  |
| dci-Format4-0-r17 SEQUENCE { |  |  |  |
| } |  |  |  |
| dci-Format4-1-r17 | Not present |  |  |
| dci-Format4-2-r17 | Not present |  |  |
| dci-Format4-1-AndFormat4-2-r17 | Not present |  |  |
| dci-Format2-7-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| searchSpaceGroupIdList-r17 | Not present |  |  |
| searchSpaceLinkingId-r17 | Not present |  |  |
| } |  |  |  |
| SearchSpaceExt-v1700[2] SEQUENCE { |  | entry 2 | MBS_Broadcast, PEI |
| monitoringSlotPeriodicityAndOffset-v1710 | Not present |  |  |
| monitoringSlotsWithinSlotGroup-r17 | Not present |  |  |
| duration-r17 | Not present |  |  |
| searchSpaceType-r17 | Not present |  |  |
| searchSpaceGroupIdList-r17 | Not present |  |  |
| searchSpaceLinkingId-r17 | Not present |  |  |
| } |  |  |  |
| SearchSpaceExt-v1700[3] SEQUENCE { |  | entry 3 | PEI |
| monitoringSlotPeriodicityAndOffset-v1710 | Not present |  |  |
| monitoringSlotsWithinSlotGroup-r17 | Not present |  |  |
| duration-r17 | Not present |  |  |
| searchSpaceType-r17 SEQUENCE { |  |  |  |
| common-r17 SEQUENCE { |  |  |  |
| dci-Format4-0-r17 | Not present |  |  |
| dci-Format4-1-r17 | Not present |  |  |
| dci-Format4-2-r17 | Not present |  |  |
| dci-Format4-1-AndFormat4-2-r17 | Not present |  |  |
| dci-Format2-7-r17 SEQUENCE { |  |  |  |
| nrofCandidates-PEI-r17 SEQUENCE { |  |  |  |
| aggregationLevel4-r17 | n2 |  |  |
| aggregationLevel8-r17 | Not present |  |  |
| aggregationLevel16-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| firstPDCCH-MonitoringOccasionOfPO-v1710 | Not present |  |  |
| pei-ConfigBWP-r17 | Not present |  |  |
| pei-ConfigBWP-r17 SEQUENCE { |  |  | PEI |
| pei-SearchSpace-r17 | SearchSpaceId with condition PEI |  |  |
| firstPDCCH-MonitoringOccasionOfPEI-O-r17 CHOICE { |  |  |  |
| sCS15KHZoneT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER { | 2 entries |  | SCS15 |
| INTEGER[1] | 0 |  |  |
| INTEGER[2] | 1 |  |  |
| 1 |  |  |  |
| } |  |  |  |
| sCS30KHZoneT-SCS15KHZhalfT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER { | 1 entry |  | SCS30 |
| INTEGER[1] | 0 |  |  |
| } |  |  |  |
| sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER { | 1 entry |  | SCS120 |
| INTEGER[1] | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| followUnifiedTCIstate-v1720 | Not present |  |  |
| applyIndicatedTCI-State-r18 | Not present |  |  |
| commonSearchSpaceListExt-r18 | Not present |  |  |
| commonSearchSpaceListExt-r18 SEQUENCE(SIZE (1..4)) OF SearchSpaceExt-v1800 { | 2 entry |  | DCI_1_3, DCI_0_3, DCI_2_9 |
| SearchSpaceExt-v1800[1] SEQUENCE { |  | entry 1 |  |
| searchSpaceType-r18 CHOICE { |  |  |  |
| common-r18 SEQUENCE { |  |  | DCI_2_9 |
| dci-Format2-9-r18 SEQUENCE { |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ue-Specific-r18 SEQUENCE { |  |  | DCI_1_3, DCI_0_3 |
| dci-FormatsMC-r18 | formats1-3 |  | DCI_1_3 |
|  | formats0-3 |  | DCI_0_3 |
|  | formats0-3-And-1-3 |  | DCI_1_3 AND DCI_0_3 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| SearchSpaceExt-v1800[2] SEQUENCE { |  | entry 2 |  |
| searchSpaceType-r18 CHOICE { |  |  |  |
| common-r18 SEQUENCE { |  |  | DCI_2_9 |
| dci-Format2-9-r18 SEQUENCE { |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ue-Specific-r18 SEQUENCE { |  |  | DCI_1_3, DCI_0_3 |
| dci-FormatsMC-r18 | formats1-3 |  | DCI_1_3 |
|  | formats0-3 |  | DCI_0_3 |
|  | formats0-3-And-1-3 |  | DCI_1_3 AND DCI_0_3 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| searchSpaceMulticastMCCH-r18 | Not present |  |  |
| searchSpaceMulticastMCCH-r18 | SearchSpaceId with condition MSS |  | MBS_Multicast_Inactive |
| searchSpaceMulticastMTCH-r18 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| EN-DC | E-UTRA-NR Dual Connectivity with E-UTRA connected to EPC |
| SCell_add | Add SCell |
| InitialBWP_SIB | Configured via DownlinkConfigCommonSIB |
| BWP-Id1 | Additional BWP 1 |
| SDT | For SDT test cases |
| PEI | Paging Early Indication is configured in the cell. |
| MBS_Broadcast | NR Cell provide MBS Broadcast service |
| DCI_2_9 | Transmission of DCI_2_9 is required |
| DCI_1_3 | Transmission of DCI_1_3 is required |
| DCI_0_3 | Transmission of DCI_0_3 is required |
| MBS_Multicast_Inactive | Used for MBS Multicast reception when UE in RRC_INACTIVE state |

<Unchanged Section skips>

–	ServingCellConfigCommonSIB

Table 4.6.3-169: ServingCellConfigCommonSIB

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| downlinkConfigCommon | DownlinkConfigCommonSIB |  |  |
|  | DownlinkConfigCommonSIB with condition PEI |  | PEI |
|  | DownlinkConfigCommonSIB with condition SDT |  | SDT |
|  | DownlinkConfigCommonSIB with condition MBS_Broadcast |  | MBS_Broadcast |
|  | DownlinkConfigCommonSIB with condition MBS_Multicast_Inactive |  | MBS_Multicast_Inactive |
| uplinkConfigCommon | UplinkConfigCommonSIB |  |  |
| supplementaryUplink | Not present |  |  |
|  | UplinkConfigCommonSIB with condition SUL_SUL |  | SUL |
| n-TimingAdvanceOffset | Not present |  |  |
| ssb-PositionsInBurst SEQUENCE { |  |  |  |
| inOneGroup | ’0100 0000’B | When carrier frequency is smaller than or equal to 3 GHz, only the 4 leftmost bits are valid | SSB#1 |
|  | ’1000 0000’B |  | SSB#0 |
| groupPresence | Not present |  | FR1 |
|  | ’1000 0000’B |  | FR2 |
| } |  |  |  |
| ssb-PeriodicityServingCell | ms20 |  |  |
| tdd-UL-DL-ConfigurationCommon | TDD-UL-DL-ConfigCommon |  | TDD |
|  | Not present |  | FDD |
| ss-PBCH-BlockPower | 0 |  |  |
| channelAccessMode-r16 | Not present |  |  |
| channelAccessMode-r16 CHOICE { |  |  | SharedSpectrum |
| semiStatic | SemiStaticChannelAccessConfig-r16 |  | pc_ul_Semi_StaticChAccess_r16 |
| dynamic | NULL |  | NOT pc_ul_Semi_StaticChAccess_r16 |
| } |  |  |  |
| discoveryBurstWindowLength-r16 | Not present |  |  |
| highSpeedConfig-r16 | Not present |  |  |
|  | HighSpeedConfig-r16 |  | R16 HST OR R17 HST |
| channelAccessMode2-r17 | Not present |  |  |
| discoveryBurstWindowLength-v1700 | Not present |  |  |
| highSpeedConfigFR2-r17 | Not present |  |  |
|  | HighSpeedConfigFR2-r17 |  | R17 HST AND FR2 |
| uplinkConfigCommon-v1700 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SUL | Supplementary uplink |
| SSB#N | Cell configured with SSB-Index set to N as defined in Table 4.4.2-2 |
| SharedSpectrum | Operation with shared spectrum channel access |
| R16 HST | For R16 HST test |
| R17 HST | For R17 HST test |
| PEI | Paging Early Indication is configured in the cell. |
| SDT | For SDT test cases |
| MBS_Broadcast | NR Cell provide MBS Broadcast service |
| MBS_Multicast_Inactive | Used for MBS Multicast reception when UE in RRC_INACTIVE state |

<End of Changed Section>